| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0242-v2 |
| --- | --- |

| Title: | CE3-related: Intra planar mode prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {mohammed.sarwer, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a method to compute bottom-row and right-column prediction samples for intra planar mode is presented. At first, the bottom-right prediction sample is generated by averaging two different predictors. Then bottom-row prediction samples are computed based on weighted average of bottom-left and bottom-right samples, and right-column prediction samples are generated based on weighted average of top-right and bottom-right samples. A boundary filtering technique for planar mode is also presented in this contribution. Compared to VTM-1.0, it is reported that the proposed planar mode achieves -0.26% (Y), -0.10% (U) and 0.00% (V) BD-rates for AI. Compared to BMS-1.0, the proposed method achieves BD-rates of -0.56% (Y), -0.55% (U) and -0.47% (V) for AI.

# Introduction

The planar prediction in VTM-1.0 and BMS-1.0 is defined as average of two linear predictions as shown in Figure 1, where W is the width of the coding block (CB) and H is the height of the CB.

Figure 1.  Planar prediction in VTM-1.0 and BMS-1.0

At first the predictors of right-column and bottom-row are generated by copying top-right and bottom-left reference samples by using Equation 1 and Equation 2, respectively.

PrW, y=R(W, -1)                                                    (1)

Pbx, H=R(-1, H)                                                     (2)

Then the horizontal and vertical predictors of (x,y) position of the block is computed by Equation 3 and Equation 4.

Phx,y=W-1-x×R-1, y+x+1×Pr(W, y)                       (3)

Pvx,y=H-1-y×Rx, -1+y+1×Pb(x, H)                        (4)

The final predicted sample is obtained by Equation 5.

Px,y= (Phx, y≪log2H+ Pvx, y≪log2W)≫shift1                (5)

In Equation 5, shift1 is defined as follows.

shift1=1+ log2W+ log2H                                             (6)

# Proposed methods

This proposal proposes two methods. The first method proposes the generation of right-column and bottom-row predictors, and the second method proposes boundary filtering of prediction samples.

(x, y)

Pbr(W, H)

R(W, -1)

R(-1, H)

R(-1, -1)

Figure 2.  Proposed planar prediction

## Generation of right-column and bottom-row predictors

The first step to generate right-column and bottom-row predictors is to predict the bottom-right (BR) sample of a CB. The bottom-right sample (Pbr(W, H)) of a CB with width = W and height = H are shown in red square in Figure 2. The bottom-right sample is defined as the average value of two bottom-right predictors in case of large CB and only one bottom-right predictor for a small CB as follows.

PbrW, H=Pbr1(W, H),  &   W×H≤32(Pbr1W, H+ Pbr2W, H+1)≫1,  &         otherwise                      (7)

Here, the first BR predictor Pbr1W, H is defined as follows.

Pbr1W, H= (w1×R-1, H+w2×RW, -1+scale2)≫shift2                     (8)

In Equation (8), shift2, w1, w2 and scale2 are defined as follows.

shift2=ceil(log2W+H)                                                       (9)

w1=                   W,  if W≤H1≪shift2-H,  else                                                 (10)

w2=                   1≪shift2-W,  if W≤HH,  else                                        (11)

scale2=1≪shift2≫1                                                        (12)

It should be noted that the multiplication operation of Equation 8 can be implemented by shift and addition operations.

The second bottom-right predictor Pbr2W, H is selected from a pre-defined candidate list. The candidate list of second predictor consists of 9 predictors (8 predictors from reference sample list and Pbr1), which are shown in black dot of Figure 3 and also tabulated in Table 1.

Table 1.  Candidates list Cbr2(i) for Pbr2W, H computation

| Index (i) | Predictor sample Cbr2 (i) |
| --- | --- |
| 0 | Pbr1W, H |
| 1 | RW/2, -1 |
| 2 | RW, -1 |
| 3 | RW+H/2, -1 |
| 4 | RW+H-1, -1 |
| 5 | R-1, H/2 |
| 6 | R-1, H |
| 7 | R-1, H+W/2 |
| 8 | R-1, H+W-1 |

Figure 3.  Candidates for Pbr2W, H

The second predictor Pbr2W, H is defined as follows.

Pbr2W, H=Cbr2(ibest)                                                          (13)

The best index, ibest, is found by minimizing the following cost.

Costi= abs(X-Cbr2i)                                                        (14)

X=clip(0, maxPixelValue, RW, -1+R-1, H-R -1, -1)                         (15)

maxPixelValue=1≪bitDepth-1                                               (16)

In Equation 14, abs() is the absolute operation and the value of i is 0 to 8 as shown in Table 1.

After computing the Pbr1W, H and Pbr2W, H, the final bottom-right predictor PbrW, H is obtained by Equation 7. Finally, right-column and bottom-row reference samples are obtained by following equations.

PrW, y=H-1-y×RW, -1+ y+1×PbrW, H≫(log2H)           (17)

Pbx, H=(W-1-x×R-1, H+ x+1×PbrW, H)≫(log2W)           (18)

Once the right-column and bottom-left predicted samples are available, Equations 3, 4, and 5 are used to generate final prediction signal.

## Boundary filtering of prediction samples

After generating the prediction signal P(x, y), the left and top boundary prediction samples are smoothed by applying boundary filtering technique. The concept of boundary filtering is illustrated in Figure 4, where NHOR is the number of top-rows need to be filtered and NVER is the number of left-columns that boundary filtering is applied. The filter coefficient fl depends on the line number. For NHOR×NVER overlapping area, 3-tap filter is used. For other samples, 2-tap filter is used.

Figure 4.  Illustration of boundary filtering technique

The number of horizontal and vertical filter line is defined as follows

NHOR= 4,  if H>42         ,  else                                                              (19)

NVER= 4,  if W>42         ,  else                                                              (20)

The filter coefficient fl is pre-defined value and depends on the line number, and it is defined as follows.

fl=8, 4, 2, 1, for l=0 to 3                                                         (21)

In Equation 21, l is the line number.

The filtered predicted samples Pf(x, y) of the proposed boundary filtering method are generated by following equations under different conditions.

if x<NVER && y<NHOR

Pfx, y=(fx*R-1, y+ fy*Rx, -1+16-fx- ffy*Px, y+ 8)≫4          (22)

if x≥NVER && y<NHOR

Pfx, y=(fy*Rx, -1+16- fy*Px, y+ 8)≫4                           (23)

if x< NVER &&y≥ NHOR

Pfx, y=(fx*R-1, y+16- fx*Px, y+ 8)≫4                           (24)

if x≥ NVER &&y≥ NHOR, There is no filtering.

Pfx, y=Px, y                                                                                                                                       25

# Simulation results

The proposed methods are implemented on BMS-1.0. All of the simulations are performed based on common test conditions [1]. Table 2 shows the results when only boundary filtering described in Section 2.2 is enabled. Table 3 shows the results when both bottom-row/right-column generations and boundary filtering are enabled.

Table 2.  Results of the proposed boundary filtering

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.21% | 0.37% | 102% | 104% | -0.67% | -1.09% | -0.94% | 100% | 106% |
| Class A2 | -0.23% | -0.18% | -0.09% | 102% | 103% | -0.61% | -0.83% | -0.69% | 100% | 104% |
| Class B | -0.20% | 0.02% | 0.13% | 101% | 104% | -0.54% | -0.62% | -0.66% | 100% | 103% |
| Class C | -0.11% | 0.11% | 0.16% | 101% | 101% | -0.20% | -0.11% | -0.03% | 100% | 101% |
| Class E | -0.25% | -0.06% | 0.25% | 101% | 101% | -0.56% | -0.60% | -0.55% | 100% | 102% |
| Overall | -0.15% | 0.03% | 0.16% | 101% | 102% | -0.50% | -0.62% | -0.55% | 100% | 103% |
| Class D | -0.08% | 0.37% | 0.26% | 101% | 99% | -0.12% | 0.03% | 0.02% | 100% | 101% |
| Class F (optional) | -0.09% | 0.16% | 0.14% | 101% | 101% | -0.18% | -0.06% | -0.06% | 100% | 101% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.04% | 0.26% | 100% | 100% | -0.30% | -0.53% | -0.49% | 100% | 100% |
| Class A2 | -0.12% | -0.08% | -0.01% | 100% | 100% | -0.26% | -0.28% | -0.27% | 100% | 100% |
| Class B | -0.07% | 0.06% | 0.10% | 100% | 99% | -0.33% | -0.32% | -0.30% | 100% | 100% |
| Class C | -0.11% | 0.16% | -0.07% | 100% | 100% | -0.12% | -0.05% | -0.24% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.06% | 0.06% | 0.06% | 100% | 100% | -0.25% | -0.28% | -0.32% | 100% | 100% |
| Class D | -0.03% | 0.21% | -0.09% | 100% | 99% | -0.08% | -0.31% | 0.15% | 100% | 100% |
| Class F (optional) | -0.08% | 0.08% | 0.08% | 100% | 100% | -0.17% | 0.03% | -0.01% | 100% | 100% |

Table 3.  Results of the proposed boundary filtering and right-column/bottom row generations

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.28% | 0.42% | 102% | 104% | -0.73% | -0.94% | -0.76% | 100% | 105% |
| Class A2 | -0.37% | -0.52% | -0.34% | 102% | 103% | -0.68% | -0.84% | -0.69% | 100% | 103% |
| Class B | -0.26% | -0.07% | -0.01% | 102% | 104% | -0.56% | -0.50% | -0.56% | 101% | 103% |
| Class C | -0.16% | -0.01% | -0.04% | 102% | 102% | -0.22% | -0.11% | 0.08% | 101% | 101% |
| Class E | -0.47% | -0.24% | 0.01% | 102% | 102% | -0.71% | -0.57% | -0.55% | 100% | 101% |
| Overall | -0.26% | -0.10% | 0.00% | 102% | 103% | -0.56% | -0.55% | -0.47% | 100% | 103% |
| Class D | -0.13% | 0.09% | 0.25% | 102% | 101% | -0.12% | -0.03% | 0.09% | 101% | 102% |
| Class F (optional) | -0.14% | 0.07% | -0.03% | 102% | 102% | -0.21% | -0.12% | -0.06% | 101% | 101% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.22% | 0.22% | 100% | 101% | -0.33% | -0.51% | -0.47% | 101% | 99% |
| Class A2 | -0.18% | -0.33% | -0.01% | 100% | 101% | -0.32% | -0.46% | -0.27% | 101% | 100% |
| Class B | -0.11% | 0.06% | 0.02% | 100% | 101% | -0.33% | -0.35% | -0.51% | 101% | 101% |
| Class C | -0.09% | -0.01% | 0.02% | 100% | 102% | -0.09% | 0.01% | -0.27% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.09% | -0.01% | 0.05% | 100% | 101% | -0.26% | -0.31% | -0.39% | 101% | 100% |
| Class D | -0.06% | 0.25% | -0.11% | 101% | 103% | -0.07% | -0.21% | -0.14% | 101% | 101% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The proposed boundary filter is also applied in DC mode. Table 4 shows the simulation results of proposed boundary filter for both planar and DC mode. Table 5 shows the results when the proposed boundary filter is combined with JVET-K240 [4] boundary filters. Both in Table 4 and Table 5, right-column/bottom row generations part of the proposed method is disabled.

The right-column and bottom-row generation part of the proposed method can be also combined with UWP [2] and PDPC [3]. Table 6 shows the simulation results of the proposed right-column/bottom row generation method combined with UWP [2]. In Table 6, the bottom-right position adjustment part described in JVET-K0055 [2] is replaced by proposed right-column/bottom row generation method. Table 7 shows the results of PDPC-planar whereas Table 8 shows the results when proposed right-column and bottom-row generation method is combined with PDPC planar. Both in Table 7 & 8, PDPC is applied only in planar mode.

Table 4.  Results of the proposed boundary filtering for both planar and DC mode (runtime is not accurate)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.28% | 0.58% | 106% | 107% | -0.79% | -1.45% | -1.21% | 102% | 109% |
| Class A2 | -0.26% | 0.00% | 0.20% | 100% | 103% | -0.82% | -1.09% | -0.79% | 102% | 110% |
| Class B | -0.39% | 0.16% | 0.24% | 102% | 104% | -0.80% | -0.87% | -0.86% | 103% | 109% |
| Class C | -0.14% | 0.64% | 0.74% | 102% | 101% | -0.22% | 0.00% | 0.00% | 102% | 106% |
| Class E | -0.39% | -0.28% | -0.04% | 102% | 100% | -0.71% | -0.82% | -0.85% | 102% | 106% |
| Overall | -0.24% | 0.19% | 0.35% | 102% | 103% | -0.66% | -0.80% | -0.71% | 102% | 108% |
| Class D | -0.10% | 0.73% | 1.02% | 101% | 102% | -0.11% | 0.13% | 0.49% | 103% | 108% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.  Results of the proposed boundary filtering for planar mode combined with JVET-K0240 [4] (runtime is not accurate)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  | -0.53% |  | -0.39% |  | -0.07% |  | 109% |  | 108% |  | -1.20% |  | -1.51% |  | -1.25% |  |  |  |  |  |
| Class A2 |  | -0.97% |  | -0.93% |  | -0.77% |  | 104% |  | 104% |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.87% |  | -0.50% |  | -0.38% |  | 105% |  | 102% |  | -1.27% |  | -0.94% |  | -0.93% |  |  |  |  |  |
| Class C |  | -0.79% |  | -0.30% |  | -0.39% |  | 104% |  | 102% |  | -0.91% |  | -0.50% |  | -0.41% |  |  |  |  |  |
| Class E |  | -1.18% |  | -0.99% |  | -0.95% |  | 106% |  | 101% |  | -1.61% |  | -1.12% |  | -1.17% |  |  |  |  |  |
| Overall |  | -0.86% |  | -0.59% |  | -0.49% |  | 105% |  | 103% |  |  |  |  |  |  |  |  |  |  |  |
| Class D |  | -0.72% |  | -0.20% |  | -0.10% |  | 105% |  | 103% |  | -0.76% |  | -0.09% |  | -0.19% |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.  Results of the proposed right-column and bottom-row generation combined with UWP [2] (runtime is not accurate)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.32% | 0.27% | 104% | 117% | -0.86% | -1.11% | -1.04% | 102% | 114% |
| Class A2 | -0.35% | -0.61% | -0.45% | 100% | 114% | -0.68% | -0.87% | -0.76% | 102% | 113% |
| Class B | -0.27% | -0.21% | -0.14% | 101% | 116% | -0.60% | -0.56% | -0.58% | 103% | 115% |
| Class C | -0.21% | -0.34% | -0.33% | 101% | 104% | -0.29% | -0.11% | -0.21% | 103% | 111% |
| Class E | -0.46% | -0.59% | -0.34% | 102% | 106% | -0.72% | -0.79% | -0.74% | 103% | 111% |
| Overall | -0.28% | -0.28% | -0.20% | 101% | 111% | -0.61% | -0.64% | -0.63% | 103% | 113% |
| Class D | -0.18% | -0.18% | -0.20% | 103% | 105% | -0.20% | -0.10% | -0.09% | 104% | 113% |
| Class F (optional) | -0.10% | -0.15% | -0.09% | 102% | 99% | -0.20% | -0.10% | -0.49% | 104% | 111% |

Table 7.  Results of PDPC-planar [3] (runtime is not accurate)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.11% | 0.29% | 103% | 109% | -0.49% | -1.00% | -0.80% | 99% | 109% |
| Class A2 | -0.23% | -0.37% | -0.28% | 98% | 104% | -0.62% | -0.91% | -0.66% | 100% | 108% |
| Class B | -0.23% | -0.08% | 0.02% | 99% | 105% | -0.57% | -0.70% | -0.65% | 100% | 107% |
| Class C | -0.19% | -0.09% | -0.19% | 99% | 98% | -0.53% | -0.58% | -0.50% | 100% | 101% |
| Class E | -0.33% | -0.06% | 0.10% | 101% | 101% | -0.77% | -0.79% | -0.68% | 99% | 100% |
| Overall | -0.21% | -0.09% | -0.02% | 100% | 103% | -0.59% | -0.77% | -0.65% | 100% | 105% |
| Class D | -0.19% | -0.06% | -0.01% | 100% | 101% | -0.21% | -0.10% | 0.02% | 101% | 105% |
| Class F (optional) | -0.13% | -0.12% | -0.07% | 102% | 99% | -0.38% | -0.49% | -0.42% | 99% | 99% |

Table 8.  Results of the proposed right-column and bottom-row generation combined with PDPC-planar [3] (runtime is not accurate)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | 0.20% | 0.27% | 103% | 104% | -0.57% | -0.79% | -0.57% | 102% | 111% |
| Class A2 | -0.36% | -0.69% | -0.51% | 99% | 102% | -0.68% | -0.95% | -0.67% | 103% | 110% |
| Class B | -0.29% | -0.15% | -0.08% | 100% | 100% | -0.60% | -0.59% | -0.57% | 102% | 114% |
| Class C | -0.23% | -0.33% | -0.28% | 100% | 100% | -0.56% | -0.55% | -0.36% | 102% | 106% |
| Class E | -0.51% | -0.23% | 0.06% | 101% | 101% | -0.87% | -0.93% | -0.68% | 101% | 103% |
| Overall | -0.30% | -0.23% | -0.12% | 100% | 101% | -0.64% | -0.73% | -0.56% | 102% | 109% |
| Class D | -0.22% | -0.13% | -0.08% | 100% | 98% | -0.23% | 0.01% | -0.06% | 104% | 109% |
| Class F (optional) | -0.17% | -0.08% | -0.04% | 102% | 97% | -0.44% | -0.40% | -0.49% | 100% | 100% |

# Conclusions

The proposed method (when both boundary filtering and bottom-row/right-col generation are enabled) achieves -0.26%/-0.10%/0.00% BD-rate savings for Y/U/V, respectively for AI compared to VTM-1.0. As compared to BMS-1.0, the proposed method achieves -0.56%/-0.55%/-0.47% BD-rate savings for Y/U/V, respectively. The proposed boundary filtering method can also be extended to DC mode. Simulation results also shows that the right-column and bottom-row generation part of the method can also combined with PDPC and UWP.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.

[2] K. Panusopone, S. Hong, Y. Yu, L. Wang, A. Segall, “CE3: Unequal Weight Planar Prediction (Test 1.5.1)”, JVET-K0055.

[3] G. Van der Auwera, V. Seregin, A. Said, A. K. Ramasubramonian, M. Karczewicz, “CE3: Simplified PDPC (Test 2.4.1)”, JVET-K0063.

[4] Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE3.2.9.1: Intra boundary filters”, JVET-K0240