| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0243-v2 |
| --- | --- |

| Title: | CE3-related: Intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {mohammed.sarwer, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a luma intra mode coding method for VTM and BMS. The most probable mode (MPM) list is re-ordered based on predefined accumulated weight of each MPM mode. The MPM mode which has highest accumulated weight is placed at the index 0 of the MPM list. The generation of non-MPM list based on predefined priority is also presented. The non-MPM list is generated from the angular MPM modes and also predefined list of modes. It is reported that under All Intra configuration the proposed method can achieve -0.58%/	-0.34%/	-0.32% Y/U/V BD-rate for VTM-1.0 and -0.23%/-0.05%/-0.08% Y/U/V BD-rate for BMS-1.0.

# Introduction

Intra prediction is a significant coding technique to reduce the spatial redundancy between the neighboring samples of the image. In VVC test model, VTM-1.0 [1], there are 35 intra prediction modes whereas in BMS-1.0 [2] test model there are 67 intra prediction modes. These large numbers of intra prediction modes demand an efficient intra mode coding method.

Figure 1.  67 intra prediction modes in BMS-1.0; In VTM-1.0 only black modes exist.

Figure 1 shows the intra prediction directions for BMS-1.0. For VTM-1.0, only modes indicated by black lines exist. In other words, red mode directions are not valid for VTM-1.0 test model.

# Proposed methods

Intra prediction modes are classified into two categories (a) MPM and (b) non-MPM. The number of MPMs for both VTM-1.0 and BMS-1.0 is 6. The number of non-MPMs is 61 in case of BMS-1.0 and 29 in case of VTM-1.0.

## Weight-based MPM list reorder:

In the proposed method, the generation of MPM list is same as BMS-1.0, where, five neighbouring intra prediction modes are used to form the MPM list [3]. Those locations of the 5 neighbouring modes, i.e., left (L), above (A), below-left (BL), above-right (AR), and above-left (AL) are shown in Figure 2. If MPM list is not full, derived modes and default intra modes are inserted in the MPM list. After generation of MPM list, the entire MPM list is reordered based on predefined weighting factors. In addition to BMS-1.0, the proposed method also applied to VTM-1.0 mode coding.

Figure 2.  Neighbouring blocks for MPM derivation

At first, the weight of the each intra mode is initialized to zero. After checking each neighboring mode in Figure 2, and associated derived and default mode, the weight of the corresponding mode is accumulated. And finally, MPM list is reordered based on the weight of the each MPM. The MPM which has the highest weight is placed at the index 0 of the MPM list. Assume, wm is the weight of a mode m. The mode number, m, is 0 to 66 for BMS-1.0. For VTM-1.0, only even numbers of m are valid. The weighting is defined as follows.

Initialize weights of all modes (wm where m=0 to 66) to zero.

- Left-Mode (LM):

  - ifLM≤30, wLM+=4

  - Otherwise, wLM+=3

- Above-Mode (AM)

  - ifAM==PLANAR OR AM≥40, wAM+=4

  - Otherwise, wAM+=3

- Planar Mode (PM)

  - ifwPM >0 , wPM+=1;

  - Otherwise, wPM+=3

- DC Mode (DM)

  - ifwDM>0, wDM+=1;

  - Otherwise, wDM+=2

- Bottom-Left Mode (BLM):

  - ifBLM<HOR ,  wBLM+=3;

  - Otherwise, wBLM+=2

- Above-Right Mode (ARM)

  - ifARM==PLANAR OR ARM≥60, wARM+=3;

  - Otherwise, wARM+=2

- Above-Left Mode  (ALM)

  - ifALM≥30 AND ALM≤38OR ALM=PLANAR, wALM+=3;

  - Otherwise, wALM +=2

- Derived Mode (DEM):

  - wDEM+=1

After generation of the MPM list, the list is reordered based on the accumulated weight of each MPM. The mode which has the highest accumulated weight is placed at the MPM list with index 0.

Offset = 2; Non-MPM-Index = 0; While (offset <= 8) For (mpmIndex = 0; mpmIndex < 6; mpmIndex++) Mode = MPM mode with index value mpmIndex If (Mode > 1) Adjacent Mode = Mode - Offset If Adjacent Mode is not available in non-mpm list Insert the Adjacent Mode in non-MPM list at index Non-MPM-Index Non-MPM-Index++; Endif Endif End For Offset = Offset * 2; End While For (k = 0; k < Total Number of Modes && Non-MPM-Index < number of non-MPM mode; k++) Mode = mode in priority list with index value k If Mode is not available in non-mpm list Insert the mode in non-MPM list at index Non-MPM-Index Non-MPM-Index++; Endif End For

Figure 3. Pseudo-code of non-MPM list generation

## Generation of non-MPM list

Figure 3 shows the pseudo-code of non-MPM list generation. After generation of MPM list, the rest of the coding modes are inserted in non-MPM list based on pre-defined priority.

–	Step 1: For each angular mode of the MPM list, put following 6 modes in the non-MPM list in following order at first.

MPM -2, MPM +2, MPM – 4, MPM + 4, MPM – 8, MPM+8

–	Step 2: Insert modes from the pre-defined priority based mode list ( ModePriorityList ) based on priority order until the non-MPM list is full. The priority based mode list is defined in equation (1) and (2) for VTM-1.0 and BMS-1.0, respectively.

ModePriorityList(VTM)(35) = {0,1,50,18,34,66,2,42,58,10,26,46,54,14,22,38,62,6,30,48,52,16,20,
44,56,12,24,40,60,8,28,36,64,4,32}                                 (1)

ModePriorityList(BMS)(67) = {0,1,50,18,34,66,2,42,58,10,26,46,54,14,22,38,62,6,30,
48,52,16,20,44,56,12,24,40,60,8,28,36,64,4,32,49,51,17,
19,47,53,15,21,45,55,13,23,43,57,11,25,35,65,3,33,37,63,
5,31,39,61,7,29,41,59,9,27}                                       (2)

## Signaling MPM list and non-MPM list:

### Signaling BMS-1.0 modes

Table 1 shows the binarization of MPMs and non-MPMs of BMS-1.0. The bins shown as underline are context coded. There are total 6 contexts used to code BMS-1.0 intra modes. The signaling and context of MPMs are same as BMS-1.0. Non-MPM lists are classified as three different categories. Modes with index 0 to 7 is category 1, mode with index 8 to 23 are classified as second category, and rest of the non-MPMs belong to third category. 3 and 4 bit fixed length code is used to signal elements in first and second category, respectively. The third category is coded as truncated binary.

Table 1.  Binarization in case of BMS-1.0

| List | Index | Binarization |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Index | 1st bin | 2nd bin | 3rd bin | 4th bin | 5th bin | 6th bin |  |
| MPM | 0 | 1 | 0 |  |  |  |  |  |
|  | 1 | 1 | 1 | 0 |  |  |  |  |
|  | 2 | 1 | 1 | 1 | 0 |  |  |  |
|  | 3 | 1 | 1 | 1 | 1 | 0 |  |  |
|  | 4 | 1 | 1 | 1 | 1 | 1 | 0 |  |
|  | 5 | 1 | 1 | 1 | 1 | 1 | 1 |  |
| Non-MPM | 0 to 7 | 0 | 1 | 3 bit FLC |  |  |  |  |
|  | 8 to 23 | 0 | 0 | 1 | 4 bit FLC |  |  |  |
|  | 24 to 60 | 0 | 0 | 0 | Truncated Binary |  |  |  |

Table 2.  Binarization in case of VTM-1.0

| List | Index | Binarization |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Index | 1st bin | 2nd bin | 3rd bin | 4th bin | 5th bin | 6th bin |  |
| MPM | 0 | 1 | 0 |  |  |  |  |  |
|  | 1 | 1 | 1 | 0 |  |  |  |  |
|  | 2 | 1 | 1 | 1 | 0 |  |  |  |
|  | 3 | 1 | 1 | 1 | 1 | 0 |  |  |
|  | 4 | 1 | 1 | 1 | 1 | 1 | 0 |  |
|  | 5 | 1 | 1 | 1 | 1 | 1 | 1 |  |
| Non-MPM | 0 to 7 | 0 | 1 | 3 bit FLC |  |  |  |  |
|  | 8 to 28 | 0 | 0 | Truncated Binary |  |  |  |  |

### Signaling VTM-1.0 modes

Table 2 shows the binarization in case of VTM-1.0. There are total 5 contexts used to code the underlines bins. In VTM-1.0, non-MPM list is classified as two categories. Modes with non-MPM-index 0 to 7 is signaled as 3 bit fixed length code, and rest of the non-MPMs are signaled as truncated binary.

# Simplification

A simplified version of the proposed method is also proposed. In the simplified version two constraints are applied.

## CTU-row constraint

In order to avoid the line buffer requirement during intra mode coding, the access to above (A), above-left (AL) and above-right (AR) coding unit (CU) are allowed only if those CUs belong to the same coding tree unit (CTU) row. If the neighboring CU is outside of current CTU-row, those neighbor modes are skipped for MPM computation.

## Mode independent contexts

In BMS-1.0 (also in proposed method, as shown in Table 1 & Table 2), three contexts are used to code 2nd, 3rd, and 4th bins of MPM mode. The selection of the context depends on the MPM of index 0 (for 2nd bin), 1 (for 3rd bin), & 2 (for 4th bin). For instance, if MPM [0] is planar or DC mode, context 1 is used for 2nd bin. If MPM [0] is one of the horizontal modes, context 2 is used to code 2nd bin. Otherwise context 3 is used to code 2nd bin.

In this constrain, in order to avoid reconstruction of intra-modes during CABAC parsing process, instead of mode dependent context, bin dependent context is used. Context 1 is used to code 2nd bin regardless of the MPMs. Similarly, Context 2 is used to code 3rd bin, and context 3 is used to code 4th bin.

The proposed simplification can also apply to other intra mode coding methods.

# Simulation results

The proposed methods are implemented in BMS-1.0. Table 3 shows the simulation results of proposed method (without simplification described in Section 3). Table 4 shows the results of proposed method when both CTU-row constraint and mode independent contexts described in Section 3 are enabled. All of the tests are performed under common test conditions described in [4].

Table 3.  Simulation results of proposed method (without simplification described in Section 3)

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.57% | -0.17% | -0.30% | 116% | 106% | -0.25% | -0.01% | -0.12% | 102% | 102% |
| Class A2 | -0.53% | -0.29% | -0.42% | 116% | 105% | -0.17% | -0.07% | -0.09% | 102% | 102% |
| Class B | -0.57% | -0.31% | -0.14% | 116% | 107% | -0.17% | 0.02% | 0.05% | 102% | 102% |
| Class C | -0.59% | -0.41% | -0.48% | 115% | 106% | -0.22% | -0.05% | -0.08% | 101% | 102% |
| Class E | -0.61% | -0.48% | -0.30% | 116% | 104% | -0.37% | -0.20% | -0.22% | 102% | 100% |
| Overall | -0.58% | -0.34% | -0.32% | 116% | 106% | -0.23% | -0.05% | -0.08% | 102% | 102% |
| Class D | -0.25% | -0.05% | -0.17% | 115% | 105% | -0.16% | 0.03% | 0.08% | 102% | 102% |
| Class F (optional) | -0.83% | -0.91% | -0.81% | 115% | 106% | 0.08% | 0.18% | 0.02% | 102% | 101% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.13% | -0.05% | 101% | 102% | -0.13% | 0.07% | -0.11% | 101% | 99% |
| Class A2 | -0.23% | -0.02% | -0.06% | 101% | 101% | -0.06% | 0.03% | 0.07% | 101% | 100% |
| Class B | -0.24% | 0.06% | 0.04% | 102% | 101% | -0.10% | 0.14% | -0.19% | 101% | 99% |
| Class C | -0.24% | -0.14% | -0.13% | 102% | 101% | -0.09% | 0.13% | -0.22% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.23% | -0.05% | -0.05% | 101% | 101% | -0.10% | 0.10% | -0.13% | 101% | 100% |
| Class D | -0.08% | 0.04% | -0.26% | 102% | 101% | -0.06% | 0.09% | 0.09% | 102% | 100% |
| Class F (optional) | -0.51% | -0.68% | -0.38% | 102% | 101% | 0.02% | 0.14% | 0.17% | 101% | 101% |

Table 4.  Simulation results of proposed method with CTU-row constraint & mode independent contexts

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.06% | -0.14% | 117% | 106% | -0.05% | 0.07% | 0.01% | 101% | 103% |
| Class A2 | -0.38% | -0.19% | -0.30% | 116% | 105% | -0.04% | 0.06% | 0.05% | 102% | 103% |
| Class B | -0.42% | -0.20% | -0.04% | 116% | 104% | 0.00% | 0.13% | 0.09% | 102% | 103% |
| Class C | -0.44% | -0.39% | -0.35% | 116% | 105% | -0.11% | 0.01% | 0.00% | 101% | 102% |
| Class E | -0.46% | -0.22% | -0.10% | 117% | 103% | -0.17% | -0.05% | 0.01% | 102% | 102% |
| Overall | -0.41% | -0.22% | -0.18% | 116% | 105% | -0.07% | 0.05% | 0.04% | 102% | 102% |
| Class D | -0.22% | 0.00% | -0.11% | 115% | 104% | -0.14% | 0.08% | -0.01% | 102% | 103% |
| Class F (optional) | -0.46% | -0.49% | -0.39% | 116% | 105% | 0.34% | 0.50% | 0.25% | 102% | 101% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.03% | 0.18% | 102% | 101% | 0.00% | 0.02% | 0.02% | 100% | 101% |
| Class A2 | -0.17% | 0.16% | 0.05% | 101% | 101% | 0.00% | 0.14% | 0.14% | 101% | 101% |
| Class B | -0.18% | 0.05% | 0.02% | 102% | 100% | -0.02% | 0.01% | -0.02% | 101% | 102% |
| Class C | -0.15% | -0.10% | -0.18% | 102% | 101% | -0.03% | 0.19% | -0.17% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.16% | 0.03% | 0.00% | 102% | 101% | -0.02% | 0.09% | -0.02% | 101% | 101% |
| Class D | -0.08% | 0.10% | -0.17% | 102% | 101% | 0.00% | -0.03% | -0.07% | 101% | 101% |
| Class F (optional) | -0.32% | -0.26% | -0.22% | 103% | 101% | 0.26% | 0.45% | 0.42% | 101% | 101% |

# Conclusions

As compared to VTM-1.0, in All Intra configuration, the proposed method achieve BD-rate of -0.58%/-0.34%/-0.32% for Y/U/V, respectively. If the CTU-row constraint and mode independent contexts are applied as simplification of the proposed method, the BD-rate of All Intra condition is -0.41%/-0.22%/-0.18% for Y/U/V, respectively. As compared to BMS-1.0, the proposed method achieve BD-rate of -0.23%/-0.05%/-0.08% for Y/U/V, respectively. The simplified proposed method can achieve -0.07%/0.05%/0.04% BD-rate of Y/U/V, respectively on top of BMS-1.0, AI configuration.

# Patent rights declaration(s)

MediaTek Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] VTM-1.0. https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_VTM/tags/VTM-1.0/

[2] BMS-1.0 https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/

[3] J. Chen, E. Alshina, G. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7 (JEM 7),” Document of Joint Video Experts Team, JVET-G1001.

[4] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.