| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0244-v1 |
| --- | --- |

| Title: | CE4.1.6: MVP pair list construction for affine inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of Core Experiment 4.1.6 (CE4.1.6) test, which applies a proposed affine inter mode and the JEM-7.0/BMS-1.0 affine merge mode. In affine inter mode, motion vectors (MVs) of two control points are predicted and signaled, and a motion vector predictor (MVP) pair is selected from a MVP pair list before signaling the MVP pair index and MV differences (MVDs). The proposed MVP pair list construction can include spatial inherited MVP pair (SMVP-pair) candidates and corner derived MVP pair (CMVP-pair) candidates into the list, and the MVP pair list size is fixed to two. SMVP-pair candidates, which are inherited from neighboring coding units (CUs) coded with affine, are newly introduced and not used in JEM-7.0/BMS-1.0 affine inter mode. CMVP-pair candidates, which are used in JEM-7.0/BMS-1.0 affine inter mode, are simplified in CE4.1.6. The proposed CMVP-pair candidates are derived from three corner MVs (CMVs): top-left (TL) CMV, top-right (TR) CMV, and bottom-left (BL) CMV, where the TL/TR/BL CMV is simply the first available neighboring CU MV at TL/TR/BL corner of the current CU. The TL and TR CMVs are further grouped to generate one CMVP-pair candidate according to affine model, and the TL and BL CMVs are further grouped to generate another CMVP-pair candidate. After attempting to add SMVP-pair and CMVP-pair candidates, if the number of candidates is smaller than two, MVPs for non-affine inter mode are inserted to the MVP pair list. It is reported that the proposed affine inter mode and the JEM-7.0/BMS-1.0 affine merge mode together can achieve -3.21% and -2.10% luma BD-rates with 32% and 59% encoding time increases and 7% and 4% decoding time increases for VTM-1.0-RA and VTM-1.0-LB, respectively. When the test is on BMS-1.0 and the anchor is BMS-1.0 with affine inter mode and affine merge mode disabled, the luma BD-rates are 2.10% and -2.04% with 11% and 28% encoding time increases and 7% and 8% decoding time increases for BMS-1.0-RA and BMS-1.0-LB, respectively.

# Introduction

In JEM-7.0/BMS-1.0, four-parameter affine inter mode with two control points is used to predict the CU with zoom and rotation motion. If affine inter mode is applied, MVs of the two control points are predicted and signaled, and a MVP pair is selected from a MVP pair list before signaling the MVP pair index and MVDs. The syntax elements of motion information including inter_dir, ref_idx, mvp_index, and two MVDs (mvd_x and mvd_y). Two modifications related to MVP pair list construction were presented in JVET-J0018 [1] and planned in CE4.1.6 as described in JVET-J1024 [2]. First, in addition to CMVP-pair candidates, it is proposed to add SMVP-pair candidates into the MVP pair list. Second, a simplified scheme for the CMVP-pair candidate generation is also proposed.

# Proposed method

An MVP pair list containing two MVP pairs is generated. The signaled mvp_index is used to select one of the two pairs. There are two kinds of MVP pair candidates. One kind is SMVP-pair candidate, and the other kind is CMVP-pair candidate. If the neighboring CUs are coded with affine, SMVP-pair candidates can be generated. The affine motion model of the neighboring affine coded block is used to generate the MVP pair. The SMVP-pair candidate, (V0x, V0y) and (V1x, V1y), is derived by the following equations.

# V0x = VB0x + (VB2x - VB0x) * (posCurCU_Y - posRefCU_Y) / RefCU_height + (VB1x - VB0x) * (posCurCU_X - posRefCU_X) / RefCU_width

# (1)

# V0y = VB0y + (VB2y - VB0y) * (posCurCU_Y - posRefCU_Y) / RefCU_height + (VB1y - VB0y) * (posCurCU_X - posRefCU_X) / RefCU_width

# (2)

# V1x = VB0x + (VB1x - VB0x) * CU_width / RefCU_width

# (3)

# V1y = VB0y + (VB1y - VB0y) * CU_width / RefCU_width

# (4)

In the above equations, VB0, VB1, and VB2 can be replaced with the TL MV, TR MV, and BL MV of any neighboring CU, respectively; (posCurCU_X, posCurCU_Y) is the position of the TL sample of the current CU relative to the TL sample of the picture; (posRefCU_X, posRefCU_Y) is the position of the TL sample of the neighboring CU relative to the TL sample of the picture. For SMVP-pair candidates, the searching order is A1B1B0A0B2, as shown in Figure 1. If the neighboring CU is coded with affine but the reference picture is different from the target reference picture, the control points’ MVs are scaled to current target picture to derive current CU’s affine model.

Figure 1.  Positions used for deriving SMVP-pair candidates

If the number of MVP pair candidates is smaller than two after searching SMVP-pair candidates, CMVP-pair candidates will be searched. The neighboring MVs at A0, A1, A2, B0, B1, C0, and C1, as shown in Figure 2, are used to derive CMVP-pair candidates. The first available MV in set A (A0, A1, and A2) and the first available MV in set B (B0 and B1) are used to calculate the first CMVP-pair candidate. The first available MV in set A and the first available MV in set C (C0 and C1) are used to calculate the second CMVP-pair candidate.

Figure 2.  Positions used for deriving CMVP-pair candidates

# Experiment results

The proposed modifications have been implemented on top of BMS-1.0. The common test conditions [3] are used for evaluation. Note that the simulations in VTM-1.0 test condition are performed by using VTM encoder configuration settings without MACRO changes in the software. Table 1 shows the simulation results of this test.

Table 1.  Results of the proposed affine inter mode and the JEM-7.0/BMS-1.0 affine merge mode.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 w/o Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.10% | -1.72% | -1.83% | 130% | 101% | -1.20% | -0.88% | -0.92% | 107% | 108% |
| Class A2 | -6.37% | -5.14% | -4.82% | 129% | 115% | -3.86% | -2.87% | -2.89% | 110% | 112% |
| Class B | -3.50% | -2.43% | -2.51% | 135% | 106% | -2.40% | -1.65% | -1.73% | 112% | 102% |
| Class C | -1.30% | -0.74% | -0.88% | 133% | 106% | -1.07% | -0.74% | -0.83% | 112% | 107% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.21% | -2.38% | -2.40% | 132% | 107% | -2.10% | -1.50% | -1.56% | 111% | 107% |
| Class D | -1.87% | -1.24% | -1.27% | 137% | 108% | -1.62% | -1.28% | -1.17% | 113% | 110% |
| Class F (optional) | -1.64% | -1.51% | -1.60% | 133% | 104% | -1.40% | -1.40% | -1.40% | 110% | 105% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 w/o Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.59% | -1.86% | -1.89% | 155% | 104% | -2.63% | -1.74% | -2.01% | 126% | 111% |
| Class C | -1.43% | -0.68% | -0.56% | 139% | 102% | -1.43% | -1.17% | -1.08% | 121% | 112% |
| Class E | -2.17% | -1.72% | -2.03% | 198% | 105% | -1.89% | -1.81% | -2.17% | 141% | 101% |
| Overall | -2.10% | -1.43% | -1.48% | 159% | 104% | -2.04% | -1.57% | -1.74% | 128% | 108% |
| Class D | -2.96% | -1.73% | -2.35% | 135% | 96% | -3.02% | -2.81% | -3.13% | 120% | 110% |
| Class F (optional) | -1.78% | -1.77% | -1.40% | 152% | 103% | -1.77% | -2.05% | -2.04% | 116% | 107% |

# Conclusions

This contribution reports the simulation results of applying the proposed affine inter mode and JEM-7.0/BMS-1.0 affine merge mode. Compared to VTM-1.0 (no affine in anchor), -3.21% and -2.10% luma BD-rates are achieved for VTM-RA and VTM-LB, respectively. Compared to BMS-1.0 disabling affine inter mode and affine merge mode in anchor, -2.10% and -2.04% luma BD-rates are achieved for BMS-RA and BMS-LB, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team, JVET-J0018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” Document of Joint Video Experts Team, JVET-J1024.

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.