| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0247-v1 |
| --- | --- |

| Title: | CE4.3.4: Removal of AMVR flag constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of Core Experiment 4.3.4 (CE4.3.4) on motion vector coding using the proposed adaptive motion vector resolution (AMVR). In JEM-7.0 AMVR, a constraint for AMVR flag is applied, and the AMVR flag is coded only when there is at least one non-zero motion vector difference (MVD). In the proposed AMVR, the AMVR flag constraint is removed, and the AMVR flag is always signaled in AMVP mode. Results of luma BD-rate, encoding time, and decoding time are as follows.

Anchor: VTM-1.0 (no AMVR); Test: VTM-1.0 + proposed AMVR
RA: {Y-BD-rate = -1.79%, EncT=121%, DecT=100%}
LB: {Y-BD-rate = -0.72%, EncT=142%, DecT=99%}

Anchor: BMS-1.0 with JEM-7.0 AMVR off; Test: BMS-1.0 with JEM-7.0 AMVR off + proposed AMVR
RA: {Y-BD-rate = -1.12%, EncT=105%, DecT=98%}
LB: {Y-BD-rate = -0.70%, EncT=115%, DecT=99%}

Anchor: VTM-1.0 + JEM-7.0 AMVR; Test: VTM-1.0 + proposed AMVR
RA: {Y-BD-rate = -0.04%, EncT=105%, DecT=100%}
LB: {Y-BD-rate = -0.04%, EncT=110%, DecT=100%}

Anchor: BMS-1.0 (JEM-7.0 AMVR on); Test: BMS-1.0 with JEM-7.0 AMVR off + proposed AMVR
RA: {Y-BD-rate = -0.02%, EncT=102%, DecT=100%}
LB: {Y-BD-rate = -0.13%, EncT=106%, DecT=100%}

Algorithm description

Adaptive motion vector difference resolutions (AMVR) is adopted in JEM-7.0, and it supports three kinds of motion vector resolutions, including quarter-luma sample, integer-luma sample, and four-luma samples, to reduce the side information of MVDs. In advanced motion vector prediction (AMVP) mode, one AMVR flag is used to indicate the resolution of MVD. One constraint is applied: the AMVR flag is coded only when there is at least one non-zero MVD. When AMVR flag is not coded, the resolution of MVD and motion vector predictor (MVP) is inferred as quarter-luma sample. In this proposal, the constraint is removed. The AMVR flag is always coded in AMVP mode to indicate the selection of MVD and MVP resolution, even if the coded MVD is zero. In JEM-7.0 AMVR, the AMVR flag is coded after all predicting information (e.g. merge flag, affine flag, MVP index, MVD). In the proposed AMVR, the AMVR flag is coded before MVD and after affine flag.

Experimental results

The proposed method has been integrated into the released BMS-1.0 [1]. All the tests were conducted according to CE4 test conditions [2, 3]. The detail results are shown in Table 1 and Table 2 with four CE4 test conditions. In Table 1, “tool on case 1” is AMVR tests with VTM-1.0 anchor result, and “tool off case 2” is AMVR tests with BMS-1.0 disabling JEM-7.0 AMVR anchor results. In Table 2, “tool off case 1” is AMVR tests with BMS-1.0 anchor result, and “tool on case 2” is AMVR tests with VTM-1.0 adding JEM-7.0 AMVR anchor results.

Table 1.  Left side: anchor = VTM-1.0 (no AMVR), test = VTM-1.0 + proposed AMVR; right side: anchor: BMS-1.0 with JEM-7.0 AMVR off, test: BMS-1.0 with JEM-7.0 AMVR off + proposed AMVR.

Table 2.  Left side: anchor = VTM-1.0 + JEM-7.0 AMVR, test = VTM-1.0 + proposed AMVR; right side: anchor: BMS-1.0 (JEM-7.0 AMVR on), test: BMS-1.0 with JEM-7.0 AMVR off + proposed AMVR.

Conclusions

In this proposal, the AMVR flag constraint is removed, and the AMVR flag is always signaled in AMVP mode. Removal of the constraint does not suffer any coding efficiency loss. Some minor coding efficiency improvement and some encoding time increase are observed. The impact on decoding time is negligible.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- [1] BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- [2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” Document of Joint Video Experts Team, JVET-J1024.

- [3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.