| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0249-v2 |
| --- | --- |

| Title: | CE5.3.3 & CE5.3.4: CABAC range sub-interval derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen Kung-Nien Yang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, kungnien.yang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In JEM CABAC engine, the rangeOne is derived by using a 512×64×9-bit look-up table, where the size of the rangeOne look-up table is 144 times of the size of the rangeLPS look-up table used in HEVC. In CE5.3.3, an alternative rangeOne derivation method is proposed. A 32×8×8-bit table is used for deriving the rangeOne value. The table size is the same as the size of the table used by HEVC. In CE5.3.4, the table look-up values are calculated by using a 6-bit by 5-bit multiplier. The derived rangeOne values of CE5.3.3 and CE5.3.4 are identical. Users can choose to use look-up table or multiplier in their implementations. Simulation results reportedly show that, compared with using the 512×64×9-bit look-up table, the average luma BD-rate increases are less than 0.04% in both BMS-1.0 and VTM-1.0 configurations. The supplementary results of using a 16×8×8-bit look-up table or a 5-bit by 5-bit multiplier are also reported as follows; compared with using the 512×64×9-bits look up table, the average luma BD-rate increases are less than 0.09% in both BMS-1.0 and VTM-1.0 configurations.

# Introduction

In JEM CABAC engine, the range is divided into rangeOne and rangeZero for coding each bin. The range sub-interval derivation is realized by using a 512×64×9-bit look-up table (LUT). The table covers probability range of (0, 1). Compared with the rangeLPS table used in HEVC which size is 64×4×8-bit, the JEM CABAC LUT size is 144 times. To reduce the table size, two methods are proposed in this contribution. In CE 5.3.4, a 32×8×8-bit table is used for deriving the rangeOne value. In CE5.3.4, the table look-up values are calculated by using a 6-bit by 5-bit multiplier.

# Proposed methods

## CE5.3.3: RangeOne derivation by using a 32×8×8-bit LUT

The range sub-interval derivation is realized using a 32×8×8-bit LUT. The range interval is divided into the rangeOne and the rangeZero. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 63 – (P>>9) : (P>>9)

rangeLPS  = LUT[ probIdx ][ ( range >> 5 ) & 7 ]

rangeOne  = ( P >= 16384 ) ? range – rangeLPS : rangeLPS

rangeZero = ( P >= 16384 ) ? rangeLPS : range – rangeLPS

In this document, P is the probability of a context, and range is the current range in binary arithmetic coder.

A supplementary test that uses a 16×8×8-bit LUT is also provided. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 31 – (P>>10) : (P>>10)

rangeLPS  = LUT[ probIdx ][ ( range >> 5 ) & 7 ]

rangeOne  = ( P >= 16384 ) ? range – rangeLPS : rangeLPS

rangeZero = ( P >= 16384 ) ? rangeLPS : range – rangeLPS

## CE5.3.4: RangeOne derivation by using a 6-bit by 5-bit multiplier

The interval subdivision is realized using a 6-bit by 5-bit multiplier. The range interval is divided into the rangeOne and the rangeZero. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 63 – (P>>9) : (P>>9)

rangeLPS  = ((2 × probIdx + 1) × (2 × (range>>5) + 1)) >> 3

rangeOne  = ( P >= 16384 ) ? range – rangeLPS : rangeLPS

rangeZero = ( P >= 16384 ) ? rangeLPS : range – rangeLPS

The value range of (2 × probIdx + 1) is (1, 63), which is a 6-bit value. The value range of (2 × (range>>5) + 1)) is (17, 31), which is a 5-bit value. The derived rangeOne and rangeZero values are identical to the values derived in CE5.3.3.

A supplementary test that uses a 5-bit by 5-bit multiplier is also provided. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 31 – (P>>10) : (P>>10)

rangeLPS  = ((2 × probIdx + 1) × (2 × (range>>5) + 1)) >> 3

rangeOne  = ( P >= 16384 ) ? range – rangeLPS : rangeLPS

rangeZero = ( P >= 16384 ) ? rangeLPS : range – rangeLPS

The value range of (2 × probIdx + 1) is (1, 31), which is a 5-bit value.

# Simulation results

The proposed methods are implemented on BMS-1.0 [1]. The results of CE5.3.3 and CE5.3.4 are shown in Table 1 and Table 2, respectively. The simulations were performed following the JVET CTC [2]. The anchor is VTM-1.0/BMS-1.0 configuration with JEM CABAC engine enabled (by setting CABACEngine : 3). The supplementary results of using a 16×8×8-bit look-up table or a 5-bit by 5-bit multiplier are also shown in Table 3 and Table 4, respectively.

Table 1.  CE5.3.3: RangeOne derivation by using a 32×8×8-bit LUT

Table 2.  CE5.3.4: RangeOne derivation by using a 5-bit by 6-bit multiplier

Table 3.  Supplementary result of CE5.3.3: RangeOne derivation by using a 16×8×8-bit LUT

Table 4.  Supplementary result CE5.3.4: RangeOne derivation by using a 5-bit by 5-bit multiplier

# Hardware synthesis results

The methods proposed in CE5.3.1, CE5.3.2, CE5.3.3, and CE5.3.4 are implemented in Verilog and synthesized by using TSMC 12nm technology. The results reportedly show that, when using the same table size (e.g. 16x16 LUT or 32x8 LUT), CE5.3.3 has the shortest critical path (highest throughput), and CE5.3.4 has the smallest area (lowest cost), as shown in Table 5.

Table 5.  Synthesis result of CE5.3.1, CE5.3.2, CE5.3.3, and CE5.3.4

|  | JEM | 5.3.1-a | 5.3.1-b | 5.3.2-a | 5.3.2-b | 5.3.3-a | 5.3.3-b | 5.3.4-a | 5.3.4-b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 512x64 LUT | 16x16 LUT | 8x8 LUT | 32x8 LUT | 16x8 LUT | 32x8 LUT | 16x8 LUT | 6b x 5b mult. | 5b x 5b mult. |
| Total Area (um2) | 3153.27 | 187.67 | 89.87 | 88.03 | 65.18 | 76.12 | 47.22 | 36.27 | 29.01 |
| Table Area (um2) | 3152.90 | 116.67 | 41.47 | 85.05 | 62.04 | 72.92 | 45.45 | 33.47 | 27.02 |
| Rest Area (um2) | 0.37 | 71.00 | 48.40 | 2.99 | 3.13 | 3.21 | 1.77 | 2.80 | 1.99 |
| Critical Path (ns) | 0.620 | 0.658 | 0.563 | 0.392 | 0.344 | 0.373 | 0.335 | 0.497 | 0.487 |

# Conclusions

In this contribution, two alternative range subinterval derivation methods are proposed. In CE5.3.3, a 32×8×8-bit table is used for deriving the rangeOne value. The table size is the same as the size of the table used by HEVC. In CE5.3.4, the table look-up values are calculated by using a 6-bit by 5-bit multiplier. The derived rangeOne values of CE5.3.3 and CE5.3.4 are identical. Simulation results reportedly show that, compared with using the 512×64×9-bit look up table, the average luma BD-rate increases are less than 0.04% in both BMS-1.0 and VTM-1.0 configurations.

# Reference

[1] BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).