| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0250-v1 |
| --- | --- |

| Title: | CE6.1.10 & CE6.3.4: AMT simplification and improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Zhi-Yi Lin, Tzu-Der Chuang , Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Adaptive multi-core transform (AMT) in JEM-7.0 is applied to luma coding blocks (CBs) of nonzero residual when the luma CB width and height are smaller than or equal to 64. Three aspects of AMT modifications are proposed.

Aspect 1: Reducing the number of transform types.
The proposed AMT only utilizes DCT-II, DST-VII, and flipped DST-VII (FDST-VII).

Aspect 2: Reducing encoding time with normative decoder changes.
In intra slices, unlike explicitly signaling AMT flag for all intra modes in JEM-7.0 AMT, the AMT flag is explicitly signaled for DC and planar modes, implicitly set to 1 for even angular modes, and implicitly set to 0 for odd angular modes. Besides, for intra coding units (CUs) and inter CUs, when AMT flag is equal to 1, the number of AMT modes is set to 1, 2, 3, or 4 according to CU type (intra or inter) and CU size, unlike always 4 in JEM-7.0 AMT. AMT index is signaled when there are more than one AMT mode.

Aspect 3: Improving coding efficiency
If the number of nonzero transform coefficients is larger than a threshold, AMT flag and AMT index can be sent; otherwise they are derived at decoder.

In the following, VTM-1.1 anchor excludes AMT, while BMS-1.1 anchor includes JEM-7.0 AMT. Luma BD-rate, encoding time, and decoding time are reported.

Results of applying aspect 1 to JEM-7.0 AMT, as planned in CE6.1.10.1
VTM-1.1-AI: {Y-BD-rate = -3.15%, EncT=191%, DecT=117%}
VTM-1.1-RA: {Y-BD-rate = -2.08%, EncT=159%, DecT=108%}
VTM-1.1-LB: {Y-BD-rate = -1.51%, EncT=159%, DecT=110%}
BMS-1.1-AI: {Y-BD-rate = +0.21%, EncT=91%, DecT=97%}
BMS-1.1-RA: {Y-BD-rate = +0.08%, EncT=93%, DecT=99%}
BMS-1.1-LB: {Y-BD-rate = -0.05%, EncT=97%, DecT=99%}

Results of applying aspect 1 and aspect 2 to JEM-7.0 AMT, as planned in CE6.1.10.2
VTM-1.1-AI: {Y-BD-rate = -2.67%, EncT=169%, DecT=121%}
VTM-1.1-RA: {Y-BD-rate = -1.80%, EncT=152%, DecT=109%}
VTM-1.1-LB: {Y-BD-rate = -1.32%, EncT=155%, DecT=109%}
BMS-1.1-AI: {Y-BD-rate = +1.03%, EncT=80%, DecT=98%}
BMS-1.1-RA: {Y-BD-rate = +0.56%, EncT=84%, DecT=100%}
BMS-1.1-LB: {Y-BD-rate = +0.23%, EncT=91%, DecT=99%}

Results of applying aspect 3 to JEM-7.0 AMT, as planned in CE6.3.4
VTM-1.1-AI: {Y-BD-rate = -3.45%, EncT=213%, DecT=124%}
VTM-1.1-RA: {Y-BD-rate = -2.25%, EncT=166%, DecT=108%}
VTM-1.1-LB: {Y-BD-rate = -1.80%, EncT=165%, DecT=107%}
BMS-1.1-AI: {Y-BD-rate = -0.01%, EncT=100%, DecT=101%}
BMS-1.1-RA: {Y-BD-rate = -0.07%, EncT=101%, DecT=99%}
BMS-1.1-LB: {Y-BD-rate = -0.33%, EncT=102%, DecT=97%}

# Introduction

The proposed AMT is based on JEM-7.0 AMT. The common parts of the proposed AMT and the JEM-7.0 AMT are described: For a luma CB of nonzero residual when the luma CB width and height are smaller than or equal to 64, an AMT flag is first signaled to indicate whether AMT is applied. If the AMT flag is signaled as 0, only DCT-II is used; otherwise an AMT index is further signaled to indicate which transform types are used for horizontal and vertical transforms. Three aspects of AMT modifications are proposed.

# Proposed methods

## Aspect 1: Reducing the number of transform types

The proposed AMT can select DCT-II, DST-VII, and flipped DST-VII (FDST-VII) for horizontal transform and vertical transform. Table 1 shows the basis functions for the three transforms. Note that, let fDST-VIIj be the transformed signal using DST-VII, then the FDST-VII is simply calculated as fDST-VIIN-1-j, where N is the transform size. AMT flag equal to 0 indicates that DCT-II is used for both horizontal transform and vertical transforms, AMT flag equal to 1 indicates that another transform type is used depending on AMT index. For an intra CU, an AMT index is signaled to indicate which of DCT-II and DST-VII is used for horizontal and vertical transforms. As for an inter CU, an AMT index is signaled to indicate which of DST-VII and FDST-VII is used for horizontal and vertical transforms. The transform sets for intra and inter coding and the mapping code words are listed in Table 2.

Table 1.  Transform basis functions of DCT-II, DST-VII, and flipped DST-VII for N-point input.

| Transform Type | Basis function Ti(j), i, j=0, 1,…, N−1 |
| --- | --- |
| DCT-II | Ti(j)=ω0∙2N∙cosπ∙i∙2j+12N where ω0=2Ni=01i≠0 |
| DST-VII | Ti(j)=42N+1∙sinπ∙(2i+1)∙(j+1)2N+1 |
| FDST-VII | Ti(j)=42N+1∙sinπ∙(2i+1)∙(N-j)2N+1 |

Table 2.  The transform sets for intra and inter coding and the mapping code words.

| CU type | AMT index | Transform set (hor,ver) | Codeword |
| --- | --- | --- | --- |
| Intra | 0 | (DST-VII, DST-VII) | 0 |
|  | 1 | (DCT-II, DST-VII) | 10 |
|  | 2 | (DST-VII, DCT-II) | 11 |
| Inter | 0 | (FDST-VII, FDST-VII) | 00 |
|  | 1 | (DST-VII, FDST-VII) | 10 |
|  | 2 | (FDST-VII, DST-VII) | 01 |
|  | 3 | (DST-VII, DST-VII) | 11 |

## Aspect 2: Reducing encoding time with normative decoder changes

Implicit AMT flag assignments are proposed as follows. In intra slices, the AMT flag is explicitly signaled for DC and planar modes, implicitly set to 1 for even angular modes, and implicitly set to 0 for odd angular modes. Moreover, when AMT flag is equal to 1, different numbers of AMT modes are allowed according to the CU size. If the horizontal CU size is smaller than or equal to 4, the type of vertical transform set is set to be DST-VII and FDST-VII for intra CU and inter CU, respectively. If the vertical CU size is smaller than or equal to 4, the type of the horizontal transform set is set to be DST-VII and FDST-VII for intra CU and inter CU, respectively.

## Aspect 3: Improving coding efficiency

It is proposed to conditionally signal AMT flag and AMT index. AMT flag is signaled when the number of nonzero transform coefficients is larger than a threshold. For inter CUs, the threshold is set to 2. For intra CUs, the threshold is set to 0. For intra CUs and inter CUs, AMT index is signaled only if the number of nonzero coefficients is larger than 2; otherwise, it is inferred as 0. To achieve the proposed conditional AMT flag signaling, the AMT flag in the syntax table is moved to a position after residual coding. This affects both the context for AMT flag and the syntax signaling of transformSkip flag since the signaling of transformSkip flag is not constrained by AMT flag. Therefore, some minor BD-rate changes are reported in AI case.

# Simulation results

CE6.1.10.1 applies aspect 1 to JEM-7.0 AMT, CE6.1.10.2 applies aspect 1 and aspect 2 to JEM-7.0 AMT, and CE6.3.4 applies aspect 3 to JEM-7.0 AMT. The proposed AMT methods have been implemented on BMS-1.1 [1]. The simulations were performed following the JVET common test conditions [2]. CE6.1.10.1 results are shown in Table 3 and Table 4, where no AMT is used in the anchor of Table 3 and JEM-7.0 AMT is enabled in the anchor of Table 4. CE6.10.2 results are shown in Table 5 and Table 6, where no AMT is used in the anchor of Table 5 and JEM-7.0 AMT is enabled in the anchor of Table 6. CE6.3.4 results are shown in Table 7 and Table 8, where no AMT is used in the anchor of Table 7 and JEM-7.0 AMT is enabled in the anchor of Table 8.

Table 3.  CE6.1.10.1: applying aspect 1 to JEM-7.0 AMT; anchor disables AMT.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.22% | -1.63% | -1.46% | 187% | 124% | -2.47% | -1.97% | -2.48% | 200% | 113% |
| Class A2 | -2.89% | -1.58% | -1.41% | 191% | 117% | -2.11% | -1.89% | -1.48% | 206% | 109% |
| Class B | -3.15% | -1.41% | -1.46% | 195% | 117% | -2.11% | -1.64% | -1.83% | 210% | 110% |
| Class C | -2.67% | -0.84% | -1.03% | 199% | 112% | -1.56% | -0.93% | -1.25% | 218% | 104% |
| Class E | -3.96% | -2.66% | -2.65% | 182% | 116% | -2.45% | -2.50% | -2.47% | 198% | 109% |
| Overall | -3.15% | -1.56% | -1.56% | 191% | 117% | -2.10% | -1.72% | -1.86% | 207% | 109% |
| Class D | -2.56% | -0.61% | -0.71% | 207% | 106% | -1.44% | -0.70% | -0.71% | 226% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 34% | #VALUE! | #VALUE! | #VALUE! | 0% | 22% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.96% | -0.15% | -0.07% | 164% | 114% | -1.73% | -0.67% | -1.29% | 169% | 103% |
| Class A2 | -2.04% | 0.04% | 0.34% | 151% | 108% | -1.69% | -0.56% | -0.08% | 155% | 102% |
| Class B | -2.38% | -0.25% | -0.46% | 157% | 108% | -1.85% | -0.84% | -0.85% | 161% | 102% |
| Class C | -1.82% | 0.32% | -0.21% | 162% | 106% | -1.28% | -0.11% | -0.31% | 166% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.08% | -0.02% | -0.16% | 159% | 108% | -1.65% | -0.56% | -0.64% | 163% | 102% |
| Class D | -1.66% | 0.39% | 0.35% | 159% | 101% | -1.22% | 0.24% | -0.14% | 163% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 14% | #VALUE! | #VALUE! | #VALUE! | 0% | 8% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.93% | 1.01% | 1.07% | 164% | 111% | -1.58% | 0.47% | 0.04% | 151% | 106% |
| Class C | -1.26% | 0.94% | 0.77% | 161% | 110% | -0.98% | 0.72% | 0.59% | 151% | 106% |
| Class E | -1.17% | 1.20% | 1.78% | 150% | 109% | -0.88% | 1.44% | 2.16% | 128% | 106% |
| Overall | -1.51% | 1.03% | 1.15% | 159% | 110% | -1.21% | 0.79% | 0.75% | 145% | 106% |
| Class D | -1.22% | 0.65% | 0.73% | 149% | 106% | -0.90% | 0.99% | 1.80% | 142% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 14% | #VALUE! | #VALUE! | #VALUE! | 0% | 10% |

Table 4.  CE6.1.10.1: applying aspect 1 to JEM-7.0 AMT, anchor enables JEM-7.0 AMT.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT |  | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.32% | -0.25% | 92% |  | 94% | 0.08% | -0.24% | -0.30% | 92% | 96% |
| Class A2 | 0.28% | 0.07% | 0.02% | 91% |  | 95% | 0.32% | 0.11% | 0.02% | 91% | 97% |
| Class B | 0.32% | 0.03% | -0.04% | 90% |  | 95% | 0.22% | 0.07% | 0.09% | 90% | 97% |
| Class C | 0.46% | 0.06% | 0.12% | 89% |  | 101% | 0.26% | 0.21% | 0.22% | 90% | 98% |
| Class E | 0.45% | 0.09% | 0.00% | 92% |  | 96% | 0.16% | 0.16% | 0.13% | 91% | 98% |
| Overall | 0.33% | -0.01% | -0.02% | 91% |  | 96% | 0.21% | 0.07% | 0.05% | 91% | 97% |
| Class D | 0.45% | 0.05% | 0.03% | 88% |  | 97% | 0.21% | 0.20% | 0.27% | 88% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% |  | 31% | #VALUE! | #VALUE! | #VALUE! | 0% | 21% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.22% | -0.22% | 96% | 98% |  | -0.01% | -0.14% | -0.34% | 92% | 99% |
| Class A2 | 0.02% | -0.10% | -0.13% | 97% | 99% |  | 0.06% | 0.15% | -0.09% | 94% | 99% |
| Class B | 0.10% | 0.02% | 0.13% | 96% | 98% |  | 0.07% | 0.09% | 0.08% | 93% | 99% |
| Class C | 0.24% | 0.30% | -0.01% | 96% | 97% |  | 0.16% | 0.41% | 0.05% | 93% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.09% | 0.02% | -0.03% | 96% | 98% |  | 0.08% | 0.14% | -0.04% | 93% | 99% |
| Class D | 0.16% | -0.12% | 0.50% | 96% | 98% |  | 0.15% | 0.11% | 0.12% | 93% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 13% |  | #VALUE! | #VALUE! | #VALUE! | 0% | 8% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.18% | 0.71% | 97% | 99% |  | -0.02% | 0.23% | -0.11% | 96% | 99% |
| Class C | 0.05% | -0.02% | -0.15% | 97% | 98% |  | -0.01% | 0.16% | 0.08% | 96% | 100% |
| Class E | 0.17% | -0.83% | -0.14% | 99% | 101% |  | -0.16% | 0.12% | 0.46% | 99% | 100% |
| Overall | 0.06% | -0.14% | 0.21% | 98% | 100% |  | -0.05% | 0.18% | 0.10% | 97% | 99% |
| Class D | 0.03% | -0.06% | -0.11% | 98% | 101% |  | 0.06% | 0.01% | 0.82% | 97% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 13% |  | #VALUE! | #VALUE! | #VALUE! | 0% | 9% |

Table 5.  CE6.1.10.2: applying aspect 1 and aspect 2 to JEM-7.0 AMT; anchor disables AMT.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.75% | -1.95% | -1.78% | 170% | 133% | -1.76% | -1.59% | -2.17% | 181% | 115% |
| Class A2 | -2.41% | -1.75% | -1.57% | 170% | 124% | -1.36% | -1.52% | -1.16% | 183% | 110% |
| Class B | -2.69% | -1.64% | -1.63% | 170% | 122% | -1.36% | -1.20% | -1.25% | 183% | 111% |
| Class C | -2.05% | -1.36% | -1.55% | 168% | 109% | -0.74% | -0.88% | -1.08% | 183% | 104% |
| Class E | -3.66% | -2.94% | -2.96% | 165% | 125% | -1.41% | -1.98% | -1.83% | 183% | 110% |
| Overall | -2.67% | -1.87% | -1.85% | 169% | 121% | -1.30% | -1.37% | -1.45% | 183% | 110% |
| Class D | -1.85% | -1.09% | -1.16% | 173% | 105% | -0.75% | -0.70% | -0.96% | 182% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 34% | #VALUE! | #VALUE! | #VALUE! | 0% | 22% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.81% | -0.29% | -0.04% | 158% | 115% | -1.24% | -0.29% | -0.76% | 151% | 104% |
| Class A2 | -1.82% | 0.16% | 0.26% | 146% | 108% | -1.37% | -0.31% | 0.05% | 142% | 104% |
| Class B | -2.11% | -0.22% | -0.49% | 151% | 110% | -1.34% | -0.28% | -0.32% | 145% | 103% |
| Class C | -1.38% | -0.06% | -0.30% | 154% | 105% | -0.76% | -0.01% | -0.10% | 149% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.80% | -0.12% | -0.20% | 152% | 109% | -1.17% | -0.22% | -0.28% | 147% | 103% |
| Class D | -1.17% | 0.13% | -0.56% | 150% | 101% | -0.78% | 0.01% | -0.36% | 146% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 14% | #VALUE! | #VALUE! | #VALUE! | 0% | 8% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.79% | 0.64% | 0.55% | 159% | 111% | -1.35% | 0.65% | 0.21% | 141% | 106% |
| Class C | -1.03% | 0.69% | 0.60% | 155% | 108% | -0.70% | 0.69% | 0.54% | 141% | 105% |
| Class E | -0.91% | 1.44% | 1.40% | 147% | 107% | -0.51% | 1.62% | 1.95% | 125% | 105% |
| Overall | -1.32% | 0.85% | 0.78% | 155% | 109% | -0.93% | 0.91% | 0.76% | 137% | 106% |
| Class D | -0.86% | 0.10% | -0.16% | 143% | 103% | -0.54% | 0.34% | 0.66% | 135% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 14% | #VALUE! | #VALUE! | #VALUE! | 0% | 10% |

Table 6.  CE6.1.10.2: applying aspect 1 and aspect 2 to JEM-7.0 AMT; anchor enables JEM-7.0 AMT.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.57% | -0.64% | -0.57% | 84% | 101% | 0.81% | 0.15% | 0.00% | 84% | 98% |
| Class A2 | 0.78% | -0.11% | -0.14% | 81% | 100% | 1.09% | 0.50% | 0.34% | 81% | 98% |
| Class B | 0.80% | -0.20% | -0.22% | 79% | 98% | 0.98% | 0.52% | 0.67% | 79% | 98% |
| Class C | 1.10% | -0.47% | -0.40% | 76% | 98% | 1.08% | 0.26% | 0.39% | 75% | 98% |
| Class E | 0.77% | -0.21% | -0.32% | 83% | 104% | 1.23% | 0.70% | 0.79% | 84% | 99% |
| Overall | 0.82% | -0.32% | -0.32% | 80% | 100% | 1.03% | 0.43% | 0.46% | 80% | 98% |
| Class D | 1.18% | -0.44% | -0.42% | 73% | 96% | 0.92% | 0.21% | 0.03% | 71% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 31% | #VALUE! | #VALUE! | #VALUE! | 0% | 21% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.37% | -0.19% | 92% | 99% | 0.49% | 0.25% | 0.19% | 83% | 99% |
| Class A2 | 0.24% | 0.02% | -0.20% | 93% | 99% | 0.40% | 0.40% | 0.05% | 85% | 101% |
| Class B | 0.38% | 0.05% | 0.09% | 92% | 100% | 0.59% | 0.66% | 0.62% | 84% | 100% |
| Class C | 0.70% | -0.09% | -0.10% | 91% | 96% | 0.69% | 0.51% | 0.26% | 83% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.38% | -0.08% | -0.07% | 92% | 99% | 0.56% | 0.48% | 0.32% | 84% | 100% |
| Class D | 0.66% | -0.38% | -0.41% | 91% | 98% | 0.60% | -0.12% | -0.10% | 83% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 13% | #VALUE! | #VALUE! | #VALUE! | 0% | 8% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.14% | -0.19% | 0.18% | 95% | 100% | 0.22% | 0.41% | 0.06% | 90% | 99% |
| Class C | 0.28% | -0.26% | -0.32% | 94% | 97% | 0.26% | 0.12% | 0.03% | 90% | 99% |
| Class E | 0.43% | -0.60% | -0.50% | 98% | 100% | 0.21% | 0.34% | 0.24% | 96% | 100% |
| Overall | 0.26% | -0.32% | -0.16% | 95% | 99% | 0.23% | 0.30% | 0.09% | 91% | 99% |
| Class D | 0.39% | -0.62% | -1.01% | 95% | 99% | 0.43% | -0.64% | -0.31% | 92% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 13% | #VALUE! | #VALUE! | #VALUE! | 0% | 9% |

Table 7:  CE6.3.4: applying aspect 3 to JEM-7.0 AMT; anchor disables AMT.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.29% | -1.34% | -1.18% | 205% | 136% | -2.55% | -1.74% | -2.16% | 217% | 119% |
| Class A2 | -3.16% | -1.61% | -1.42% | 213% | 126% | -2.42% | -2.00% | -1.51% | 225% | 114% |
| Class B | -3.46% | -1.41% | -1.43% | 218% | 128% | -2.33% | -1.68% | -1.95% | 233% | 115% |
| Class C | -3.11% | -0.91% | -1.17% | 225% | 111% | -1.82% | -1.07% | -1.34% | 245% | 106% |
| Class E | -4.37% | -2.75% | -2.74% | 200% | 121% | -2.61% | -2.62% | -2.65% | 219% | 111% |
| Overall | -3.45% | -1.55% | -1.55% | 213% | 124% | -2.32% | -1.77% | -1.89% | 229% | 113% |
| Class D | -2.99% | -0.74% | -0.71% | 238% | 107% | -1.66% | -0.67% | -0.91% | 258% | 104% |
| Class F | -1.92% | -0.91% | -0.81% | 212% | 108% | -1.26% | -0.98% | -0.91% | 242% | 105% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.96% | -0.16% | -0.03% | 172% | 114% | -1.78% | -0.59% | -1.19% | 184% | 104% |
| Class A2 | -2.10% | -0.08% | 0.27% | 157% | 108% | -1.78% | -0.81% | -0.27% | 167% | 102% |
| Class B | -2.59% | -0.50% | -0.66% | 165% | 109% | -2.02% | -1.24% | -1.12% | 174% | 102% |
| Class C | -2.15% | -0.13% | -0.42% | 170% | 104% | -1.55% | -0.58% | -0.71% | 180% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.25% | -0.25% | -0.28% | 166% | 108% | -1.80% | -0.85% | -0.86% | 176% | 102% |
| Class D | -1.89% | 0.17% | -0.19% | 168% | 103% | -1.43% | -0.19% | -0.52% | 176% | 101% |
| Class F | -1.72% | -0.47% | -0.34% | 178% | 103% | -1.29% | -0.57% | -0.40% | 190% | 103% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 + AMT disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.09% | 0.45% | 0.55% | 171% | 110% | -1.76% | -0.02% | 0.01% | 158% | 103% |
| Class C | -1.58% | 0.56% | 0.40% | 167% | 105% | -1.28% | 0.35% | 0.17% | 159% | 103% |
| Class E | -1.62% | 0.90% | 0.88% | 153% | 105% | -1.19% | 0.41% | 2.04% | 130% | 103% |
| Overall | -1.80% | 0.60% | 0.58% | 165% | 107% | -1.46% | 0.21% | 0.57% | 151% | 103% |
| Class D | -1.34% | 0.51% | 0.19% | 154% | 103% | -1.14% | 0.53% | 0.41% | 148% | 101% |
| Class F | -1.41% | 0.04% | -0.06% | 170% | 105% | -0.92% | -0.66% | -0.11% | 161% | 102% |

Table 8:  CE6.3.4: applying aspect 3 to JEM-7.0 AMT; anchor enables JEM-7.0 AMT.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.03% | 100% | 103% | -0.01% | -0.01% | 0.02% | 100% | 101% |
| Class A2 | -0.01% | 0.04% | 0.01% | 100% | 103% | 0.00% | 0.00% | -0.01% | 100% | 101% |
| Class B | 0.00% | 0.03% | -0.01% | 100% | 103% | 0.00% | 0.02% | -0.04% | 100% | 101% |
| Class C | 0.02% | -0.01% | -0.01% | 100% | 100% | -0.02% | 0.06% | 0.12% | 101% | 100% |
| Class E | 0.03% | -0.01% | -0.09% | 100% | 101% | 0.00% | 0.03% | -0.05% | 100% | 100% |
| Overall | 0.01% | 0.01% | -0.01% | 100% | 102% | -0.01% | 0.03% | 0.01% | 100% | 101% |
| Class D | 0.01% | -0.09% | 0.03% | 101% | 99% | 0.00% | 0.23% | 0.07% | 101% | 100% |
| Class F | -0.03% | -0.08% | 0.08% | 100% | 100% | -0.07% | -0.03% | -0.25% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.24% | -0.17% | 101% | 99% | -0.05% | -0.10% | -0.20% | 101% | 98% |
| Class A2 | -0.05% | -0.22% | -0.19% | 101% | 99% | -0.02% | -0.17% | -0.25% | 101% | 100% |
| Class B | -0.11% | -0.23% | -0.07% | 101% | 100% | -0.09% | -0.30% | -0.23% | 101% | 99% |
| Class C | -0.09% | -0.16% | -0.22% | 100% | 95% | -0.10% | -0.06% | -0.37% | 102% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.08% | -0.21% | -0.16% | 101% | 98% | -0.07% | -0.17% | -0.27% | 101% | 99% |
| Class D | -0.07% | -0.33% | -0.03% | 101% | 99% | -0.05% | -0.32% | -0.28% | 102% | 99% |
| Class F | -0.15% | -0.13% | 0.08% | 101% | 98% | -0.12% | -0.06% | 0.05% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 + AMT enabled |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.17% | -0.38% | 0.19% | 101% | 97% | -0.20% | -0.50% | 0.12% | 101% | 97% |
| Class C | -0.28% | -0.39% | -0.52% | 101% | 95% | -0.32% | -0.20% | -0.39% | 102% | 97% |
| Class E | -0.28% | -1.12% | -1.02% | 101% | 98% | -0.55% | -1.02% | -0.24% | 102% | 98% |
| Overall | -0.23% | -0.57% | -0.35% | 101% | 97% | -0.33% | -0.53% | -0.14% | 102% | 97% |
| Class D | -0.10% | -0.21% | -0.70% | 101% | 99% | -0.21% | -0.01% | -0.83% | 102% | 98% |
| Class F | -0.37% | -0.05% | -0.26% | 101% | 99% | -0.22% | -0.78% | -0.43% | 102% | 97% |

# Conclusions

Three aspects of AMT modifications are proposed: aspect 1: Reducing the number of transform types; aspect 2: Reducing encoding time with normative decoder changes; aspect 3: Improving coding efficiency. Simulation results show that aspect 1 can reduce 3-9% encoding time with about 0.1% luma BD-rate, while the transform complexity is significantly reduced; aspect 2 can reduce 9-20% encoding time with about 0.6% luma BD-rate, while the decoding complexity impact is little; aspect 3 can achieve -0.1% luma BD-rate, while the encoding and decoding complexity impacts are both little.

# References

[1] BMS-1.1: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).