| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0252-v1 |
| --- | --- |

| Title: | CE7.3.3: Derivation of chroma QP from luma QP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment 7.3.3 (CE7.3.3) for derivation of the chroma quantization parameter value from the associated luma quantization parameter (QP) value. The proposed method utilizes the same pre-defined function for deriving the chroma quantization parameter Qpc from the input QP index qPi. However, the new syntax elements are added for setting the constraint on the maximum allowed difference between qPi and Qpc. Experimental results show the proposed method can provide an additional mechanism for balancing bit and distortion distribution among the three color components.

# Introduction

In the High Efficiency Video Coding (HEVC) standard as specified in [1], the chroma quantization parameter Qpc is derived by Table 1, where qPi is equal to the associated luma quantization parameter plus the chroma QP offset value signaled in the picture parameter set (PPS) and/or the slice header. The derivation of chroma QP value from the associated luma QP value can be adjusted by signaling different chroma QP offset values. A positive chroma QP offset value will result in a coarser quantizer for the associated chroma component.

Table 1: Specification of QpC as a function of qPi

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 35 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

# Proposed methods

In the proposed method [2], derivation of the chroma quantization parameter value from the associated luma quantization parameter can be further controlled by a variable maxDiffQpiQpc, which determines the maximum allowed difference between the input index qPi and the output value Q’pc of the chroma QP mapping function specified in Table 1. The chroma quantization parameter Qpc is derived by

Qpc = qPi – min ( maxDiffQpiQpc, qPi – Q’pc ).

The variable maxDiffQpiQpc with a value range from 0 to 6 can be signaled in the PPS and the slice header for cb and cr components, respectively. The encoder configuration parameters MaxDiffQpiQpcCb and MaxDiffQpiQpcCr are added to set the corresponding PPS syntax element values for cb and cr components, respectively.

# Experimental results

The proposed method was integrated into the BMS-1.0 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified BMS-1.0 reference software. All the coding experiments were conducted according to the CE7 test conditions specified in [4, 5].

Table 2 summarize the simulation results for CE7.3.3 under the VTM-1.0 configurations (with all BMS tools off) and the BMS-1.0 configurations (with all BMS tools on), respectively. The encoder configuration parameters MaxDiffQpiQpcCb and MaxDiffQpiQpcCr are both set equal to 2.

The detailed simulation results can be found in the attached spreadsheets.

Table 2: Summary of the average BD bitrate and run time results for CE7.3.3 with MaxDiffQpiQpcCb and MaxDiffQpiQpcCr both set equal to 2

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- High Efficiency Video Coding (HEVC), Rec. ITU-T H.265 and ISO/IEC 23008-2, Jan. 2013 (and later editions).

- C.-W. Hsu, et al, “Description of SDR video coding technology proposal by MediaTek,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0018, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- The software BMS-1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1027, 10th Meeting: San Diego, US, 10–20 Apr. 2018.