| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0253-v1 |
| --- | --- |

| Title: | CE9.1.2: DMVR early termination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chi Su, Tzu-Der Chuang, ChingYeh Chen, Yu-Wen Huang, Shaw-Min Lei, No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yu-chi.su, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an early termination method for decoder-side motion vector refinement (DMVR). In the proposed method, the motion vector refinement for the motion vector from list 1 (L1) is skipped based on the motion refinement results for the motion vector from list 0 (L0). Under VTM-1.0 with DMVR enabled, the early termination can save 4% decoding time. Under BMS-1.0 with DMVR enabled, the early termination can save 1 % decoding time. Statistical results show that 52% DMVR coding units (CUs) apply the early termination. No noticeable BD-rate change is caused by the early termination.

# Introduction

DMVR is proposed in JVET-D0029 [1]. In DMVR, the two motion vectors (MV0 from L0 and MV1 from L1) in bi-prediction mode are further refined to find a better predictor. The motion vector refinement is based on a bilateral template matching process. A template is generated by using the bi-prediction from the reference blocks of MV0 and MV1 as shown in Figure 1. Then the template is used as a new current block and perform the motion estimation to find a better matched block in the L0 reference picture and the L1 reference picture from L1 separately. The refined MVs are MV0’ and MV1’. Then the refined MVs are used to generate a final bi-prediction block for the current block.

Figure 1. DMVR based on bilateral template matching process.

# Proposed early termination

In this contribution, an early termination is introduced to reduce the computation complexity of DMVR. In DMVR, after generating the template, MV refinement for MV0 from list 0 is performed to generate a new motion vector, called MV0’. If the MV0’ is equal to MV0, then the motion refinement for MV1 from list 1 is skipped.

# Experimental results

The proposed method was implemented on top of BMS-1.0 reference software [2]. Table 1 shows the results of the proposed DMVR over VTM-1.0 (DMVR disabled in the anchor) and over BMS-1.0 (JEM DMVR enabled in the anchor). Table 2 shows the results of the early termination on VTM-1.0 with DMVR. In addition, statistical results show that 52.7% and 52.4% of DMVR CUs apply the proposed early termination under VTM-1.0 and BMS-1.0.

Table1. Results of the proposed DMVR. (VTM-1.0 has no DMVR; BMS-1.0 has JEM DMVR.)

Table2. Results of the proposed early termination. (Anchor: VTM-1.0 with JEM DMVR)

# Conclusions

In this contribution, an early termination method for the L1 motion refinement in DMVR based on the L0 motion refinement result is proposed. The early termination has almost no impact on BD-rate and can save 1%-4% decoding time when DMVR is enabled.

# Patent rights declaration

MediaTek, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- [1] X. Chen, J. An, J. Zheng, “Decoder-side motion vector refinement based on bilateral template matching,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0029, October 2016.

- [2] BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/