| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0254-v1 |
| --- | --- |

| Title: | CE9.2.1: Bilateral matching merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In this contribution, a bilateral matching merge mode is proposed. In the proposed bilateral matching merge mode, a motion vector (MV) candidate list is generated and a starting MV candidate index is explicitly signaled. The bilateral matching is applied to find the best refined MV around the starting MV candidate. The candidate list is generated by using merge candidate list generation process without including the sub-CU merge candidates. The similar MV candidates (luma MV differences smaller than a half luma sample) are also removed. Both CU-level search and sub-CU-level search are applied. The CU-level search and sub-CU-level search share the same search window for reducing memory bandwidth requirement. The bilateral matching MV derivation is disabled for 4x4, 4x8, and 8x4 CUs for reducing the worst case memory bandwidth. Simulation results reportedly show -5.72%, -2.35%, -0.43%, and -0.34% luma BD-rates with 172%, 68%, 51%, and 32% decoding time increases for RA-VTM-1.0, RA-BMS-1.0, LB-VTM-1.0, LB-BMS-1.0, respectively. The supplementary results of disabling sub-CU-level search in the proposed bilateral matching merge mode reportedly show -4.76%, -1.58%, -0.46%, and -0.41% luma BD-rates with 114%, 31%, 31%, and 18% decoding time increases for RA-VTM-1.0, RA-BMS-1.0, LB-VTM-1.0, LB-BMS-1.0, respectively.

Introduction

In JEM pattern-matched MV derivation (PMMVD), there are two decoder-side derived merge mode, which are the template-matching merge mode and bilateral matching merge mode. The template-matching merge mode uses the neighboring reconstructed samples as template to find a best matched template in the reference picture. The bilateral matching merge mode finds a best matched block pair in two reference pictures (one in list 0 (L0), and the other in list 1 (L1)). In both merge modes, the decoder needs to evaluate several MV candidates to determine a starting MV candidate for CU-level search. In sub-CU-level search, in addition to the best CU-level MV, several MV candidates are added. The decoder needs to evaluate these MV candidates to find the best MV, which requires a lot of memory bandwidth. To reduce the memory bandwidth requirement, a bilateral matching merge mode with explicitly signaled starting MV candidate is proposed in this contribution.

Proposed methods

Starting MV candidate signalling

In the PMMVD of JEM, up to 22 MV pairs are evaluated to determine the starting MV candidate, which requires a lot of memory bandwidth to load the reference block pairs. In JEM, the MVs in each MV pair must be in the same motion trajectory. To reduce the complexity and the memory bandwidth requirement, if the bilateral matching merge mode is selected, a MV candidate list is first generated and a starting MV candidate index is explicitly signaled to indicate the starting MV candidate. The original merge candidate derivation process is reused for generating the starting MV candidates except that the sub-CU merge candidates, e.g., the affine candidates and ATMVP candidates, are excluded. In non-low-delay B picture, if the selected MV candidate is a bi-predicted MV candidate, two reference blocks pointed by the MV candidate are used for bilateral matching even if the MVs are not in the same motion trajectory of constant velocity. In low-delay B picture, a bi-prediction MV candidate is divided into two independent uni-prediction MV candidates. If the selected MV candidate is a uni-predicted MV, the uni-predicted MV and its mirrored MV is used for bilateral matching. In the MV candidate generation, the redundant MV candidate checking is replaced by the similar MV candidate checking. The MV with both the luma MV differences smaller than a half luma sample is removed.

CU-level search and sub-CU-level search

In the proposed bilateral matching merge mode, both CU-level and sub-CU-level search are applied. The CU-level search starts from the explicitly signaled starting MV candidate. A three-step diamond search is applied. The search algorithm is exactly the same as that used in JEM bilateral matching except the MV pair do not need to be always in the same motion trajectory of constant velocity. One CU-level best MV candidate is determined by the search algorithm.

For sub-CU-level search, the current CU is divided into several sub-blocks. The minimum sub-block size is 4x4. In JEM, for each sub-block, up to 9 starting MV candidates (including the CU-level best MV candidate) are used for determining the best starting MV candidate. In this contribution, to reduce the complexity and memory bandwidth requirement, for each sub-block, the CU-level best MV candidate is directly used as the starting candidate. A two-step adaptive cross search, which is the same as the sub-block search used by JEM PMMVD, is applied to find a best sub-block best MV candidate. The sub-block best MV candidate is the final MV for each sub-block.

Further memory bandwidth reduction

Signaling the starting MV candidate can already achieve significant memory bandwidth reduction. In this subsection, further memory bandwidth reduction techniques are proposed.

Smaller blocks require the more memory bandwidth for motion compensation. To reduce the worst case memory bandwidth requirement, the bilateral matching merge mode is disabled for 4x4, 4x8, and 8x4 CUs. For the CU size larger than 64 (e.g. 8x8 block), the search range for CU-level and sub-CU-level is ±8 samples. For the CU size equal to 64, the search range for CU-level and sub-CU-level is ±4 samples.

To reduce the memory bandwidth of sub-CU-level search, it is proposed to reuse the CU-level search window, e.g., the ±8 samples around the CU-level signaled starting MV candidate, for sub-CU-level search. The search candidate outside of the CU-level search window is marked as invalid. In this way, there is no additional memory bandwidth for sub-CU-level search. The worst case and average case bandwidth requirement are both reduced.

Simulation results

The proposed methods are implemented on BMS-1.0 [1]. The results of CE9.2.1 bilateral matching merge mode are shown in Table 1. The simulations were performed following the JVET common test conditions [2]. The anchors are the default VTM-1.0/BMS-1.0 configuration. The supplementary results of CE9.2.1 bilateral matching merge mode without the sub-CU-level search are shown in Table 2.

Table 1.  Results of CE9.2.1 bilateral matching merge mode.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -5.22% | -4.15% | -4.50% | 164% | 259% | -1.76% | -1.50% | -1.78% | 124% | 155% |
| Class A2 | -9.90% | -8.10% | -8.31% | 164% | 329% | -3.83% | -2.94% | -3.04% | 129% | 183% |
| Class B | -5.15% | -3.93% | -3.98% | 175% | 272% | -2.15% | -1.71% | -1.72% | 129% | 166% |
| Class C | -3.67% | -3.32% | -3.49% | 179% | 244% | -1.94% | -1.64% | -1.73% | 129% | 170% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -5.72% | -4.64% | -4.82% | 172% | 272% | -2.35% | -1.90% | -2.00% | 128% | 168% |
| Class D | -4.08% | -3.42% | -3.69% | 181% | 260% | -2.41% | -2.04% | -1.99% | 130% | 188% |
| Class F (optional) | -1.64% | -1.86% | -1.85% | 169% | 164% | -0.62% | -0.78% | -0.73% | 122% | 142% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.45% | -0.76% | -1.08% | 243% | 138% | -0.28% | -0.86% | -0.34% | 148% | 117% |
| Class C | -0.30% | -0.84% | -0.92% | 227% | 143% | -0.35% | -0.74% | -1.14% | 148% | 127% |
| Class E | -0.55% | -0.47% | -0.41% | 237% | 190% | -0.44% | -0.12% | -0.48% | 152% | 169% |
| Overall | -0.43% | -0.71% | -0.86% | 236% | 151% | -0.34% | -0.64% | -0.64% | 149% | 132% |
| Class D | -0.34% | -1.09% | -1.51% | 218% | 161% | -0.45% | -0.64% | -1.23% | 149% | 147% |
| Class F (optional) | -0.08% | -0.63% | -0.34% | 202% | 133% | 0.29% | -0.13% | 0.22% | 135% | 124% |

Table 2.  Results of CE9.2.1 bilateral matching merge mode without sub-CU-level search.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.70% | -3.65% | -4.05% | 153% | 215% | -1.35% | -1.09% | -1.31% | 117% | 130% |
| Class A2 | -8.25% | -6.92% | -6.97% | 153% | 259% | -2.48% | -1.90% | -1.95% | 119% | 140% |
| Class B | -4.23% | -3.27% | -3.36% | 156% | 204% | -1.41% | -1.17% | -1.07% | 118% | 131% |
| Class C | -2.86% | -2.70% | -2.76% | 158% | 195% | -1.30% | -1.04% | -1.19% | 117% | 127% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -4.76% | -3.92% | -4.06% | 155% | 214% | -1.58% | -1.27% | -1.33% | 118% | 131% |
| Class D | -3.11% | -2.72% | -3.00% | 159% | 205% | -1.53% | -1.18% | -1.20% | 117% | 133% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.54% | -0.60% | -0.89% | 175% | 122% | -0.34% | -0.80% | -0.51% | 125% | 109% |
| Class C | -0.38% | -0.88% | -1.00% | 167% | 124% | -0.46% | -0.83% | -1.13% | 125% | 115% |
| Class E | -0.43% | 0.14% | 0.20% | 173% | 158% | -0.49% | 0.56% | 0.00% | 127% | 140% |
| Overall | -0.46% | -0.51% | -0.66% | 172% | 131% | -0.41% | -0.47% | -0.59% | 126% | 118% |
| Class D | -0.44% | -1.14% | -1.24% | 163% | 135% | -0.51% | 0.00% | -0.67% | 125% | 120% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, a bilateral matching merge mode with explicitly signaled starting MV candidate is proposed. Both CU-level search and sub-CU-level search are applied. The CU-level search and sub-CU-level search share the same search window for reducing memory bandwidth requirement. The bilateral matching MV derivation is disabled for 4x4, 4x8, and 8x4 CUs for reducing the worst case bandwidth. Simulation results reportedly show -5.72% and -2.35% luma BD-rates with 172% and 68% decoding time increases in VTM-1.0 and BMS-1.0 configurations respectively in RA, and -0.43% and -0.34% luma BD-rates with 51% and 32% decoding time increases in VTM-1.0 and BMS-1.0 configurations respectively in LB. The supplementary results of disabling sub-CU-level search reportedly show -4.76% and -1.58% luma BD-rates with 114% and 31% decoding time increases in VTM-1.0 and BMS-1.0 configurations respectively in RA, and -0.46% and -0.41% luma BD-rates with 31% and 18% decoding time increases in VTM-1.0 and BMS-1.0 configurations respectively in LB.

References

[1] BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).