| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0255-v1 |
| --- | --- |

| Title: | CE9.5.2: BIO with simplified gradient calculation, adaptive BIO granularity, and applying BIO to chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chen-Yen Lai, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City, 30078, Taiwan | Tel: Email: | +886-3-5670766 {chingyeh.chen, jenny.lai, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In this contribution, three aspects of bi-prediction optical flow (BIO) modifications as planned in Core Experiment 9.5.2 (CE9.5.2) are proposed. The first aspect is simplified gradient calculation. Compared to JEM-7.0 BIO, gradients of fewer samples are used for one BIO block in this proposal. Beside, gradients are calculated directly on interpolated samples to be used for inter prediction, i.e., samples of motion compensation (MC) results, using a phase independent gradient filter, i.e., one gradient filter for all integer and fractional motion vectors (MVs), while in JEM-7.0 one additional set of interpolation filters different from MC interpolation filters and one additional set of phase dependent gradient filters are used. In addition, the number of reference picture samples accessed during MC for BIO is kept the same as that for non-BIO by applying padding, so the proposed BIO does not increase any MC memory bandwidth. Finally, the division operation is replaced with shift operations only. The second aspect is adaptive BIO granularity (i.e., adaptive BIO block size) according to picture size. BIO derives displacement vectors at BIO block level (i.e., one displacement vector for all samples in one BIO block). The BIO block size is reduced from 4x4 to 2x2 for small pictures, which only impacts Class D sequences more and has zero or negligible effects for rest sequences in the test set. The third aspect is applying BIO to chroma components. Luma displacement vectors are reused and scaled for chroma components. The results are summarized as follows.

BIO with simplified gradient calculation (aspect 1)
Overall
VTM-1.0-RA: {Y-BD-rate=-2.86%, U-BD-rate=-0.94%, V-BD-rate=-0.65%, EncT=108%, DecT=146%}
BMS-1.0-RA: {Y-BD-rate=-1.28%, U-BD-rate=-0.49%, V-BD-rate=-0.42%, EncT=104%, DecT=118%}
Class D
VTM-1.0-RA: {Y-BD-rate=-4.05%, U-BD-rate=-1.83%, V-BD-rate=-1.65%, EncT=110%, DecT=141%}
BMS-1.0-RA: {Y-BD-rate=-1.91%, U-BD-rate=-1.11%, V-BD-rate=-1.02%, EncT=104%, DecT=117%}

BIO with simplified gradient calculation (aspect 1) and adaptive BIO granularity (aspect 2)
Overall
VTM-1.0-RA: {Y-BD-rate=-2.85%, U-BD-rate=-0.92%, V-BD-rate=-0.66%, EncT=111%, DecT=156%}
BMS-1.0-RA: {Y-BD-rate=-1.28%, U-BD-rate=-0.48%, V-BD-rate=-0.41%, EncT=105%, DecT=122%}
Class D
VTM-1.0-RA: {Y-BD-rate=-4.29%, U-BD-rate=-1.86%, V-BD-rate=-1.76%, EncT=117%, DecT=165%}
BMS-1.0-RA: {Y-BD-rate=-2.05%, U-BD-rate=-1.04%, V-BD-rate=-0.95%, EncT=106%, DecT=128%}

BIO with simplified gradient calculation (aspect 1) and applying BIO to chroma components (aspect 3)
Overall
VTM-1.0-RA: {Y-BD-rate=-2.90%, U-BD-rate=-2.23%, V-BD-rate=-2.30%, EncT=110%, DecT=155%}
BMS-1.0-RA: {Y-BD-rate=-1.31%, U-BD-rate=-1.15%, V-BD-rate=-1.25%, EncT=105%, DecT=122%}
Class D
VTM-1.0-RA: {Y-BD-rate=-4.04%, U-BD-rate=-3.13%, V-BD-rate=-3.35%, EncT=112%, DecT=150%}
BMS-1.0-RA: {Y-BD-rate=-1.91%, U-BD-rate=-1.71%, V-BD-rate=-1.73%, EncT=105%, DecT=120%}

BIO with simplified gradient calculation (aspect 1), adaptive BIO granularity (aspect 2), and applying BIO to chroma components (aspect 3)
Overall
VTM-1.0-RA: {Y-BD-rate=-2.90%, U-BD-rate=-2.22%, V-BD-rate=-2.30%, EncT=113%, DecT=168%}
BMS-1.0-RA: {Y-BD-rate=-1.31%, U-BD-rate=-1.16%, V-BD-rate=-1.23%, EncT=106%, DecT=127%}
Class D
VTM-1.0-RA: {Y-BD-rate=-4.29%, U-BD-rate=-3.12%, V-BD-rate=-3.26%, EncT=121%, DecT=177%}
BMS-1.0-RA: {Y-BD-rate=-2.09%, U-BD-rate=-1.86%, V-BD-rate=-1.93%, EncT=107%, DecT=133%}

Proposed methods

BIO is proposed in VCEG-AZ07 and adopted in JEM. The concept of BIO is to utilize the assumption of optical flow and steady motion to further refine the final predictor in bi-prediction. The decoding process of the BIO in this CE test is similar to the BIO in JEM-7.0. A block-based design of BIO is applied in this proposal as the JEM-7.0 BIO. Three aspects of modifications are proposed.

Simplified gradient calculation

Assume the current coding block (CB) size is equal to PxQ, and the BIO block size is MxM (i.e., samples in one MxM block share one displacement vector derived by BIO), as shown in Figure 1. First, the required gradients for one BIO block are reduced from gradients of (2+M+2)x(2+M+2) samples in JEM-7.0 to gradients of (1+M+1)x(1+M+1) samples in this proposal. Next, the proposed BIO calculates gradients in x and y directions directly by applying one fixed gradient filter with coefficients equal to [-1, 0, 1] to interpolated samples (i.e., samples of MC results), while JEM-7.0 BIO selects “one gradient filter from one set of 6-tap gradient filters” and “one interpolation filter from one set of 6-tap interpolation filters that are different from motion compensation interpolation filters” based on the fractional position of MV and apply the selected gradient filter and interpolation filter to integer reference picture samples for calculating gradients. To calculate gradients of (1+M+1)x(1+M+1) samples with [-1, 0, 1] gradient filter applied to MC results, (2+M+2)x(2+M+2) MC samples are needed. Considering that the MV is the same for all MxM BIO blocks in one CU of size PxQ, (2+P+2)x(2+Q+2) MC samples have to be generated, and (3+2+P+2+4)x(3+2+Q+2+4) reference picture samples have to be accessed, which increases memory bandwidth because only (3+P+4)x(3+Q+4) reference picture samples are accessed when BIO is off. In order to avoid increasing memory bandwidth for MC, only the (3+P+4)x(3+Q+4) reference picture samples for MC when BIO is off are allowed to be accessed when BIO is on. Reference picture samples outside the (3+P+4)x(3+Q+4) region are replaced using by padding. As for the division operation, it is replaced with right shift operations only.

Figure 1.  Illustration of data access for BIO.

Adaptive BIO granularity

Block-based BIO is designed to share one displacement vector for all samples in one BIO block. In the proposed adaptive BIO granularity, if the number of luma samples in one picture is smaller than or equal to 921600 (i.e., 1280x720), the BIO block size is reduced from 4x4 to 2x2.

Applying BIO to chroma components

BIO is also extended to chroma components in this contribution. For chroma components, the displacement vector is reused from luma component, and the gradient filter is also modified from 3-tap filter ([-1, 0, 1]) to 2-tap filter with coefficients equal to [-1, 1] at CU boundaries, to avoid increase of MC memory bandwidth.

Experiment result

The proposed methods have been implemented on top of BMS-1.0 [1]. The common test conditions [2] are used to evaluate the coding performance. The simulations in VTM-1.0 test condition are performed by using VTM-1.0 encoder configuration settings only without MACRO changes in the software. In this test, several sub-tests are provided to evaluate the performances of each modification. Table 1 shows the simulation results of BIO with simplified gradient calculation. Table 2 shows the simulation results of BIO with simplified gradient calculation and adaptive BIO granularity. Table 3 shows the simulation results of BIO with simplified gradient calculation and applying BIO to chroma components. Table 4 is the combined result of BIO with simplified gradient calculation, adaptive BIO granularity, and applying BIO to chroma components.

Table 1.  BIO with simplified gradient calculation

Table 2.  BIO with simplified gradient calculation and adaptive BIO granularity

Table 3.  BIO with simplified gradient calculation and applying BIO to chroma components

Table 4.  BIO with simplified gradient calculation, adaptive BIO granularity, and applying BIO to chroma components

Conclusions

This contribution reports the simulation results of three BIO modifications presented in JVET-J0018, as planned in CE9.5.2. BIO with simplified gradient calculation achieves -2.86% and -1.28% luma BD-rates for VTM-1.0-RA and BMS-1.0-RA, respectively, with 8% and 4% encoding time increases and 46% and 18% decoding time increases. Based on BIO with simplified gradient calculation, adaptive BIO granularity is added, and it achieves additional 0.24% and 0.14% luma BD-rate savings for Class D for VTM-1.0-RA and BMS-1.0-RA and no BD-rate impact for rest sequences. Applying BIO to chroma components on top of the simplified gradient calculation in BIO provides additional 1.47% and 0.75% chroma BD-rate reductions for VTM-1.0-RA and BMS-1.0-RA, respectively.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

[1] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei (MediaTek), “Description of SDR video coding technology proposal by MediaTek”, JVET-J0018, JVET 10th Meeting: San Diego, US, 10–20 April, 2018.

[2] S. Esenlik, Y. W. Chen, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-J0029, JVET 10th Meeting: San Diego, US, 10–20 April, 2018.

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-J1010.