| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0256-v2 |
| --- | --- |

| Title: | CE9-related: MV prediction modification for decoder-side MV derivation tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Decoder-side motion vector derivation (DMVD) tools can cause serious degradation of processing throughput in hardware decoders. In DMVD, an initial motion vector (MV) is first obtained with simple processes without any need to access reference picture samples, and then the initial MV is refined within a search range centered at the initial MV using refinement processes that need to access reference picture samples to obtain the final MV. In this contribution, a MV prediction modification is proposed for DMVD as follows. When reconstructing MV(s) of a non-DMVD current block or initial MV(s) of a DMVD current block, “MVs from non-DMVD previous blocks” and “initial MVs from DMVD previous blocks” (instead of final MVs from DMVD previous blocks) can be referenced. With the proposed method, at parsing stage of a pipelined hardware decoder, all reference picture samples needed to perform DMVD can be pre-fetched using initial MVs of DMVD blocks. To reduce coding efficiency degradation, final MV(s) of a DMVD previous block can be referenced when the DMVD previous block is above the current coding tree unit (CTU) row or in a previously decoded picture. Final MV(s) of DMVD blocks are used in overlapped block motion compensation (OBMC) and deblocking processes. Simulation results show that decoder-side motion vector refinement (DMVR) and OBMC with the proposed method can achieve -2.59% luma BD-rate for VTM-1.0-RA, and 72% of DMVR and OBMC coding gain is preserved by applying the proposed method. It is also shown that DMVR, OBMC, and Core Experiment 9.2.1 (CE9.2.1) bilateral matching merge mode with the proposed method can achieve -5.81% luma BD-rate for VTM-1.0-RA, and 83% of DMVR, OBMC, and CE9.2.1 bilateral matching merge mode coding gain is preserved by applying the proposed method.

# Problem definition

In a hardware video decoder, reference pictures are usually stored in external off-chip DRAM, so motion compensation (MC) takes a lot of clock cycles and long latency to acquire reference picture samples. In order to solve the long MC latency issue, pre-fetching reference picture samples is applied in the pipelined hardware decoder architecture. MVs of one pipelined data unit (DU; e.g., one 64x64 luma block and its two 32x32 chroma blocks for VTM-1.0 decoder), DUT, are reconstructed at parsing stage after parsing related MV information, and the reference picture samples needed to decode DUT are pre-fetched from DRAM to a local on-chip memory before DUT enters the next pipeline stage, e.g., reconstruction stage. When the parsing stage processes DUT, the reconstruction stage processes the previous data unit, DUT-1; when the parsing stage processes the next data unit, DUT+1, the reconstruction stage processes DUT; …, and so on. In this way, reconstruction processes (e.g., inverse quantization, inverse transform, add residual to prediction…) can be finished quickly without waiting for fetching reference picture samples needed to decode DUT because of the pre-fetching at parsing stage, which can greatly improve hardware decoder processing throughput. However, when any DMVD tool (e.g., DMVR, CE9.2.1 bilateral matching merge mode), is applied, reconstructing MVs and fetching reference picture samples from DRAM have to be done in the same pipeline stage, which fails to apply the mechanism of pre-fetching reference picture samples. Consequently the hardware decoder processing throughput is significantly decreased.

# Proposed method

In this contribution, a MV prediction modification is proposed for DMVD tools to allow pre-fetching reference picture samples. In a DMVD tool, an initial MV is first obtained with simple processes without any need to access reference picture samples, and then the initial MV is refined within a search range centered at the initial MV using refinement processes that need to access reference picture samples to obtain the final MV. It is proposed to use initial MV(s) of each DMVD block to predict MVs of non-DMVD future blocks and initial MVs of DMVD future blocks. In other words, when reconstructing MV(s) of a non-DMVD current block or initial MV(s) of a DMVD current block, “MVs from non-DMVD previous blocks” and “initial MVs from DMVD previous blocks” (instead of final MVs from DMVD previous blocks) can be referenced. With the proposed method, at parsing stage, all reference picture samples needed to perform DMVD can be pre-fetched using initial MVs of DMVD blocks.

To further improve coding efficiency, an exceptional condition for the proposed MV prediction modification is further proposed: during MV prediction, if a DMVD previous block is above the current CTU row or in a previously decoded picture, final MV(s) of the DMVD previous block can be referenced.

As for applying deblocking and OBMC to DMVD blocks, the final MVs are used.

# Simulation results

The proposed method is implemented on BMS-1.0 [1]. The simulations were performed following the JVET common test conditions [2]. The anchors are the default VTM-1.0/BMS-1.0 configuration without DMVR (set the “DMVR : 0” in BMS condition).

## Applying the proposed method to DMVR

The results of enabling DMVR are shown in Table 1. DMVR is enabled by setting “DMVR : 1” in configure files. It is shown that -2.65% and -1.44% luma BD-rates are achieved for VTM-1.0-RA and BMS-1.0-RA, respectively. The results of enabling DMVR with the proposed method are shown in Table 2. It is shown that -1.58% and -0.95% luma BD-rates for VTM-1.0-RA and BMS-1.0-RA, respectively. 60% of DMVR coding gain is preserved by the proposed method for solving the pipeline issue.

Table 1.  Results of enabling DMVR on VTM configuration

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.11% | -2.37% | -2.70% | 106% | 133% | -1.84% | -1.47% | -1.80% | 101% | 113% |
| Class A2 | -4.82% | -4.72% | -4.75% | 105% | 141% | -2.37% | -2.48% | -2.55% | 101% | 110% |
| Class B | -2.08% | -2.01% | -2.11% | 109% | 143% | -1.02% | -1.14% | -1.18% | 101% | 115% |
| Class C | -1.38% | -1.69% | -1.80% | 109% | 129% | -0.98% | -1.32% | -1.28% | 100% | 107% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.65% | -2.54% | -2.67% | 108% | 137% | -1.44% | -1.52% | -1.60% | 101% | 111% |
| Class D | -1.25% | -1.56% | -1.77% | 114% | 135% | -0.95% | -1.22% | -1.37% | 99% | 105% |
| Class F (optional) | -0.52% | -0.67% | -0.71% | 107% | 119% | -0.20% | -0.34% | -0.37% | 100% | 104% |

Table 2.  Results of enabling DMVR with the proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.84% | -1.47% | -1.61% | 109% | 125% | -1.19% | -0.96% | -1.22% | 104% | 113% |
| Class A2 | -2.79% | -2.78% | -2.77% | 106% | 133% | -1.54% | -1.67% | -1.65% | 105% | 109% |
| Class B | -1.23% | -1.29% | -1.28% | 111% | 127% | -0.68% | -0.83% | -0.80% | 105% | 111% |
| Class C | -0.91% | -1.13% | -1.14% | 112% | 132% | -0.68% | -0.86% | -0.90% | 103% | 114% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.58% | -1.58% | -1.61% | 110% | 129% | -0.95% | -1.03% | -1.08% | 104% | 112% |
| Class D | -0.85% | -1.02% | -1.16% | 116% | 136% | -0.65% | -0.88% | -0.90% | 104% | 115% |
| Class F (optional) | -0.30% | -0.40% | -0.40% | 111% | 126% | -0.17% | -0.24% | -0.27% | 103% | 112% |

## Applying the proposed method to DMVR and OBMC

The results of enabling DMVR and OBMC are shown in Table 3. The OBMC is enabled by setting “OBMC : 1” in the configure file. It is shown that -3.60% and -2.65% luma BD-rates are achieved for VTM-1.0-RA and BMS-1.0-RA, respectively. The results of enabling DMVR and OBMC with the proposed method are shown in Table 4. It is shown that -2.59% and -2.20% luma BD-rates are achieved for VTM-1.0-RA and BMS-1.0-RA, respectively. 72% of DMVR and OBMC coding gain is preserved by the proposed method for solving the pipeline issue.

Table 3.  Results of the DMVR and OBMC

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.49% | -3.78% | -4.47% | 115% | 157% | -2.30% | -2.55% | -3.04% | 105% | 130% |
| Class A2 | -5.71% | -6.44% | -6.28% | 117% | 177% | -3.57% | -4.55% | -5.19% | 109% | 152% |
| Class B | -3.00% | -3.83% | -3.88% | 122% | 168% | -2.23% | -3.46% | -3.31% | 109% | 152% |
| Class C | -2.84% | -4.06% | -4.25% | 128% | 162% | -2.75% | -4.09% | -4.13% | 108% | 145% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.60% | -4.40% | -4.58% | 121% | 166% | -2.65% | -3.67% | -3.85% | 108% | 146% |
| Class D | -2.59% | -3.44% | -3.63% | 134% | 167% | -2.79% | -4.10% | -4.13% | 111% | 153% |
| Class F (optional) | -0.33% | -1.23% | -1.32% | 121% | 137% | -0.35% | -1.43% | -1.41% | 106% | 129% |

Table 4.  Results of the DMVR and OBMC with the proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.24% | -2.97% | -3.49% | 116% | 162% | -1.66% | -2.06% | -2.45% | 107% | 146% |
| Class A2 | -3.78% | -4.69% | -4.51% | 118% | 179% | -2.80% | -3.76% | -4.42% | 109% | 174% |
| Class B | -2.22% | -3.23% | -3.16% | 125% | 171% | -1.92% | -3.23% | -3.02% | 110% | 165% |
| Class C | -2.42% | -3.60% | -3.77% | 129% | 162% | -2.50% | -3.74% | -3.76% | 111% | 151% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.59% | -3.57% | -3.66% | 123% | 168% | -2.20% | -3.24% | -3.38% | 109% | 159% |
| Class D | -2.25% | -3.00% | -3.14% | 136% | 170% | -2.58% | -3.72% | -3.77% | 111% | 152% |
| Class F (optional) | -0.13% | -0.97% | -1.09% | 121% | 140% | -0.35% | -1.39% | -1.35% | 108% | 138% |

## Applying the proposed method to DMVR, OBMC, and CE9.2.1 bilateral matching merge mode

The results of enabling DMVR, OBMC, and CE9.2.1 bilateral matching merge mode are shown in Table 5. The proposed method is integrated on CE9.2.1 software [3]. It is shown that -7.03% and -5.00% luma BD-rates are achieved for VTM-1.0-RA and BMS-1.0-RA, respectively. The results of enabling DMVR and OBMC with the proposed method are shown in Table 6. It is shown that -5.81% and -4.26% luma BD-rates are achieved for VTM-1.0-RA and BMS-1.0-RA, respectively. 83% of DMVR, OBMC, and CE9.2.1 bilateral matching merge mode coding gain is preserved by the proposed method for solving the pipeline issue.

Table 5.  Results of enabling DMVR, OBMC, and CE9.2.1 bilateral matching merge mode

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -5.99% | -5.85% | -6.71% | 190% | 283% | -4.01% | -3.96% | -4.77% | 130% | 183% |
| Class A2 | -11.37% | -10.42% | -10.52% | 198% | 399% | -7.39% | -7.47% | -8.18% | 141% | 229% |
| Class B | -6.34% | -6.17% | -6.20% | 217% | 329% | -4.40% | -5.13% | -5.07% | 141% | 218% |
| Class C | -5.42% | -6.11% | -6.47% | 227% | 328% | -4.68% | -5.91% | -5.96% | 140% | 233% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -7.03% | -6.94% | -7.24% | 210% | 331% | -5.00% | -5.57% | -5.87% | 138% | 216% |
| Class D | -5.83% | -5.92% | -6.02% | 242% | 366% | -5.29% | -5.91% | -6.13% | 141% | 256% |
| Class F (optional) | -1.45% | -2.48% | -2.45% | 200% | 201% | -0.93% | -2.17% | -2.08% | 131% | 179% |

Table 6.  Results of enabling DMVR, OBMC, and CE9.2.1 bilateral matching merge mode with the proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.73% | -4.92% | -5.58% | 192% | 353% | -3.30% | -3.47% | -4.07% | 130% | 188% |
| Class A2 | -9.32% | -8.76% | -8.72% | 205% | 477% | -6.25% | -6.51% | -7.22% | 142% | 235% |
| Class B | -5.28% | -5.33% | -5.33% | 218% | 386% | -3.76% | -4.62% | -4.46% | 142% | 224% |
| Class C | -4.67% | -5.41% | -5.75% | 235% | 344% | -4.11% | -5.33% | -5.42% | 142% | 239% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -5.81% | -5.96% | -6.17% | 214% | 384% | -4.26% | -4.96% | -5.19% | 139% | 222% |
| Class D | -5.06% | -5.27% | -5.32% | 244% | 372% | -4.74% | -5.27% | -5.56% | 145% | 264% |
| Class F (optional) | -0.97% | -1.98% | -1.96% | 207% | 212% | -0.72% | -1.96% | -1.88% | 129% | 176% |

# Conclusions

To solve the hardware pipeline issue caused by DMVD tools, a MV prediction modification is proposed for DMVD. With the proposed method, pre-fetching reference picture samples can be applied as conventional hardware decoders without DMVD. In addition, the proposed method can preserve a significant portion of DMVD coding gain.

# References

[1] BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

[3] https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE9/CE9-2.1/

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).