| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0259-v1 |
| --- | --- |

| Title: | CE10-related: OBMC bandwidth reduction and line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents proposed methods to reduce overlapped block motion compensation (OBMC) memory bandwidth and line buffer. In order to remove the additional memory bandwidth required by OBMC, padding is applied to additional reference picture samples at the right-most w’ columns and bottom-most h’ rows. They are not fetched and replaced with generated samples by replicating the right-most column and the bottom-most row of the original fetched reference picture samples for fractional sample accuracy motion compensation (MC), where w’ and h’ are the width and height of OBMC region, respectively. For blocks coded with sub-block mode, the same padding method can be applied to reduce the required bandwidth. As for line buffer usage, the number of blended lines for above block boundary is reduced from 4 to 2, if current block is at coding tree unit (CTU) row boundary. Applying padding for OBMC at CU boundary suffers x.xx%, x.xx%, x.xx%, and x.xx% BD-rates for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively. Applying padding for OBMC at CU boundary and reducing the number of blended lines at CTU row boundary induce x.xx%, x.xx%, x.xx%, and x.xx% BD-rates for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively.

# Introduction

In OBMC, for an inter block, its boundary will be blended with predictors generated by neighboring motion vectors (MVs) as shown in Figure 1. To reduce memory bandwidth, OBMC is done by doing the above 4-row MC and left 4-column MC with the neighboring blocks, which is illustrated in Figure 2.

Figure 1.  Illustration of OBMC

Figure 2.  MC of current block

For each block, if 8-tap interpolation filters are applied, it requires to fetch (3+W+4)*(3+H+4) samples from memory to perform fractional sample accuracy MC. If OBMC is applied, the number of samples need to be fetched from memory will be increased to (3+W+w’+4)*(3+H+h’+4), as shown in Figure 3 and Figure 4, where w’ and h’ are the width and height of OBMC region respectively. Therefore, additional memory bandwidth will be introduced if OBMC is applied. Moreover, the additional bandwidth is related to the sub-block size of OBMC. If current block is coded with sub-block mode (e.g., alternative temporal motion vector prediction (ATMVP) mode, affine mode), for each sub-block, the samples required for MC will be increased from (3+W_subblk +4)* (3+H_subblk +4) to (3+W_subblk +2*w’ +4)* (3+H_subblk +2*h’ +4), as shown in Figure 5.

Figure 3.  MC of current block without OBMC

Figure 4.  MC of current block with OBMC

Figure 5.  MC of current sub-block with OBMC

Considering the usage of line buffers, there will be additional line buffers required to store the MC results for neighboring blocks if the above 4-row MC and left 4-column MC are done with the neighboring blocks as shown in Figure 2.

# Proposed methods

## OBMC bandwidth reduction at block boundaries

As introduced above, to reduce the additional bandwidth introduced by block boundary OBMC, it is proposed to pad the reference samples with the size (3+W+4)*(3+H+4) to (3+W+w’+4)*(3+H+h’+4) at right and bottom boundaries, where w’ and h’ are the width and height of OBMC region respectively. In Fig. 6, the w’ columns in orange region, which are the additional samples required to derive OBMC_block_R, are generated by replicating the samples in the right-most column of the blue region. The h’ rows in green region, which are the additional samples required to derive OBMC_block_B, are generated by replicating the samples in the bottom row of the blue region. In this way, there is no need to fetch additional samples for block boundary OBMC for fractional sample accuracy MC of current block.

Figure 6. Reference samples with padding for block boundary OBMC.

## OBMC bandwidth reduction at sub-block boundaries

As introduced above, to reduce the additional bandwidth introduced by sub-block OBMC, it is proposed to pad the reference picture samples of size (3+W_subblk+4) * (3+H_subblk+4) to (3+W_subblk+2*w’+4) * (3+H_subblk+2*h’+4) at above, right, bottom, and left boundaries, where w’ and h’ are the width and height of OBMC region respectively. In Figure 7, the w’ columns in orange region, which are the additional samples required to derive OBMC_block_R, are generated by replicating the samples in the right-most column of the blue region. The w’ columns in red region, which are the additional samples required to derive OBMC_block_L, are generated by replicating the samples in the left-most column of the blue region. The h’ rows in purple region, which are the additional samples required to derive OBMC_block_A, are generated by replicating the samples in the top row of the blue region. The h’ rows in green region, which are the additional samples required to derive OBMC_block_B, are generated by replicating the samples in the bottom row of the blue region. In this way, there is no need to fetch additional samples for sub-block OBMC for fractional sample accuracy MC of current block.

Figure 7. Reference samples with padding for sub-block OBMC.

## OBMC line buffer reduction

To reduce the line buffers across CTUs, if current block is at CTU row boundary, then the number of blended lines for OBMC at above block boundary is reduced to 2.

# Experiment results

The proposed modifications have been implemented on top of BMS-1.0 [1]. The common test conditions [2] are used for evaluation. Note that the simulations in VTM-1.0 test condition are performed by using VTM-1.0 encoder configuration settings without MACRO changes in the software. Table 1 shows the simulation results of OBMC bandwidth reduction at CU boundaries. Table 2 shows the simulation results of OBMC bandwidth reduction at CU boundaries and OBMC buffer reduction. Table 3 shows the simulation results of OBMC bandwidth reduction at CU boundaries and sub-block boundaries and OBMC buffer reduction.

Table 1.  OBMC bandwidth reduction at CU boundaries. (to be updated)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -1.59% | -1.78% | 102% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | #NUM! |
| Class A2 | -1.00% | -1.61% | -1.28% | 100% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | #NUM! |
| Class B | -0.95% | -1.46% | -1.40% | 105% | #NUM! | -1.20% | -1.97% | -2.88% | 56% | #NUM! |
| Class C | -1.47% | -1.94% | -2.08% | 109% | 133% | -1.98% | -6.32% | -6.06% | 7% | 62% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.99% | -1.65% | -1.63% | 104% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | #NUM! |
| Class D | -1.30% | -1.40% | -1.19% | 110% | 133% | -1.79% | -2.26% | -2.33% | 112% | 160% |
| Class F (optional) | 0.10% | -0.56% | -0.61% | 106% | 125% | #VALUE! | #VALUE! | #VALUE! | 6% | 53% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 1% | 21% | #VALUE! | #VALUE! | #VALUE! | 0% | 1% |
| Class C | -1.54% | -1.42% | -1.31% | 104% | 127% | -2.68% | -6.65% | -6.88% | 26% | 94% |
| Class E | -1.23% | -0.86% | -0.71% | 104% | 132% | -1.87% | -2.05% | -1.27% | 43% | 112% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 15% | 60% | #VALUE! | #VALUE! | #VALUE! | 0% | 17% |
| Class D | -1.71% | -1.01% | -0.32% | 104% | 118% | -2.47% | -1.43% | -1.87% | 109% | 166% |
| Class F (optional) | 0.34% | -0.15% | -0.11% | 108% | 132% | #VALUE! | #VALUE! | #VALUE! | 6% | 59% |

Table 2.  OBMC bandwidth reduction at CU boundaries and OBMC buffer reduction (to be updated)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | -3.99% | -4.83% | 36% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| Class A2 | -0.99% | -1.48% | -1.35% | 100% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | #NUM! |
| Class B | -0.92% | -1.49% | -1.39% | 105% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 8% | #NUM! |
| Class C | -1.45% | -1.98% | -2.09% | 107% | 130% | #VALUE! | #VALUE! | #VALUE! | 2% | 41% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.85% | -2.12% | -2.26% | 84% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | #NUM! |
| Class D | -1.29% | -1.40% | -1.27% | 110% | 133% | -1.78% | -2.33% | -2.25% | 109% | 154% |
| Class F (optional) | 0.12% | -0.56% | -0.56% | 106% | 127% | #VALUE! | #VALUE! | #VALUE! | 1% | 35% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 11% | #VALUE! | #VALUE! | #VALUE! | 0% | 1% |
| Class C | -1.54% | -1.39% | -1.32% | 103% | 127% | #VALUE! | #VALUE! | #VALUE! | 3% | 53% |
| Class E | -1.18% | -1.00% | -0.79% | 106% | 137% | -2.81% | -5.26% | 1.16% | 7% | 59% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 8% | 47% | #VALUE! | #VALUE! | #VALUE! | 0% | 11% |
| Class D | -1.74% | -0.74% | 0.07% | 104% | 119% | -2.48% | -2.02% | -2.06% | 110% | 168% |
| Class F (optional) | 0.19% | -0.54% | 0.21% | 108% | 134% | #VALUE! | #VALUE! | #VALUE! | 2% | 38% |

Table 3. OBMC bandwidth reduction at CU boundaries and sub-block boundaries and OBMC buffer reduction (to be updated)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.51% | -5.26% | -5.00% | 13% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| Class A2 | -0.27% | -2.93% | -2.28% | 12% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| Class B | -0.92% | -1.49% | -1.39% | 105% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 1% | #NUM! |
| Class C | -1.45% | -1.98% | -2.09% | 109% | 136% | #VALUE! | #VALUE! | #VALUE! | 0% | 18% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.64% | -2.66% | -2.48% | 45% | #NUM! | #VALUE! | #VALUE! | #VALUE! | 0% | #NUM! |
| Class D | -1.29% | -1.40% | -1.27% | 112% | 137% | -1.75% | -2.22% | -2.08% | 108% | 155% |
| Class F (optional) | 0.02% | -1.11% | -0.94% | 53% | 98% | #VALUE! | #VALUE! | #VALUE! | 0% | 20% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 5% | #VALUE! | #VALUE! | #VALUE! | 0% | 1% |
| Class C | -1.54% | -1.39% | -1.32% | 102% | 126% | #VALUE! | #VALUE! | #VALUE! | 0% | 24% |
| Class E | -1.18% | -1.00% | -0.79% | 102% | 130% | #VALUE! | #VALUE! | #VALUE! | 1% | 33% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 3% | 34% | #VALUE! | #VALUE! | #VALUE! | 0% | 7% |
| Class D | -1.74% | -0.74% | 0.07% | 102% | 115% | -2.42% | -1.35% | -1.53% | 110% | 177% |
| Class F (optional) | 0.12% | -1.70% | 0.26% | 54% | 103% | #VALUE! | #VALUE! | #VALUE! | 0% | 21% |

# Conclusions

This contribution presents proposed methods to reduce OBMC memory bandwidth and line buffer. In order to remove the additional memory bandwidth required by OBMC, the additional reference picture samples at the right-most w’ columns and bottom h’ rows are not fetched and replaced with padding results. This method can be extended to CU coded with sub-block mode. The number of blended lines for above block boundary is reduced from 4 to 2, when current block is at CTU row boundary, in order to reduce OBMC line buffer. Applying padding for OBMC at CU boundary suffers x.xx%, x.xx%, x.xx%, and x.xx% BD-rates for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively. Applying padding for OBMC at CU boundary and reducing the number of blended lines at CTU row boundary induce x.xx%, x.xx%, x.xx%, and x.xx% BD-rates for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively. Apply padding for OBMC at CU boundary and subblock boundary and reducing the number of blended lines at CTU row boundary introduce x.xx%, x.xx%, x.xx%, and x.xx% BD-rates for VTM-1.0-RA, VTM-1.0-LB, BMS-1.0-RA, and BMS-1.0-LB, respectively.

# References

[1] BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).