| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0260-v1 |
| --- | --- |

| Title: | Flexible Tiles |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson Jack Enhorn | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com jack.enhorn@ericsson.com |
| Source: | Ericsson |  |  |  |

_____________________________

# Abstract

This contribution proposes to include tiles into VVC. The tile sizes are proposed to be signaled individually, either by copying the tile size from the previous tile size in decoding order or by one tile width and one tile height code word. This is reported to enable tile partition structures that are not restricted by the HEVC rule that tile boundaries must span across the entire picture.

The following HEVC tile properties are included in the proposal:

- That CABAC is initialized for each tile, flushed after each tile and that the tiles are byte aligned

- That tiles break predictions as in HEVC

- That the tile structure is specified in the PPS

- That tile pointers are mandatory for all tiles except the first in a slice

- That either all CTUs in a tile belongs to the same slice or all CTUs in a slice belong to the same tile

- That the initial QP of a tile is set as in HEVC

- That tiles are CTU aligned

The following tile properties are also proposed:

- That the number of tiles in a picture is signaled in the PPS

- That the tile sizes are specified in subtile units to reduce the signaling bit cost

- That tiles are specified individually in subtile raster scan order by their individual tile size either by copying the tile size from the previous tile size in decoding order or by one tile width and one tile height UVLC code word each

# Introduction

Tiles were first proposed for HEVC in JCTVC-D227 [1] and developed through a number of input contributions such as JCTVC-D378 [2], JCTVC-E408 [3] and JCTVC-F335 [4]. Using tiles, pictures in HEVC can be partitioned into rows and columns of samples where a tile is an intersection of a row and a column. The tiles in HEVC are always aligned with CTU boundaries. Figure 1 shows an example of a tile partitioning using 4 tile rows and 5 tile columns resulting in a total of 20 tiles for the picture.

Figure 1 – An example Tile partition

The tile structure in HEVC is signaled in the picture parameter set (PPS) by specifying the heights of rows and the widths of columns. Individual rows and columns can have different sizes, but the partitioning always span across the entire picture, from left to right or from top to bottom.

Table 1 – HEVC Tile syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| tiles_enabled_flag | u(1) |
| ... |  |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| ... |  |

The PPS syntax used for specifying the tile structure in HEVC is listed in Table 1. First there is a flag specifying whether tiles are used or not. If the flag is set, the number of tiles columns and rows are specified. The uniform_spacing_flag is a flag specifying whether the column widths and row heights are explicitly signaled or whether a pre-defined method to space the tile borders evenly should be used. If explicit signaling is indicated, the column widths are signaled one-by-one followed by the row heights. Those are signaled in CTU units. Finally, the loop_filter_across_tiles_enabled_flag flag specifies whether in-loop filters across tile boundaries are turned on or off for all tile boundaries in the picture.

Similar to slices, there is no decoding dependency between tiles of the same picture. This includes intra prediction, context selection and motion vector prediction. One exception is that in-loop filtering dependencies are generally allowed between tiles. Those dependencies can however be disabled by the loop_filter_across_tiles_enabled_flag flag. In contrast to slices, tiles do not require as much header data. The header overhead per tile consist of the signaling of bit-stream offsets which are present in the slice header. These bit-stream offsets indicate the starting points of each tile in a picture. A decoder may decode the starting points to split the coded picture into coded tiles and distribute them for parallel decoding. In HEVC, the inclusion of bit-stream offsets in the slice header is mandatory when tiles are enabled. The combination of tiles and slices is restricted in HEVC. Either all CTUs in a tile belongs to the same slice or all CTUs in a slice belong to the same tile.

The main use for HEVC tiles is to partition pictures into independent segments with minimal compression efficiency losses. One particular use-case for tiles is to partition pictures for viewport aware 360-degree streaming. The source video is then partitioned and encoded using motion constraint tile sets (MCTS) that can be decoded independently of neighboring tile sets. The decoder decodes a subset of the tile sets depending on the viewport direction. Some implementations [6][7], uses tiles in different spatial resolutions such that the picture partition structure consists of a mix of tiles and slices.

# Problem statement

Tiles in its current form in HEVC are restricted by the use of rows and columns. This means that all tile boundaries span across the entire picture, either from left to right or from top to bottom. This limits the flexibility of tiles. For example, the left tile partition in Figure 2 is not supported in HEVC.

Figure 2 – An example flexible Tile partition

In order to virtually support such a tile structure, one may specify tile partition shown to the right in Figure 2. However, that tile structure contains a lot more tile boundaries which harms compression efficiency and may cause unnecessary tiling artifacts. Also, if the tiles are to be completely spatially independent, HEVC only allows for either enabling or disabling in-loop filtering dependencies for all tile boundaries of a picture. Therefore, a lot of unnecessary in-loop filtering restrictions may occur.

# Proposal

We propose flexible tiles for VVC where rectangular shaped tiles of varying sizes are supported. For example, the example shown to the left in Figure 2 can be supported by the proposal.

We propose to inherit the following aspects from the HEVC tiles:

- That CABAC is initialized for each tile, flushed after each tile and that the tiles are byte aligned

- That tiles break predictions as in HEVC

- That the tile structure is specified in the PPS

- That tile pointers are mandatory for all tiles except the first in a slice

- That either all CTUs in a tile belongs to the same slice or all CTUs in a slice belong to the same tile

- That the initial QP of a tile is set as in HEVC

- That tiles are CTU aligned

- That the minimum tile width is set equal to 256 and the minimum tile height is set equal to 64

We propose the following additions and modifications:

- That the number of tiles in a picture is signaled in the PPS

- That the tile sizes are specified in subtile units to reduce the signaling bit cost

- That tiles are specified individually in subtile raster scan order by their individual tile size either by copying the tile size from the previous tile size in decoding order or by one tile width and one tile height UVLC code word each

# Proposed syntax and semantics and level constraints

Table 2 – Proposed Tile syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| tiles_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| number_of_tiles_in_picture_minus2 | ue(v) |
| subtile_width_minus1 | ue(v) |
| subtile_height_minus1 | ue(v) |
| for ( i=0; i < number_of_tiles_in_picture_minus2+2 ; i++ ) { |  |
| if ( i > 0 ) |  |
| use_previous_tile_size_flag | u(1) |
| if ( use_previous_tile_size_flag == 0) { |  |
| tile_width_minus1[ i ] | ue(v) |
| tile_height_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| ... |  |
| } |  |

tiles_enabled_flag equal to 1 specifies that there is more than one tile in each picture referring to the PPS. tiles_enabled_flag equal to 0 specifies that there is only one tile in each picture referring to the PPS.

It is a requirement of bitstream conformance that the value of tiles_enabled_flag shall be the same for all PPSs that are activated within a CVS.

If tiles_enabled_flag is equal to 1, the value of the variable tile_id and the value of the two-dimentional array TileId are specified as follows:

tile_id = 0

for( y= 0; y < PicHeightInCtbsY; y++ )

for( x = 0; x < PicWidthInCtbsY; x++ )

TileId[ x ][ y ] = “no tile”

number_of_tiles_in_picture_minus2 plus 2 specifies the number of tiles in a picture. number_of_tiles_in_picture_minus2 shall be in the range of 0 to PicSizeInCtbsY − 2, inclusive.

subtile_width_minus1 plus 1 specifies the width of the subtile unit in coding tree units. subtile_width_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive.

subtile_height_minus1 plus 1 specifies the height of the subtile unit in coding tree units. subtile_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive.

use_previous_tile_size_flag equal to 1 specifies that the size of the current tile is equal to the size of the previous tile. use_previous_tile_size_flag equal to 0 specifies that the size of the current tile is not equal to the size of the previous tile. When not present, the value of use_previous_tile_size_flag is inferred to be equal to 0.

When use_previous_tile_size_flag is equal to 1, the value of the i’th element of the arrays tile_height_minus1 and tile_width_minus1 are derived as follows:

tile_height_minus1[ i ]  = tile_height_minus1[ i - 1 ]

tile_width_minus1[ i ] = tile_width_minus1[ i - 1 ]

tile_width_minus1[ i ] plus 1 specifies the width of the i-th tile in tile units.

tile_height_minus1[ i ] plus 1 specifies the height of the i-th tile in tile units.

The value of the variable tile_id and the value of the two-dimentional array CTU_tile_assigment are derived as follows:

for( tile_coordinate_y = 0; tile_coordinate_y < PicHeightInCtbsY; tile_coordinate_y++ )

for( tile_coordinate_x = 0; tile_coordinate_x < PicWidthInCtbsY; tile_coordinate_x++ )

if( TileId[ tile_coordinate_x ][ tile_coordinate_y ] == “no tile” )

break

for( ctu_y = 0 ; ctu_y  < ( tile_height_minus1[ i ] + 1 ) * ( subtile_height_minus1 + 1 ) && (tile_coordinate_y + ctu_y ) < PicHeightInCtbsY ; ctu_y++ )

for( ctu_x = 0 ; ctu_x < ( tile_width_minus1[ i ] + 1 ) * ( subtile_width_minus1 + 1 ) && (tile_coordinate_x + ctu_x ) < PicWidthInCtbsY ; ctu_x++ )

TileId[ tile_coordinate_x + ctu_x ][ tile_coordinate_y + ctu_y ] = tile_id

tile_id++

It is a requirement of bitstream conformance that the break statement is reached for each value of i.

It is a requirement of bitstream conformance that the value of TileId[ tile_coordinate_x + ctu_x ][ tile_coordinate_y + ctu_y ] shall be equal to “no tile” prior to the assignment to the value tile_id.

It is a requirement of bitstream conformance that after all assignments of TileId, TileId[ x ][ y ] shall not be equal to “no tile” for x ranging from 0 to PicWidthInCtbsY-1 inclusive and y ranging from 0 to PicHeightInCtbsY-1 inclusive.

NOTE X – The derivation of the variable tile_id and the value of the two-dimentional array CTU_tile_assigment as specified in the semantics of tile_height_minus1[ i ] are exectuted for each value of i. This means that the derivation is done after each loop of the variable i, including when use_previous tile_size_flag is not present, when use_previous tile_size_flag is equal to 0 and when use_previous tile_size_flag is equal to 1. Likewise, the bitstream conformance condition that the value of TileId[ tile_coordinate_x + ctu_x ][ tile_coordinate_y + ctu_y ] shall be equal to “no tile” prior to the assignment to the value tile_id shall be true for any assignment done in any loop.

Table 3 – Proposed level constraints

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols | Max # of tiles MaxTiles |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 | 4 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 | 6 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 | 25 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 | 25 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 | 110 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 | 110 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 | 110 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 | 440 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 | 440 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 | 440 |

# References

[1] A. Fuldseth, “Replacing slices with tiles for high level parallelism,” JCT-VC Document JCTVC-D227, Daegu, January 2011.

[2] M. Horowitz and S. Xu, “Generalized slices,” JCT-VC Document JCTVC-D378, Daegu, January 2011.

[3] A. Fuldseth, M. Horowitz, S. Xu, and M. Zhou, “Tiles,” JCT-VC Document JCTVC-E408 r1, Geneva, March 2011.

[4] A. Fuldseth, M. Horowitz, S. Xu, A. Segall and M. Zhou, “Tiles,” JCT-VC Document JCTVC-F335, Torino, July 2011.

[5] K. Misra, A. Segall, M. Horowitz, Shilin Xu, A. Fuldseth, Minhua Zhou, "An Overview of Tiles in HEVC", Selected Topics in Signal Processing IEEE Journal of, vol. 7, no. 6, Dec 2013, pp. 969-977.

[6] R. Skupin, Y. Sanchez, D. Podborski, C. Hellge, and T. Schierl, "HEVC tile based streaming to head mounted displays," IEEE Annual Consumer Communications & Networking Conference. IEEE, 2017, pp. 613-615.

[7] A. Zare, A. Aminlou, M. Hannuksela, "6K Effective Resolution with 4K HEVC Decoding Capability for OMAF-compliant 360° Video Streaming", 23rd Packet Video Workshop, 2018, pp. 72-77.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).