| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0262 |
| --- | --- |

| Title: | Cross-check of JVET-K0196: CE3-related: Non-linear weighted intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Jianle Chen Russian Research Center, 1/7, Altufievskoe shosse, Moscow | Tel: Email: | +7-925-008-1847 sergey.ikonin@huawei.com chernyak.roman@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution is a cross-check report of JVET-K0196 on CE3-related: Non-linear weighted intra prediction. Tool’s description and simulation results provided by the proponent were verified. No mismatches compared to the reported results were found beside small deviations in encoding and decoding times. Source code modifications are consistent with the description.

# Test results

Simulations were performed according to JVET CTC [1] using software provided by proponent.

Table 1. Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.51% | -2.33% | -2.05% | 278% | 123% |
| Class A2 | -2.68% | -1.51% | -1.74% | 278% | 129% |
| Class B | -3.11% | -1.22% | -0.88% | 277% | 134% |
| Class C | -2.66% | -1.24% | -1.50% | 269% | 133% |
| Class E | -5.57% | -2.42% | -2.77% | 274% | 138% |
| Overall | -3.42% | -1.66% | -1.67% | 275% | 132% |
| Class D | -2.82% | -1.22% | -1.24% | 265% | 137% |
| Class F (optional) | -1.92% | -1.25% | -1.45% | 272% | 122% |

Table 2. Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.75% | -0.34% | -0.35% | 127% | 105% |
| Class A2 | -1.43% | 1.02% | 0.77% | 122% | 105% |
| Class B | -1.83% | 1.05% | 1.12% | 126% | 107% |
| Class C | -1.61% | 0.04% | -0.15% | 127% | 108% |
| Class E |  |  |  |  |  |
| Overall | -1.68% | 0.50% | 0.42% | 126% | 106% |
| Class D | -1.39% | 0.60% | -0.11% | 127% | 107% |
| Class F (optional) | -1.35% | -0.62% | -0.76% | 139% | 105% |

Table 3. Result for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.40% | -3.96% | -4.02% | 150% | 114% |
| Class A2 | -2.22% | -2.43% | -1.93% | 147% | 115% |
| Class B | -2.29% | -2.16% | -2.65% | 147% | 117% |
| Class C | -1.60% | -1.78% | -1.83% | 140% | 117% |
| Class E | -3.68% | -2.63% | -2.47% | 151% | 117% |
| Overall | -2.54% | -2.50% | -2.55% | 147% | 116% |
| Class D | -1.46% | -1.60% | -1.54% | 136% | 120% |
| Class F (optional) | -1.66% | -1.57% | -1.84% | 147% | 112% |

Table 4. Result for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.92% | -2.45% | -2.95% | 114% | 103% |
| Class A2 | -1.09% | -1.22% | -0.33% | 114% | 102% |
| Class B | -1.40% | -1.01% | -1.55% | 115% | 102% |
| Class C | -0.98% | -1.27% | -1.73% | 116% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.33% | -1.41% | -1.64% | 115% | 103% |
| Class D | -0.80% | -1.12% | -1.09% | 115% | 103% |
| Class F (optional) | -1.11% | -1.30% | -1.20% | 120% | 103% |

# Source code analysis

The modifications in the source code are consistent with a description presented in the text of the proposal. New code is written in the same style as a base code. Nevertheless а further cleanup and refactoring are suggested to increase code readability especially in the part of processing steps required for prediction code generation. There are some not used code and code that touches not related parts of base software. As was explained by proponent that code left from development stage and will be removed in further revisions.

# Conclusion

The results received within this cross-check are the same with provided by proponent. The source code corresponds with tool’s description given

# Reference

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, USA, April 2018.

- Philipp Helle et al., “CE3-related: Non-linear weighted intra prediction”, JVET-K0196, Ljubljana, Slovenia, July 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.