| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0264r2 |
| --- | --- |

| Title: | CE6: Report of CE6.1.3 (Transform reduction in AMT), with further reduction via DST-IV inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) Contact(s): | K. Naser, F. Le Léannec, E. François | Tel: Email: | karam.naser@technicolor.com fabrice.leleannec@technicolor.com edouard.francois@technicolor.com |
| ource: | Technicolor |  |  |

_____________________________

# Abstract

This contribution relates to adaptive multiple transforms (AMT) design implemented in BMS1.1. It first reports results of transform reduction as described in CE 6.1 – Primary Transforms (test 6.1.3 Transform Reduction). It further proposes inheriting DST-IV from the existing implementation of DCT-II. This reportedly leads to reduced memory and/or hardware area for specific coding implementations requiring DST-IV computation.

## Introduction

In CE6.1.3, it is proposed to start from the AMT design of [2] and to further reduce the transforms used in ‎[2]. CE6.1.3 also modifies the corresponding mapping between the intra-prediction mode, block size and transform set.

Specifically, the transforms that are used in ‎[2] (in addition to DCT-II) are:

- DST-I,

- DST-IV,

- DST-VII,

- DCT-V,

- DCT-VIII

Compared to the BMS AMT design, [2] further includes DST-IV.

CE6.1.3 proposes to reduce this set to include only:

- DST-IV,

- DST-VII,

- DCT-VIII

- DST-VII, that is actually further omitted as it can be inferred from DCT-VIII by reverse ordering and sign changing operations.

## Further simplification: inheriting DST-IV from DCT-II

DST-IV has been frequently included in the AMT design to improve the bitrate saving, e.g. ‎[2]‎[3], which necessarily requires the implementation of new transform. However, it is possible to show DST-IV is already implemented in DCT-II, which means that neither extra implementation, nor a new transform matrix is needed.

As described in ‎[4], the DCT-II transform matrix of size 2Nx2N can be decomposed into NxN DCT-II and NxN DCT-IV, where the even-indexed basis functions belong to DCT-II and the odd ones belong to DCT-IV. In VTM software, the 8x8 DCT-II is scaled by a factor of 8×28, which results in the following values:

| 256 | 256 | 256 | 256 | 256 | 256 | 256 | 256 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 355 | 301 | 201 | 71 | -71 | -201 | -301 | -355 |
| 334 | 139 | -139 | -334 | -334 | -139 | 139 | 334 |
| 301 | -71 | -355 | -201 | 201 | 355 | 71 | -301 |
| 256 | -256 | -256 | 256 | 256 | -256 | -256 | 256 |
| 201 | -355 | 71 | 301 | -301 | -71 | 355 | -201 |
| 139 | -334 | 334 | -139 | -139 | 334 | -334 | 139 |
| 71 | -201 | 301 | -355 | 355 | -301 | 201 | -71 |

The even-ordered basis functions, in bold, are exactly the basis functions of DCT-IV scaled by 4×28, which is the proper scale according to VTM implementation. Finally, the DST-IV basis functions can be obtained by reversing the order of DCT-IV basis functions, and sign-changing the odd basis functions. That is, the scaled DST-IV basis functions are:

| 71 | 201 | 301 | 355 |
| --- | --- | --- | --- |
| 201 | 355 | 71 | -301 |
| 301 | 71 | -355 | 201 |
| 355 | -301 | 201 | -71 |

Accordingly, the transform matrix of NxN DCT-IV is already available in the matrix of 2Nx2N DCT-II. The algorithm that is needed to obtain DCT-IV from DCT-II is described by a simple pseudo-code function below:

Function Coeff = Tr_DCT-IV_SizeNxN(In)

In = ReverseOrder(In)

Coeff = Tr_DCT-II_Size2Nx2N(In, OddCoef)

Return SignChange(Coeff);

End

# Experimental Results

The implementation with DCT-IV inheriting naturally fully matches the CE6.1.3 results reported below. BD-rates are exactly the same, while it was observed that the encoding / decoding time is not impacted.

The proposed technique is implemented on BMS1.1 and VTM1.1 and tested under All Intra, Low-delay-B, and Random Access configurations. The reference is the VTM1.1 (without AMT) or BMS1.1 (with full AMT design). The test solution is VTM1.0 or BMS1.1, with the AMT design described in section 1.3. The results are shown in Tables below.

The benefits of the proposed AMT scheme in VTM is clearly visible, with a psnr-Y BD-rate gain of 3.6% in AI, 2.2% in RA, and 1.6% in LDB. Compared to the default AMT design of BMS1.1, gains of 0.20% in AI, 0.05% in RA and 0.08% in LDB are observed.

## Experiments in VTM

Table 1 Comparison of Proposed method with VTM 1.0 (All Intra)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.44% | -1.35% | -1.24% | 194% | 137% |
| Class A2 | -3.29% | -1.41% | -1.35% | 200% | 129% |
| Class B | -3.52% | -1.49% | -1.56% | 209% | 126% |
| Class C | -3.39% | -1.17% | -1.43% | 222% | 114% |
| Class E | -4.46% | -2.95% | -3.04% | 197% | 123% |
| Overall | -3.60% | -1.63% | -1.69% | 206% | 125% |
| Class D | -3.34% | -1.07% | -1.01% | 233% | 121% |

Table 2 Comparison of Proposed method with VTM 1.0 (Random Access)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.97% | -0.05% | 0.19% | 168% | 125% |
| Class A2 | -2.04% | 0.39% | 0.55% | 152% | 118% |
| Class B | -2.51% | -0.17% | -0.43% | 161% | 117% |
| Class C | -2.15% | -0.11% | -0.28% | 169% | 113% |
| Class E |  |  |  |  |  |
| Overall | -2.21% | -0.02% | -0.07% | 163% | 118% |
| Class D | -1.97% | 0.52% | -0.31% | 168% | 110% |

Table 3 Comparison of Proposed method with VTM 1.0 (low delay B)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.94% | 0.89% | 0.98% | 170% | 120% |
| Class C | -1.36% | 0.85% | 0.91% | 161% | 120% |
| Class E | -1.18% | 0.48% | 1.30% | 153% | 120% |
| Overall | -1.56% | 0.77% | 1.04% | 163% | 120% |
| Class D | -1.24% | 1.37% | 0.31% | 146% | 115% |

## Experiments in BMS

Table 4 Comparison of Proposed method with BMS 1.1 (All Intra)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.13% | 0.04% | -0.11% | 98% | 102% |
| Class A2 | -0.10% | 0.18% | 0.08% | 102% | 102% |
| Class B | -0.09% | 0.02% | -0.07% | 100% | 101% |
| Class C | -0.38% | -0.30% | -0.33% | 105% | 98% |
| Class E | -0.35% | -0.31% | -0.44% | 101% | 99% |
| Overall | -0.20% | -0.08% | -0.17% | 101% | 100% |
| Class D | -0.47% | -0.39% | -0.42% | 102% | 101% |

Table 5 Comparison of Proposed method with BMS 1.1 (Random Access)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.04% | -0.18% | -0.14% | 101% | 105% |
| Class A2 | 0.02% | 0.13% | 0.21% | 99% | 106% |
| Class B | -0.03% | 0.05% | -0.06% | 102% | 103% |
| Class C | -0.12% | -0.10% | -0.22% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.02% | -0.06% | 101% | 103% |
| Class D | -0.19% | -0.35% | -0.16% | 107% | 96% |

Table 6 Comparison of Proposed method with BMS 1.1 (low delay B)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.01% | 0.12% | 104% | 104% |
| Class C | -0.06% | 0.20% | 0.06% | 102% | 101% |
| Class E | -0.20% | -0.49% | 0.44% | 103% | 103% |
| Overall | -0.08% | -0.06% | 0.18% | 103% | 103% |
| Class D | -0.01% | -0.19% | 0.20% | 101% | 100% |

# Conclusion

This contribution proposes an implementation of AMT limiting the number of stored transforms, when DST-IV implementation is needed. It is based on the mathematical relationships between DCT-II, DCT-IV and DST-IV.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Amir Said, Xin Zhao, ” Description of Core Experiment 6: Transforms and transform signalling” JVET-J1026, San Diego, US, 10–20 Apr. 2018.

- “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions” JVET-J0021, San Diego, US, 10–20 Apr. 2018.

- “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version” JVET-J0022, San Diego, US, 10–20 Apr. 2018.

- V. Britanak, K. R. Rao, and P. Yip, “Discrete Cosine and Sine Transforms: General Properties, Fast Algorithms and Integer Approximations”, Academic Press - Elsevier, Oxford, UK, 2007.