| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0265 |
| --- | --- |

| Title: | CE6-related: Reduction of the number of core transforms in AMT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | K. Naser, F. Le Léannec, E. François | Tel: Email: | karam.naser@technicolor.com fabrice.leleannec@technicolor.com edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes several changes to the current AMT design of BMS software, aiming at reducing the number of core transforms. This reduction is based on the mathematical properties of the discrete trigonometric transforms as well as the observed similarity between the basis functions.

# Introduction

The current implementation of AMT in BMS1.1 includes the following set of transforms:

- DCT-V

- DST-I

- DST-VII

- DCT-VIII

Apart from DST-I, the rest of transforms are considered as odd transforms, because their basis functions are based on odd length of discrete Fourier transform. This leads to difficulties in their implementation since their fast algorithms are inherited from odd length DFT, which itself has much slower implementation than the even DFT. Thus, this contribution proposes to replace these transforms by others, which are based on even FFT.

First, let’s define the following matrices to establish the necessary mathematical relationship between transforms:

- Reverse ordering matrix (R): R is a sparse matrix whose antidiagonal coefficients are zeros.

- Sign changing matrix (S): S is a sparse matrix whose diagonal elements are equal to (-1)i, where i is the row index (from 0 to N).

With those matrices, we can define the two operations:

- STR: is an operation that starts with a reverse ordering (R) of the input vector, then transform operation (T), then sign changing (S) of odd-ordered coefficients.

- RTS: is an operation that starts with a sign changing (S) of odd-ordered samples of the input vector, then transform operation (T), then reverse ordering (R) of the resulting coefficients.

The following relationships exist:

- RTS of DCT-II is DST-II

- STR of DCT-IV is DST-IV

- DCT-IV can be inherited from DCT-II, without any additional operation ‎[1].

Using all these relationships, we can approximate the current AMT transforms by others that are either inherited or inferred from other transforms.

# Proposed Approximations

When looking at the first basis functions of the current 4 transforms of BMS1.1 AMT, namely DST-I, DST-VII, DCT-V and DCT-VIII, it can be observed that there is a high similarity with the first basis functions of DST-II, DST-IV, DCT-II and DCT-IV, respectively. This is demonstrated in Figure 1. This allows the following approximation and simplification of the design

- DST-I can be approximated by DST-II, where the latter is inferred from DCT-II by RTS operation.

- DCT-V can be approximated by DCT-II.

- DCT-VIII can be approximated by DCT-IV, which is inherited from DCT-II.

- DST-VII can be approximated by DST-IV, which is inferred from DCT-IV by STR operation.

Figure 1 The first basis functions of EMT transform and the corresponding approximate basis from other transforms.

These approximations significantly simplify the AMT design, since only DCT-II is required, and the rest of transforms are either inherited of inferred from this transform. This can also highly speed up the encoder/decoder as many DCT-II fast algorithms exist, including the current partial butterfly implementation. In terms of memory requirement, solely the transform matrix of DCT-II needs to be stored, thus reducing the overall memory to 1/5.

In practice, it was noted that the approximation of DCT-V by DCT-II can be used for residual vector of length larger than 8. Thus, the results for replacing DCT-V by DCT-II are only reported for such condition.

# Results

The proposed approximations where implemented on top of BMS software 1.1, with Random Access configuration. In the following subsections, we provide the individual results of replacing each transform, as well as the results of replacing all transforms by their approximate ones.

## Replacing DST-I by DST-II

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.03% | 0.06% | 0.05% |
| Class A2 | 0.06% | 0.26% | 0.17% |
| Class B | 0.02% | 0.25% | -0.11% |
| Class C | -0.01% | 0.05% | -0.04% |
| Class E |  |  |  |
| Overall | 0.02% | 0.16% | -0.01% |
| Class D | -0.02% | 0.30% | 0.35% |

## Replacing DCT-V by DCT-II

The replacement of DCT-V by DCT-II only applies for residual vector of length larger than 8.

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.01% | 0.17% | 0.08% |
| Class A2 | 0.01% | 0.13% | 0.11% |
| Class B | 0.03% | 0.15% | 0.04% |
| Class C | 0.03% | 0.31% | -0.18% |
| Class E |  |  |  |
| Overall | 0.02% | 0.19% | 0.00% |
| Class D | 0.04% | -0.12% | -0.19% |

## Replacing DCT-VIII by DCT-IV and DST-VII by DST-IV

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.23% | 0.35% | 0.22% |
| Class A2 | 0.25% | 0.54% | 0.19% |
| Class B | 0.09% | 0.05% | 0.21% |
| Class C | -0.15% | -0.03% | -0.36% |
| Class E |  |  |  |
| Overall | 0.09% | 0.19% | 0.06% |
| Class D | -0.25% | -0.29% | -0.36% |

## Putting all together (replacing DST-I by DST-II, DCT-V by DCT-II, DCT-VIII by DCT-IV and DCT-IV)

The replacement of DCT-V by DCT-II only applies for residual vector of length larger than 8.

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.26% | 0.47% | 0.43% |
| Class A2 | 0.36% | 0.68% | 0.34% |
| Class B | 0.18% | 0.47% | 0.19% |
| Class C | -0.13% | 0.13% | -0.29% |
| Class E |  |  |  |
| Overall | 0.15% | 0.42% | 0.14% |
| Class D | -0.18% | -0.16% | -0.48% |

# Conclusion

This contribution provided some approximations of the AMT transforms by others that can be inherited or inferred from DCT-II. Numerical results showed that replacing each individual transform has a low impact on the rate-distortion performance (0.02 to 0.09 % luma loss), whereas the replacement all of them result in 0.15% luma loss, which is a little higher but negligible in terms of the huge memory and/or computational requirement.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- K. Naser, F. Leleannec and E. Francois "CE6: Report of CE6.1.3 (Transform reduction in AMT), with further reduction via DST-4 inheritance”, JVET-K0264, Ljubljana, SI, 10–18 July 2018.