| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0266-v1 |
| --- | --- |

| Title: | CE3: Non-linear weighted intra prediction (Test 6.1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Merkle, Jonathan Pfaff, Philipp Helle, Roman Rischke, Michael Schäfer, Björn Stallenberger, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-433 philipp.merkle@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the results on the tool under test in CE3 (JVET-J1023) for studying the performance of the intra prediction modes proposed in JVET-J0037 on top of VTM and BMS.

# Introduction

The proposed method comprises a set of intra prediction modes. These modes perform the following two main steps: First, a set of features is extracted from reconstructed samples. Second, these features are used to select an affine linear combination of predefined image patterns as the prediction signal. It is noted that the proposed method will be referred to as non-linear weighted intra prediction (NWIP) modes in the following.

# Proposed method

Non-linear weighted intra prediction (NWIP) has been integrated to the test model according to the description of sub-test CE3.6 [1].

Figure 1 shows an overview of the prediction process: The reconstructed samples are used for a non-linear feature extraction. These features are identical for all NWIP modes of a given block size. For a specific mode, the sparse prediction signal in the frequency domain is then derived by a linear weighted combination of these features. Finally, the predicted samples of the block are achieved by applying the inverse DCT.

Figure 1. Prediction of MxN intra block from reconstructed samples using NWIP

For mode signaling, NWIP also applies the most probable mode (MPM) concept known from conventional intra prediction but estimates three MPMs based on reconstructed samples in a separate non-linear process.

The additional transforms described in JVET-J0037 [2] have not been integrated to the test model for test CE3-6.1.1, such that NWIP blocks use the DCT for VTM configuration and EMT but no NSST for BMS configuration.

# Results

According to the CE3 description in JVET-J1023 [1] and the common test conditions in JVET-J1010 [3], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with BMS-1.0 software in VTM and BMS configuration. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 1. Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -2.28% | -1.89% | -1.71% | 269% | 134% |
| Class A2 | -2.29% | -1.24% | -1.50% | 265% | 143% |
| Class B | -2.71% | -1.06% | -0.73% | 265% | 152% |
| Class C | -2.31% | -1.18% | -1.36% | 260% | 147% |
| Class E | -4.68% | -2.04% | -2.49% | 268% | 145% |
| Overall | -2.81% | -1.42% | -1.46% | 265% | 145% |
| Class D | -2.46% | -1.05% | -1.08% | 256% | 154% |
| Class F (optional) |  |  |  |  |  |

Table 2. Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.04% | -0.47% | -0.45% | 124% | 112% |
| Class A2 | -1.06% | 0.63% | 0.51% | 119% | 105% |
| Class B | -1.52% | 0.76% | 0.62% | 124% | 113% |
| Class C | -1.34% | -0.05% | -0.13% | 125% | 111% |
| Class E |  |  |  |  |  |
| Overall | -1.28% | 0.27% | 0.18% | 123% | 111% |
| Class D | -1.20% | 0.26% | -0.25% | 126% | 108% |
| Class F (optional) |  |  |  |  |  |

Table 3. Result for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -2.28% | -3.03% | -3.00% | 146% | 118% |
| Class A2 | -1.88% | -1.92% | -1.52% | 145% | 119% |
| Class B | -1.94% | -1.65% | -2.08% | 144% | 123% |
| Class C | -1.32% | -1.30% | -1.24% | 140% | 122% |
| Class E | -2.94% | -2.02% | -2.08% | 151% | 121% |
| Overall | -2.02% | -1.91% | -1.95% | 145% | 121% |
| Class D | -1.20% | -1.16% | -1.18% | 137% | 126% |
| Class F (optional) |  |  |  |  |  |

Table 4. Result for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.26% | -1.91% | -2.05% | 113% | 104% |
| Class A2 | -0.91% | -0.94% | -0.32% | 114% | 103% |
| Class B | -1.18% | -0.85% | -1.35% | 115% | 104% |
| Class C | -0.79% | -1.09% | -1.32% | 117% | 104% |
| Class E |  |  |  |  |  |
| Overall | -1.04% | -1.14% | -1.28% | 115% | 104% |
| Class D | -0.66% | -0.85% | -0.60% | 117% | 105% |
| Class F (optional) |  |  |  |  |  |

# Cross check

The cross check of test 6.1.1 was done by Technicolor, Huawei, and HiSilicon.

# Conclusion

The results show that the proposed NWIP modes lead to considerable coding gains for all configurations, with more than 30% of blocks coded with NWIP for both VTM and BMS AI configuration. The gains for BMS configuration are smaller than for the VTM configuration because NWIP does not use dedicated secondary transforms like NSST.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, April 2018.

- J. Pfaff et al., “Intra Prediction Modes based on Neural Networks,” JVET-J0037, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).