| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0267-v1 |
| --- | --- |

| Title: | Non CE4 : Virtual Temporal Affine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin , A. Robert, T. Poirier, F. Leleannec | Tel: Email: | firstname.lastname@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the results of a modified temporal affine merge candidate using virtual affine model from collocated candidates. We report the results of implementation in JVET VTM-1.0 and BMS-1.0. For tool ON/tool OFF test, simulation results reportedly show an average luma BD-rate gain of -xxx%, resp. -xxx%, in Random Access (RA), resp. Low-delay (LDB) configuration, compared with TM, and -xxx%, resp. -xxx%, in RA, resp. LDB configuration, compared to BMS. Moreover, additional tests on affine class sequences show an average luma BD-rate gain in RA of -xxx% for TM, resp. -xxxx%, for BMS.

# Introduction

In J0022, new affine merge candidates were introduced. Temporal affine candidates were added to the merge candidates list, when using unified affine merge (see CE4-2.4 results). A constraint is then added on the temporal motion buffer, which should also contain the affine PU information. In this contribution, we show the results using affine temporal candidates built without additional information. The study of the temporal motion buffer resolution is also studied.

# Virtual candidate

The process of deriving the virtual temporal affine candidate is the same as the one described in J0021- “2.9.2 Affine motion vector prediction”, except the sub-blocks used for the model derivation are the collocated corners of the current block.

Table 1 Temporal affine sub-blocks

The reference image used for the motion compensation is the one pointed to by the corner A in the collocated block, or the first image in the reference list if it does not exist in the reference list.

The candidate is added to the merge candidates list. The unified merge candidates list is described in J0021 “2.9.3	Unified affine/normal merge candidate list”. In this contribution, only default merge candidates and additional affine candidates are used as shown in the table.

|  | VTM | BMS | Proposed added candidates |
| --- | --- | --- | --- |
| Candidate list | - spatial positions in {a1, b1, b0, a0} - spatial positions in {b2} - a temporal position {c} - bi-directional combinations - zero MV for each reference picture | 7 candidates, among: - spatial positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - spatial positions in {b2} - a temporal position {c} - bi-directional combinations - zero MV for each reference picture | - spatial positions in {a1, b1, b0, a0} - spatial affine positions in {a1, b1, b0, a0} - ATMVP (is 1 candidate, where CU is divided into 4x4 sub-CUs) - constructed affine candidate based on spatial non-affine positions - spatial positions in {b2} - spatial affine positions in {b2, a2, b3} - a temporal position {c} - temporal virtual affine positions in {c} - bi-directional combinations - zero MV for each reference picture |

# Results

All tests are done with SIMD=SSE42 options with BMS 1.0 software in VTM or BMS configuration.

## Temporal buffer resolution 8x8

### Over VTM

Anchor is VTM 1.0. The temporal motion buffer resolution is 8x8.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.91% | -2.76% | -2.91% | 156% |  |
| Class C | -1.59% | -1.10% | -1.07% | 150% |  |
| Overall |  |  |  |  |  |
| Class D | -2.14% | -1.33% | -1.50% | 147% |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Affine 1080p |  |  |  |  |  |
| Affine 720p | -14.90% | -11.21% | -11.75% | 210% |  |
| Overall |  |  |  |  |  |

### Over VTM tool OFF

Anchor is VTM 1.0, with only the affine temporal candidate removed, the temporal motion buffer resolution is 8x8.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.01% | -0.07% | 101% |  |
| Class C | -0.13% | -0.14% | 0.02% | 101% |  |
| Overall |  |  |  |  |  |
| Class D | -0.22% | -0.15% | -0.19% | 102% |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Affine 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Affine 720p | -0.51% | -0.49% | -1.15% | 100% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Over BMS without affine tool

Anchor is BMS without affine tool.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

## Over BMS tool OFF

Anchor is BMS 1.0, with only the affine temporal candidate removed, the temporal motion buffer resolution is 8x8.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

# Conclusion

The overall gains of affine modification in J0022 is presented in this document. With only marginal complexity increase at decoder the method brings around xxx% BD-rate gain over VTM for the luma component in Random Access configuration. xxxx

# References

[1] E. François, F. Le Léannec, P. Bordes, A. Robert, T. Poirier, F. Racapé, F. Galpin and al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0022, 10th Meeting, San Diego, April 2018.

[2] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1024, 10th Meeting, San Diego, April 2018.

[3] “Affine Transform prediction for next generation video coding”, COM16-C1016R1, ITU, Sept. 2015.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).