| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0269-v1 |
| --- | --- |

| Title: | CE10: Multi-Hypothesis Inter Prediction (Tests 1.5 – 1.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Winken Heiko Schwarz Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-570 martin.winken@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports results for tests CE10-1.5 – CE10-1.8 on multi-hypothesis inter prediction.  The total number of motion-compensated prediction signals (per block) which constitute the overall prediction signal is limited to 3 for tests CE10-1.5/CE10-1.7 and limited to 4 for tests CE10-1.6/CE10-1.8.  For tests CE10-1.7/CE10-1.8, multi-hypothesis is restricted to be available only for blocks that are larger than 8x8 luma samples.  For tests CE10-1.5/CE10-1.6, there is no such restriction.  For the Random Access configuration the average Y BD bit rate saving are -1.63%, -1.91%, -1.55%, and -1.84% relative to the VTM anchor with an relative encoder run-time of 127%, 130%, 122%, and 125%, corresponding to the tests CE10-1.5,6,7,8.

# Introduction

In the multi-hypothesis inter prediction mode, one or more additional prediction signals are signaled, in addition to the conventional uni/bi prediction signal.  The resulting overall prediction signal is obtained by sample-wise weighted superposition.  With the uni/bi prediction signal puni/bi and the first additional inter prediction signal/hypothesis h3, the resulting prediction signal p3 is obtained as follows:

p3=(1-α)puni/bi+αh3

The changes to the prediction unit syntax structure are shown below:

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| … |  |
| if( ! cu_skip_flag[ x0 ][ y0 ] ) { |  |
| i = 0 |  |
| readMore = 1 |  |
| while( i < MaxNumAdditionalHypotheses && readMore ) { |  |
| additional_hypothesis_flag[ x0 ][ y0 ][ i ] | ae(v) |
| if( additional_hypothesis_flag[ x0 ][ y0 ][ i ] ) { |  |
| ref_idx_add_hyp[ x0 ][ y0 ][ i ] | ae(v) |
| mvd_coding( x0, y0, 2+i ) |  |
| mvp_add_hyp_flag[ x0 ][ y0 ][ i ] | ae(v) |
| add_hyp_weight_idx[ x0 ][ y0 ][ i ] | ae(v) |
| } |  |
| readMore = additional_hypothesis_flag[ x0 ][ y0 ][ i ] |  |
| i++ |  |
| } |  |
| } |  |
| } |  |

The weighting factor α is specified by the syntax element add_hyp_weight_idx, according to the following mapping:

| add_hyp_weight_idx | α |
| --- | --- |
| 0 | 1/4 |
| 1 | -1/8 |

Note that for the additional prediction signals, the concept of prediction list0/list1 is abolished, and instead one combined list is used.  This combined list is generated by alternatingly inserting reference frames from list0 and list1 with increasing reference index, omitting reference frames which have already been inserted, such that double entries are avoided.

Analogously to above, more than one additional prediction signals can be used.  The resulting overall prediction signal is accumulated iteratively with each additional prediction signal.

pn+1=1-αn+1pn+αn+1hn+1

The resulting overall prediction signal is obtained as the last pn (i.e., the pn having the largest index n).

Note that also for inter prediction blocks using MERGE mode (but not SKIP mode), additional inter prediction signals can be specified.  Further note, that in case of MERGE, not only the uni/bi prediction parameters, but also the additional prediction parameters of the selected merging candidate can be used for the current block.

## Multi-Hypothesis Motion Estimation

First, the inter modes with no explicitly signaled additional inter prediction parameters are tested.  For the best of these modes (i.e., the one having lowest RD cost), an additional inter prediction hypothesis is searched.  For that purpose, for all combinations of the following parameters, a motion estimation with a restricted search range of 16 is performed:

- Weighting factor α

- Reference frame for the additional prediction hypothesis

For determining the best combination of these two parameters, a simplified RD cost using Hadamard distortion measure and approximated bit rate is used.  The chosen parameter combination is then used to compute a more accurate RD cost, using forward transform and quantization, which is compared against the so-far best found coding mode for the current block.  If the resulting RD cost is lower than the current best, an additional hypothesis is signaled for the current block.  Note that there is an upper limit for the number of additional hypotheses, such that for blocks using MERGE mode, the resulting total number of merged and explicitly signaled additional hypotheses does not exceed this limit. E.g. in the case of this limit equal to 1, for MERGE blocks where the merging candidate already has one additional inter prediction hypothesis, no further additional hypothesis is tested.  If this limit has not yet been reached for the current block and if the last tested additional hypothesis resulted in a reduced RD cost, the multi-hypothesis estimation process starts all over again, adding one more additional hypothesis also to be tested.

# Software and Settings

The source code was made available to all CE10 participants in the following branches:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-1.5

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-1.6

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-1.7

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-1.8

The encoder can be run using the default config files, all necessary settings are accordingly pre-defined in the corresponding branches.

# Test results

According to the CE10 description in JVET-J1030 [1] and the common test conditions in JVET-J1010 [2], simulations were performed for random-access (RA) and low-delay B (LB) configuration with BMS-1.0 software in VTM and BMS configuration. The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

## Test CE10-1.5

For this test, the total number of hypotheses is limited to 3.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.84% | -0.91% | -0.99% | 121% | 103% | -0.46% | -0.74% | -0.81% | 104% | 101% |
| Class A2 | -1.04% | -0.81% | -0.72% | 126% | 102% | -0.48% | -0.60% | -0.53% | 108% | 102% |
| Class B | -2.48% | -1.56% | -1.42% | 128% | 105% | -1.57% | -1.26% | -1.23% | 107% | 99% |
| Class C | -1.60% | -0.78% | -0.94% | 133% | 105% | -0.77% | -0.52% | -0.58% | 109% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.63% | -1.07% | -1.07% | 127% | 104% | -0.92% | -0.83% | -0.83% | 107% | 100% |
| Class D | -1.68% | -0.35% | -0.51% | 138% | 101% | -0.65% | -0.45% | -0.50% | 111% | 101% |
| Class F (optional) | -1.70% | -1.14% | -1.07% | 131% | 101% | -1.43% | -1.13% | -1.00% | 107% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.19% | -0.92% | -0.77% | 141% | 106% | -1.20% | -0.96% | -0.57% | 113% | 100% |
| Class C | -1.93% | -0.92% | -1.15% | 146% | 107% | -0.80% | -0.25% | -0.51% | 114% | 98% |
| Class E | -1.96% | 1.19% | 1.05% | 149% | 108% | -0.65% | 0.06% | 0.07% | 116% | 98% |
| Overall | -2.05% | -0.39% | -0.44% | 145% | 107% | -0.93% | -0.47% | -0.39% | 114% | 99% |
| Class D | -1.20% | -0.84% | -0.70% | 146% | 104% | -0.69% | -0.08% | -1.23% | 116% | 99% |
| Class F (optional) | -3.70% | -2.70% | -2.31% | 141% | 102% | -2.37% | -1.91% | -1.53% | 110% | 99% |

## Test CE10-1.6

For this test, the total number of hypotheses is limited to 4.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | -1.07% | -1.21% | 121% | 102% | -0.54% | -0.70% | -0.89% | 104% | 103% |
| Class A2 | -1.20% | -0.88% | -0.78% | 129% | 102% | -0.56% | -0.65% | -0.70% | 109% | 101% |
| Class B | -2.91% | -1.79% | -1.57% | 130% | 105% | -1.86% | -1.42% | -1.40% | 108% | 99% |
| Class C | -1.89% | -0.95% | -1.03% | 137% | 106% | -0.87% | -0.63% | -0.62% | 110% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.91% | -1.24% | -1.19% | 130% | 104% | -1.07% | -0.91% | -0.95% | 108% | 100% |
| Class D | -2.18% | -0.55% | -0.75% | 143% | 103% | -0.70% | -0.42% | -0.42% | 112% | 100% |
| Class F (optional) | -1.94% | -1.26% | -1.11% | 134% | 101% | -1.62% | -1.23% | -1.07% | 107% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.54% | -1.14% | -0.77% | 144% | 107% | -1.48% | -1.07% | -0.62% | 113% | 98% |
| Class C | -2.08% | -1.09% | -1.21% | 147% | 108% | -0.99% | -0.44% | -0.40% | 116% | 99% |
| Class E | -2.15% | 1.79% | 1.19% | 155% | 110% | -0.75% | 0.93% | 1.14% | 117% | 99% |
| Overall | -2.29% | -0.39% | -0.43% | 147% | 108% | -1.13% | -0.36% | -0.11% | 115% | 99% |
| Class D | -1.41% | -1.11% | -0.88% | 151% | 104% | -0.75% | -0.21% | -1.17% | 118% | 99% |
| Class F (optional) | -4.27% | -3.44% | -2.88% | 144% | 101% | -2.83% | -2.02% | -2.00% | 110% | 98% |

## Test CE10-1.7

For this test, the total number of hypotheses is limited to 3 and availability of multi-hypothesis prediction is restricted to blocks which are larger than 8x8 luma samples.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.92% | -1.02% | 118% | 102% | -0.44% | -0.65% | -0.79% | 105% | 102% |
| Class A2 | -1.03% | -0.79% | -0.63% | 122% | 102% | -0.50% | -0.58% | -0.60% | 107% | 101% |
| Class B | -2.35% | -1.57% | -1.45% | 123% | 108% | -1.51% | -1.27% | -1.24% | 106% | 99% |
| Class C | -1.53% | -0.77% | -0.79% | 125% | 103% | -0.74% | -0.40% | -0.57% | 107% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.55% | -1.07% | -1.02% | 122% | 104% | -0.89% | -0.78% | -0.84% | 106% | 100% |
| Class D | -1.62% | -0.32% | -0.48% | 126% | 101% | -0.64% | -0.32% | -0.36% | 108% | 100% |
| Class F (optional) | -1.59% | -0.96% | -0.90% | 126% | 99% | -1.33% | -1.11% | -0.94% | 105% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.14% | -1.02% | -0.82% | 133% | 106% | -1.17% | -1.00% | -0.49% | 111% | 99% |
| Class C | -1.87% | -0.76% | -1.13% | 131% | 105% | -0.81% | -0.16% | -0.51% | 111% | 98% |
| Class E | -2.09% | 1.28% | 0.68% | 144% | 109% | -0.71% | 0.47% | 1.14% | 115% | 98% |
| Overall | -2.04% | -0.36% | -0.54% | 135% | 107% | -0.94% | -0.35% | -0.09% | 112% | 98% |
| Class D | -1.18% | -0.91% | -0.86% | 129% | 106% | -0.66% | 0.00% | -0.64% | 111% | 97% |
| Class F (optional) | -3.60% | -2.49% | -1.87% | 133% | 101% | -2.22% | -1.73% | -1.93% | 109% | 100% |

## Test CE10-1.8

For this test, the total number of hypotheses is limited to 4 and availability of multi-hypothesis prediction is restricted to blocks which are larger than 8x8 luma samples.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | -1.15% | -1.25% | 119% | 101% | -0.50% | -0.81% | -0.87% | 105% | 101% |
| Class A2 | -1.21% | -0.87% | -0.72% | 124% | 103% | -0.59% | -0.66% | -0.67% | 108% | 101% |
| Class B | -2.77% | -1.86% | -1.63% | 126% | 110% | -1.76% | -1.39% | -1.35% | 107% | 100% |
| Class C | -1.80% | -0.98% | -1.02% | 129% | 102% | -0.85% | -0.46% | -0.56% | 109% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.84% | -1.29% | -1.21% | 125% | 105% | -1.03% | -0.88% | -0.91% | 107% | 100% |
| Class D | -2.09% | -0.52% | -0.69% | 131% | 101% | -0.69% | -0.48% | -0.28% | 109% | 100% |
| Class F (optional) | -1.83% | -1.17% | -1.07% | 128% | 101% | -1.52% | -1.20% | -1.04% | 106% | 98% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.47% | -1.14% | -1.00% | 135% | 106% | -1.39% | -1.09% | -0.43% | 113% | 100% |
| Class C | -2.05% | -0.75% | -1.04% | 135% | 110% | -0.94% | -0.35% | -0.36% | 114% | 100% |
| Class E | -2.22% | 1.60% | 1.12% | 148% | 109% | -0.71% | 1.13% | 0.72% | 116% | 100% |
| Overall | -2.27% | -0.32% | -0.48% | 138% | 108% | -1.07% | -0.29% | -0.12% | 114% | 100% |
| Class D | -1.39% | -0.63% | -0.97% | 135% | 105% | -0.72% | 0.05% | -0.70% | 114% | 98% |
| Class F (optional) | -4.09% | -2.85% | -2.45% | 137% | 103% | -2.55% | -2.08% | -2.10% | 108% | 97% |

# Memory bandwidth

## Average number of hypotheses and reference samples per prediction sample

In the following, the average numbers of prediction hypotheses per inter predicted sample as well as the average number of reference samples which are required for generating one inter predicted sample are shown.  The number of reference samples can be higher than the number of hypotheses because of the sub-pel interpolation filter.  Average values among different rate points and/or sequences are computed using the geometric mean (similar as done for the encoder/decoder run time in the Common Test Conditions [2]).

### BMS-as-VTM anchor

| Hypotheses |  |  |  |  |  |  | Ref.samples / Pred.sample |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |  | RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |  | A1 | 2.5 | 2.1 | 2.0 | 2.0 | 2.1 |
| A2 | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |  | A2 | 2.7 | 2.4 | 2.2 | 2.1 | 2.3 |
| B | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |  | B | 2.7 | 2.3 | 2.2 | 2.1 | 2.3 |
| C | 1.8 | 1.7 | 1.7 | 1.7 | 1.7 |  | C | 2.9 | 2.5 | 2.3 | 2.1 | 2.4 |
| AVG | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |  | AVG | 2.7 | 2.3 | 2.2 | 2.1 | 2.3 |
| D | 1.8 | 1.8 | 1.7 | 1.8 | 1.8 |  | D | 3.3 | 2.8 | 2.5 | 2.3 | 2.7 |
| F | 1.6 | 1.7 | 1.7 | 1.7 | 1.7 |  | F | 1.9 | 1.9 | 1.9 | 1.8 | 1.9 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |  | LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 1.7 | 1.6 | 1.5 | 1.4 | 1.5 |  | B | 2.9 | 2.2 | 1.9 | 1.7 | 2.1 |
| C | 1.6 | 1.5 | 1.4 | 1.4 | 1.5 |  | C | 2.9 | 2.4 | 2.1 | 1.8 | 2.3 |
| E | 1.4 | 1.3 | 1.2 | 1.1 | 1.3 |  | E | 1.7 | 1.5 | 1.3 | 1.2 | 1.4 |
| AVG | 1.6 | 1.5 | 1.4 | 1.3 | 1.4 |  | AVG | 2.5 | 2.0 | 1.8 | 1.6 | 2.0 |
| D | 1.6 | 1.5 | 1.4 | 1.3 | 1.5 |  | D | 3.4 | 2.7 | 2.2 | 1.9 | 2.5 |
| F | 1.2 | 1.2 | 1.2 | 1.1 | 1.2 |  | F | 1.5 | 1.4 | 1.3 | 1.3 | 1.4 |

### Test CE10-1.5

| Hypotheses |  |  |  |  |  |  | Ref.samples / Pred.sample |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |  | RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 2.2 | 2.0 | 1.8 | 1.8 | 2.0 |  | A1 | 3.0 | 2.3 | 2.1 | 2.0 | 2.3 |
| A2 | 2.2 | 2.0 | 1.9 | 1.8 | 2.0 |  | A2 | 3.2 | 2.6 | 2.3 | 2.1 | 2.5 |
| B | 2.3 | 2.1 | 2.0 | 1.9 | 2.1 |  | B | 3.4 | 2.7 | 2.4 | 2.2 | 2.6 |
| C | 2.2 | 2.0 | 1.9 | 1.8 | 2.0 |  | C | 3.5 | 2.9 | 2.5 | 2.2 | 2.7 |
| AVG | 2.2 | 2.0 | 1.9 | 1.8 | 2.0 |  | AVG | 3.3 | 2.6 | 2.3 | 2.1 | 2.6 |
| D | 2.1 | 2.0 | 1.9 | 1.8 | 2.0 |  | D | 3.8 | 3.1 | 2.7 | 2.4 | 3.0 |
| F | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |  | F | 2.1 | 2.0 | 1.9 | 1.9 | 2.0 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |  | LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 2.0 | 1.8 | 1.6 | 1.5 | 1.7 |  | B | 3.2 | 2.4 | 2.1 | 1.8 | 2.3 |
| C | 2.0 | 1.8 | 1.6 | 1.4 | 1.7 |  | C | 3.4 | 2.7 | 2.2 | 1.9 | 2.5 |
| E | 1.6 | 1.4 | 1.3 | 1.2 | 1.4 |  | E | 1.9 | 1.6 | 1.4 | 1.3 | 1.5 |
| AVG | 1.9 | 1.7 | 1.5 | 1.4 | 1.6 |  | AVG | 2.9 | 2.3 | 1.9 | 1.7 | 2.1 |
| D | 1.9 | 1.7 | 1.5 | 1.4 | 1.6 |  | D | 3.9 | 3.0 | 2.4 | 2.0 | 2.7 |
| F | 1.3 | 1.3 | 1.2 | 1.2 | 1.3 |  | F | 1.6 | 1.5 | 1.4 | 1.3 | 1.5 |

### Test CE10-1.6

| Hypotheses |  |  |  |  |  |  | Ref.samples / Pred.sample |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |  | RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 2.4 | 2.0 | 1.9 | 1.8 | 2.0 |  | A1 | 3.2 | 2.4 | 2.1 | 2.0 | 2.4 |
| A2 | 2.4 | 2.1 | 1.9 | 1.9 | 2.1 |  | A2 | 3.5 | 2.7 | 2.4 | 2.2 | 2.6 |
| B | 2.6 | 2.3 | 2.1 | 1.9 | 2.2 |  | B | 3.7 | 2.8 | 2.5 | 2.2 | 2.8 |
| C | 2.4 | 2.1 | 1.9 | 1.8 | 2.1 |  | C | 3.8 | 3.0 | 2.5 | 2.2 | 2.8 |
| AVG | 2.5 | 2.1 | 2.0 | 1.8 | 2.1 |  | AVG | 3.6 | 2.8 | 2.4 | 2.2 | 2.7 |
| D | 2.3 | 2.1 | 2.0 | 1.9 | 2.1 |  | D | 4.2 | 3.3 | 2.8 | 2.4 | 3.1 |
| F | 1.9 | 1.8 | 1.8 | 1.8 | 1.8 |  | F | 2.2 | 2.1 | 2.0 | 1.9 | 2.0 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |  | LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 2.1 | 1.9 | 1.7 | 1.5 | 1.8 |  | B | 3.4 | 2.5 | 2.1 | 1.8 | 2.4 |
| C | 2.1 | 1.8 | 1.6 | 1.4 | 1.7 |  | C | 3.7 | 2.8 | 2.3 | 1.9 | 2.6 |
| E | 1.7 | 1.5 | 1.3 | 1.2 | 1.4 |  | E | 2.0 | 1.7 | 1.4 | 1.3 | 1.6 |
| AVG | 2.0 | 1.7 | 1.6 | 1.4 | 1.7 |  | AVG | 3.1 | 2.4 | 2.0 | 1.7 | 2.2 |
| D | 2.0 | 1.7 | 1.5 | 1.4 | 1.6 |  | D | 4.0 | 3.1 | 2.4 | 2.0 | 2.8 |
| F | 1.4 | 1.3 | 1.3 | 1.2 | 1.3 |  | F | 1.7 | 1.5 | 1.4 | 1.3 | 1.5 |

### Test CE10-1.7

| Hypotheses |  |  |  |  |  |  | Ref.samples / Pred.sample |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |  | RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 2.2 | 2.0 | 1.8 | 1.8 | 1.9 |  | A1 | 2.9 | 2.3 | 2.1 | 2.0 | 2.3 |
| A2 | 2.2 | 2.0 | 1.9 | 1.8 | 2.0 |  | A2 | 3.2 | 2.6 | 2.3 | 2.1 | 2.5 |
| B | 2.3 | 2.1 | 2.0 | 1.9 | 2.1 |  | B | 3.3 | 2.7 | 2.4 | 2.2 | 2.6 |
| C | 2.2 | 2.0 | 1.9 | 1.8 | 2.0 |  | C | 3.4 | 2.8 | 2.5 | 2.2 | 2.7 |
| AVG | 2.2 | 2.0 | 1.9 | 1.8 | 2.0 |  | AVG | 3.2 | 2.6 | 2.3 | 2.1 | 2.6 |
| D | 2.1 | 2.0 | 1.9 | 1.8 | 1.9 |  | D | 3.7 | 3.1 | 2.7 | 2.4 | 2.9 |
| F | 1.8 | 1.8 | 1.8 | 1.8 | 1.8 |  | F | 2.1 | 2.0 | 1.9 | 1.9 | 2.0 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |  | LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 2.0 | 1.8 | 1.6 | 1.5 | 1.7 |  | B | 3.1 | 2.4 | 2.1 | 1.8 | 2.3 |
| C | 2.0 | 1.8 | 1.6 | 1.4 | 1.7 |  | C | 3.4 | 2.7 | 2.2 | 1.9 | 2.5 |
| E | 1.6 | 1.4 | 1.3 | 1.2 | 1.4 |  | E | 1.9 | 1.6 | 1.4 | 1.3 | 1.5 |
| AVG | 1.9 | 1.7 | 1.5 | 1.4 | 1.6 |  | AVG | 2.8 | 2.3 | 1.9 | 1.7 | 2.1 |
| D | 1.9 | 1.7 | 1.5 | 1.4 | 1.6 |  | D | 3.7 | 3.0 | 2.4 | 2.0 | 2.7 |
| F | 1.3 | 1.3 | 1.2 | 1.2 | 1.3 |  | F | 1.6 | 1.5 | 1.4 | 1.3 | 1.5 |

### Test CE10-1.8

| Hypotheses |  |  |  |  |  |  | Ref.samples / Pred.sample |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |  | RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 2.4 | 2.0 | 1.9 | 1.8 | 2.0 |  | A1 | 3.2 | 2.4 | 2.1 | 2.0 | 2.4 |
| A2 | 2.4 | 2.1 | 1.9 | 1.9 | 2.1 |  | A2 | 3.4 | 2.7 | 2.4 | 2.2 | 2.6 |
| B | 2.5 | 2.3 | 2.1 | 1.9 | 2.2 |  | B | 3.6 | 2.8 | 2.5 | 2.2 | 2.7 |
| C | 2.4 | 2.1 | 1.9 | 1.8 | 2.1 |  | C | 3.7 | 3.0 | 2.5 | 2.2 | 2.8 |
| AVG | 2.5 | 2.1 | 2.0 | 1.9 | 2.1 |  | AVG | 3.5 | 2.7 | 2.4 | 2.2 | 2.7 |
| D | 2.3 | 2.1 | 2.0 | 1.9 | 2.1 |  | D | 4.0 | 3.3 | 2.8 | 2.5 | 3.1 |
| F | 1.9 | 1.8 | 1.8 | 1.8 | 1.8 |  | F | 2.2 | 2.1 | 2.0 | 1.9 | 2.0 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |  | LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 2.1 | 1.9 | 1.7 | 1.5 | 1.8 |  | B | 3.3 | 2.5 | 2.1 | 1.8 | 2.4 |
| C | 2.1 | 1.8 | 1.6 | 1.4 | 1.7 |  | C | 3.5 | 2.8 | 2.3 | 1.9 | 2.6 |
| E | 1.7 | 1.5 | 1.3 | 1.2 | 1.4 |  | E | 2.0 | 1.7 | 1.4 | 1.3 | 1.6 |
| AVG | 2.0 | 1.7 | 1.6 | 1.4 | 1.7 |  | AVG | 3.0 | 2.3 | 2.0 | 1.7 | 2.2 |
| D | 2.0 | 1.7 | 1.5 | 1.4 | 1.6 |  | D | 3.9 | 3.0 | 2.4 | 2.0 | 2.7 |
| F | 1.4 | 1.3 | 1.3 | 1.2 | 1.3 |  | F | 1.7 | 1.5 | 1.4 | 1.3 | 1.5 |

## Average reference picture access data rates

In the following, it is shown how many Mbit/s have to be fetched from the reference pictures in order to obtain the motion-compensated prediction hypotheses.  It is assumed that for accessing one sample from the reference picture 2 byte (16 bit) have to be fetched, even though only 10 bit are used.

### BMS-as-VTM anchor

| Reference Picture Data Rate [MBit/s] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 18,168 | 16,770 | 16,205 | 16,017 | 16,769 |
| A2 | 26,490 | 24,825 | 23,488 | 22,484 | 24,276 |
| B | 6,837 | 5,951 | 5,581 | 5,388 | 5,914 |
| C | 1,172 | 1,021 | 936 | 882 | 997 |
| AVG | 6,811 | 6,088 | 5,720 | 5,503 | 6,011 |
| D | 337 | 288 | 259 | 244 | 280 |
| F | 908 | 903 | 900 | 904 | 904 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 7,466 | 5,725 | 5,014 | 4,609 | 5,606 |
| C | 1,230 | 1,015 | 873 | 767 | 956 |
| E | 2,290 | 1,920 | 1,736 | 1,606 | 1,871 |
| AVG | 3,046 | 2,447 | 2,148 | 1,948 | 2,363 |
| D | 364 | 291 | 238 | 203 | 268 |
| F | 723 | 688 | 665 | 645 | 680 |

### Test CE10-1.5

| Reference Picture Data Rate [MBit/s] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 22,093 | 18,402 | 16,979 | 16,328 | 18,323 |
| A2 | 32,084 | 27,060 | 24,531 | 22,903 | 26,428 |
| B | 8,452 | 6,882 | 6,188 | 5,693 | 6,728 |
| C | 1,418 | 1,167 | 1,012 | 913 | 1,112 |
| AVG | 8,310 | 6,864 | 6,154 | 5,699 | 6,688 |
| D | 392 | 319 | 277 | 252 | 306 |
| F | 1,001 | 969 | 943 | 930 | 961 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 8,452 | 6,367 | 5,472 | 4,860 | 6,151 |
| C | 1,440 | 1,153 | 946 | 805 | 1,060 |
| E | 2,574 | 2,149 | 1,868 | 1,666 | 2,037 |
| AVG | 3,481 | 2,745 | 2,330 | 2,042 | 2,597 |
| D | 409 | 317 | 254 | 211 | 289 |
| F | 798 | 746 | 707 | 673 | 729 |

### Test CE10-1.6

| Reference Picture Data Rate [MBit/s] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 24,364 | 18,967 | 17,182 | 16,359 | 18,984 |
| A2 | 34,531 | 27,929 | 24,923 | 23,054 | 27,284 |
| B | 9,299 | 7,318 | 6,428 | 5,775 | 7,089 |
| C | 1,537 | 1,227 | 1,043 | 924 | 1,161 |
| AVG | 9,069 | 7,189 | 6,319 | 5,754 | 6,978 |
| D | 427 | 340 | 288 | 256 | 322 |
| F | 1,030 | 987 | 953 | 937 | 976 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 8,987 | 6,647 | 5,624 | 4,923 | 6,377 |
| C | 1,532 | 1,192 | 964 | 807 | 1,092 |
| E | 2,681 | 2,201 | 1,886 | 1,671 | 2,077 |
| AVG | 3,682 | 2,844 | 2,377 | 2,057 | 2,675 |
| D | 428 | 325 | 257 | 210 | 294 |
| F | 824 | 764 | 719 | 682 | 745 |

### Test CE10-1.7

| Reference Picture Data Rate [MBit/s] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 21,725 | 18,390 | 16,972 | 16,309 | 18,236 |
| A2 | 31,791 | 27,046 | 24,550 | 22,891 | 26,365 |
| B | 8,254 | 6,869 | 6,185 | 5,691 | 6,684 |
| C | 1,381 | 1,161 | 1,011 | 914 | 1,103 |
| AVG | 8,143 | 6,848 | 6,152 | 5,698 | 6,649 |
| D | 380 | 318 | 277 | 252 | 303 |
| F | 994 | 967 | 941 | 929 | 957 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 8,270 | 6,352 | 5,467 | 4,863 | 6,113 |
| C | 1,407 | 1,145 | 948 | 802 | 1,052 |
| E | 2,567 | 2,152 | 1,868 | 1,665 | 2,036 |
| AVG | 3,420 | 2,737 | 2,331 | 2,040 | 2,583 |
| D | 396 | 315 | 254 | 211 | 286 |
| F | 790 | 742 | 704 | 672 | 726 |

### Test CE10-1.8

| Reference Picture Data Rate [MBit/s] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| RA | QP22 | QP27 | QP32 | QP37 | AVG |
| A1 | 23,775 | 18,941 | 17,159 | 16,382 | 18,863 |
| A2 | 34,125 | 27,900 | 24,934 | 23,041 | 27,195 |
| B | 9,033 | 7,304 | 6,426 | 5,775 | 7,034 |
| C | 1,496 | 1,224 | 1,041 | 925 | 1,152 |
| AVG | 8,853 | 7,175 | 6,313 | 5,757 | 6,932 |
| D | 412 | 338 | 289 | 256 | 319 |
| F | 1,023 | 985 | 951 | 935 | 973 |
| LB | QP22 | QP27 | QP32 | QP37 | AVG |
| B | 8,728 | 6,626 | 5,630 | 4,915 | 6,325 |
| C | 1,487 | 1,184 | 964 | 807 | 1,082 |
| E | 2,679 | 2,202 | 1,891 | 1,670 | 2,077 |
| AVG | 3,602 | 2,834 | 2,380 | 2,055 | 2,658 |
| D | 411 | 323 | 257 | 211 | 291 |
| F | 816 | 760 | 717 | 681 | 742 |

# Conclusion

Experimental results for multi-hypothesis inter prediction show that by using up to 3 hypotheses, average Y BD bit rate savings of -1.63% can be achieved for Random Access (RA) and -2.05% for Low-delay B (LB), relative to the VTM anchor.  When allowing up to 4 hypotheses, these gains are further increased to -1.91% and -2.29%, respectively.  When restricting the multi-hypothesis prediction to blocks that are larger than 8x8 luma samples, the coding gains are -1.55% and -2.04% for RA and LB with up to 3 hypotheses, and -1.84% and -2.27% for RA and LB with up to 4 hypotheses, all relative to the VTM anchor.  In the latter case, the measured encoder run time is 125% for RA and 138% for LB.  We therefore propose to consider multi-hypothesis inter prediction for adoption into the VTM.

# References

- C.-W. Hsu, M. Winken, X. Xiu, “Description of Core Experiment 10: Combined and multi-hypothesis prediction,” JVET-J1030, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).