| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0270 |
| --- | --- |

| Title: | CE10-related: Diagonal motion partitions on top of MTT block structure |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

In JVET-H0087, diagonal motion partitions (DMPs) were proposed on top of quadtree plus binary tree (QTBT) block structure. In this contribution, the same method for inter prediction is proposed on top of multi-type tree (MTT) block structure. In the proposed partitioning method, a coding unit (CU) is split into two diagonal motion partitions. The proposed method includes only two diagonal directions, but it can represent various arbitrary partitions on top of MTT block structure. The proposed DMPs can achieve 1.28% and 1.75% BD-rate reduction over VTM-1.1 for random access and low-delay B configurations, respectively.

# Introduction

To improve prediction performance, several arbitrary block partitioning methods have been proposed [1]-[5]. Especially, geometry motion partition (GEO) was proposed during JCT-VC activities, and it could achieve 1.9-5.0% BD-rate reduction over TMuC and HM [3]-[5]. In GEO, a square block is divided into two regions, and the boundary separating the two regions is defined by a straight line. The line is represented by two polar coordinates parameters ( and ). Figure 1 shows an example of arbitrary block partitions with GEO. Even though the meaningful coding gains could be achieved, but it was not adopted in HEVC standard. There were two main concerns that one was signaling overhead and the other was complexity increment.

Figure 1. An example of arbitrary block partitions with GEO

# Technical description

In JVET-H0087, diagonal motion partitions (DMPs) were proposed on top of quadtree plus binary tree (QTBT) block structure [6]. Moreover, same shapes of partitioning for skip and merge mode were also proposed in the last meeting [7]. In this contribution, the same method as [6] is proposed on top of multi-type tree (MTT) block structure. The proposed partitioning method uses only two diagonal partitions, left and right diagonal, for each coding unit (CU), but it can cover the various partitions in geometry motion partition (GEO). The motivation of the proposed partitioning method is that binary or ternary split CUs can help represent arbitrary motion partitions. It means that the binary or ternary split CUs can make rectangular shape blocks which have different block width and height compared with only using the quadtree (QT) block structure, and the various angle and distance parameters of GEO can be represented by only two diagonal partitions combined with MTT block structure. Therefore, the proposed DMPs can reduce signaling overhead with MTT split syntaxes.

## Representation of diagonally partitioned blocks

Figure 2 represents two diagonal motion partitions in the proposed method. As shown in Figure 2, the proposed method uses straight lines to split a CU into two diagonal motion partitions. In this contribution, the straight lines can be represented by simple linear equations as below:

- Left diagonal: 							(1)

- Right diagonal:							(2)

where (x,y) is the sample position within a CU, w is CU width, and h is CU height. This contribution assumes that the origin of 2D-coordinate is the top-left position of a CU.

(a) Left diagonal			 (b) Right diagonal

Figure 2. Two diagonal motion partitions in the proposed method

To split a CU into two diagonal partitions, it has a division problem of samples within the straight lines. The problem means that it cannot divide a sample into two regions. To solve the division problem, the proposed method applies mask based motion compensation and overlapping the samples within the straight lines. The mask represents that each sample in a CU is belonged to which diagonal partitions and, it is pre-defined according to block sizes and directions of diagonal partitions. The masks for the partition 0 (P0) and the partition 1 (P1) are evaluated as follows:

(3)

(a) DMP mask for P0			 		(b) DMP mask for P1

Figure 3. An example of right diagonal masks for a 16x8 CU

Figure 3 illustrates an example of right diagonal mask for a 16x8 CU. As shown in equation (3) and Figure 3, the proposed method applies overlapping the samples within the straight line. The final prediction of the CU is can be stated as:

(4)

where PDMP represents the final prediction with the proposed method, and PP0 and PP1 are the prediction blocks for partition 0 and partition 1, respectively. shift is a shift factor for scaling down, in this case, it is equal to 1.

## Overlapped block motion compensation (OBMC) for DMP

To decrease discontinuities on the prediction boundaries, the proposed method applies overlapped block motion compensation (OBMC). The proposed OBMC for DMP applies additional overlapping the samples nearby the straight line which splits a CU into two diagonal motion partitions. In this contribution, OBMC for DMP is always applied for every CU size. Figure 4 presents an example of right diagonal masks applying the proposed OBMC for DMP.

(a) DMP mask for P0					(b) DMP mask for P1

Figure 4. An example of right diagonal masks applying the proposed OBMC for DMP

## Modifications of AMVP and MERGE candidates

In this contribution, a CU coded by diagonal motion partition (DMP) applies modified AMVP and MERGE candidates. In Figure 6, the potential AMVP and MERGE candidates for DMP coded CU are presented. For the DMP coded CU, the proposed method removes un-correlated spatial and temporal candidates in AMVP and MERGE list. On the other hand, additional temporal candidates can be considered as shown in Figure 6. For each DMP block, the maximum number of candidates for AMVP list is equal to 2 and, that of MERGE is equal to 5.

- Left diagonal

- Right diagonal

Figure 6. Modified AMVP and MERGE candidates for diagonal motion partitions (Blue: PART_0, Green: PART_1, Cyan: PART_0 and PART_1)

## Constraints of block sizes for DMPs

In this contribution, the proposed method includes constraints of block sizes for DMPs to reduce complexity and bit overhead. The conditions of constraints as below:

- minSize = min(log2(width), log2(height))

- maxSize = max(log2(width), log2(height))

- If minSize > 2 and maxSize – minSize <3, DMPs can be applied;

- Otherwise, DMPs do not used.

It means that the minimum size of CU width and height should be larger than or equal to 8 and the difference of ratios of CU width and height should be smaller than 3.

## Signaling DMP parameters

To indicate DMP block and its motion data, the proposed DMP parameters as below:

- CU-level: DMP flag and DMP direction

- DMP-level: merge flag, merge index, motion data (inter prediction direction, reference indices, motion vector differences, and AMVP indices)

Figure 7 represents an example of replacing GEO parameters on top of MTT block structure. As shown in Figure 7, a DMP flag and a DMP direction combined with a block split flag syntax and a block split type syntax can be used to represent the diagonally partitioned block.

Figure 7. An example of replacing GEO parameters on top of MTT block structure

# Performance evaluation

The proposed method was evaluated under JVET common test conditions (CTC) [8]. The proposed method was integrated on VTM-1.1 and BMS-1.1. Table 1 and Table 2 show BD-rate reductions of the proposed diagonal motion partitions on top of MTT block structure against VTM-1.1 and BMS-1.1. With the proposed method, 1.28% and 1.75% BD-rate reductions are achieved for random access (RA) and low-delay B (LDB) configurations, respectively.

Table 1. BD-rate reduction of the proposed method for random access

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.71% | -1.22% | -1.66% | 222% |  |  |  |  |  |  |
| Class A2 | -1.41% | -1.75% | -1.72% | 228% |  |  |  |  |  |  |
| Class B | -1.13% | -1.63% | -1.47% | 221% |  |  |  |  |  |  |
| Class C | -1.79% | -2.54% | -2.66% | 208% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.28% | -1.81% | -1.87% | 219% |  |  |  |  |  |  |
| Class D | -1.39% | -1.74% | -1.85% | 196% |  |  |  |  |  |  |

Table 2. BD-rate reduction of the proposed method for low-delay B

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.31% | -1.27% | -0.59% | 218% |  |  |  |  |  |  |
| Class C | -1.68% | -2.13% | -2.11% | 210% |  |  |  |  |  |  |
| Class E | -2.57% | -0.31% | 1.40% | 245% |  |  |  |  |  |  |
| Overall | -1.75% | -1.32% | -0.60% | 222% |  |  |  |  |  |  |
| Class D | -1.38% | -1.19% | -1.34% | 190% |  |  |  |  |  |  |

# Conclusion

In this contribution, diagonal motion partitions on top of multi-type tree (MTT) block structure are proposed. The proposed method can achieve 1.28% and 1.75% BD-rate reduction over VTM-1.1 for random access (RA) and low-delay B (LDB) configuration.

# References

- O. Divorra, P. Yin, C. Gomila, “Geometry-adaptive block partitioning,” ITU-T Q.6/SG16 VCEG, VCEG-AF10, San Jose, US, Apr. 2007.

- J. Chen, S. Lee, K.-H. Lee, and W.-J. Han, “Object boundary based motion partition for video coding,” Proc. of IEEE International Symposium on Circuits and Systems, Nov. 2007.

- P. Chen, W. Chien, R. Panchal, and M. Karczewicz, “Geometry motion partition,” JCTVC-B049, 2nd JCT-VC meeting, Geneva, CH, Jul. 2010.

- P. Chen, L. Guo, and M. Karczewicz, “Geometry motion partition,” JCTVC-C301, 3rd JCT-VC meeting, Guangzhou, CN, Oct. 2010.

- E. Francois, P. Bordes, L. Guo, and M. Karczewicz, “CE2: Simplified geometry block partitioning,” JCTVC-D230, 4th JCT-VC meeting, Daegu, KR, Jan. 2011.

- Y. Ahn, H. Ryu, and D. Sim, “Diagonal motio partitions on top of QTBT block structure,” JVET-H0087, 8th JVET meeting, Macau, CN, Oct. 2017.

- C.-W. Hsu, M. Winken, and X. Xiu, “Decription of Core Experiment 10: Combined and multi-hypothesis prediction,” JVET-J1030, 10th JVET meeting, San Diego, US, April 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 10th JVET meeting, San Diego, US, April 2018.

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).