| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0272-v1 |
| --- | --- |

| Title: | CE6.1.2: Efficient Implementations of AMT with Transform Adjustment Stages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Yung-Hsuan Chao, Hilmi E. Egilmez, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE6 test 6.1.2. A unified transform approach is proposed to approximate the four types of DCTs and DSTs: DCT-8, DST-7, DCT-5, and DST-1 in AMT for primary transform, where each transform is approximated with an adjustment stage as pre-processing and a DCT-2 variant (sign changes and reflection of DCT-2). The results show approximately 2% and 5% reduction for encoding and decoding time against BMS1.1 anchor, and less than 0.05% loss in BD rate.

# CE Description

In this core experiment, contribution proposed in JVET-J0066 is implemented for block size 16x16, 32,32 and 64x64. The proposed transform method approximates different types of DCTs and DSTs in JEM7 (DST-7, DCT-8, DST-1, DCT-5) by applying adjustment stages to a transform in the DCT-2 family (DCT-2, DCT-3, DST-2, and DST-3), as shown in Fig. 1.

The adjustment stages are defined by sparse block-band orthogonal matrices. In this contribution, “6-taps” sparse block-band matrices are considered, i.e. there are only 6 non-zero elements in each row of the adjustment matrix. The adjustment is a pre-processing stage in the forward AMT transform (as shown in Fig. 1), and its inverse is a post-processing stage in the inverse AMT transform.

Figure 1: Adjustment stages to DCT-2 family

The DCT-2 transform in HEVC is implemented with the partial-butterfly structure, which is also employed in the implementation of DCT-2 family in this contribution. Besides, in the submitted software, only the elements needed for partial butterfly computation are stored. The memory needed is approximately 1/3 of the memory size needed to store full DCT-2 matrix.

# Experimental Results

The software is implemented based on BMS-1.1. The BMS-1.1 software with VTM configuration is used as VTM anchor and the proposed method with VTM configuration and configurations EMT=3 and EMTFast =3 is used as the test set. For BMS, the BMS-1.1 software with BMS configuration is used as BMS anchor and the proposed method with BMS configuration is used as the test set. The test is under CTC [3] and the results are summarized in Table 1 and 2.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 (EMT=3 by default) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.27% | -1.34% | -1.18% | 201% | 114% | -0.02% | 0.10% | -0.04% | 97% | 86% |
| Class A2 | -3.10% | -1.67% | -1.45% | 207% | 109% | 0.02% | 0.06% | 0.03% | 99% | 89% |
| Class B | -3.40% | -1.44% | -1.47% | 211% | 110% | 0.02% | 0.16% | 0.08% | 98% | 92% |
| Class C | -3.09% | -0.96% | -1.20% | 217% | 106% | -0.01% | 0.05% | 0.02% | 99% | 96% |
| Class E | -4.29% | -2.68% | -2.71% | 197% | 109% | 0.01% | 0.02% | -0.07% | 99% | 91% |
| Overall | -3.41% | -1.56% | -1.57% | 208% | 110% | 0.00% | 0.09% | 0.01% | 98% | 91% |
| Class D | -3.00% | -0.60% | -0.69% | 231% | 106% | 0.02% | 0.20% | 0.03% | 99% | 98% |
| Class F (optional) | -1.87% | -0.78% | -0.79% | 207% | 104% | -0.01% | 0.09% | 0.00% | 101% | 102% |

Table 1: BMS and VTM results for all intra (AI)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 (EMT=3 by default) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.91% | -0.06% | 0.07% | 161% | 105% | 0.02% | 0.11% | -0.05% | 93% | 95% |
| Class A2 | -2.01% | 0.16% | 0.38% | 150% | 104% | 0.02% | 0.08% | -0.04% | 95% | 97% |
| Class B | -2.48% | -0.23% | -0.50% | 152% | 103% | 0.01% | 0.15% | 0.08% | 95% | 98% |
| Class C | -2.05% | 0.17% | -0.15% | 158% | 104% | 0.03% | 0.12% | -0.26% | 96% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.16% | -0.01% | -0.11% | 155% | 104% | 0.02% | 0.12% | -0.06% | 95% | 98% |
| Class D | -1.81% | 0.38% | -0.08% | 156% | 102% | 0.04% | 0.14% | 0.11% | 98% | 100% |
| Class F (optional) | -1.51% | -0.16% | -0.18% | 166% | 103% | 0.02% | 0.02% | -0.08% | 97% | 99% |

Table 2: BMS and VTM results for random access (RA)

Simulation results in Table 1-2 show less than 0.05% loss in luma BD-rate with around 2% and 5% reduction for encoding and decoding time in BMS configuration. In the software, DST-7, DCT-8, DST-1 and DCT-5 of size 16x16, 32x32 are implemented with DCT variance with adjustment, namely, only DCT-2 matrices are needed to be stored instead of the full matrices for all five transform types in AMT.  All the transforms are optimized with partial butterfly structure, similar to the implementation of DCT-2 in HEVC. For adjustment stages, only the non-zero coefficients of band matrices are stored, i.e. 6xN elements for NxN blocks. Since the band matrices associated to DCT-8 is a reflection of the band matrices of DCT-7, only one of them is stored.

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said, H. Egilmez, V. Seregin, and M. Karczewicz, “Complexity Reduction for Adaptive Multiple Transforms (AMT) using Adjustment Stages” 9th meeting, San Diego, CA, USA, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).