| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0272-v2 |
| --- | --- |

| Title: | CE6.1.2: Efficient Implementations of AMT with Transform Adjustment Stages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Yung-Hsuan Chao, Hilmi E. Egilmez, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE6 test 6.1.2. A unified transform approach is proposed to approximate the four types of DCTs and DSTs: DCT-8, DST-7, DCT-5, and DST-1 in AMT for primary transform, where each transform is approximated with an adjustment stage as pre-processing and a DCT-2 variant (sign changes and reflection of DCT-2). The results show approximately 2% and 5% reduction for encoding and decoding time against BMS1.1 anchor, and less than 0.05% loss in BD rate.

# CE Description

In this core experiment, contribution proposed in JVET-J0066 is implemented for block size 16x16, 32,32 and 64x64. The proposed transform method approximates different types of DCTs and DSTs in JEM7 (DST-7, DCT-8, DST-1, DCT-5) by applying adjustment stages to a transform in the DCT-2 family (DCT-2, DCT-3, DST-2, and DST-3), as shown in Fig. 1.

The adjustment stages are defined by sparse block-band orthogonal matrices. In this contribution, “6-taps” sparse block-band matrices are considered, i.e. there are only 6 non-zero elements in each row of the adjustment matrix. The adjustment is a pre-processing stage in the forward AMT transform (as shown in Fig. 1), and its inverse is a post-processing stage in the inverse AMT transform.

Figure 1: Adjustment stages to DCT-2 family

The DCT-2 transform in HEVC is implemented with the partial-butterfly structure, which is also employed in the implementation of DCT-2 family in this contribution. Besides, in the submitted software, only the elements needed for partial butterfly computation are stored. The memory needed is approximately 1/3 of the memory size needed to store full DCT-2 matrix.

# Complexity Metrics

## Operation counts for individual transforms

The estimated operation counts for 1D forward/inverse transforms in this CE are shown in Table 1.

The operation counts include the number of operations needed for implementing DCT-2 family and adjustment stages (multiplication of sparse band matrices)

|  | # of Multiplication |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 8 | 64 | 182 | 534 | 1750 |
| DCT8 | 8 | 64 | 182 | 534 | 1750 |
| DST1 | 8 | 64 | 182 | 534 | 875 |
| DCT5 | 16 | 64 | 182 | 534 | 875 |
| DCT2 | 8 | 24 | 86 | 342 | 683 |

|  | # of Addition |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 15 | 65 | 210 | 594 | 1842 |
| DCT8 | 15 | 65 | 210 | 594 | 1842 |
| DST1 | 13 | 65 | 210 | 594 | 1192 |
| DCT5 | 17 | 65 | 210 | 594 | 1192 |
| DCT2 | 13 | 37 | 113 | 401 | 807 |

|  | # of Shift |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 5 | 9 | 34 | 66 | 98 |
| DCT8 | 5 | 9 | 34 | 66 | 98 |
| DST1 | 5 | 9 | 34 | 66 | 98 |
| DCT5 | 5 | 9 | 34 | 66 | 98 |
| DCT2 | 5 | 9 | 17 | 33 | 33 |

Table 1: Estimated operation counts for 1D forward/inverse transforms

## Memory requirements for storing transform cores

The estimated number of elements and bits needed for transform are shown in Table 2.

|  | # of elements in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 16 | 64 | 96 | 192 | 384 | 752 |
| DCT8 | 16 | 64 | 0 | 0 | 0 | 80 |
| DST1 | 16 | 64 | 96 | 192 | 384 | 752 |
| DCT5 | 16 | 64 | 96 | 192 | 384 | 752 |
| DCT2 | 16 | 64 | 86 | 342 | 1366 | 1874 |
| Summary (for each size) | 80 | 320 | 374 | 918 | 2518 | 4210 |

|  | # of bits in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 160 | 640 | 960 | 1920 | 3840 | 7520 |
| DCT8 | 160 | 640 | 0 | 0 | 0 | 800 |
| DST1 | 160 | 640 | 960 | 1920 | 3840 | 7520 |
| DCT5 | 160 | 640 | 960 | 1920 | 3840 | 7520 |
| DCT2 | 160 | 640 | 860 | 3420 | 13660 | 18740 |
| Summary (for each size) | 800 | 3200 | 3740 | 9180 | 25180 | 42100 |

Table 2: Estimated number of elements and bits for each transform matrix

Note that the band matrix associated with DCT-8 is a reflection of the band matrix of DCT-7, and therefore no additional memory is needed to store its band matrices.

## Specify the minimum bit-precision required by the transforms

Same as for BMS

## Specify if a transform requires multiple iterations

No additional iteration is required from BMS

## Other operations and memory requirements relevant to the proposed tool

None

## Encoder and decoder times

Listed in the experimental results in section 3.

## List of all combinations of transform and transforms block sizes used for primary transformation

Same as for BMS

## Provide analysis of implementation and arithmetic commonalities of proposed transforms

The pre-processing and post processing with adjustment stages in encoder and decoder are implemented with transform multiplication (only non-zero elements are involved)

For DCT-2, the transform is implemented with partial butterflies as in BMS.

For other types of DCT/DSTs, partial butterflies are used for DCT-2 family applied for block size 16x16, 32x32, and 64x64, while for block size 8x8, matrix multiplication is used. Same as for BMS, 4x4 transforms for DST-1, DST-7, and DCT-8 are implemented with partial butterflies and 4x4 DCT-5 is implemented with matrix multiplication. In Table 4, the implementation types described for each transform are summarized.

| Size | 2 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- |
| DST7 |  | PB | MM | PB | PB | PB |
| DCT8 |  | PB | MM | PB | PB | PB |
| DST1 |  | PB | MM | PB | PB | PB |
| DCT5 |  | MM | MM | PB | PB | PB |
| DCT2 | PB | PB | PB | PB | PB | PB |

Table 4: Analysis of implementation (PB: partial butterflies, MM: matrix multiplication)

# Experimental Results

The software is implemented based on BMS-1.1. The BMS-1.1 software with VTM configuration is used as VTM anchor and the proposed method with VTM configuration and configurations EMT=3 and EMTFast =3 is used as the test set. For BMS, the BMS-1.1 software with BMS configuration is used as BMS anchor and the proposed method with BMS configuration is used as the test set. The test is under CTC [3] and the results are summarized in Tables 1-3.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 (EMT=3 by default) |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.27% | -1.34% | -1.18% | 201% | 114% | -0.02% | 0.10% | -0.04% | 97% | 86% |
| Class A2 | -3.10% | -1.67% | -1.45% | 207% | 109% | 0.02% | 0.06% | 0.03% | 99% | 89% |
| Class B | -3.40% | -1.44% | -1.47% | 211% | 110% | 0.02% | 0.16% | 0.08% | 98% | 92% |
| Class C | -3.09% | -0.96% | -1.20% | 217% | 106% | -0.01% | 0.05% | 0.02% | 99% | 96% |
| Class E | -4.29% | -2.68% | -2.71% | 197% | 109% | 0.01% | 0.02% | -0.07% | 99% | 91% |
| Overall | -3.41% | -1.56% | -1.57% | 208% | 110% | 0.00% | 0.09% | 0.01% | 98% | 91% |
| Class D | -3.00% | -0.60% | -0.69% | 231% | 106% | 0.02% | 0.20% | 0.03% | 99% | 98% |
| Class F (optional) | -1.87% | -0.78% | -0.79% | 207% | 104% | -0.01% | 0.09% | 0.00% | 101% | 102% |

Table 1: BMS and VTM results for all intra (AI)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 (EMT=3 by default) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.91% | -0.06% | 0.07% | 161% | 105% | 0.02% | 0.11% | -0.05% | 93% | 95% |
| Class A2 | -2.01% | 0.16% | 0.38% | 150% | 104% | 0.02% | 0.08% | -0.04% | 95% | 97% |
| Class B | -2.48% | -0.23% | -0.50% | 152% | 103% | 0.01% | 0.15% | 0.08% | 95% | 98% |
| Class C | -2.05% | 0.17% | -0.15% | 158% | 104% | 0.03% | 0.12% | -0.26% | 96% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.16% | -0.01% | -0.11% | 155% | 104% | 0.02% | 0.12% | -0.06% | 95% | 98% |
| Class D | -1.81% | 0.38% | -0.08% | 156% | 102% | 0.04% | 0.14% | 0.11% | 98% | 100% |
| Class F (optional) | -1.51% | -0.16% | -0.18% | 166% | 103% | 0.02% | 0.02% | -0.08% | 97% | 99% |

Table 2: BMS and VTM results for random access (RA)

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 (EMT=3 by default) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.91% | 0.76% | 0.49% | 154% | 101% | 0.00% | -0.05% | -0.09% | 94% | 94% |
| Class C | -1.29% | 0.92% | 0.88% | 153% | 104% | -0.06% | 0.15% | 0.05% | 96% | 96% |
| Class E | -1.22% | 1.60% | 1.23% | 138% | 100% | -0.28% | -0.52% | 0.25% | 95% | 99% |
| Overall | -1.53% | 1.02% | 0.81% | 150% | 102% | -0.09% | -0.10% | 0.04% | 94% | 96% |
| Class D | -1.23% | 0.67% | 0.65% | 144% | 100% | 0.07% | 0.23% | 0.08% | 97% | 98% |
| Class F (optional) | -1.17% | -0.26% | -0.09% | 155% | 100% | -0.12% | 0.27% | -0.17% | 95% | 96% |

Table 3: BMS and VTM results for low delay B (LDB)

Simulation results in Table 1-2 show less than 0.05% loss in luma BD-rate with around 2% and 5% reduction for encoding and decoding time in BMS configuration.

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said, H. Egilmez, V. Seregin, and M. Karczewicz, “Complexity Reduction for Adaptive Multiple Transforms (AMT) using Adjustment Stages” 9th meeting, San Diego, CA, USA, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).