| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0273-v1 |
| --- | --- |

| Title: | CE5-related: Implementation considerations for entropy coding engine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 frank@bossentech.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Minor modifications are proposed to the entropy coding core engine such as to enable a wider variety of implementations. These newly enabled implementations may be beneficial in both SW and HW. For example, it is claimed that SW implementations with reduced per bin cycle counts are enabled. It is asserted that the proposed changes do not noticeably impact compression efficiency.

# Introduction

AVC and HEVC use a common arithmetic coding engine. This engine was designed to rely on lookup tables for subinterval size computation and probability state update. Improvements to compression efficiency have been studied in CE5 [1] where higher precision arithmetic and no lookup tables are used for state updates. Modifications relative to techniques studied in CE5 are proposed here.

# Proposed changes

Two minor changes are proposed: the first change relates to the count update function and the second change relates to the subinterval range computation.

## Count update function

Experiments in CE5 typically define a state update rule for probability estimation as:

if (symbol)

q = q + ((32768 − q) >> n)

else

q = q − (q >> n)

where q is a count or probability estimate and n is an adaptation rate.

Expressed as such it includes a branch and two distinct paths which is generally undesirable in SW as it may lead to branch misprediction. This misprediction issue may be compounded by the fact that the value of “symbol” is needed early in the computation, increasing the chances that a branch is executed speculatively. For example, a function implementing the above rule would typically compile to:

testl   %esi, %esi

je      L7

movl    $32768, %eax

subl    %edi, %eax

shrx    %edx, %eax, %eax

addl    %edi, %eax

ret

L7:

shrx    %edx, %edi, %edx

movl    %edi, %eax

subl    %edx, %eax

ret

In HW a multiplexer would typically be used to select one of two outcomes, which isn’t ideal either.

An alternative branchless implementation can be achieved with:

t = q − ((32768 − (1 << n) + 1) & symbol)

q = q − (t >> n)

where the value of “symbol” is assumed to be either 0 or −1.

While such an implementation avoids issues associated with branching, it requires the value of “symbol” to be known early in the computation. Indeed, four operations need to be executed in sequence once “symbol” is available: &, −, >>, and −. This is disadvantageous in a decoder where latency matters.

It is proposed to slightly modify the update rule to the following, by replacing the constant 32768 by 32767:

if (symbol)

q = q + ((32767 − q) >> n)

else

q = q − (q >> n)

All the implementations described above can still be realized. Additional implementations, as described below, become realizable.

For example, one could still use conditional execution while delaying the use of the “symbol” variable:

q = q − (q >> n)

if (symbol)

q = q + (32767 >> n)

The use of the “symbol” value can similarly be delayed in the branchless form:

q = q − (q >> n)

q = q + ((32767 >> n) & symbol)

In this case only two operations (instead of four) are dependent on the value of “symbol”: & and +.

Also note that 32767 − x may be slightly easier to compute in HW than 32768 − x as each bit of the result can be computed independently (no carry propagation).

## Subinterval range computation

Experiments in CE5 include methods for deriving the subinterval range using either a table lookup or an actual multiplication. Some methods support both implementations as such a feature is indeed desirable. The lookup table shouldn’t be too large. A table with 32x8 entries seems to be a popular choice and matches the number of entries used in AVC and HEVC. Implementations using an actual multiplication should also be as simple as possible.

Consider having probability estimates qa and qb resulting from two different estimators, each estimate being 15 bits wide, and a range value r which is 9 bits wide (same as AVC and HEVC).

Let us further define q as qa+qb, and qLPS as q ^ ((q & 32768) ? 65535 : 0). Note that this can easily be implemented in C if the type of q is defined as int16_t: qLPS = q ^ (q >> 15). Additionally, let r3 = r >> 5 and q5 = qLPS >> 10.

The subinterval range rLPS may then be computed as:

rLPS = ((2 * r3 + 1) * (2 * q5 + 1) + 4) >> 3

With additional clipping to the range 4 .. 32 × (r3 − 4), one obtains the table used in experiment CE5.B2.

It is proposed to modify the computation of the range and use the following expression which is simpler and doesn’t require any clipping:

rLPS = (r3 * q5 >> 1) + k

where k is a constant that should have two properties:

- It is larger than 0 such that rLPS cannot take value 0;

- It should be such that r − rLPS cannot be less than 128. This property guarantees that during renormalization after processing an MPS, the range register is shifted left by at most one bit position. The worst case happens for r = 256 and qLPS = 32767.

The value of k that satisfies the second property is: 256 − 124 + k ≥ 128. Thus, k must be ≤ 4.

Considering the equation from CE5.B2 mentioned above, k should be E[0.5 × (0.5× r3 + 0.5× q5 + 0.25 + 1)], or about 0.5 × (0.5×11.5 + 0.5×15.5 + 0.25 + 1) = 7.375. However, this value exceeds the previously established bound, and we thus settle for k = 4.

# Results

Two types of results are presented: compression efficiency and throughput.

## Compression efficiency

The following results were obtained with the software provided in CE/J_SanDiego/CE5/CE5-1.2.0. The reference “my ref” below include the software with the following parameters: CABACEngine=4 and CIPF=0.

Results of the probability update modification:

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 VTM Cfg |  |  |  |  | Over my ref |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.79% | -0.72% | -0.97% | 100% | 98% | 0.01% | 0.01% | -0.03% | 96% | 95% |
| Class A2 | -0.88% | -0.53% | -0.67% | 104% | 102% | 0.01% | 0.03% | -0.03% | 91% | 93% |
| Class B | -0.74% | -0.60% | -0.45% | 105% | 101% | 0.00% | -0.05% | 0.04% | 93% | 94% |
| Class C | -0.71% | -0.67% | -0.52% | 108% | 105% | -0.01% | -0.03% | 0.09% | 83% | 84% |
| Class E | -0.71% | -0.64% | -0.52% | 111% | 110% | 0.00% | 0.00% | 0.03% | 85% | 87% |
| Overall | -0.76% | -0.63% | -0.60% | 105% | 103% | 0.00% | -0.01% | 0.03% | 89% | 90% |
| Class D | -0.70% | -0.66% | -0.61% | 100% | 99% | 0.01% | -0.05% | -0.12% | 100% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 VTM Cfg |  |  |  |  | Over my ref |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | -0.75% | -1.01% | 110% | 103% | -0.03% | 0.03% | -0.07% | 101% | 100% |
| Class A2 | -0.62% | -0.32% | -0.45% | 112% | 104% | -0.01% | 0.00% | 0.02% | 106% | 103% |
| Class B | -0.66% | -0.46% | -0.39% | 109% | 106% | 0.00% | -0.06% | -0.10% | 100% | 99% |
| Class C | -0.71% | -0.62% | -0.34% | 107% | 105% | 0.01% | 0.15% | 0.17% | 103% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.66% | -0.53% | -0.51% | 109% | 105% | -0.01% | 0.03% | 0.00% | 102% | 101% |
| Class D | -0.63% | -0.17% | -0.14% | 103% | 103% | 0.06% | -0.03% | -0.02% | 102% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 VTM Cfg |  |  |  |  | Over my ref |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.65% | 0.56% | 0.80% | 101% | 98% | 0.00% | 0.05% | 0.03% | 99% | 99% |
| Class C | -0.66% | -0.49% | -0.63% | 107% | 104% | 0.02% | 0.01% | -0.24% | 100% | 99% |
| Class E | -0.18% | 0.57% | 1.02% | 99% | 99% | 0.06% | 0.52% | 0.35% | 99% | 99% |
| Overall | -0.53% | 0.21% | 0.38% | 103% | 100% | 0.02% | 0.15% | 0.02% | 99% | 99% |
| Class D | -0.73% | -0.04% | 0.54% | 109% | 106% | 0.02% | -0.53% | -0.05% | 107% | 106% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Results of the combined probability update and subrange interval computation modifications:

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 VTM Cfg |  |  |  |  | Over my ref |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.77% | -0.70% | -0.95% | 103% | 96% | 0.03% | 0.03% | -0.01% | 98% | 93% |
| Class A2 | -0.86% | -0.50% | -0.64% | 107% | 99% | 0.03% | 0.06% | -0.01% | 93% | 90% |
| Class B | -0.71% | -0.57% | -0.43% | 111% | 102% | 0.03% | -0.03% | 0.07% | 98% | 94% |
| Class C | -0.67% | -0.63% | -0.48% | 104% | 103% | 0.02% | 0.01% | 0.12% | 80% | 82% |
| Class E | -0.68% | -0.62% | -0.49% | 105% | 105% | 0.03% | 0.03% | 0.06% | 80% | 84% |
| Overall | -0.73% | -0.60% | -0.57% | 106% | 101% | 0.03% | 0.01% | 0.05% | 90% | 89% |
| Class D | -0.66% | -0.62% | -0.57% | 113% | 108% | 0.05% | -0.01% | -0.08% | 114% | 110% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 VTM Cfg |  |  |  |  | Over my ref |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.62% | -0.72% | -0.99% | 106% | 104% | 0.00% | 0.06% | -0.04% | 97% | 100% |
| Class A2 | -0.59% | -0.29% | -0.42% | 106% | 101% | 0.02% | 0.03% | 0.05% | 100% | 99% |
| Class B | -0.64% | -0.43% | -0.37% | 107% | 104% | 0.02% | -0.04% | -0.07% | 99% | 97% |
| Class C | -0.67% | -0.58% | -0.29% | 106% | 104% | 0.06% | 0.20% | 0.21% | 102% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.63% | -0.50% | -0.48% | 106% | 103% | 0.03% | 0.06% | 0.03% | 99% | 99% |
| Class D | -0.59% | -0.12% | -0.10% | 102% | 102% | 0.10% | 0.01% | 0.03% | 101% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 VTM Cfg |  |  |  |  | Over my ref |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.62% | 0.59% | 0.82% | 98% | 96% | 0.03% | 0.08% | 0.06% | 96% | 98% |
| Class C | -0.61% | -0.44% | -0.58% | 102% | 98% | 0.07% | 0.06% | -0.19% | 96% | 92% |
| Class E | -0.17% | 0.57% | 1.03% | 96% | 95% | 0.07% | 0.53% | 0.36% | 96% | 94% |
| Overall | -0.50% | 0.24% | 0.41% | 99% | 96% | 0.05% | 0.18% | 0.05% | 96% | 95% |
| Class D | -0.68% | 0.00% | 0.58% | 100% | 97% | 0.06% | -0.49% | 0.00% | 99% | 97% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

These results demonstrate that there is no noticeable impact on compression efficiency from the proposed modifications.

## Complexity and throughput

The following setup is used to measure complexity and throughput of entropy decoder implementations. An arithmetic decoder processes a bitstream comprising N bins, where the context associated with each bin is determined by counting the number of bins with value ‘1’ within the last 15 processed bins (16 contexts are used). The length N of the bin stream varies from 26 to 218. Each bin stream is randomly generated. The figure below shows how the number of CPU cycles per bin varies with the sequence length. Several implementations are considered, with combinations of table-based and multiplier-based subinterval range computation, probability updates with and without branches, and probability updates with early or late use of ‘symbol’.

Only the “table, branch, early” and “table, branchless, early” implementations are realizable without the modifications proposed in this document. All implementations are realizable with the proposed modifications.

In each case the sequence is decoded multiple times such that a total of 224 bins are decoded. For short sequences branch prediction can be very accurate, as the same sequence is decoded multiple times. As the sequence length increases the accuracy of the branch predictor decreases. The increased frequency of mispredictions results in a noticeable increase of the cycle count of about 35%, as seen in the “table, branch, early” and “mult, branch, early” curves.

The same phenomenon is not observed with the “late branch” implementations. Inspection of the assembly code reveals that a conditional move is used instead of a branch, which would explain the difference in behaviour.

It should also be noted that the implementations using a straight multiplication are generally slightly faster than their equivalents that use a table lookup, and that branchless implementations with a late use of “symbol” are faster than branchless implementations with an early use of “symbol”. These observations support the adoption of the proposed modifications.

It should further be noted that all implementations are highly optimized yielding throughputs in excess of 100Mbit/s.

# Summary

Two changes are proposed:

- Modify the constant 2b to 2b−1 (e.g., 32768 to 32767) in the probability estimate update function

- Modify the subinterval range computation for the LPS symbol to ((r >> 5) * (qLPS >> (b − 5)) >> 1) + 4. Note that, alternatively, this equation can be implemented using a 32×8×8 = 2048 bit lookup table.

Both modifications enable a wider range of implementations without having a noticeable impact on compression efficiency.

# References

[1] Core Experiment 5: Arithmetic Coding Engine, JVET-J1025, April 2018.

# Patent rights declaration(s)

Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).