| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0274-v4 |
| --- | --- |

| Title: | CE2 related: Reduced complexity bilateral filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström Per Wennersten Jack Enhorn Du Liu Kenneth Andersson Rickard Sjöberg Torshamnsgatan 23 164 80 Stockholm Sweden | Tel: Email: | +46702672192 {jacob.strom, per.wennersten, jack.enhorn, du.liu, kenneth.r.andersson rickard.sjoberg}@ ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a modified version of the bilateral filter from JVET-F0034, JVET-F0096 and JVET-J0021. The main modification is a reduction in the size of the look-up table (LUT) that is used to store the filter coefficients. The contribution claims to reduce the total size of the stored variables (including the LUT) from 2783 bytes in JVET-F0096 to 816 bytes, a reduction of 71%. The proposal states that this is achieved by approximating the 34 rows in the LUT (one row is used for every qp) by four rows plus shifting. The contribution further claims that the need for a division table is removed by using the approximation proposed in JVET-J0021. The non-local filtering for inter blocks proposed in JVET-J0021 is reportedly also used. The BD rate figures for an implementation in BMS 1.0 are reported to be -0.33% / -0.52% / -0.60% for AI/RA/LD respectively, and the VTM figures are reported to be -0.33% / -0.81% / -0.59% for AI/RA/LD respectively. The BMS decoder run times are reported to be 101% / 101% / 101% for AI/RA/LD and the VTM decoder run times are reported to be 104% / 103% / 103%.

# Introduction

The bilateral filter presented in JVET-D0069, JVET-E0032, JVET-F0034 and JVET-F0096 was included into the JEM test model at the Hobart meeting. Already then it was raised that for some hardware implementations, the filter may be situated in the critical path for small intra blocks, since prediction is done from filtered blocks. At the last meeting in San Diego, this point was raised again, and we had a discussion with one of the interested parties to see what can be done about it.

When decoding a small intra block such as a 4×4 block, the decoder may have to use four filtered samples from the current block to predict the next block. In order not to increase the latency too much, a hardware implementer may therefore choose to filter these four samples in parallel. As is seen in Appendix A, this will require seven instantiations of the LUT and four instantiations of the division table. Therefore, if implementing JVET-F0096, the total table size may therefore be 7*2202+4*576 = 17718 bytes. This amount of on-chip storage was identified as being costly by one of the interested parties.

Therefore, in this proposal, we have concentrated our efforts on reducing the size of the LUT. To filter four samples with the current proposal, only 7*816 = 5712 bytes of LUT storage is needed for the seven instantiations needed. This is a substantial reduction of the needed LUT storage, and one that should reduce the cost of a hardware implementation of the filter.

# Structure of the LUT

Following JVET-J0021/CE2-1.3, we use an approximation of the bilateral filter in order to avoid the division table. This is done slightly differently from the implementation of JVET-J0021. For details on how this is done, see Appendix B. The resulting filter equation for a non-border sample of an intra block is

IF=IC+dσd×ρqpΔIA×ΔIA+ρqpΔIB×ΔIB+ρqpΔIL×ΔIL+ρqpΔIR×ΔIR,    (Eq. 1)

where

IC is the intensity of the center sample (the sample to be filtered)

IA, IB, IL and IR are the intensity values of the sample above, below, left and right respectively

ΔIA=IA-IC, ΔIB=IB-IC, ΔIL=IL-IC, ΔIR=IR-IC, and where

dσd=256255 e- 12σd21+4e- 12σd2      (Eq. 2)

is a one-dimensional look-up table of four values and where

ρqpΔI=255256e- ΔI28qp-172=LUT(qp,ΔI)    (Eq. 3)

is a two-dimensional look-up table. For quantization parameter (qp) values of 17 and lower, the filtering is not used, so there are 34 different possible qp values for which filtering is done and this is regardless of the bit depth. For 10-bit data, the value ΔI can vary from 0 to 1023. In Figure 1, we have plotted the LUT value for three different qps, where 8 fractional bits are used for the LUT value.

Figure 1. The look-up values LUT(qp,x) for three different values of qp, representing three rows in the two-dimensional LUT. Note how the curves look like stretched versions of each other.

From the diagram in Figure 1 it looks like the curve for qp = 34 is just a stretched version of that for qp = 51. This turns out to be exactly the case. As is seen in Appendix C, the formula for LUT34, ΔI=LUT(51, 2ΔI), i.e., we can get the blue curve by taking every second point in the black curve. In general we have that

LUTqp, ΔI=LUT51, c*ΔI,    (Eq. 4)

where

c=51-17qp-17.    (Eq. 5)

Thus, instead of storing every row in the LUT, it is possible to only store the last row LUT(51, ΔI) and use Equation 4 and 5 to get the LUT value for any value of qp. However, this solution comes at the cost of a multiplication between c and ΔI, which may be costly in terms of silicon area. Therefore, we restrict the c-value to a power of two, for which the multiplication is a trivial bit-shift. As an example, if we only store row 33 (i.e., we store  LUT(33,ΔI)), we can approximate the rows for qp=18, 19, 21, 25, 33 and 49 exactly using c-values of 16, 8, 4, 2, 1 and 0.5. However, this will give a big gap between qp 33 and 49, where the rows will not be approximated well. For this reason we propose instead to store the following four different rows using 8 fractional bits:

// LUT(50,ΔI): 
UChar LUT50 = {255, 255, 255, 255, 255, 254, 254, 254, 253, 253, 252, 251, 251, 250, 249, 248, 248, 247, 246, 245, 244, 242, 241, 240, 239, 237, 236, 235, 233, 232, 230, 228, 227, 225, 223, 222, 220, 218, 216, 214, 212, 210, 208, 206, 204, 202, 200, 198, 196, 194, 191, 189, 187, 185, 182, 180, 178, 176, 173, 171, 169, 166, 164, 162, 159, 157, 155, 152, 150, 148, 145, 143, 141, 138, 136, 134, 131, 129, 127, 125, 122, 120, 118, 116, 113, 111, 109, 107, 105, 103, 101, 99, 97, 94, 92, 90, 89, 87, 85, 83, 81, 79, 77, 75, 74, 72, 70, 69, 67, 65, 64, 62, 60, 59, 57, 56, 54, 53, 52, 50, 49, 47, 46, 45, 44, 42, 41, 40, 39, 38, 37, 36, 35, 33, 32, 31, 31, 30, 29, 28, 27, 26, 25, 24, 24, 23, 22, 21, 21, 20, 19, 19, 18, 17, 17, 16, 16, 15, 15, 14, 14, 13, 13, 12, 12, 11, 11, 10, 10, 10, 9, 9, 9, 8, 8, 8, 7, 7, 7, 6, 6, 6, 6, 5, 5, 5, 5, 5, 4, 4, 4, 4, 4, 4, 3, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0};  // 235 values

// LUT46,ΔI: 
UChar LUT46 = {255, 255, 255, 255, 254, 254, 254, 253, 253, 252, 251, 250, 250, 249, 248, 247, 245, 244, 243, 242, 240, 239, 237, 236, 234, 232, 231, 229, 227, 225, 223, 221, 219, 217, 215, 213, 210, 208, 206, 203, 201, 199, 196, 194, 191, 189, 186, 184, 181, 178, 176, 173, 171, 168, 165, 163, 160, 157, 155, 152, 149, 147, 144, 141, 139, 136, 133, 131, 128, 126, 123, 121, 118, 115, 113, 111, 108, 106, 103, 101, 98, 96, 94, 92, 89, 87, 85, 83, 81, 79, 77, 74, 72, 71, 69, 67, 65, 63, 61, 59, 58, 56, 54, 53, 51, 50, 48, 47, 45, 44, 42, 41, 40, 38, 37, 36, 35, 33, 32, 31, 30, 29, 28, 27, 26, 25, 24, 23, 22, 21, 21, 20, 19, 18, 18, 17, 16, 16, 15, 14, 14, 13, 13, 12, 12, 11, 11, 10, 10, 9, 9, 9, 8, 8, 8, 7, 7, 7, 6, 6, 6, 5, 5, 5, 5, 4, 4, 4, 4, 4, 3, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0};  // 206 values

// LUT42,ΔI:
UChar LUT42 ={ 255, 255, 255, 255, 254, 254, 253, 253, 252, 251, 250, 249, 248, 247, 245, 244, 242, 241, 239, 237, 235, 233, 231, 229, 227, 225, 223, 220, 218, 216, 213, 210, 208, 205, 202, 200, 197, 194, 191, 188, 185, 182, 179, 176, 173, 170, 167, 164, 161, 158, 155, 152, 148, 145, 142, 139, 136, 133, 130, 127, 124, 121, 118, 115, 112, 110, 107, 104, 101, 98, 96, 93, 90, 88, 85, 83, 80, 78, 76, 73, 71, 69, 66, 64, 62, 60, 58, 56, 54, 52, 50, 49, 47, 45, 44, 42, 40, 39, 37, 36, 35, 33, 32, 31, 29, 28, 27, 26, 25, 24, 23, 22, 21, 20, 19, 18, 17, 17, 16, 15, 14, 14, 13, 12, 12, 11, 11, 10, 10, 9, 9, 8, 8, 7, 7, 7, 6, 6, 6, 5, 5, 5, 5, 4, 4, 4, 4, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0}; // 178 values

// LUT38,ΔI:
UChar LUT38 = { 255, 255, 255, 254, 254, 253, 252, 251, 250, 249, 248, 246, 245, 243, 241, 239, 237, 235, 233, 230, 228, 225, 222, 219, 217, 214, 211, 207, 204, 201, 198, 194, 191, 187, 184, 180, 177, 173, 169, 166, 162, 158, 155, 151, 147, 144, 140, 136, 133, 129, 126, 122, 118, 115, 112, 108, 105, 102, 98, 95, 92, 89, 86, 83, 80, 77, 74, 71, 69, 66, 64, 61, 59, 56, 54, 52, 50, 47, 45, 43, 42, 40, 38, 36, 35, 33, 31, 30, 28, 27, 26, 24, 23, 22, 21, 20, 19, 18, 17, 16, 15, 14, 13, 13, 12, 11, 11, 10, 9, 9, 8, 8, 7, 7, 6, 6, 6, 5, 5, 5, 4, 4, 4, 4, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0}; // 150 values

In order not to have to store more than the first zero, we also need to keep track of how long each of these four LUT rows is, which is another four bytes:

sizeRows[4] = {234, 206, 178, 150};

For every row, we also need a two-bit number to keep track of which of the four base-rows to use;

baseRow[34] = {3, 0, 2, 0, 3, 2, 1, 0, 0, 3, 3, 2, 2, 1, 1, 0, 0, 0, 3, 3, 3, 3, 2, 2, 2, 2, 1, 1, 1, 1, 0, 0, 0, 0},

and finally we also need to know how many bits to shift

shiftValue[34] = {4, 4, 3, 3, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0};

which is a vector of three-bit numbers.

# Intra filtering

To filter the samples of an intra block, first the correct LUT row must be selected.

UChar **row_ptrs = {LUT50, LUT46, LUT42, LUT38};
UChar current_row = baseRow[qp];
UChar *LUTptr = row_ptrs[current_row];
maxVal = size_rows[current_row];
shift = shiftValue[current_row];

Then every sample is filtered using Equation 1:

deltaIA = IA – IC;
influenceA = LUTptr[min(maxVal, abs(deltaIA) ≪ shift)]*deltaIA;
deltaIB = IB – IC;
influenceB = LUTptr[min(maxVal, abs(deltaIB) ≪ shift)]*deltaIB;
deltaIL = IL – IC;
influenceL = LUTptr[min(maxVal, abs(deltaIL) ≪ shift)]*deltaIL;
deltaIR = IR – IC;
influenceR = LUTptr[min(maxVal, abs(deltaIR) ≪ shift)]*deltaIR;

influenceSum = influenceA + influenceB + influenceL + influenceR;

IF = IC + (distanceFactor * influenceSum + 131072) ≫ 18

The influence values can be reused between samples so on average less than two table look-ups are needed per sample, rather than the four seen above.

For intra filtering, samples outside the block are regarded unavailable. This means that for samples situated on the border or in a corner of the block, some of the influence-variables will not be available. As an example, for a sample on the right edge, influenceR will not be available. In that case it is simply omitted from the calculation of the sum:

influenceSum = influenceA + influenceB + influenceL;

A different distanceFactor will be used to compensate for the smaller sum. Thus the distanceFactor depends on whether it is a border/corner sample. The distanceFactor also depends on the size of the block. It is fetched from the following table.

|  | σd=0.80 for 4x4 blocks | σd=0.59 4xn and nx4, n>4 | σd=0.56 8xn and nx8, n ≥ 8 | σd=0.46 16xn and nx16, n ≥ 16 |
| --- | --- | --- | --- | --- |
| dσdintra | 166 | 125 | 115 | 70 |
| dσdintra border | 198 | 143 | 130 | 75 |
| dσdintra corner | 246 | 166 | 148 | 81 |

# Inter filtering

For the inter filtering, non-local filtering from JVET-J0021 is used. The main difference is that instead of using the delta value for the look-up table, an averaged filtered value is used.

Assume that we want to filter the sample a3,3 shown below.

| a1,1 | a1,2 | a1,3 | a1,4 | a1,5 | ⋯ |
| --- | --- | --- | --- | --- | --- |
| a2,1 | a2,2 | a2,3 | a2,4 | a2,5 |  |
| a3,1 | a3,2 | a3,3 | a3,4 | a3,5 |  |
| a4,1 | a4,2 | a4,3 | a4,4 | a4,5 |  |
| a5,1 | a5,2 | a5,3 | a5,4 | a5,5 |  |
| ⋮ |  |  |  |  | ⋱ |

If this were the intra filter described above, deltaR would be calculated as ΔIR=a3,4-a3,3, and the absolute value of that would be used for the look-up:

deltaIR = a34-a33;
influenceR = LUTptr[min(maxVal, abs(deltaIR) ≪ shift)]*deltaIR;

However, following JVET-J0021/CE-2.1.3, we instead calculate the average absolute value;

NLR=19i=2..4j=2..4ai+1,j-ai,j ,  (Eqn 6)

and this is the value used for the lookup:

NL_R = ( (abs(a23-a22) + abs(a24-a23) + abs(a25-a24) +
          abs(a33-a32) + abs(a34-a33) + abs(a35-a34) +
          abs(a43-a42) + abs(a44-a43) + abs(a45-a44)) * 114 ) >> 10;
deltaIR = a34-a33;
influenceR = LUTptr[min(maxVal, abs(NL_R) ≪ shift)]*deltaIR;

Following again JVET-J0021/CE-2.1.3, 114/1024 is used to approximate 1/9. The influence from the other directions are treated similarly:

NL_A = ( (abs(a12-a22) + abs(a13-a23) + abs(a14-a24) +
          abs(a22-a32) + abs(a23-a33) + abs(a34-a35) +
          abs(a32-a42) + abs(a33-a43) + abs(a44-a45)) * 114 ) >> 10;
deltaIA = a23-a33;
influenceA = LUTptr[min(maxVal, abs(NL_A) << shift)]*deltaIA;

NL_B = ( (abs(a52-a42) + abs(a53-a43) + abs(a54-a44) + 
          abs(a42-a32) + abs(a43-a33) + abs(a44-a34) +
          abs(a32-a22) + abs(a33-a23) + abs(a34-a24)) * 114 ) >> 10;
deltaIB = a43-a33;
influenceB = LUTptr[min(maxVal, abs(NL_B) << shift)]*deltaIB;

NL_L = ( (abs(a21-a22) + abs(a31-a32) + abs(a41-a42) + 
          abs(a22-a23) + abs(a32-a33) + abs(a42-a43) +
          abs(a23-a24) + abs(a33-a34) + abs(a43-a44)) * 114 ) >> 10;
deltaIL = a32-a33;
influenceL = LUTptr[min(maxVal, abs(NL_L) << shift)]*deltaIL;

It should be noted that a lot of computation can be saved between samples. This is not done in the above pseudo code for clarity. How this can be done is described in Appendix D. Finally the filtered sample is calculated as

influenceSum = influenceA + influenceB + influenceL + influenceR;
IF = IC + (distanceFactor * influenceSum + 131072) ≫ 18

For inter blocks, the surrounding samples are typically available. Therefore there is no difference between corner/border samples and interior samples for inter blocks. The distanceFactor depends only on the block size, and is taken from the following table:

|  | σd=0.71 for 4x4 blocks | σd=0.58 4xn and nx4, n>4 | σd=0.55 8xn and nx8, n ≥ 8 |
| --- | --- | --- | --- |
| dσdinter | 154 | 122 | 111 |

If the surrounding samples are not available, the block is filtered as an intra block. In this case the distanceFactor is different in corners and borders, and the following table is used to get the value of distanceFactor:

|  | σd=0.71 for 4x4 blocks | σd=0.58 4xn and nx4, n>4 | σd=0.55 8xn and nx8, n ≥ 8 |
| --- | --- | --- | --- |
| dσdinter border | 180 | 139 | 125 |
| dσdinter corner | 219 | 160 | 142 |

# Total storage need

The storage needed for the 8-bit values of the four LUT rows is 235+206+178+150 = 769 bytes .

The sizeRows vector is four 8-bit values, i.e., 4 bytes.

Each value in baseRow is two bits and each value in shiftValue is three bits. Both of these have 34 entries which yields 34*(3+2)  = 170 bits, i.e, 22 bytes.

For distanceFactor there are twelve 8-bit values for intra and nine 8-bit values for inter, i.e., 21 bytes.

In total the storage need for one instantiation of the filter becomes 769+4+22+21 = 816 bytes.

For comparison, we do a similar calculation for JVET-F0034/JVET-F0096. In Section 4 of JVET-F0034, it is described that the LUT values themselves take up 2168 bytes.

The equivalent of sizeRows takes up 34 8-bit values, or 34 bytes.

The division table takes up another 576 bytes, as described in Section 5.2 of JVET-F0096.

The centerweight table from Section 3 of JVET-F0096 needs another five bytes.

In total JVET-F0034/JVET-F0096 would need 2168+34+576+5 = 2783 bytes. Thus the proposed method has reduced the storage by (2783-816)/2783 = 71%.

# Results

The filter was implemented in BMS-1.0. For the BMS test, the bms-configuration files were used. For the VTM test, the same BMS-1.0 software was used, but with vtm-configuration files used. In all cases (both tests and anchors) the SEIDecodedPictureHash flag was set to one. The decoder run time measurements were done on the same computer and are therefore more reliable than the encoder run time measurements.

## All-intra results

For VTM:

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | 0.09% | 0.05% | 97% | 106% |
| Class A2 | -0.31% | -0.06% | 0.10% | 109% | 105% |
| Class B | -0.32% | 0.09% | 0.01% | 104% | 103% |
| Class C | -0.39% | -0.06% | -0.21% | 100% | 104% |
| Class E | -0.39% | -0.45% | -0.32% | 101% | 104% |
| Overall | -0.33% | -0.06% | -0.07% | 102% | 104% |
| Class D | -0.40% | -0.07% | 0.00% | 105% | 102% |

For BMS:

|  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.13% | -0.18% | 103% | 101% |
| Class A2 | -0.35% | -0.19% | 0.07% | 110% | 101% |
| Class B | -0.25% | -0.07% | -0.22% | 99% | 101% |
| Class C | -0.53% | -0.25% | -0.30% | 103% | 102% |
| Class E | -0.38% | -0.38% | -0.44% | 102% | 101% |
| Overall | -0.33% | -0.19% | -0.22% | 103% | 101% |
| Class D | -0.37% | 0.05% | -0.09% | 97% | 102% |

## Random access results

For VTM:

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | -0.16% | -0.20% | 109% | 104% |
| Class A2 | -0.88% | -0.40% | -0.33% | 108% | 104% |
| Class B | -1.01% | -0.05% | -0.13% | 109% | 102% |
| Class C | -0.68% | -0.14% | -0.39% | 106% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.81% | -0.17% | -0.25% | 108% | 103% |
| Class D | -0.63% | -0.26% | -0.23% | 105% | 101% |

For BMS:

|  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.10% | -0.12% | 106% | 101% |
| Class A2 | -0.58% | -0.42% | -0.04% | 106% | 102% |
| Class B | -0.60% | -0.05% | -0.29% | 106% | 101% |
| Class C | -0.60% | 0.00% | -0.51% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.12% | -0.26% | 105% | 101% |
| Class D | -0.52% | -0.08% | -0.59% | 99% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |

## Low delay results

For VTM:

|  | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.71% | 0.56% | 0.61% | 107% | 105% |
| Class C | -0.59% | 0.36% | 0.44% | 111% | 104% |
| Class E | -0.37% | 0.59% | 0.18% | 105% | 100% |
| Overall | -0.59% | 0.50% | 0.45% | 108% | 103% |
| Class D | -0.63% | -0.01% | 0.27% | 106% | 101% |

For BMS:

|  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.47% | 0.38% | 0.98% | 104% | 101% |
| Class C | -0.69% | 0.50% | 0.48% | 110% | 102% |
| Class E | -0.70% | -0.28% | -0.19% | 105% | 99% |
| Overall | -0.60% | 0.26% | 0.52% | 106% | 101% |
| Class D | -0.73% | 0.39% | -0.08% | 98% | 101% |

# Cycle count estimation

At the meeting it was requested to make a cycle count estimation of the filter. It is summarized in Figure 2 below.

Figure 2: Cycle count estimation of filtering an 8×8 block. Arrows represent differences between intensity values. Note that since the samples to the right of the block edge are padded, the green arrows can be reused for that sample column. Since multiplication typically take have more than one cycle of latency, we assume n cycles for the first multiplication (“Multiply by dI”) of 8 bits times 10 bits, and we assume m cycles of latency for the second multiplication (“Multiply with distance value and shift”) of 8 times 20 bits. All filtered pixels are available after (16+n+m) cycles, giving 2 + (n+m)/8 cycles per sample.

Going through the diagram in Figure 2, the first step is to calculate all the intensity differences that are represented by the green arrows in parallel. The next cycle (2), these adders can instead be used to calculate the black arrows, and at the same time the absolute value of the green arrows can be calculated. In cycle (3), these absolute values are added pairwise as a first step of creating the average value. At the same time the absolute values of the black arrows are calculated. In cycle (4) the black arrows are pairwise added. There is now enough information available to create the first LUT argument: The LUT argument for the top pixel marked by light blue needs summation of the top rows of arrows. The two top black arrows were pairwise added in the previous step, so they are now added to the third black arrow. The same is done for the green arrows. Then the black sum and two times the green sum are added together and rightshifted to create the LUT argument. This happens in cycle (5). In cycle (6) the LUT is applied. In cycle (7) this is multiplied by dI. This takes n cycles. In cycle (7+n) this is added to the influence value, which also takes in values in the left-right direction. In cycle (8+n) the influence value is multiplied by the distance value and shifted, taking m cycles. In cycle (8+n+m) this is added to the original sample value. Thus in the next cycle, (9+n+m), all filtered sample values of the first column are available. Since this is pipelined over columns, already in the next clock cycle, (10+n+m), the second column is available. The last pixel column is available in cycle (16+n+m). Counted as cycles per filtered sample, this means (16+n+m)/8 = 2+(n+m)/8. If multiplying a value by a k-bit value takes log2(k) cycles, n equals log2(8) = 3 since the LUT value is an 8-bit value. Likewise m = 3 since the distance value is also eight bits. In this case the average cycle count per filtered pixel would be 2+(3+3)/8 = 2.75. The added latency before the first column of pixels would be available would be 9+n+m = 9+3+3 = 15 cycles.

# Appendix A

In this appendix we describe in detail how many instantiations of the LUT are needed in order to filter a single sample, if maximum parallelism is required. We also go through how many instantiations of the LUT we need to filter four connected samples in parallel.

## Single sample with maximum parallelism

A single sample is filtered using Equation 1, which is repeated here for the convenience of the reader:

IF=IC+dσd×ρqpΔIA×ΔIA+ρqpΔIB×ΔIB+ρqpΔIL×ΔIL+ρqpΔIR×ΔIR.    (Eq. 1)

We will concentrate on the right factor in brackets, where

IC is the intensity of the center sample (the sample to be filtered)

IA, IB, IL and IR are the intensity values of the sample above, below, left and right respectively

ΔIA=IA-IC, ΔIB=IB-IC, ΔIL=IL-IC, and ΔIR=IR-IC.

In this case, the deltas ΔIA, ΔIB, ΔIL,  and ΔIR,  are potentially all different, and the weights ρqpΔIA, ρqpΔIB, ρqpΔIL and ρqpΔIR must therefore be fetched from different parts of the LUT. Typically, a LUT implementation cannot read out two different values at once. Hence in this case we would need four LUT instantiations in order to be able to read out four different values at once.

Thus, a maximally parallel version of a single sample would need four instantiations of the LUT.

## Column of samples with maximum parallelism

In the following, we will describe how it is possible to get away with using only seven instantiations of a LUT when filtering a row or a column of four samples in parallel.

Assume we need to filter the right-most column of the following 4×4 block

|  |  | S | T |
| --- | --- | --- | --- |
|  |  | U | V |
|  |  | W | X |
|  |  | Y | Z |

Here we have denoted the top right sample with “T”, the sample to its left is denoted with “S” etc. The intensity of sample T is denoted IT, the intensity of sample S is denoted IS etc.

To filter sample T we first calculate the intensity difference between its neighboring samples S and T: ΔITS=IS-IT and ΔITV=IV-IT. We can now use ΔITS to get the weight ρqpΔITS=ρqpΔITS and likewise for the weight ρqpΔITV.  The filtered sample T can now be calculated using

ITfiltered=IT+dσd×ρqpΔITSΔITS+ρqpΔITVΔITV.     (Eqn A1)

Next we filter sample V. To do so, we need to calculate the weights for the three surrounding samples T, U and X. We start with the above sample, ΔIVT=IT-IV, and we can now calculate the weight ρqpΔIVT.  However, since ΔIVT=IT-IV= -IV-IT= -ΔITV, this means that ρqpΔIVT=ρqpΔIVT=ρqpΔITV=ρqpΔITV, and this weight was already calculated above. Hence we can reuse ρqpΔITV and we don’t need another instantiation of the LUT. For the other two weights, ρqpΔIVU and ρqpΔIVX, we need instantiations of the LUT, bringing the number to four so far.

Likewise, when filtering sample X, we can use ρqpΔIXV=ρqpΔIVX, and we only need two more instantiations for ρqpΔIXW and ρqpΔIXZ, bringing the total so far to six.

For the last sample, we can use ρqpΔIZX=ρqpΔIXZ, and we only need one more instantiation for ρqpΔIZY, bringing the total number of instantiations needed to seven.

It should also be noted that the product ρqpΔIVT×ΔIVT=ρqpΔITV×-ΔITV=-ρqpΔITV×ΔITV. Hence it is possible to save not only the look-up of the weight ρqpΔIVT, but also to avoid the multiplication ρqpΔIVT×ΔIVT and instead replace it with a negation of the previously calculated value ρqpΔITV×ΔITV. This means that to filter the four samples T, V, X and Z, a total of seven multipliers must be used. This is not counting the multiplications with dσd of which there is one per sample, i.e., four in total.

Thus, in total seven instantiations of the LUT are needed, and eleven multiplications (seven for multiplying by ΔI and four for multiplying by dσd). Likewise, for JVET-F0096, seven instantiations of the LUT are needed (requiring 2202*7 = 15414 bytes), seven multiplications and four division tables (requiring 576*4 = 2304 bytes). In total for JVET-F0096 we need 17718 bytes to filter four samples in parallel. In contrast, the current proposal requires only 816*7 = 5712 bytes.

# Appendix B

This appendix explains how the division can be avoided as is done in JVET-J0021/CE-2.1.3.

To filter a sample, contribution JVET-F0096 uses

IF=IC+wLΔIL+wRΔIR+wAΔIA+wBΔIBwC+wL+ wR+ wA+wB,                (Eq. B1)

where IF is the filtered sample, IC is the intensity of the center sample (the sample to be filtered), IL, IR, IA and IB are the intensity of the samples to the left, to the right, above and below respectively. The delta values are differences against the center sample;  ΔIL=IL-IC and ΔIR=IR-IC, etc. The weights are calculated as

wX=e- 12σd2 - ΔIX2 2σr2,        (Eq. B2)

where X can be A, B, L or R. Thus Equation B.1 can be written as

IF=IC+e- 12σd2 e- ΔIA22σr2ΔIA+e- 12σd2 e- ΔIB22σr2ΔIB+e- 12σd2 e- ΔIL22σr2ΔIL+e- 12σd2 e- ΔIR22σr2ΔIR1+e- 12σd2 e- ΔIA22σr2+e- 12σd2 e- ΔIB22σr2+e- 12σd2 e- ΔIL22σr2+e- 12σd2 e- ΔIR22σr2.    (Eq. B3)

During the filtering of a block, the variables σd and σr are kept constant. However, the intensity differences ΔI are changing with every sample. This means that the denominator in Equation B3 will be different for every sample, and a division is needed. To avoid this, the factors e- ΔIA22σr2are simply removed from the denominator in JVET-J0021, yielding

IF=IC+e- 12σd2 e- ΔIA22σr2ΔIA+e- 12σd2 e- ΔIB22σr2ΔIB+e- 12σd2 e- ΔIL22σr2ΔIL+e- 12σd2 e- ΔIR22σr2ΔIR1+e- 12σd2 +e- 12σd2 +e- 12σd2 +e- 12σd2 .     (Eq. B4)

In JVET-J0021, this is further simplified to

IF=IC+Wσd,σrΔIA×ΔIA+Wσd,σrΔIB×ΔIB+Wσd,σrΔIL×ΔIL+Wσd,σrΔIR×ΔIR.       (Eq. B5)

where

Wσd,σrΔI=e- 12σd2 1+4e- 12σd2 e- ΔIX22σr2.      (Eq. B6)

It is clear from Equation B5 that this version of the filter only has four multiplications, one for each ΔI. For the implementation of JVET-J0021, Wσd,σrΔI is stored as a look-up table. However, since this table is three-dimensional (it depends on σd, σr and ΔI) such a LUT will become very big: σd can take on 6 values, σr can take on 34 values and ΔI up to 1023 values, giving a total LUT size of 6*34*1023 = 208692 values.

Instead we simplify Equation B4 to

IF=IC+dσd×ρσrΔIA×ΔIA+ρσrΔIB×ΔIB+ρσrΔIL×ΔIL+ρσrΔIR×ΔIR,    (Eq. B7)

where

ρσrΔI=255256e- ΔI22σr2     (Eq. B8)

is a two-dimensional look-up table and

dσd=256255 e- 12σd21+4e- 12σd2      (Eq. B9)

is a one-dimensional look-up table of 6 values. This version has introduced an extra multiplication by dσd, but on the other hand it avoids a three-dimensional LUT.  For borders we use a different version of Equation B9 where the denominator is 1+3e- 12σd2 and for corners we use a denominator of 1+2e- 12σd2. The factor 255256 is used to avoid having to store the value 1.0 in the LUT. Using eight fractional bits to store ρσrΔI the largest possible number is then 255 (8 bits) instead of 256 (9 bits). Since σr=12*qp-17 when we use 10-bit intensity values, we can rewrite Equation B8 as

ρσrΔI=255256e- ΔI28qp-172.     (Eq. B10)

Note that even though ten fractional bits are used to store dσ_d, the largest possible value is smaller than 0.25, and therefore only the lowest eight bits need to be stored.

# Appendix C

This appendix explains why two rows of the LUT can be obtained from each other using a simple stretch.

Equation 3 describes how the values in the non-approximated LUT are calculated. It is repeated here for the convenience of the reader:

LUTqp,ΔI=ρqpΔI=255256e- ΔI28qp-172       (Eq. C1)

In particular, the formula for the case when qp = 34 is

LUT34,ΔI=255256e- ΔI2834-172.       (Eq. C2)

But the exponent ΔI2834-172 can, if we multiply both numerator and denominator by 51-17234-172 be written as

ΔI251-17234-172834-17251-17234-172=ΔI51-1734-172851-172=ΔI*c2851-172 ,       (Eq. C3)

where c=51-1734-17=2. Hence

LUT34, ΔI= 255256e- ΔI2834-172=255256e- ΔI*c2851-172=LUT51,ΔI*c.        (Eq. C4)

Thus, instead of using LUT(34, ΔI), we can get the same result by just taking every secondo value in LUT51,ΔI instead. (It is every second, since c happens to be exactly 2 in this case.)

# Appendix D

In this section we discuss the reusing of difference calculations for calculating the average absolute difference value. It produces the same result as JVET-J0021 but uses fewer arithmetic operations and memory accesses. As an example, assume we want to obtain the weight for the “below” sample.

Figure D2. Instead of just using the absolute difference between the sample to be filtered and the above sample, the differences marked with the nine arrows are averaged, and that is used for the weight look-up.

In Figure D1 we can see that nine absolute differences are formed between samples. These are then averaged to provide the weight for the sample below. However, when filtering the next sample, these differences can be reused, as can be seen in Figure D2.

Figure D2.As can be seen in the figure, when filtering the next sample to the right the difference values overlap. The sum for sample (x+1,y), (i.e., the black and green arrows of the right block) is equal to the sum for sample (x,y) (i.e., the arrows of the left block) minus the red arrows and plus the green arrows.

In detail, the new sum is the previous sum minus three values (marked with red in the image) plus three values (marked with green in the image). Furthermore, the red values were previously added so they can be retrieved from a register instead of being calculated. Hence only three absolute differences (one for each green arrow) need to be calculated per sample to obtain the sum, instead of nine values.

The same logic applies to the sample to the right. For the “above” weight, it is possible to reuse the “below” weight of the previous row. Similarly, for the “left” weight, the “right” weight one pixel to the left can be reused. Disgregarding borders and edges therefore, only six absolute differences need to be performed per sample, instead of eighteen.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).