| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0276-v1 |
| --- | --- |

| Title: | CE3-related: Mode-dependent multiple reference line intra prediction with bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Benjamin Bross Paul Keydel Heiko Schwarz Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes to add bi-prediction to mode-dependent multiple-reference line intra prediction as tested in sub-CE 3.5, test 5.4.4 and reported in JVET-K0051. An angular prediction that uses extend reference lines can be averaged with PLANAR prediction using the nearest reference lines. When the reference line index indicates that extended reference samples are used in angular intra prediction, an additional flag indicates whether bi-prediction with PLANAR mode applies or not. The results show that MRL bi-prediction achieves -1.17% (AI) and -0.54% (RA) BD-rate Y, 181% (AI) and 114% (RA) enc. time as well as 99% (AI) and 100% (RA) decoding time for the VTM configuration. It is proposed to study MRL bi-prediction in a new CE on MRL.

# Introduction

MRL bi-prediction as proposed in this document is built on top of test 5.4.4 for mode-dependent MRL with explicit signaling from sub-CE 3.5 of CE3 on intra prediction and mode coding [1] as described in JVET-K0051 [2] . Section 2 provides details of the proposed technique and section 3 reports detailed results.

# Mode-dependent MRL with Bi-Prediction

The proposed bi-prediction can be seen as a combination or weighted sum of two prediction signals derived from two different reference lines:

pbi(x,y)= w0 p0(x,y) + wi pi(x,y)

The prediction p0(x,y) is generated with PLANAR intra prediction using the nearest reference lines (mrlIdx=0), the prediction pi(x,y) is generated with MRL intra prediction (angular prediction) restricted to extended reference lines, i.e. with an associated reference line offset (mrlIdx) that must be greater than 0. The weights w0 and w1are selected to be 0.5.

In case extended reference lines are used (mrl_idx > 0), an additional mrl_bipred_flag flag indicates whether bi-prediction is used on top of MRL or not. The flag is entropy-coded flag using one context.

At the encoder, the search for each MRL index (mrlIdx={1,3}) is performed twice. Once without bi-prediction and once with bi-prediction. In order to reduce encoder runtimes, early skip conditions are implemented to prevent the encoder from testing unlikely modes. There are two threshold-based skip conditions that skip testing MRL bi-prediction for an mrlIdx > 0 if:

- the RD costs of the best MRL mode for mrlIdx are 10% larger than the RD cost of the best mode so far

- the distortion of the previously tested PLANAR mode for mrlIdx=0 is 10% larger than the distortion of the best MRL mode for mrlIdx.

# Results

According to the common test conditions in JVET-J1010 [3], simulations were performed for intra-only (AI) and random-access (RA) configuration with BMS-1.0 software in VTM and BMS configuration. The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

The results reported in Table 2 and Table 3 have been cross-checked as reported in JVET-Kxxxx [3].

Table 2 Results for VTM configuration

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -1.65% | -1.07% | -0.89% | 189% | 99% | -0.82% | -0.98% | -0.92% | 118% | 97% |
| Class A2 | -1.09% | -0.62% | -0.39% | 187% | 97% | -0.46% | -0.69% | -0.49% | 111% | 100% |
| Class B | -1.14% | -0.45% | -0.75% | 183% | 101% | -0.53% | -0.50% | -0.99% | 113% | 101% |
| Class C | -0.78% | -0.41% | -0.54% | 182% | 100% | -0.40% | -0.42% | -0.45% | 112% | 101% |
| Class E | -1.00% | -0.94% | -0.87% | 178% | 98% |  |  |  |  |  |
| Overall | -1.12% | -0.66% | -0.69% | 183% | 99% | -0.54% | -0.61% | -0.73% | 113% | 100% |
| Class D | -0.56% | -0.15% | -0.25% | 183% | 103% | -0.21% | -0.12% | -0.47% | 112% | 98% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Table 3 Results for BMS configuration

| BMS | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| Class A1 | -0.93% | -0.90% | -0.86% | 133% | 98% | -0.49% | -0.77% | -0.50% | 120% | 98% |
| Class A2 | -0.67% | -0.39% | -0.07% | 132% | 93% | -0.31% | -0.48% | -0.16% | 116% | 98% |
| Class B | -0.77% | -0.30% | -0.41% | 133% | 95% | -0.40% | -0.14% | -0.43% | 117% | 95% |
| Class C | -0.59% | -0.31% | -0.46% | 131% | 94% | -0.25% | 0.01% | -0.36% | 116% | 96% |
| Class E | -0.66% | -0.55% | -0.58% | 132% | 95% |  |  |  |  |  |
| Overall | -0.72% | -0.46% | -0.47% | 132% | 95% | -0.36% | -0.29% | -0.37% | 117% | 97% |
| Class D | -0.35% | -0.14% | -0.15% | 131% | 93% | -0.09% | -0.31% | -0.34% | 116% | 95% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, April 2018.

- B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Multiple reference line intra prediction (Test 5.4.1, 5.4.2, 5.4.3 and 5.4.4),” JVET-K0051, Ljubljana, SI, July 2018.

- X-Check JVET-Kxxxx

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).