| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0279-v1 |
| --- | --- |

| Title: | CE4: Restricted merge (Test 4.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Winken Heiko Schwarz Detlev Marpe Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-570 martin.winken@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports results for test CE4-2.2 on restricted merge.  The restricted merge mode allows a modification of bi-predictive merging candidates to uni-prediction.  In merge mode (but not in SKIP mode) in B slices, first it is indicated whether restricted mode is to be applied, and if yes, it is indicated which reference picture list is to be used.  For the Random Access configuration, the average Y BD bit rate saving is -0.21%  relative to the VTM anchor with an relative encoder run-time of 139% and -0.20% relative the the BMS anchor with an relative encoder run-time of 120%.  For the Low-delay B configuration, no coding gain is observed relative to the VTM anchor, and an average Y BD bit rate saving of -0.10% with an relative encoder run-time of 131% relative to the BMS anchor.

# Introduction

In merge mode (but not SKIP mode) in B slices, the merging candidate list can be modified such that it only contains merging candidates using uni prediction.  The application of this “restricted merge mode” as well as the designated prediction list are indicated via a syntax element merge_ref_pic_list_idc, which can take the values LIST_0, LIST_1, and LIST_NONE.  When merge_ref_pic_list_idc != LIST_NONE, the merging candidate list modification process proceeds as specified by the following ordered steps:

- All merging candidates using uni prediction are removed from the merging candidate list.

- For the remaining bi-prediction merging candidates, the prediction list utilization flags are modified, such that the designated prediction list is used (e.g., for merge_ref_pic_list_idc == LIST_0, predFlagL1N is set to 0, with N taking all bi prediction merging candidates).

- Possibly empty entries at the end of the merging candidate list (due to the removal in step 1) are filled with zero motion vector entries having the appropriate prediction list utilization flag and iterating over all reference frames of this prediction list.

The syntax element merge_ref_pic_list_idc can take three different values. It is coded using a truncated unary binarization. The first bin indicates whether restricted merge or the conventional merge mode is used (as in VTM/BMS).  If the first bin is equal to 1, the second bin indicates which of the two motion hypotheses (list 0 or list 1) is used. Both bins are coded with a dedicated context model.

# Software and Settings

The source code was made available to all CE4 participants in the following branch:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE4/CE4-2.2

The encoder can be run using the default config files, all necessary settings are accordingly pre-defined in the corresponding branch.

# Test results

According to the CE4 description in JVET-J1024 [1] and the common test conditions in JVET-J1010 [2], simulations were performed for random-access (RA) and low-delay B (LB) configuration with BMS-1.0 software in VTM and BMS configuration. The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

## Test CE4-2.2

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.28% | -0.36% | 137% | 103% | -0.20% | -0.22% | -0.23% | 119% | 101% |
| Class A2 | -0.13% | -0.31% | -0.25% | 136% | 101% | -0.09% | -0.29% | -0.22% | 120% | 100% |
| Class B | -0.21% | -0.26% | -0.23% | 139% | 104% | -0.21% | -0.20% | -0.21% | 120% | 99% |
| Class C | -0.25% | -0.41% | -0.46% | 145% | 101% | -0.28% | -0.40% | -0.47% | 121% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.21% | -0.32% | -0.32% | 139% | 102% | -0.20% | -0.28% | -0.29% | 120% | 100% |
| Class D | -0.16% | -0.32% | -0.40% | 148% | 99% | -0.17% | -0.45% | -0.49% | 122% | 105% |
| Class F (optional) | -0.36% | -0.20% | -0.23% | 141% | 101% | -0.29% | -0.35% | -0.32% | 118% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.06% | 0.07% | 144% | 103% | -0.08% | -0.23% | 0.21% | 130% | 98% |
| Class C | 0.00% | -0.13% | 0.02% | 145% | 105% | -0.09% | -0.01% | -0.28% | 132% | 98% |
| Class E | 0.04% | 0.33% | -0.26% | 145% | 102% | -0.12% | -0.35% | -0.12% | 132% | 100% |
| Overall | 0.01% | 0.02% | -0.03% | 145% | 104% | -0.10% | -0.19% | -0.03% | 131% | 99% |
| Class D | 0.03% | -0.36% | -0.25% | 143% | 102% | 0.06% | 0.27% | -0.27% | 132% | 97% |
| Class F (optional) | -0.34% | -0.39% | -0.37% | 147% | 101% | -0.08% | 0.13% | -0.11% | 128% | 97% |

# Conclusion

A restricted merge mode has been presented, which allows restriction of bi-predictive merging candidates to uni-prediction.  For the Random Access configuration, the average Y BD bit rate saving is -0.21%  relative to the VTM anchor with an relative encoder run-time of 139% and -0.20% relative the the BMS anchor with an relative encoder run-time of 120%.  For the Low-delay B configuration, no coding gain is observed relative to the VTM anchor, and an average Y BD bit rate saving of -0.10% with an relative encoder run-time of 131% relative to the BMS anchor.

# References

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding,” JVET-J1024, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).