| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0281-v4 |
| --- | --- |

| Title: | Inter-Component Context Modelling for Coded Block Flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rudat, P. Haase, H. Schwarz, D. Marpe, T. Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 23 588 034 heiko.schwarz@hhi.fraunhofer.de christian.rudat@hhi.fraunhofer.de paul.haase@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This proposal presents a method for a coding scheme to be used for coding Coded Block Flags. It is reported that average bit rate savings of 0.2% (up to 0.4%) can be achieved  in random access coding configuration whereas, the measured execution time of neither the encoder nor the decoder has changed significantly.

# Introduction

In current video coding standards, syntax elements may be coded by context adaptive arithmetic coding. In this contribution an inter-component context derivation method used for coding the Coded Block Flag (CBF) is proposed. Experimental results are presented.

# Inter-Component Context Modelling for CBF Coding

Conventionally, the CBFs of the color components (for example Y,Cb,Cr) of a given Transform-Unit (TU) are coded independently in the following sequence: Cb, Cr, Y. The presented context derivation method for coding of Coded Block Flags (CBFs) uses information of previously coded CBFs of other color-components within the same Transform-Unit (TU).

## Coding of CBF of Cr-Compopent

Conventionally, the CBF Syntax Element for the Cr-Component is coded directly after the CBF Syntax Element for the Cb-Component. In this proposal the Context for coding the Cr-CBF is determined directly by the Cb-CBF, like

…

cbf_cb = decodeBin(context_cbf_cb)

if(cbf_cb == false){

context_cbf_cr = c_0

}else{

context_cbf_cr = c_1

}

cbf_cr = decodeBin(context_cbf_cr)

…

## Coding of CBF of Cb-Component (Intra-CUs only)

In this proposal, the coding order of the CBFs for each color component of TUs within Intra-Predicted Prediction-Units (PUs) is Y, Cb, Cr. The already coded Y-CBF is used directly to determine the context for coding of the Cb-CBF, like

…

cbf_y = decodeBin(context_cbf_Y)

if(cbf_y == false){

context_cbf_cb = c_2

}else{

context_cbf_cb = c_3

}

cbf_cb = decodeBin(context_cbf_cb)

…

# Simulation Results

In this section, experimental results in terms of Bjøntegaard Delta (BD) bit rate savings [3] are presented as an average over a testset of sequences given by the Common Test Conditions (CTC) [2]. The Software “BMS-1.0” was used to perform the simulations (precompile macro “#HHI_CBF_CODING”. For each sequence, the first 49 frames were coded (Simulations with full number of frames are still running). The encoder configurations “vtm” and “bms” (in both cases with the additional paramaters CbQpOffset=1 and CrQpOffset=1) were used, whereas “vtm” results in a coding environment similar to “HEVC/H.265” with the exception of incorporating the QT+BTS partitioning scheme as described in [4].

## Test Conditions VTM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.26% | -0.37% | 100% | 100% |
| Class A2 | -0.24% | 0.19% | -1.03% | 100% | 101% |
| Class B | -0.20% | -1.34% | -0.39% | 99% | 98% |
| Class C | -0.22% | -0.62% | -0.49% | 100% | 100% |
| Class E | -0.16% | -1.28% | -0.87% | 99% | 98% |
| Overall | -0.24% | -0.73% | -0.60% | 100% | 99% |
| Class D | -0.20% | -0.15% | -0.47% | 102% | 103% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | 0.68% | 0.17% | 101% | 101% |
| Class A2 | -0.09% | 0.33% | -0.74% | 99% | 99% |
| Class B | -0.26% | -0.42% | -0.05% | 100% | 102% |
| Class C | -0.11% | -0.27% | -0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.01% | -0.16% | 100% | 100% |
| Class D | 0.01% | -0.96% | -1.68% | 101% | 104% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -1.51% | 0.21% | 101% | 102% |
| Class C | -0.05% | 0.07% | -0.13% | 100% | 99% |
| Class E | 0.14% | -2.31% | -0.96% | 99% | 95% |
| Overall | -0.05% | -1.19% | -0.20% | 100% | 99% |
| Class D | 0.00% | -1.54% | -1.86% | 101% | 106% |
| Class F (optional) |  |  |  |  |  |

## Test Conditions BMS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.47% | -0.70% | 100% | 100% |
| Class A2 | -0.12% | 0.44% | -0.91% | 99% | 101% |
| Class B | -0.13% | -1.36% | 0.60% | 100% | 102% |
| Class C | -0.07% | -1.02% | 0.33% | 101% | 102% |
| Class E | -0.11% | -2.21% | 0.67% | 101% | 100% |
| Overall | -0.12% | -0.82% | 0.08% | 100% | 101% |
| Class D | -0.13% | -1.42% | 0.02% | 100% | 103% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | 0.66% | -0.02% | 101% | 101% |
| Class A2 | -0.14% | 0.50% | -0.37% | 100% | 101% |
| Class B | -0.05% | -1.16% | 0.92% | 100% | 101% |
| Class C | -0.08% | -0.51% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.29% | 0.23% | 100% | 101% |
| Class D | -0.21% | -0.43% | -0.06% | 102% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.98% | 1.24% | 101% | 101% |
| Class C | -0.06% | -0.71% | 0.45% | 100% | 100% |
| Class E | -0.01% | -1.29% | -0.49% | 100% | 101% |
| Overall | -0.03% | -0.97% | 0.54% | 100% | 101% |
| Class D | -0.12% | -0.15% | 0.48% | 100% | 102% |
| Class F (optional) |  |  |  |  |  |

# Conclusions

It is reported that bit rate savings with an average of 0.2% and up to 0.4% (vtm, randomaccess) can be achieved by coding CBFs dependently through color components. The measured execution time of the encoder and decoder stays virtually the same.

We therefore propose to consider Inter-Component Context Modelling for Coded Block Flag for the upcoming standardization work.

# References

A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” JVET, doc. JVET-H1002, October 2017.

J. Boyce, K. Sühring, X. Li, V.Seregin “JVET common test conditions and software reference configurations,” JVET, doc. JVET-J1010, April 2018.

G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” VCEG-M33, Austin, Mar. 2001.

Ma, et. al, “Quadtree plus binary tree with shifting,” JVET-J0035, Apr. 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).