| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July, 2018 | Document: JVET-K0281-v4 |
| --- | --- |

| Title: | Inter-Component Context Modelling for Coded Block Flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Rudat, Paul Haase, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 23 588 034 christian.rudat@hhi.fraunhofer.de paul.haase@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The document describes an inter-component context modelling for chroma coded block flags. The context for the coded block flag for the Cr component is determined by the value of the coded block flag for the Cb component. For intra-coded CUs, the coding order of coded block flags is modified (the coded block flags are transmitted in the order Y, Cb, Cr) and the context for the coded block flag for the Cb component is determined by the values of the coded block flag for the Y component. Compared to VTM-1, average bit rate savings of 0.25% for all intra and 0.15% for random access are reported.

# Inter-Component Context Modelling for CBF Coding

In HEVC, VTM-1, and BMS-1, the coded block flags for the individual colour components (Y, Cb, Cr) of a transform unit (TU) are coded independently of each other. The coded block flags are transmitted in the following order: Cb, Cr, Y.

In this document, we propose to select the context for a coded block flag depending on the values of already transmitted coded block flags (for other colour components) for the same transform unit.

## Coded block flag (CBF) for the Cr component

In a first version, only the context modelling for the coded block flag (syntax element coded_block_flag) of the Cr component is modified. One of two probability models is selected based on the value of the already transmitted coded block flag for the Cb component of the same transform unit. The context index, which identifies the probability model used, is set according to

ctxId = codedBlockFlagCb,

where codedBlockFlagCb represents the value of the syntax element coded_block_flag for the Cb component of the same transform unit

…

cbf_cb = decodeBin(context_cbf_cb)

if(cbf_cb == false){

context_cbf_cr = c_0

}else{

context_cbf_cr = c_1

}

cbf_cr = decodeBin(context_cbf_cr)

…

.

The modified context selection is used for both intra- and inter-picture predicted coding/transform units.

The described context modelling for the coded block flag of the Cr component is included in the transform coefficient coding presented in JVET-K0071. It was used in the version tested in CE7-1.2.

Extension to

## Cb component for Intra CUs

In an extended version of the proposal, the inter-component context modelling is additionally applied to the coded block flag of the first chroma component (Cb) for intra-picture coded CUs/TUs.

For that purpose, the coding order of coded block flags for intra-picture coded TUs is modified. If a TU represents a TU of an intra-picture coded CU, the coded block flags are coded in the following order:

Cbf coding order: Y, Cb, Cr

Similarly as for the Cr components, one of two probability models is selected for coding the coded block flag of the Cb component. The context index, which specifies the probability model used, is derived according to

ctxId = codedBlockFlagY,

where codedBlockFlagY represents the value of the syntax element coded_block_flag for the Y component of the same transform unit.

For inter-picture coded CUs, the coding order and context modelling are not modified (i.e., only the context modelling for the Cr component described above is used).

…

cbf_y = decodeBin(context_cbf_Y)

if(cbf_y == false){

context_cbf_cb = c_2

}else{

context_cbf_cb = c_3

}

cbf_cb = decodeBin(context_cbf_cb)

…

# Simulation Results

In this section, experimental results in terms of Bjøntegaard Delta (BD) bit rate savings [2] are presented as an average over the set of sequences specified in the Common Test Conditions (CTC) [1]. Simulations were conducted for both, the VTM and BMS configurations. Detailed results are provided in the accompanying Excel documents.

Table 1 summarizes the average results for the VTM configuration. The average results for the BMS configuration are provided in Table 2.

## Table 1: Average test results for the VTM configuration (relative the VTM-1, using BMS-1 in VTM configuration).

Table 2: Average test results for the BMS configuration (relative the BMS-1).

# Conclusions

In this document, an inter-component context modelling for chroma coded block flags is presented. Relative to VTM, average bit rate savings of 0.25% for all intra and 0.15% for random access are measured.

# References

J. Boyce, K. Sühring, X. Li, V.Seregin “JVET common test conditions and software reference configurations,” JVET, doc. JVET-J1010, April 2018.

G. Bjøntegaard, “Calculation of average PSNR differences between RD-curves,” VCEG-M33, Austin, Mar. 2001.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).