| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0282 |
| --- | --- |

| Title: | CE5: Context adaptive counter-based probability estimation (test 2.3.0) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jing Cui, Shanshe Wang, Siwei Ma Peking University Xiaozhen Zheng DJI | Tel: Email: Tel: Email: | swma@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

This contribution reports test results for the arithmetic coding CABAC modification method proposed in document JVET-J0011[1]. The proposed probability estimation method of CABAC is implemented in VVC reference software VTM1.0 and BMS 1.0 and the experimental results for AI, RA and LDB configurations are tested under CTC. The proposed method achieves about 0.66% and 0.53% BD-rate saving over VTM1.0 and BMS1.0 at AI configuration, while the performances for RA and LDB have 0.13% / 0.05% and 0.22%/0.04% BD-rate increase over VTM1.0 and BMS1.0, respectively.

Introduction

In JVET-J0011[1], a counter-based “double-window” CABAC probability estimation method is proposed, where each context model additionally includes a variable counter to record the cycle time of current context in current slice. For the given context, if its cycle count is larger than the pre-defined threshold, the sliding window parameter will be added by 1. In addition, the initialization of probability is modified according to the value of initVal of each context. Two window parameters which is relative to probability is decided at the beginning of each slice.

Proposed Method in JVET-J0011

The proposed context adaptive counter-based probability estimation scheme whose initialization takes place once per slice.

For the given context, one context counter c is included in the context buffer. The counter c is increased when the given context is used in the CABAC process. Two sliding window parameter a and b is assigned as the following:

If c<Thr, (a1,b1) = (a0,b0)

If c≥Thr, (a1,b1) = (a0+1,b0+1)

Where a0 and b0 are the initialized values. The a1 and b1 are the adjusted value when the proposed scheme is used. Thr is the counter threshold, which can be set as 32, 64, 128 and 512. The default value is set to 512 in J0011.

In addition, the initializations of window parterres a and b are performed at the beginning of each slice, where all the context relative variables are initialized and prepared for the CABAC process. In implementation:

If (initVal < 64)  a0 = 8, b0 = 4;

If(initVal >= 64)  a0 = (initVal >> 4) & 0xF; b0 = (initVal >> 5) & 0xF;

assert((a0>1) && (a0<10) &&(b0>1) && (b0<10));

Experimental results

The software is implemented based on BMS-1.0. And it is committed in SVN server [3]. The test are evaluated follow by common test condition [4], and its results are summarized in Table 1 and Table 2.

Table 1. Performance of Proposed Method for AI configuration

Table 2. Performance of Proposed Method for RA and LDB configuration

Conclusion

This contribution proposes a probability estimation method, which is able to improve the coding performance for AI configuration with about 0.6% BD-rate reduction. Due to limited parameter test, the performance under RA and LDB is not as good as that of AI. More parameter training is needed in further.

References

[1] Z. Wang, X. Meng, C. Jia, et al. “Description of SDR video coding technology proposal by DJI and Peking University,” JVET-J0011, April., 2018.

[2], “Description of CE5: Arithmetic Coding Engine” JVET-J1025, JVET 10th Meeting: San Diego, US, April 2018.

[3] https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE5/CE5-2.3.0/

[4] K. Suehring, X. Li, “JVET common test conditions and software reference configurations,” JVET-C1010, May, 2016.

.

Patent rights declaration(s)

SZ DJI Technology Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).