| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0285-v3 |
| --- | --- |

| Title: | CE2.4.2.1 Multiple-feature based adaptive loop filter (JVET-K0285) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wang-Q Lim, Johannes Erfurt, Mischa Siekmann, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-229 wang.lim@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The main idea of adaptive loop filter (ALF) is to apply a classification to obtain multiple classes, which gives a partition of a set of all pixel locations. After that, a Wiener filter is applied for each of those classes. Therefore, the performance of ALF essentially relies on how its classification behaves. Multiple-feature based (classifications) adaptive loop filter (MCALF) extends a classification in ALF by applying more than one classifier at the encoder to group all reconstructed samples and then to select a classifier with the best RD-performance to carry out the classification process. Overall, MCALF gives about 5.61%, bitrate reduction for luma over VTM-1.0 and about 5.34% over BMS-1.0 (without ALF) for random access keeping nearly the same complexity of ALF.

# Multiple-feature based adaptive loop filter (MCALF)

Multiple-feature based (classification) adaptive loop filter (MCALF) uses the same approach for filtering and signalling of filter coefficients as ALF. However, the classification for the luma component is modified. In general, a classification of the reconstructed samples s'(i,j) is given as

CkD=i,j∈I :Di,j=k       for

where I is a set of all sample locations of the reconstructed samples s'(i,j).  D is a classifier that assigns a class index  to each sample location (i,j).  And a class CkD is the collection of all sample to which the class index k is assigned by the classifier D. K specifies the number of classes. The MCALF supports four different classifiers, each of which provides K=25 classes.  The classifier that is used in the decoder is specified by the syntax element classification_idx, which is transmitted at a slice level. Given the classes CkD, with , the adaptive loop filter proceeds as in JEM. Depending on the value of classification_idx, the following applies:

- If classification_idx is equal to 0, the classifier DG specified for ALF in JEM is used. The classifier provides K=25 classes. The samples are classified based on the direction and activity of local gradient in the reconstructed picture.

- Otherwise, if classification_idx is equal to 1, a sample-based feature descriptor DS is used

as classifier.  DS(i,j) simply takes the quantized sample value of each sample s'i,j according to

where B is the sample bit depth, the number of classes K is set equal K=25, and the operator

specifies rounding to the largest integer not larger than the argument.

- Otherwise, if classification_idx is equal to 2, a ranking with sample-based feature descriptor  DR1 is used. The ranking with sample-based feature descriptor DR1 is specified as follows. Let r8i,j be a descriptor that ranks a sample value s'i,j in order of its magnitude compared to its 8 neighboring samples,

r8i,j=k=i-1i+1l=j-1j+1 s'i,j<s'k,l  ?  1  : 0 .

The descriptor r8i,j can take values in the range of 0 to 8, inclusive. If the sample s'i,j is the largest sample inside the 3×3 block centered around i,j, the value of r8i,j is equal to 0. If s'i,j is the second largest sample, r8i,j is equal to 1, etc.  After that, the value of r8i,j is quantized to 5 distinct values, 0,1,..,4 by min⁡(58⋅r8i,j,4). Then the dynamic range for the samples is divided into 5 bands with about equal size and inside each band, the quantized rank of the sample in its local neighborhood is used as additional criterion, which gives 5×5 = 25 classes.

- Otherwise (classification_idx is equal to 3), a ranking with local variation-based feature descriptor DR2 is used. The feature descriptor DR2 applies the same procedure as DR1 except that the dynamic range for local variation vi,j instead of  the samples is divided into 4 bands, where  the local variation vi,j at each sample location (i,j) is given as

vi,j=|4*s'i,j-(s'i-1,j+s'i+1,j+s'i,j+1+s'(i,j-1))|

Those 4 bands are specified by some fixed threshold parameters T1,T2, T3.

# Test results

According to the CE2 description in JVET-J1022 [1] and the common test conditions in JVET-J1010 [2], simulations were performed for intra-only (AI), random-access (RA), low-delay B (LB) and low-delay P (LP) configurations, with BMS-1.0 software in VTM and BMS configuration. The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | Off | AVX2 | GCC 7.2.1 | Linux |

## 2.1  MCALF with 4 classifiers

Table 2 Result for VTM vs VTM+MCALF – AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -3.32% | -3.55% | -4.06% | 103% | 162% |
| Class D | -2.71% | -2.85% | -3.38% | 105% | 140% |
| Class F (optional) | -2.68% | -1.02% | -1.12% | 103% | 138% |

Table 3 Result for VTM vs VTM+MCALF – RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.61% | -2.22% | -1.66% | 111% | 197% |
| Class D | -5.95% | -2.11% | -2.47% | 120% | 156% |
| Class F (optional) | -3.01% | -0.85% | -0.99% | 125% | 148% |

Table 4 Result for VTM vs VTM+MCALF – LB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.19% | -1.74% | -2.04% | 117% | 167% |
| Class D | -4.72% | -1.54% | -1.98% | 116% | 148% |
| Class F (optional) | -5.71% | -2.52% | -1.79% | 129% | 145% |

Table 5 Result for VTM vs VTM+MCALF – LP configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -7.74% | -2.54% | -3.19% | 121% | 166% |
| Class D | -3.80% | -0.96% | -1.11% | 123% | 155% |
| Class F (optional) | -5.56% | -2.30% | -2.33% | 138% | 150% |

Table 6 Result for BMS-ALF vs BMS-ALF + MCALF – AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -2.77% | -3.43% | -3.57% | 100% | 145% |
| Class D | -1.98% | -2.60% | -2.96% | 100% | 126% |
| Class F (optional) | -2.39% | -1.32% | -1.21% | 100% | 133% |

Table 7 Result for BMS-ALF vs BMS-ALF + MCALF – RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.34% | -2.19% | -1.31% | 102% | 159% |
| Class D | -5.79% | -2.01% | -2.11% | 104% | 130% |
| Class F (optional) | -2.82% | -1.18% | -1.16% | 106% | 137% |

Table 8 Result for BMS-ALF vs BMS-ALF + MCALF – LB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -4.86% | -1.76% | -1.83% | 104% | 148% |
| Class D | -5.01% | -1.89% | -2.14% | 105% | 138% |
| Class F (optional) | -5.27% | -2.15% | -2.51% | 107% | 139% |

Table 9 Result for BMS-ALF vs BMS-ALF + MCALF – LP configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -7.38% | -2.06% | -2.16% | 104% | 150% |
| Class D | -4.00% | -1.06% | -1.31% | 106% | 141% |
| Class F (optional) | -5.33% | -1.93% | -2.33% | 108% | 137% |

2.2  MCALF with only two classifiers DG and  DS

Table 10 Result for VTM vs VTM+MCALF – AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -3.31% | -3.55% | -4.06% | 102% | 165% |
| Class D | -2.71% | -2.85% | -3.38% | 103% | 140% |
| Class F (optional) | -2.57% | -1.03% | -1.12% | 101% | 143% |

Table 11 Result for VTM vs VTM+MCALF – RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.53% | -2.21% | -1.67% | 109% | 200% |
| Class D | -5.94% | -2.12% | -2.48% | 117% | 157% |
| Class F (optional) | -2.64% | -0.87% | -0.95% | 121% | 141% |

Table 12 Result for VTM vs VTM+MCALF – LB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -4.97% | -1.96% | -2.13% | 115% | 167% |
| Class D | -4.69% | -2.02% | -2.16% | 113% | 152% |
| Class F (optional) | -5.17% | -2.27% | -1.96% | 125% | 146% |

Table 13 Result for BMS-ALF vs BMS-ALF + MCALF – AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -2.76% | -3.43% | -3.57% | 100% | 144% |
| Class D | -1.99% | -2.60% | -2.96% | 100% | 127% |
| Class F (optional) | -2.22% | -1.32% | -1.22% | 101% | 136% |

Table 14 Result for BMS-ALF vs BMS-ALF + MCALF – RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.26% | -2.22% | -1.32% | 102% | 157% |
| Class D | -5.79% | -2.00% | -2.12% | 103% | 130% |
| Class F (optional) | -2.43% | -1.09% | -1.07% | 105% | 134% |

Table 15 Result for BMS-ALF vs BMS-ALF + MCALF – LB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -4.71% | -1.75% | -1.73% | 104% | 149% |
| Class D | -4.93% | -2.14% | -1.95% | 103% | 137% |
| Class F (optional) | -4.88% | -2.23% | -2.15% | 106% | 140% |

2.3  MCALF with only two classifiers DG and  DR1

Table 16 Result for VTM vs VTM+MCALF – AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -3.31% | -3.55% | -4.06% | 102% | 164% |
| Class D | -2.71% | -2.85% | -3.38% | 103% | 139% |
| Class F (optional) | -2.36% | -1.03% | -1.12% | 102% | 140% |

Table 17 Result for VTM vs VTM+MCALF – RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.60% | -2.21% | -1.64% | 109% | 197% |
| Class D | -5.81% | -2.11% | -2.51% | 115% | 160% |
| Class F (optional) | -2.69% | -0.84% | -1.00% | 121% | 146% |

Table 18 Result for VTM vs VTM+MCALF – LB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.14% | -1.99% | -1.94% | 113% | 165% |
| Class D | -4.62% | -1.99% | -2.51% | 113% | 152% |
| Class F (optional) | -5.03% | -2.28% | -1.96% | 124% | 147% |

Table 19 Result for BMS-ALF vs BMS-ALF + MCALF – AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -2.76% | -3.43% | -3.57% | 100% | 145% |
| Class D | -1.99% | -2.60% | -2.96% | 100% | 127% |
| Class F (optional) | -2.09% | -1.32% | -1.21% | 100% | 134% |

Table 20 Result for BMS-ALF vs BMS-ALF + MCALF – RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.34% | -2.17% | -1.31% | 102% | 159% |
| Class D | -5.68% | -2.10% | -2.20% | 103% | 132% |
| Class F (optional) | -2.50% | -1.22% | -1.13% | 105% | 139% |

Table 21 Result for BMS-ALF vs BMS-ALF + MCALF – LB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -4.83% | -1.76% | -1.60% | 103% | 149% |
| Class D | -4.93% | -2.22% | -1.99% | 103% | 138% |
| Class F (optional) | -4.54% | -2.29% | -1.97% | 106% | 139% |

2.4  MCALF with only two classifiers DG and  DR2

Table 22 Result for VTM vs VTM+MCALF – AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -3.30% | -3.55% | -4.06% | 102% | 166% |
| Class D | -2.71% | -2.85% | -3.38% | 103% | 139% |
| Class F (optional) | -2.32% | -1.03% | -1.13% | 102% | 142% |

Table 23 Result for VTM vs VTM+MCALF – RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.53% | -2.24% | -1.66% | 110% | 201% |
| Class D | -5.78% | -2.12% | -2.50% | 116% | 162% |
| Class F (optional) | -2.54% | -0.87% | -0.98% | 121% | 144% |

Table 24 Result for VTM vs VTM+MCALF – LB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -4.90% | -1.67% | -2.04% | 115% | 169% |
| Class D | -4.63% | -2.02% | -1.87% | 112% | 152% |
| Class F (optional) | -4.63% | -2.28% | -1.24% | 125% | 147% |

Table 25 Result for BMS-ALF vs BMS-ALF + MCALF – AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -2.75% | -3.43% | -3.57% | 100% | 146% |
| Class D | -1.99% | -2.60% | -2.96% | 100% | 127% |
| Class F (optional) | -2.05% | -1.32% | -1.22% | 100% | 134% |

Table 26 Result for BMS-ALF vs BMS-ALF + MCALF – RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -5.27% | -2.24% | -1.32% | 102% | 158% |
| Class D | -5.66% | -2.03% | -2.15% | 103% | 133% |
| Class F (optional) | -2.41% | -1.21% | -1.17% | 105% | 136% |

Table 27 Result for BMS-ALF vs BMS-ALF + MCALF – LB configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Overall | -4.63% | -1.89% | -1.88% | 104% | 151% |
| Class D | -4.86% | -1.69% | -1.59% | 103% | 140% |
| Class F (optional) | -4.43% | -2.46% | -1.55% | 106% | 138% |

# References

- L. Zhang, K. Andersson, C.-Y. Chen “Description of Core Experiment 2 (CE2): In-Loop Filters”, JVET-J1022, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.