| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0287-v1 |
| --- | --- |

| Title: | CE1-2.0.11: Picture Boundary Handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Zhijie Zhao, Anand Meher Kotra, Jianle Chen Riesstr. 25 80992 Munich Germany | Tel: Email: | +49 89 158 8344083 han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

A versatile boundary partition method is proposed. In the proposed method, the same syntax signaling is applied for the normal CUs and the CUs located at picture boundary. Compare to VTM-1.0, the proposed method in CE1 achieves overall -0.04%, -0.31%, and -0.49% luma BD rate saving, respectively for AI, RA and LDB configurations with 100%, 100% and 102% encoding time. Compare to BMS-1.0, the proposed method have overall -0.05%, -0.35%, and -0.65% luma BD rate saving, respectively for AI, RA and LDB configurations, with 100%, 102% and 104% encoding time.

In addition to the test in CE1 which applies QT partitioning followed by BT at the picture boundary, the results of application of only BT at the picture boundary is also presented in the document. This alternative configuration of the proposal achieves overall -0.05%, -0.31%, and -0.51% luma coding gain compared to VTM-1.0, and overall -0.06%, -0.37%, and -0.68% luma coding gain compared to BMS-1.0 for AI, RA and LDB configurations respectively.

# Introduction

The proposal presents a new method of picture boundary handling. The proposed method use unchanged regular block syntax to partition boundary located CTUs/CUs, block both in the encoder and decoder, the semantics of the partition syntax could be different for boundary and non-boundary located CTUs/CUs.

# Proposed Method

### Proposed Method in CE1

Since the current forced quadtree boundary partition solution (solution in VTM-1.0) is not optimized. In our proposal, we proposed the boundary partition method using regular block partition syntax to keep the continuity CABAC engine as well as matching the picture boundary

The versatile boundary partition obtains the following rules (both encoder and decoder):

- Using exactly same partition syntax of the normal block (non-boundary) (for instance, VTM-1.0 like Fig. 1) for boundary located block, the syntax need to be unchanged.

- If the no split mode is parsed for the boundary CU, used forced boundary partition (FBP) to match the picture boundary.

- After forced boundary partition (non-singling boundary partition), no further partition.

0

0

0

0

0

1

1

1

1

1

QT

No split/FBP

Horizontal

Vertical

HTT

HBT

VTT

VBT

Fig. 1: unchanged syntax and changed semantic for versatile boundary partition

The forced boundary partition of proposed method in CE1-2.0.11 is descripted as follow:

- If the size of block is larger than the maximal allowed BT size, forced QT is used to perform the FBP in the current forced partition level;

- Otherwise, if the bottom-right sample of current CU is located below the bottom picture boundary, and not extended the right boundary, forced horizontal BT is used to perform the FBP in the current forced partition level;

- Otherwise, if the bottom-right sample of current CU is located at the right side of the right picture boundary, and not below the bottom boundary, forced vertical BT is used to perform the FBP in the current forced partition level;

- Otherwise, if the bottom-right sample of current CU is located at the right side of the right picture boundary and below the bottom boundary, forced QT is used to perform the FBP in the current forced partition level.

### Experiment results of proposed method in CE1

The proposed method is implemented on top of the BMS 1.0 software. The experiment results are to be produced under the common test conditions. Further, the following two configurations are to be used for the experiments, as mention in JVET-J1021 [2]:

- BMS1.0:
VVC software with Benchmark set (BMS) configuration as provided in the repository.

- VTM1.0:
Same software as in BMS1.0 configuration, but with disabling the BMS tools at config level.

Table 1: Experiment results of proposed method in CE1 on top of BMS1.0

Table 2: Experiment results of proposed method in CE1 on top of VTM1.0

### Alternative forced boundary partition of the proposed method in CE1

In the alternative configuration of the proposed method, the rules of versatile boundary partition is keeping modified, namely unchanged syntax between normal block and the boundary block partition.

The rules of forced boundary partition is replaced as follow:

- If the bottom-right sample of current CU is located below the bottom picture boundary, forced horizontal BT is used to perform the FBP in the current forced partition level;

- Otherwise, if the bottom-right sample of current CU is located at the right side of the right picture boundary, forced vertical BT is used to perform the FBP in the current forced partition level.

### Experiment results of the alternative forced boundary partition of the proposed method in CE1

Table 3: Experiment results of alternative forced boundary partition of the proposed method in CE1 on top of BMS1.0

Table 4: Experiment results of alternative forced boundary partition of the proposed method in CE1 on top of VTM1.0

# Conclusion

A new versatile boundary partition method in CE1 is proposed to achieve better performance of the picture boundary handling. Experiment results report that the proposed method saves BD-rate overall -0.04%, -0.31%, and -0.49% of luma component for AI, RA and LDB configuration compare to VTM anchor; and Compare to BMS anchor, the proposed method saves BD-rate overall -0.05%, -0.35%, and -0.65% of luma component for AI, RA and LDB configuration.

The alternative forced boundary partition of the proposed method in CE1 achieves overall -0.05%, -0.31%, and -0.51% luma coding gain compare to VTM-1.0, and overall -0.06%, -0.37%, and -0.68% luma coding gain compare to BMS-1.0 for AI, RA and LDB configuration.

# Reference

[1] “Versatile Video Coding (Draft 1)”, B. Bross, doc. JVET-J1001, April 2018

[2] “Description of Core Experiment 1 (CE1): Partitioning”, J. Ma, F. Le Léannec, M. W. Park, doc. JVET-J1021, April 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).