| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0288 |
| --- | --- |

| Title: | CE9-related: Memory bandwidth reduction for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Stephan Wenger, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, swenger, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Decoder side motion vector refinement (DMVR) techniques were proposed to refine motion information at decoder side. With DMVR, an initial MV pair for a block is first identified at decoder. Then the MV pair is refined within a search range. As the final MV may be at any position within the search range, the memory bandwidth requirement of DMVR mode would be higher than regular inter mode at decoder side. In this contribution, a memory bandwidth reduction method is proposed for DMVR. It is reported the memory bandwidth of DMVR can be kept the same as regular inter mode with the proposed method at the cost of 0.06% average luma BD-rate increase.

# Introduction

Decoder side motion vector refinement (DMVR) techniques were proposed to derive motion information at decoder side. With DMVR, an initial MV pair for a block is first identified at decoder. Then the MV pair is refined within a search range. However, due to the introduction of search range, the memory bandwidth requirement for DMVR mode is higher than conventional inter mode.

# Proposed method

Figure 1 shows the memory access for interpolation in motion compensation (MC) of a conventional inter mode. For a W×H block, a (W+Tap_MC-1)×(H+Tap_MC-1) block needs to be fetched into memory in a generic case, where Tap_MC represents the tap of interpolation filter.

Figure 1 Memory access for interpolation in motion compensation

Figure 2 shows the memory access for a W×H block in DMVR mode, where SR represents the search range during MV refinement. From the figure, totally a (W+2×SR+Tap_MC-1)×(H+2×SR+Tap_MC-1) block needs to fetched into memory in a generic case if the same interpolation filter is used in both MV refinement process and final MC interpolation. Clearly, it requires higher memory bandwidth than that of conventioal inter mode, especially when SR is not small.

Figure 2 Memory access for a block in DMVR

However, if different interpolation filters are used for MV refinement and final MC interpolation, the memory bandwidth requirement may be reduced.

Let Tap_SR and Tap_MC-DMVR represent the tap of interpolation filter used in MV refinement process and final MC interpolation for DMVR mode, respectively. The memory access for MV refinement is (W+2×SR+Tap_SR-1)×(H+2×SR+Tap_SR-1). If a worst case of memory access is further assumed after MV refinement that the final MV is at the corner of the search range such as (-SR, -SR), a (W+Tap_MC-DMVR-1)×(H+Tap_MC-DMVR-1) block needs to be further fetched from position (-SR, -SR). Considering the block overlap between MV refinement and final MC, the total memory access is to load a block from position (-SR-Tap_MC-DMVR/2, -SR-Tap_MC-DMVR/2) to (W-1+SR+Tap_SR/2-1, H-1+SR+Tap_SR/2-1), namely a block with size (W+2×SR+Tap_MC-DMVR/2+Tap_SR/2-1)× (H+2×SR+Tap_MC-DMVR/2+Tap_SR/2-1).

To keep the memory access the same as that of the conventional inter mode, i.e., (W+Tap_MC-1)×(H+Tap_MC-1), the following constraint is necessary

2×SR+Tap_MC-DMVR/2+Tap_SR/2 = Tap_MC 					(1)

In VTM-1 and BMS-1 [1], Tap_MC=8. If we have SR=2, Tap_MC-DMVR=6, Tap_SR=2, (1) is satisfied. Therefore, it is proposed to use bilinear interpolation filter for MV refinement and set search range SR to 2 pixels. Moreover, it is proposed to use 6-tap interpolation filter for final MC process for DMVR blocks. In the current implementation, the 6-tap interpolation filter is reused the one used by BIO in BMS-1 [1]. Different 6-tap interpolation filter may also be used.

# Simulation results

The proposed method is implemented on top of CE-9.2.6, where the search range of 2 pixels and bilinear interpolation filter are used for MV refinement process. On top of CE-9.2.6, 6-tap interpolation filter is used for the final MC process of DMVR mode. It should be noted that the proposed method can be combined with any DMVR technique.

First reference: BMS-1 + VTM configuration

Second reference: BMS-1 + VTM configuration + CE-9.2.6

Test: BMS-1 + VTM configuration + CE-9.2.6 + proposed constraint

It is observed from the results that the proposed method only leads to 0.06% luma BD-rate increase on average.

# Conclusions

In this contribution, a memory bandwidth reduction method is proposed for DMVR. With the proposed method, the memory bandwidth requirement by DMVR is kept the same as regular inter modes at the cost of 0.06% luma BD-rate increase on average. It is proposed to further study the method in CE.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1/