| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SL, 10–20 July 2018 | Document: JVET-K0289 |
| --- | --- |

| Title: | CE3-related: Wide angular intra prediction for non-square blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Shan Liu, Xin Zhao, and Xiang Li | Email: | {leolzhao, shanl, xinzzhao, xlxiangli}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to adaptively replace several conventional angular intra prediction modes with wide angular intra prediction modes for non-square blocks. With the proposed method, the intra mode coding process is kept unchanged. The replaced intra prediction modes are signaled using the original signaling method though a remapping is needed after parsing. Simulation results reportedly show that, with almost no impact on the encoder or decoder run-time, average -0.27% and -0.16% BD-rate gain is achieved on top of VTM-1.0 under all intra (AI) and random access (RA) configuration, and -0.XX% and -0.XX% BD-rate gain is achieved on top of BMS-1.0 under AI and RA configuration, respectively.

Introduction

In JVET-J0012 [1], wide angles beyond the range of prediction directions covered by conventional intra prediction modes are proposed, which are called wide angular intra prediction modes. And these wide angles are only applied for non-square blocks as follows:

•	Angles going beyond 45 degree in top-right direction if block width is larger than block height

•	Angles going beyond 45 degree in bottom-left direction if block height is larger than block width

Each wide-angular intra prediction direction is associated with one conventional intra prediction direction, which captures the same prediction direction, but using opposite side of reference samples (left column or top row). For example, in Figure 1, wide-angular modes 35 and 36 are associated with conventional intra prediction mode 3 and 4, respectively.

Signaling for wide angles is implemented by sending a 1-bin identifier for those directions that have an available wide-angle “flip mode”. The actual sample prediction process follows the one in VTM-1, but the effective prediction direction for the wide angular modes is calculated by inverting the signaled prediction direction and switching the reference arrays (using top instead of left or left instead of top).

The figure below illustrates the process for the hypothetical case of 33 HEVC directions and limiting the operation to two modes closest to the diagonal modes. In this example, if the block width is larger than height, mode 3 and mode 4 would have extra flags indicating whether to use the conventional mode or flipped wide angular directions with mode 35 and mode 36.

Figure 1: Illustration of the signaled modes and corresponding wide-angle modes.

Proposed wide angular intra prediction for non-square blocks

For non-square blocks, it is proposed to replace several conventional angular intra prediction modes with wide angular modes. The replaced modes are signaled using the original method and remapped to the indexes of wide angular modes after parsing. The total number of intra prediction modes is unchanged, i.e., 35 as in VTM-1.0, or 67 as in BMS-1.0, and the intra mode coding is unchanged.

In case of 35 intra prediction modes, the replaced intra prediction modes are illustrated in below Table 1.

Table 1: Intra prediction modes replaced by wide-angular modes when 35 intra prediction modes are used.

| Condition | Replaced intra prediction modes |
| --- | --- |
| W / H == 2 | Modes 2, 3, 4 |
| W / H > 2 | Modes 2, 3, 4, 5, 6 |
| W / H == 1 | None |
| H / W == 1/2 | Modes 32, 33, 34 |
| H / W < 1/2 | Modes 30, 31, 32, 33, 34 |

In case of 67 intra prediction modes, the replaced intra prediction modes are illustrated in below Table 2.

Table 2: Intra prediction modes replaced by wide-angular modes when 67 intra prediction modes are used.

| Condition | Replaced intra prediction modes |
| --- | --- |
| W / H == 2 | Modes 2,3,4,5,6,7 |
| W / H > 2 | Modes 2,3,4,5,6,7,8,9,10,11 |
| W / H == 1 | None |
| H / W == 1/2 | Modes 61,62,63,64,65,66 |
| H / W < 1/2 | Mode 57,58,59,60,61,62,63,64,65,66 |

Figure 2 shows an example of how angular intra modes are replaced with wide angular modes for non-square blocks. In this example, mode 2 and mode 3 are replaced with wide angle mode 35 and mode 36, where the direction of mode 35 is pointing to the opposite direction of mode 3, and the direction of mode 36 is pointing to the opposite direction of mode 4.

Figure 2: Illustration of the replaced angular modes and added wide angular modes

Experimental results

The proposed scheme is implemented on top of BMS-1.0 software [2]. The simulations are performed following AI and RA configurations on Linux cluster, using the common test condition described in [3].

Table 3: Coding performance of proposed method over VTM-1.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.19% | -0.26% |  |  | -0.18% | -0.29% | -0.26% |  |  |
| Class A2 | -0.22% | -0.25% | -0.28% |  |  | -0.15% | -0.21% | -0.23% |  |  |
| Class B | -0.24% | -0.38% | -0.34% |  |  | -0.15% | -0.24% | -0.29% |  |  |
| Class C | -0.30% | -0.46% | -0.59% |  |  | -0.17% | -0.22% | -0.36% |  |  |
| Class E | -0.40% | -0.53% | -0.38% |  |  |  |  |  |  |  |
| Overall | -0.27% | -0.37% | -0.38% |  |  | -0.16% | -0.24% | -0.29% |  |  |
| Class D | -0.30% | -0.46% | -0.59% |  |  | -0.12% | -0.17% | -0.27% |  |  |

Table 4: Coding performance of proposed method over BMS-1.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | -0.12% | -0.07% | -0.21% |  |  |
| Class A2 |  |  |  |  |  | -0.10% | -0.10% | -0.02% |  |  |
| Class B | -0.18% | -0.18% | -0.26% |  |  | -0.14% | -0.15% | -0.22% |  |  |
| Class C | -0.22% | -0.29% | -0.26% |  |  | -0.07% | 0.08% | -0.54% |  |  |
| Class E | -0.31% | -0.32% | -0.36% |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  | -0.11% | -0.06% | -0.26% |  |  |
| Class D | -0.22% | -0.11% | -0.16% |  |  | -0.08% | -0.35% | -0.11% |  |  |

Conclusion

In this contribution, it is proposed to adaptively replace several conventional angular intra prediction modes with wide angular intra prediction modes for non-square blocks. With the proposed method, average -0.27% coding gain can be achieved on top of VTM-1.0 for intra coding. It is proposed to adopt this proposal into VTM.

References

- Rickard Sjöberg et. al, “Description of SDR and HDR video coding technology proposal by Ericsson and Nokia,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J0012, San Diego, CA, USA, 10–20 April 2018.

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, San Diego, CA, USA, 10–20 Apr. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).