| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SL, 10–20 July 2018 | Document: JVET-K0289_v1 |
| --- | --- |

| Title: | CE3-related: Wide angular intra prediction for non-square blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Shan Liu, Xin Zhao, and Xiang Li | Email: | {leolzhao, shanl, xinzzhao, xlxiangli}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to adaptively replace several conventional angular intra prediction modes with wide angular intra prediction modes for non-square blocks. With the proposed method, the intra mode coding process is kept unchanged. The replaced intra prediction modes are signaled using the original signaling method though a remapping is needed after parsing. Simulation results reportedly show that, with almost no impact on the encoder or decoder run-time, average -0.27% and -0.16% BD-rate gain is achieved on top of VTM-1.0 under all intra (AI) and random access (RA) configuration, and -0.20% and -0.11% BD-rate gain is achieved on top of BMS-1.0 under AI and RA configuration, respectively.

# Spec text

The following texts describe intra prediction process using wide angular prediction for rectangular block, changes are made on top of HEVC specification draft JCTVC-L1003 section 8.4.4.2.6, texts with line-through indicate deleted texts, yellow highlighted texts indicate added texts.

##### 8.4.4.2.2 Reference sample substitution process for intra sample prediction

Inputs to this process are:

- reference samples p[ x ][ y ] with x = −1, y = −1.. nTbS Height * 2 −1 and x = 0.. nTbS Width  * 2 −1, y = −1, for intra sample prediction,

- a transform block size nTbS,

- a variable nWidth and nHeight specifying the transform block width and height

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the modified reference samples p[ x ][ y ] with x = −1, y = −1.. nTbS Height * 2 −1 and x = 0.. nTbS Width  * 2 −1, y = −1 for intra sample prediction.

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The values of the samples p[ x ][ y ] with x = −1, y = −1.. nTbS Height * 2 −1 and x = 0.. nTbS Width  * 2 −1, y = −1 are modified as follows:

- If all samples p[ x ][ y ] with x = −1, y = −1.. nTbS Height  * 2 −1 and x = 0.. nTbS Width  * 2 −1, y = −1 are marked as "not available for intra prediction", the value 1  <<  ( bitDepth − 1 ) is substituted for the values of all samples p[ x ][ y ].

- Otherwise (at least one but not all samples p[ x ][ y ] are marked as "not available for intra prediction"), the following ordered steps are performed:

- When p[ −1 ][ nTbS Height  * 2 −1 ] is marked as "not available for intra prediction", search sequentially starting from x = −1, y = nTbS Height  * 2 −1 to x = −1, y = −1, then from x = 0, y = −1 to x = nTbS Width  * 2 −1, y = −1. Once a sample p[ x ][ y ] marked as "available for intra prediction" is found, the search is terminated and the value of p[ x ][ y ] is assigned to p[ −1 ][ nTbS Width  * 2 −1 ].

- Search sequentially starting from x = −1, y = nTbS Height  * 2 −1 to x = −1, y = −1, when p[ x ][ y ] is marked as "not available for intra prediction", the value of p[ x ][ y + 1 ] is substituted for the value of p[ x ][ y ].

- For x = 0..nTbS Width  * 2 −1, y = −1, when p[ x ][ y ] is marked as "not available for intra prediction", the value of p[ x − 1 ][ y ] is substituted for the value of p[ x ][ y ].

All samples p[ x ][ y ] with x = −1, y = −1..nTbS Height  * 2 - 1 and x = 0..nTbS Width  * 2 −1, y = −1 are marked as "available for intra prediction".

##### 8.4.4.2.3 Filtering process of neighbouring samples

Inputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS Height * 2 −1 and x = 0..nTbS Width  * 2 −1, y = −1,

- a transform block size nTbS,

- a variable nWidth and nHeight specifying the transform block width and height

Outputs of this process are the filtered samples pF[ x ][ y ], with x = −1, y = −1..nTbS Height * 2 −1 and x = 0.. nTbS Width  * 2 −1, y = −1.

The variable nTbS is set to ( nWidth + nHeight ) /2.

The variable filterFlag is derived as follows:

–	If one or more of the following conditions are true, filterFlag is set equal to 0:

- predModeIntra is equal to INTRA_DC.

- nTbS is equal 4.

–	Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra – 2650 ), Abs( predModeIntra − 1018 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-3.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

Table 8-3 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes

|  | nTbS = 8 | nTbS = 16 | nTbS = 32 |
| --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 7 | 1 | 0 |

When filterFlag is equal to 1, the following applies:

–	The variable biIntFlag is derived as follows:

–	If all of the following conditions are true, biIntFlag is set equal to 1:

- strong_intra_smoothing_enabled_flag is equal to 1

- nTbS is equal to 32

- Abs( p[ −1 ][ −1 ] + p[ nTbS * 2 −1 ][ −1 ] – 2 * p[ nTbS −1 ][ −1 ] ) < ( 1  <<  ( BitDepthY − 5 ) )

- Abs( p[ −1 ][ −1 ] + p[ −1 ][ nTbS * 2 −1 ] – 2 * p[ −1 ][ nTbS −1 ] ) < ( 1  <<  ( BitDepthY − 5 ) )

–	Otherwise, biIntFlag is set equal to 0.

–	The filtering is performed as follows:

–	If biIntFlag is equal to 1, the filtered sample values pF[ x ][ y ] with x = −1, y = −1..63 and x = 0..63, y = −1 are derived as follows:

pF[ −1 ][ −1 ] = p[ −1 ][ −1 ]		(8-30)

pF[ −1 ][ y ] = ( ( 63 − y ) * p[ −1 ][ −1 ] + ( y + 1 ) * p[ −1 ][ 63 ] + 32 )  >>  6 for y = 0..62		 	 	 (8-31)

pF[ −1 ][ 63 ] = p[ −1 ][ 63 ]		(8-32)

pF[ x ][ −1 ] = ( ( 63 − x ) * p[ −1 ][ −1 ] + ( x + 1 ) * p[ 63 ][ −1 ] + 32 )  >>  6 for x = 0..62		 	 	 (8-33)

pF[ 63 ][ −1 ] = p[ 63 ][ −1 ]		(8-34)

–	Otherwise (biIntFlag is equal to 0), the filtered sample values pF[ x ][ y ] with x = −1, y = −1.. nTbS Height * 2 −1  and x = 0.. nTbS Width  * 2 −1, y = −1 are derived as follows:

pF[ −1 ][ −1 ] = ( p[ −1 ][ 0 ] + 2 * p[ −1 ][ −1 ] + p[ 0 ][ −1 ] + 2 )  >>  2	(8-35)

pF[ −1 ][ y ] = ( p[ −1 ][ y + 1 ] + 2 * p[ −1 ][ y ] + p[ −1 ][ y − 1 ] + 2 )  >>  2 for y = 0.. nTbS Height * 2 − 2		 (8-36)

pF[ −1 ][ nTbS Height * 2 −1 ] = p[ −1 ][ nTbS Height * 2 −1 ]	(8-37)

pF[ x ][ −1 ] = ( p[ x − 1 ][ −1 ] + 2 * p[ x ][ −1 ] + p[ x + 1 ][ −1 ] + 2 )  >>  2 for x = 0.. nTbS Width  * 2 − 2		 (8-38)

pF[nTbS Width  * 2 −1 ][ −1 ] = p[ nTbS Width  * 2 −1 ][ −1 ]	(8-39)

##### 8.4.4.2.6 Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1.. nTbS Height * 2 −1 and x = 0.. nTbS Width  * 2 −1, y = −1,

- a transform block size nTbS,

- a variable nWidth and nHeight specifying the transform block width and height

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Figure 8-2 illustrates the total 33 43 intra angles and Table 8-4 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Figure 8-2 – Intra prediction angle definition (informative)

Table 8-4 – Specification of intraPredAngle

| predModeIntra | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | 114 | 79 | 60 | 49 | 39 |  | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 |
| predModeIntra | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 |
| intraPredAngle | −5 | −9 | −13 | −17 | −21 | −26 | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 |
| predModeIntra | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |  |  |  |  |  |  |
| intraPredAngle | 9 | 13 | 17 | 21 | 26 | 32 | 39 | 49 | 60 | 79 | 114 |  |  |  |  |  |  |

Table 8-5 further specifies the mapping table between predModeIntra and the inverse angle parameter invAngle.

Table 8-5 – Specification of invAngle

| predModeIntra | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| predModeIntra | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

When nWidth is not equal to nHeight, predModeIntra is mapped as follows:

- If nWidth/nHeight <= 2 and 2 <= predModeIntra <= 4, predModeIntra = predModeIntra + 33

- If nWidth/nHeight > 2 and 2 <= predModeIntra <= 6, predModeIntra = predModeIntra + 33

- If nHeight/nWidth <= 2 and 32 <= predModeIntra <= 34, predModeIntra = predModeIntra – 35

- If nHeight/nWidth > 2 and 30 <= predModeIntra <= 34, predModeIntra = predModeIntra – 35

The values of the prediction samples predSamples[ x ][ y ], with x = 0.. n TbS Width − 1, y = 0.. nTbS Height  − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0.. nTbS Width 	(8-47)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( n TbS Height * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x = −1..( n TbS Height  * intraPredAngle )  >>  5	(8-48)

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nTbS Width  + 1..2 * nTbS Width 	(8-49)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbS Width − 1, y = 0..nTbS Height  − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>	(8-50)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31	(8-51)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-52)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]	(8-53)

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0 and nTbS is less than 32, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-54)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0.. nTbS Height 	(8-55)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS Width * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = −1..( nTbS Width  * intraPredAngle )  >>  5	(8-56)

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nTbS Height  + 1..2 * nTbS Height 	(8-57)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbS Width − 1, y = 0..nTbS Height  − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5	(8-58)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31	(8-59)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-60)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]	(8-61)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0 and nTbS is less than 32, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-62)