| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K292-v1 |
| --- | --- |

| Title: | CE6-related: Compound orthonormal transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Zhaobin Zhang, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, v_zhaobin, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In Adaptive Multiple Transform (AMT) of BMS, besides DCT-2, four additional transform types including DCT-5/8 and DST-1/7 have been applied for primary transformation. Increased number of transform types requires additional cost for storing multiple transform cores and implementing different transform types. In this contribution, a method of embedding 16-point and 32-point DST-7 into 64-point DCT-2 is proposed, namely Compound Orthonormal Transform (COT). With the proposed method, 16-point and 32-point DST-7 transform cores can be extracted from the proposed 64-point DCT-2 alike transform core so that additional cost of storing DST-7 transform cores is greatly reduced. It is reportedly shown that, when only DST-7/DCT-8 is applied in AMT and maximum block size applying AMT is limited to 32x32, the proposed method gives 0.15%, 0.12% and 0.10% BD-Rate(Y) increase over VTM for All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively. On top of BMS, 0.09%, 0.08% and 0.04% BD-Rate(Y) increase is reported for AI, RA and LDB configurations, respectively, and the memory cost of DST-7/DCT-8 is reduced from 1700 bytes to 100 bytes.

# Problem Statement

In AMT, besides DCT-II, four additional primary transform types, including DCT-5/8 and DST-1/7 are used. Additional cost is needed for applying AMT, including storing and implementing the additional transform types. For example, the current design of AMT requires 20460 bytes memory (DST-7 and DCT-8 are considered to share the same set of bases) to store the primary transform cores. Different from DCT-2, most transform types used in AMT are based on matrix multiplication.

In CE6, several methods for simplifying AMT have been proposed, including only applying DST-7 and DCT-8 in AMT (CE6-1.5), and limiting the maximum block size of AMT to 32x32 (CE6-1.6). However, with these two simplifications, AMT still needs 1700 bytes memory to store the additional transform cores, and AMT need to be implemented separately from DCT-2 since different transform cores are used.

# Proposed method

It is observed that partial of the odd bases (odd basis refers to basis vector associated with an odd index, the first basis is associated with index 0) in DCT-2 is very similar to the scaled DCT-8 bases.

In Figure 1, the second, fourth, sixth, and eighth basis vectors of 64-point DCT-2 are shown, as indicated by solid curves. In addition, the first, second, third and fourth basis vector of 32-point DCT-8, which is scaled by a constant 1/sqrt(2), are also shown in Figure 1, as indicated by dotted curves. From Figure 1 it can be seen that, the first half of the second, fourth, sixth, and eighth basis vectors of 64-point DCT-2 are very close to the first, second, third and fourth basis vectors of scaled 32-point DCT-8 .

Figure 1: The 2nd, 4th, 6th and 8th basis of 64-point DCT-2 (solid curve) and 1st,2nd, 3rd, 4th basis of scaled 32-point DCT-8 (dash curve).

Based on the above observation, it is proposed to replace the odd bases with scaled DCT-8 bases concatenated with their mirrored versions (flipping and sign change), as shown in below Figure 2.

Figure 2 The 2nd, 4th, 6th and 8th basis of 64-point DCT-2 (solid curve), the 1st, 2nd, 3rd, 4th basis of scaled 32-point DCT-8 (dash curve), and the flipped 1st, 2nd, 3rd, 4th basis of scaled 32-point DCT-8 with inverse sign (dot-dash curve).

To achieve the above goal, a 64-point primary transform core is proposed as shown below (only the left half of the transform matrix is shown, the right half can be derived according to the symmetry/anti- symmetry property of DCT-2 core).

where

{aa, ab, ac, ad, ae, af, ag, ah, ai, aj, ak, al, am, an, ao, ap, aq, ar, as, at, au, av, aw, ax, ay, az, ba, bb, bc, bd, be, bf, bg, bh, bi, bj, bk, bl, bm, bn, bo, bp, bq, br, bs, bt, bu, bv, bw, bx, by, bz, ca, cb, cc, cd, ce, cf, cg, ch, ci, cj, ck} =

{ 64, 83, 36, 89, 75, 50, 18, 90, 87, 80, 70, 57, 43, 25, 9, 90, 89, 87, 83, 81, 77, 72, 66, 62, 56, 49, 41, 33, 25, 17, 9, 90, 90, 89, 88, 88, 86, 85, 84, 82, 80, 78, 77, 74, 72, 68, 66, 63, 60, 56, 53, 50, 45, 42, 38, 34, 30, 26, 21, 17, 13, 9, 4 }.

With the above proposed 64-point COT core, we can extract the following 2-point DCT-II core:

4-point DCT-II core:

8-point DCT-II core:

16-point DCT-II core:

32-point COT core:

In addition, we can also extract the 16-point DCT-8 core:

32-point DCT-8 core:

With the proposed method, applying AMT requires only one 4-point and one 8-point DST-7 core, which consumes 100 bytes in total. In addition, part of the 64-point primary transform logic can be reused to implement 16- and 32-point DST-7/DCT-8.

Furthermore, by applying proposed method, the primary transform can be further speed up using the fast DST-7 method proposed in JVET-K0291 [4]. For example, the multiplication operations of 64-point COT is 39% less than 64-point DCT-2. In a 2N-point DCT-2 implementation based on partial butterfly, part of the coefficeints in the transform core are associated with pure multiplications. For example, in partial butterfly, the calculations related to odd bases (the second, fourth, sixth…basis) of DCT-2 are implemented by N add/subs followed by an NxN matrix multiplication. With the proposed method, since the odd bases in the 2N-point DCT-2 core is replaced by N-point DCT-8 core, the NxN matrix multiplication can be implemented alternatively using the fast method described in JVET-K0291. A comparison on the operation counts between DCT-2 and proposed COT is shown below table:

|  | Partial butterfly (HEVC) |  |  | Proposed COT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Transform size | Mult | Add/Sub | Shift | Mult | Add | Shift |
| 4-point | 6 | 8 | 4 |  |  |  |
| 8-point | 22 | 28 | 8 |  |  |  |
| 16-point | 86 | 100 | 16 |  |  |  |
| 32-point | 342 | 372 | 32 | 213 | 287 | 32 |
| 64-point | 1366 | 1428 | 64 | 833 | 1069 | 64 |

# Experimental results

The proposed scheme is implemented on top of BMS-1.1 [1]. Simulations are performed under AI, RA and LDB configurations. Three tests are performed. The first test (Test #1) is actually the combined tests of CE6-1.5 (only applying DST-7/DCT-8 in AMT) and CE6-1.6 (limiting maximum block size in AMT to 32x32), which shows and CE6-1.6 the coding loss when only DST-7/DCT-8 is used for blocks up to 32x32. On top of the first test, the second test (Test #2) and the third test (Test #3) further show the coding performance of applying proposed method under VTM and BMS configuration, respectively.

Test #1

Anchor: BMS-1.1, w/ AMT, VTM configuration

Tested: BMS-1.1, w/ AMT, w/ CE6-1.5, w/ CE6-1.6, VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.26% | 0.22% | 101% | 98% | 0.42% | 0.05% | 0.06% | 84% | 95% |  |  |  |  |  |
| Class A2 | 0.14% | 0.08% | -0.01% | 100% | 98% | 0.19% | 0.09% | 0.07% | 88% | 98% |  |  |  |  |  |
| Class B | 0.23% | -0.01% | 0.03% | 101% | 101% | 0.18% | 0.18% | 0.23% | 89% | 100% | 0.20% | -0.11% | 0.21% | 86% | 97% |
| Class C | 0.12% | -0.08% | -0.08% | 100% | 101% | 0.09% | 0.11% | 0.15% | 91% | 100% | 0.06% | -0.27% | -0.02% | 91% | 101% |
| Class E | 0.28% | -0.05% | -0.05% | 100% | 99% |  |  |  |  |  | 0.37% | -0.73% | -0.12% | 83% | 99% |
| Overall | 0.19% | 0.03% | 0.02% | 100% | 100% | 0.21% | 0.12% | 0.14% | 88% | 99% | 0.20% | -0.32% | 0.05% | 87% | 98% |
| Class D | 0.11% | 0.02% | -0.09% | 100% | 103% | 0.01% | -0.05% | -0.10% | 93% | 100% | 0.07% | 0.17% | -0.27% | 94% | 103% |
| Class F | 0.13% | -0.01% | 0.03% | 99% | 98% | 0.13% | 0.07% | 0.09% | 89% | 97% | 0.11% | -0.02% | 0.01% | 89% | 98% |

Test #2

Anchor: BMS-1.1 w/ AMT, w/ CE6-1.5, w/ CE6-1.6, VTM configuration

Tested: BMS-1.1 w/ AMT, w/ CE6-1.5, w/ CE6-1.6, w/ proposed method, VTM configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | 0.26% | 0.33% | 100% | 101% | 0.20% | 0.10% | 0.14% | 102% | 100% |  |  |  |  |  |
| Class A2 | 0.38% | 0.24% | 0.18% | 101% | 101% | 0.22% | 0.03% | -0.02% | 100% | 100% |  |  |  |  |  |
| Class B | 0.07% | 0.16% | 0.11% | 100% | 100% | 0.10% | -0.08% | 0.13% | 100% | 100% | 0.12% | 0.19% | 0.44% | 99% | 101% |
| Class C | 0.03% | 0.02% | 0.09% | 100% | 101% | 0.03% | -0.03% | -0.10% | 101% | 100% | 0.08% | 0.16% | -0.02% | 100% | 101% |
| Class E | 0.15% | 0.19% | 0.11% | 100% | 101% |  |  |  |  |  | 0.10% | 0.07% | -1.12% | 100% | 101% |
| Overall | 0.15% | 0.16% | 0.15% | 100% | 101% | 0.12% | -0.01% | 0.04% | 101% | 100% | 0.10% | 0.15% | -0.11% | 100% | 101% |
| Class D | 0.03% | -0.18% | 0.06% | 100% | 101% | 0.06% | -0.14% | 0.03% | 100% | 102% | 0.02% | -0.38% | 0.39% | 99% | 97% |
| Class F | 0.00% | -0.03% | -0.04% | 99% | 98% | 0.00% | 0.20% | 0.01% | 100% | 100% | -0.06% | 0.25% | -0.19% | 98% | 100% |

Test #3

Anchor: BMS-1.1 w/ AMT w/ CE6-1.5, w/ CE6-1.6, BMS configuration

Tested: BMS-1.1 w/ AMT w/ CE6-1.5, w/ CE6-1.6 w/ proposed method, BMS configuration

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.35% | 0.25% | 99% | 97% | 0.15% | 0.07% | 0.18% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.12% | 0.17% | 0.09% | 101% | 100% | 0.08% | -0.04% | -0.15% | 100% | 100% |  |  |  |  |  |
| Class B | 0.05% | 0.12% | 0.01% | 100% | 98% | 0.07% | 0.07% | 0.19% | 101% | 101% | 0.07% | -0.07% | -0.15% | 98% | 99% |
| Class C | 0.05% | 0.07% | 0.04% | 99% | 100% | 0.03% | -0.05% | 0.16% | 101% | 100% | 0.05% | 0.05% | 0.11% | 101% | 98% |
| Class E | 0.13% | 0.27% | 0.25% | 105% | 107% |  |  |  |  |  | -0.01% | -1.24% | 0.39% | 99% | 99% |
| Overall | 0.09% | 0.18% | 0.11% | 101% | 100% | 0.08% | 0.02% | 0.11% | 101% | 101% | 0.04% | -0.32% | 0.07% | 99% | 99% |
| Class D | 0.04% | 0.05% | -0.02% | 102% | 103% | 0.09% | 0.00% | 0.02% | 99% | 98% | 0.00% | 0.18% | -0.25% | 101% | 100% |
| Class F | 0.00% | 0.08% | -0.15% | 101% | 108% | 0.03% | -0.17% | 0.06% | 100% | 100% | 0.07% | 0.24% | -0.62% | 100% | 101% |

# Conclusions

In this contribution, a Compound Orthonormal Transform method is proposed. With the proposed method, the additional memory cost of applying DST-7/DCT-8 transform cores is reduced to 100 bytes from 1700 bytes. In addition, comparing to 64-point DCT-2, 40% reduction of multiplications can be achieved by using proposed 64-point COT together with the fast DST-7/DCT-8 method proposed in JVET-K0291.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1/

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, US, 10–20 Apr. 2018.

- A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling”, JVET-J1026, San Diego, US, 10–20 Apr. 2018.

- Z. Zhang, X. Zhao, X. Li, S. Liu, “CE6-related: Fast DST-7/DCT-8 with dual implementation support”, JVET-J1026, Ljubljana, SI, 10–18 July 2018.