| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0293_v2 |
| --- | --- |

| Title: | CE3 related: Simplifications for chroma intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it is proposed that only DM and CCLM modes (when enabled) are used for 2xN or Nx2 chroma blocks. Simulation results reportedly show that average -0.05% luma BD-rate and around 0.5% chroma BD-rate with 19% overall encoding time savings is achieved for VTM-1.0 under all intra (AI) configuration, and -0.02% luma BD-rate and 0.17% chroma BD-rate with 8% encoding time savings is achieved for BMS-1.0 under AI configuration.

# Introduction

In current VVC test mode VTM-1.0 [1], chroma intra coding modes are the same as those in HEVC, including DM and 4 additional angular intra prediction modes. When the chroma intra prediction mode is DM, the intra prediction mode of the co-located luma component is used. Otherwise, the default modes, including Planar, Vertical, Horizontal and DC, are added. In case of DM being equal to any of the default modes, that default mode is replaced by VDIA_IDX (mode 34).

In current BMS-1.0, the cross component linear model (CCLM) mode is also applied for chroma intra coding. The CCLM modes include one LM mode, one multi-model LM (MMLM) and 4 multi-filter LM (MFLM) modes.

# Proposed method

In both VTM-1.0 and BMS-1.0, the minimum chroma block is 2×2 for YUV 4:2:0 formats. However, all the chroma intra modes are used even for those small blocks, which is costly and leads to higher encoder complexity.

To address the above issue, it is proposed to only keep DM and CCLM (in BMS-1.0) for 2xN or Nx2 chroma blocks, where N can be 2, 4, 8, 16 or 32.

# Experimental results

The proposed scheme is implemented on top of BMS-1.0 [1]. The simulations are performed following all intra (AI) and random access (RA) configuration on Linux cluster, and the test sequences are those defined in the common test condition described in [2].

Table 1: Coding performance of proposed method comparing to VTM-1.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.21% | 0.12% | 79% | 95% | -0.03% | 0.13% | 0.27% | 95% | 97% |
| Class A2 | -0.07% | 0.57% | 0.45% | 81% | 99% | -0.03% | 0.43% | 0.41% | 95% | 96% |
| Class B | -0.04% | 0.41% | 0.66% | 80% | 93% | 0.00% | 0.50% | 0.54% | 96% | 95% |
| Class C | -0.05% | 1.24% | 1.53% | 86% | 97% | -0.01% | 1.19% | 1.62% | 97% | 94% |
| Class E | 0.00% | 0.38% | 0.85% | 79% | 93% |  |  |  |  |  |
| Overall | -0.05% | 0.58% | 0.76% | 81% | 95% | -0.02% | 0.59% | 0.75% | 96% | 95% |
| Class D | 0.00% | 1.59% | 2.05% | 80% | 91% | 0.01% | 1.83% | 1.85% | 97% | 92% |

Table 2: Coding performance of proposed method comparing to BMS-1.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% |  | 0.08% |  | 0.07% |  | 92% |  | 100% |  | 0.01% |  | 0.13% |  | 0.13% |  | 100% |  | 101% |  |
| Class A2 | -0.01% |  | 0.16% |  | 0.04% |  | 90% |  | 99% |  | 0.02% |  | 0.02% |  | 0.08% |  | 99% |  | 100% |  |
| Class B | -0.03% |  | 0.18% |  | 0.14% |  | 92% |  | 100% |  | -0.01% |  | 0.17% |  | 0.12% |  | 98% |  | 98% |  |
| Class C | -0.05% |  | 0.24% |  | 0.35% |  | 94% |  | 101% |  | 0.01% |  | 0.37% |  | -0.03% |  | 98% |  | 98% |  |
| Class E | 0.00% |  | 0.19% |  | 0.11% |  | 90% |  | 100% |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% |  | 0.17% |  | 0.16% |  | 92% |  | 100% |  | 0.01% |  | 0.19% |  | 0.08% |  | 99% |  | 99% |  |
| Class D | -0.01% |  | 0.32% |  | 0.30% |  | 92% |  | 103% |  | 0.02% |  | 0.40% |  | 0.30% |  | 97% |  | 94% |  |

# Conclusions

In this contribution, it is proposed that only DM and CCLM modes (when enabled) are used for 2xN or Nx2 chroma blocks. The proposed method achieves -0.05% coding gain for luma component and 0.50% loss for chroma component. It is proposed to adopt this proposal into VTM.

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J1010, San Diego, CA, USA, 10–20 Apr. 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).