| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0295-v1 |
| --- | --- |

| Title: | CE9-related: Constrained Decoder Side Motion Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Meng Xu, Shan Liu | Tel: Email: | {xlxiangli, mengxxu, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Decoder side motion vector derivation (DMVD) techniques were proposed to derive motion information at decoder side after parsing stage. As motion vector (MV) of an inter block may depend on the results of its spatial neighbor’s MV derivation, parsing/determining the initial MV of the current block and prefetching reference block cannot be started until the MV derivation process is finished for the previous block, which leads to serious latency in hardware pipeline. It is reported that the issue can be solved with the method proposed in this contribution at the cost of about 0.6% luma BD-rate increase for VTM-1 + DMVR. It is also reported that the method can be combined with other DMVD methods with similar luma BD-rate increase even when other DMVD method provides higher coding gain.

# Introduction

Decoder side motion vector derivation (DMVD) techniques were proposed to derive motion information at decoder side so that a lot of bits can be saved for motion vector (MV). These methods, such as FRUC and DMVR, showed good coding performance and were studied in CE-9 [2]. However, such methods introduce dependency between the initial MV of the current block and the final MV of the previous block, which leads to pipeline latency issue in hardware design.

# Problem statement

For an inter block in DMVD mode, the decoding process may be in three steps:

- parse/determine initial motion vector MV_init and prefetch reference block;

- refine MV to get final motion vector MV_final based on block matching or other measurement;

- motion compensation with MV_final;

After MV refinement (step 2), MV_final is written back to picture motion field so that the refined MVs may be used as MV predictors for later blocks. Unfortunately, this leads to difficulty in efficient architecture design, especially for hardware.

Figure 1 sample pipeline stages for DMVD methods

Figure 1 shows a sample figure of pipeline stages for DMVD methods like DMVR. In the figure, three major modules represent the three decoding steps for DMVD methods.

CU0 is processed by the three major modules at different time stages, i.e., from T0 to T2. In hardware pipeline design, it is more efficient to have each major module work at each time stage so that within a certain period more CUs can be processed. However, due to MV_final of CU0 may be used as a predictor for the MV_init of CU1, the MV parsing and reference block prefetching of CU1 cannot be started by the first module until MV_final of CU0 is determined in the second module at T1 stage, as shown by the arrows in the figure. Therefore, the parsing and prefetching modules have to wait at T1, which further lead to the other two major modules idle in following time stages. Clearly, it is not efficient for practical codec.

# Proposed method

It can be observed from Figure 1 that the dependency between MV_final of C0 and MV_init of C1 leads to the issue. Therefore, it is proposed to remove such dependency.

First, the CU (CU_prev) which is coded right before the current CU (CU_cur) is identified before decoding/encoding CU_cur. When parsing/determining MV_init of CU_cur, if a MV predictor is from CU_prev and CU_prev is in DMVD mode, the predictor is marked as unavailable for the CU_cur. Consequently, the MV_init of CU_cur is not dependent on MV_final of CU_prev and the dependency is removed.

In BMS-1, DMVR mode is a special merge candidate and there is no specific signaling for DMVR. Moreover, it is possible that the final MV of a block is unchanged after MV refinement. Unfortunately, it is difficult to distinguish whether MV_final would be different from MV_init without actual MV refinement. Therefore, CU_prev is regarded as in DMVD mode if all the following conditions are true in the current implementation.

- CU_prev is in merge mode

- CU_prev is not affine

- CU_prev has two valid but different reference frames

# Simulation results

The proposed method is implemented on top of BMS-1. The methods were tested with two DMVD methods, i.e., DMVR in BMS-1 and DMVR in CE-9.2.5

## Test with DMVR in BMS-1

First reference: BMS-1 + VTM configuration

Second reference: BMS-1 + VTM configuration + DMVR

Test: BMS-1 + VTM configuration + DMVR + proposed constraint

It can be observed from the results that the proposed method keeps the most gain of DMVR. Compared to the DMVR in BMS-1, 0.62% luma BD-rate increase is observed. This is the cost to break the dependency between MV_init of the current block and MV_final of the previously coded block.

## Test with DMVR in CE-9.2.5

First reference: BMS-1 + VTM configuration

Second reference: BMS-1 + VTM configuration + CE-9.2.5

Test: BMS-1 + VTM configuration + CE-9.2.5 + proposed constraint

From the tests above, it can be concluded that the proposed method can be combined with different DMVD methods at similar cost. CE-9.2.5 shows higher coding gain than DMVR in BMS-1 at the cost of longer encoding time. However, the average relative loss introduced by the proposed method is stable though slightly smaller, i.e., 0.58% on average.

# Conclusions

An issue in decoder side motion vector derivation methods is identified in this contribution, i.e., the MV refinement process imposes dependency between the initial MV of the current block and the final MV of the previous block which leads to serious delay issue in hardware pipeline design. A method to break such dependency is proposed. Simulations shows that the latency issue is solved with around 0.6% luma BD-rate increase. Moreover, the method can be combined with different DMVD methods. It is proposed to further study this method in CE.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.1/

- S. Esenlik, Y. W. Chen, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-J1029, San Diego, US, 10–20 Apr. 2018.