| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET- K0298 |
| --- | --- |

| Title: | CE12: report of dynamic range adaptation and DRA refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy, A.K. Ramasubramonian, M. Karczewicz (Qualcomm) E. François, C. Chevance, F. Hiron (Technicolor) | Email: | dmytror@qti.qualcomm.com edouard.francois@technicolor.com |
| Source: | Qualcomm, Technicolor |  |  |

_____________________________

# Abstract

The contribution report results from CE12.1 – CE12.4 tests on out-of-loop mapping for HDR content as proposed in JVET-J0021[1]/J0022 [2]. Proposed dynamic range adaptation (DRA) is applied on input video data, Y’CbCr 4:2:0 picture (10 bits per sample) prior to coding (pre-processing). Inverse DRA is implemented after decoding loop as a post-processing (output process). The complete DRA design includes three parts: DRA mapping, chroma sample refinement, harmonization of DRA scaling with coding QP.

In the CE12.1 and CE12.2 tests, the proposed out-of-loop DRA mapping with chroma sample refinement was tested against the HDR anchor and against in-loop mapping.

In the test CE12.1: for AI fixed QP configuration the proposed method reportedly shows 0.7% and 18.8% of bitrate reduction on average for luma and chroma components respectively (in terms of PSNR L100 and PSNR DeltaE). For RA fixed QP configuration, the proposed method reportedly shows 6.2% of BD-rate reduction for luma (PSNR L100) with 10.6% of BD-rate penalty on chroma (PSNR DeltaE), comparing to HDR anchor.

In the CE12.3 and CE12.4 tests, the complete out-of-loop DRA design comprising the proposed DRA mapping, with chroma sample refinement, and harmonization of DRA scaling with coding QP, was tested against the HDR anchor and against in-loop mapping.

In the test CE12.3: for AI fixed QP configuration, the proposed method reportedly shows 3.1% and 11.7% of BD-rate reduction on average for luma and chroma components respectively (in terms of PSNR L100 and PSNR DeltaE), comparing to HDR anchor. For RA fixed QP configuration, the proposed method reportedly shows 6.6% of BD-rate reduction for luma (PSNR L100) with around 10.3% of BD-rate penalty on chroma (PSNR DeltaE), comparing to HDR anchor.

# Introduction

Dynamic range adaptation (DRA) was proposed in JVET-J0021 and JVET-J0022 for HDR category, and further studied in CE12. The current contribution aims at reporting results from different variants of this technology.

# Tools description

## Luma and cross-component chroma DRA

Dynamic Range Adjustment (DRA) is range-adaptive scaling of luma and chroma samples. Scaling is applied at the pre-processing stage and its inverse is applied at the decoder side as post-processing. Inverse DRA operations at post-processing are described as:

XY=xY*SYi+OYi

XCb=xCb*SCbj+OCbj

XCr=xCr*SCrk+OCrk

where the terms S and O define the piece-wise linear function of the DRA for a particular dynamic range partition of color component, x is an input sample (with xY, xCb and xCr being the Y, Cb and Cr sample), X is the post-processed sample (with XY, XCb and XCr being the Y, Cb and Cr components of the post-processed sample) and i, j and k are indices of the dynamic range partition to which the Y, Cb and Cr components, respectively, of sample x belong.

Cross-Component DRA (CC-DRA) is carried to preserve luma/chroma bitrate allocation for different brightness ranges following the application of the DRA on input chroma samples. At the decoder side, inverse CC-DRA precedes the inverse DRA process. In this approach, for each decoded chroma sample xCx, a chroma scaling component S2CxY is derived from luma inverse DRA scales SYi of the associated luma sample:

S2CxY=funSYxY

The CC-DRA scale S2CxY is obtained by linear interpolation of luma DRA scales of adjacent intervals, e.g. SYi and SYi+1.

The derived CC-DRA scales are applied to a zero-mean chroma samples xCb and xCr based on the luma values xY associated with these chroma samples.

X2Cx=xCx-2bitDepth-1*S2CxY+2bitDepth-1

In practice, in decoder, inverse DRA can be implemented using simple 1D look-up-tables (plotted in Figure 1 and Figure 2).

XY=LUTinvDraY[ xY]

X2Cx=xCx-2bitDepth-1*LUTinvDraCxY+2bitDepth-1

Figure 1. 1D LUT for inverse DRA of luma.

Figure 2. 1D LUTs for inverse DRA of chroma.

## DRA with harmonization of DRA chroma scale and QP

DRA scales and scalar quantizer harmonization consists in adjusting the CC-DRA scale S2CxY by a multiplication factor S3CxQP to preserve a luma/chroma bit allocation assumed by the core coding design for a given QP settings introduced by chroma QP offset table (cf Table 8-10 of HEVC specification).

The CC-DRA applies to chroma samples xCb and xCr as follows:

X2Cx=xCx-2bitDepth-1*S2CxY*S3CxQP+2bitDepth-1

The term S3CxQP aims at compensating the impact of the luma-based QP derivation, which eventually impacts the chroma QP, noted Qpcx. Qpcx is dependent on luma QP, QpY, similar to Table 8-10 of HEVC specification:

qPi = function( QpY, chromaQpOffsetBitstream )

Qpcx = qPi + chromaQpTable

where qPi is the QP index derived from luma QP value QpY, chromaQpOffsetBitstream is the cumulative chroma QP offset signalled in the bitstream and chromaQpTable is an offset introduced by Table 8-10.

## DRA refinement

DRA refinement process aims at compensating the distortion resulting from the compression that may impact the inverse DRA process. The process applies per slice. The inverse DRA LUTs are corrected by the refinement LUTs, modeled by piece-wise linear functions.

LUTinvDraY_ref[ k ]=Min( 2Blum – 1, Max(0, LUTrefineY[ k ]*LUTinvDraY[ k ])

LUTinvDraCx_ref[ k ]=LUTrefineCx[ k ]*LUTinvDraCx[ k ])

The look-up-tables LUTrefineY, LUTrefineCb, LUTrefineCr are built from piece-wise linear (PWL) tables, that can be signaled per slice. The pivot points of PWL tables are equi-distant, and the tables size is equal to 2S+1, S being a parameter signaled in the slice header. When a table is not signaled for a component, the table previously decoded is used.

# Results

The tools described above are implemented within the BMS-1.0 software, configured using the macros as VTM1.0 (JEM_TOOLS macros deactivated). Reference are HDR anchors as defined in the HDR CTCs. Complete simulation results are provided in the attachments to this contribution and summary for all requested metrics are provided in this document.

## CE12-1 test results:

Summary of the simulation results produced in CE12.1 (DRA+chroma refinement) test are provided in Table 1 and Table 2 for AI and RA fixed QP configuration.

Table 1. CE12.1 set of technologies vs HDR anchor, AI fixed QP constraints

|  | BD-rate %, |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -19.0% | -2.9% | -1.1% | -8.8% | -34.5% | 5.9% | 7.6% | -6.8% |
| SunRise | -26.1% | -0.1% | 3.8% | -2.9% | -44.9% | 8.4% | 4.8% | 4.3% |
| ShowGirl2 | -15.5% | 1.2% | 2.9% | -23.2% | 17.3% | 7.4% | -17.2% | 35.4% |
| BalloonFestival | -6.0% | -0.5% | 2.5% | -3.5% | -32.9% | 8.1% | 6.4% | -7.0% |
| Hurdles | -37.7% | -2.4% | -0.9% | -30.4% | -77.6% | 2.8% | -10.9% | -66.4% |
| Starting | -21.9% | -1.5% | 1.1% | -2.9% | -56.0% | 5.2% | 6.9% | -33.7% |
| Cosmos1 | -5.7% | 1.6% | 3.4% | -9.4% | -59.6% | 4.7% | -14.5% | -63.8% |
| Average HDR-B | -18.8% | -0.7% | 1.7% | -11.6% | -41.1% | 6.1% | -2.4% | -19.7% |

Table 2. CE12.1 set of technologies vs HDR anchor, RA fixed QP constraints

|  | BD-rate % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 22.8% | -6.3% | -4.8% | 35.3% | 1058.2% | 1.9% | 66.2% | 94.3% |
| SunRise | 18.5% | -5.5% | -2.0% | 44.2% | 211.6% | 3.0% | 60.2% | 268.5% |
| ShowGirl2 | 10.2% | -2.7% | -1.2% | 23.2% | 33.7% | 2.9% | 34.2% | 53.2% |
| BalloonFestival | 16.6% | -7.2% | -4.8% | 19.0% | 136.2% | 0.0% | 37.6% | 179.4% |
| Hurdles | -16.4% | -7.7% | -5.9% | -7.9% | -44.2% | -2.5% | 16.0% | -2.4% |
| Starting | 14.4% | -9.0% | -6.5% | 24.6% | 51.5% | -2.6% | 41.8% | 104.1% |
| Cosmos1 | 8.4% | -5.2% | -2.9% | 107.0% | 36.7% | -1.3% | 80.5% | 29.0% |
| Average HDR-B | 10.6% | -6.2% | -4.0% | 35.1% | 212.0% | 0.2% | 48.1% | 103.7% |

## CE12-3 test results:

Summary of the simulation results produced in CE12.3 configuration (DRA+scale harmonizing+chroma refinement) are provided in Table 3 and Table 4 for AI and RA fixed QP configuration.

Table 3. CE12.3 set of technologies vs HDR anchor, AI fixed QP constrains

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -10.2% | -4.0% | -2.3% | -13.3% | -27.0% | 4.7% | 2.4% | -3.4% |
| SunRise | -13.4% | -2.7% | 1.1% | -7.1% | -19.7% | 5.9% | 0.9% | -9.6% |
| ShowGirl2 | -8.7% | -0.7% | 0.8% | -17.8% | -31.8% | 5.2% | -11.1% | -24.1% |
| BalloonFestival | -2.5% | -3.6% | -0.6% | -4.5% | -8.6% | 5.1% | 4.7% | 5.6% |
| Hurdles | -29.1% | -4.1% | -2.7% | -24.9% | -43.5% | 0.9% | -8.5% | -28.6% |
| Starting | -14.2% | -4.8% | -2.2% | -7.2% | -22.9% | 1.9% | 2.9% | -8.9% |
| Cosmos1 | -3.5% | -1.7% | 0.5% | -4.7% | -48.1% | 2.0% | -10.2% | -51.0% |
| Average HDR-B | -11.7% | -3.1% | -0.8% | -11.3% | -28.8% | 3.7% | -2.7% | -17.2% |

Table 4. CE12.3 set of technologies vs HDR anchor, RA fixed QP constrains

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 23.0% | -6.5% | -5.0% | 37.7% | 171.8% | 1.7% | 67.1% | 266.1% |
| SunRise | 15.6% | -5.9% | -2.4% | 45.0% | 149.6% | 2.7% | 60.3% | 208.0% |
| ShowGirl2 | 10.0% | -2.9% | -1.4% | 19.6% | 30.5% | 2.7% | 29.6% | 50.5% |
| BalloonFestival | 16.1% | -7.5% | -5.1% | 20.3% | 93.7% | -0.4% | 38.7% | 121.7% |
| Hurdles | -15.9% | -8.0% | -6.2% | -8.2% | -40.9% | -2.8% | 14.6% | -4.8% |
| Starting | 14.7% | -9.5% | -7.0% | 26.0% | 38.2% | -3.2% | 42.5% | 77.3% |
| Cosmos1 | 8.8% | -5.7% | -3.3% | 100.9% | 36.6% | -1.7% | 76.6% | 29.0% |
| Average HDR-B | 10.3% | -6.6% | -4.4% | 34.5% | 68.5% | -0.1% | 47.1% | 106.8% |

# Analysis of results

Complete simulation results are provided in the attachments to this contribution and summary for all requested metrics are provided in this document. Coding scheme tested in Core Experiment CE12.3 implements complete mapping method proposed by the proponents, and its results (Table 3 and Table 4) are discussed in the current section.

An important aspect for results interpretation is the validity of HDR metrics. At the last JVET meeting, BoG on HDR coding discussed results of statistical analysis conducted over CfP MOS score results and reported objective results for CfP responses. Presented statistical results and discussion suggested that HDR metrics (DeltaE 100 and PSNRL100) seems to provide higher correlations with MOS scores across all HDR CfP response and rate points, in comparison to wPSNR and PSNR metrics. Due to this information, results in DeltaE100 and PSNR L100 metrics will be used in the current reporting and discussion.

Analysis of AI simulation results presented in Table 3, suggests that proposed technology provides consistent gain in PSNR L100 and DeltaE for all sequences and across all QP points. These results can be interpreted as a performance assessment for the proposed mapping technology, since temporal psychovisual components of video coding (namely QP cascading) are not affecting these results.

Analysis of RA simulation results presented in Table 4, indicates that proposed technology provides 6.6% BD-rate reduction in luma component, with 10% penalty for chroma. It can be argued that the chroma penalty originates from the fact that chroma QP offset (responsible for luma/chroma bitrate allocation) deployed by the HDR anchor relies on the input QP value and does not follow the QP cascading (namely QP increase for B frames) utilized in RA configuration and thus “over improve” the quality of the chroma in the anchor configuration.  However, if needed, penalty in chroma can be obviously removed by redistributing bits from luma to chroma by using generic chroma QP offset technique.

# Additional per-tool results

This contribution also provides additional simulation results to assess the performance of two components consisting the proposal: DRA scale and coding QP harmonization and DRA refinement.

## DRA scale and QP harmonization:

Comparing technologies in CE12.3 and CE12.1, allows to assess the performance of the DRA scale and QP harmonization on top of DRA and DRA-refinement.

Table 5. Performance of the DRA scale and QP harmonization component compared against the CE12.1 configuration (Harmonization disabled), AI fixed QP constrains

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 9.6% | -1.1% | -1.2% | -2.9% | -8.1% | -1.2% | -4.3% | -1.4% |
| SunRise | 14.0% | -2.5% | -2.6% | -3.6% | 5.0% | -2.4% | -3.2% | -21.2% |
| ShowGirl2 | 6.5% | -1.8% | -2.0% | 7.0% | -14.5% | -2.0% | 7.7% | -23.6% |
| BalloonFestival | 3.4% | -3.0% | -3.0% | 0.3% | 19.2% | -3.0% | 0.0% | 16.2% |
| Hurdles | 6.3% | -1.7% | -1.7% | 2.0% | 13.8% | -1.8% | 1.2% | 15.9% |
| Starting | 7.3% | -3.2% | -3.2% | -1.9% | 23.5% | -3.2% | -2.3% | 24.5% |
| Cosmos1 | 2.2% | -3.2% | -2.8% | 6.3% | 7.7% | -2.7% | 6.1% | 8.6% |
| Average HDR-B | 7.0% | -2.4% | -2.4% | 1.0% | 6.7% | -2.3% | 0.7% | 2.7% |

Table 6. Performance of the DRA scale and QP harmonization component compared against the CE12.1 configuration (Harmonization disabled), RA fixed QP constrains

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | 1.1% | -0.2% | -0.2% | 2.4% | -72.5% | -0.2% | 2.5% | 97.8% |
| SunRise | 1.0% | -0.4% | -0.4% | 1.4% | -3.3% | -0.3% | 1.3% | -5.2% |
| ShowGirl2 | 0.4% | -0.2% | -0.2% | -0.9% | 2.4% | -0.2% | -1.3% | 3.9% |
| BalloonFestival | 0.0% | -0.4% | -0.4% | 1.2% | -3.2% | -0.4% | 1.2% | -4.2% |
| Hurdles | 0.7% | -0.3% | -0.3% | 1.7% | 1.7% | -0.3% | 1.6% | 1.4% |
| Starting | 1.8% | -0.5% | -0.6% | 1.5% | -0.8% | -0.6% | 1.6% | -2.2% |
| Cosmos1 | 0.5% | -0.5% | -0.4% | 0.1% | 1.5% | -0.4% | 0.3% | 1.0% |
| Average HDR-B | 0.8% | -0.3% | -0.4% | 1.1% | -10.6% | -0.3% | 1.0% | 13.2% |

## DRA Refinement:

The tables below depict the benefits of the refinement step only, by comparing performance of (DRA+scale harmonizing+chroma refinement) versus (DRA+scale harmonizing).

Table 5. Performance of the DRA refinement, AI fixed QP constraints

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -1.1% | 0.0% | 0.0% | -4.1% | -3.1% | -0.1% | -6.8% | -3.0% |
| SunRise | -2.6% | 0.2% | 0.2% | -4.8% | -4.9% | 0.1% | -5.6% | -6.0% |
| ShowGirl2 | -2.6% | -0.2% | -0.1% | -12.7% | -17.8% | 0.0% | -12.4% | -20.9% |
| BalloonFestival | -0.6% | 0.1% | 0.1% | -4.8% | -3.2% | 0.0% | -8.0% | -5.4% |
| Hurdles | -13.6% | 0.1% | 0.0% | -8.4% | -18.5% | 0.0% | -5.7% | -13.0% |
| Starting | -4.2% | 0.1% | 0.1% | -2.3% | -7.3% | 0.1% | -3.3% | -4.9% |
| Cosmos1 | -3.8% | -0.3% | -1.0% | -31.7% | -51.6% | -1.1% | -33.6% | -55.6% |
| Average HDR-B | -4.1% | 0.0% | -0.1% | -9.8% | -15.2% | -0.1% | -10.8% | -15.6% |

Table 6. Performance of the DRA refinement, RA fixed QP constraints

|  | BD-rate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV |
| Market | -1.2% | 0.0% | 0.0% | -4.5% | -5.5% | 0.0% | -6.9% | -5.1% |
| SunRise | -2.9% | 0.2% | 0.2% | -4.9% | -6.5% | 0.2% | -5.3% | -7.6% |
| ShowGirl2 | -2.0% | -0.1% | 0.0% | -15.7% | -21.5% | 0.0% | -15.6% | -24.3% |
| BalloonFestival | -1.2% | 0.0% | 0.0% | -7.7% | -6.6% | 0.0% | -12.6% | -10.5% |
| Hurdles | -19.9% | 0.1% | 0.0% | -9.0% | -42.4% | 0.0% | -6.4% | -31.7% |
| Starting | -4.8% | 0.1% | 0.1% | -2.2% | -19.6% | 0.1% | -3.1% | -12.0% |
| Cosmos1 | -4.3% | -0.9% | -0.7% | -47.9% | -55.0% | -0.9% | -48.7% | -57.0% |
| Average HDR-B | -5.2% | -0.1% | 0.0% | -13.1% | -22.4% | -0.1% | -14.1% | -21.2% |

# Conclusion

This contribution report results from CE12.1 – CE12.4 tests on out-of-loop mapping for HDR content as proposed in JVET-J0021[1]/J0022 [2]. Simulation results show that complete method proposed to JVET provides a consistent gain of 3% and 11% of BD-rate reduction in AI configuration, and 6.6% BD-rate gain for luma with 10% of BD-rate penalty chroma. It is argued that if needed, penalty in chroma can be suppressed with “inexpensive” bits redistribution from luma to chroma by using generic chroma QP offset technique.

# References

- Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions, 10th JVET Meeting, San Diego, California, USA, April 2018, JVET-J0021.

- Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version, 10th JVET Meeting, San Diego, California, USA, April 2018, JVET-J0022.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).