| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0300-v1 |
| --- | --- |

| Title: | Design goals for tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Alireza Zare, Maryam Homayouni, Ramin Ghaznavi-Youvalari, Alireza Aminlou Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes that the VVC tile design should enable

- Encoding of motion-constrained tile sets (MCTSs) that are more efficient than HEVC MCTSs in terms of rate-distortion penalty;

- Avoiding visible MCTS boundaries with as small processing cost as possible;

- Intra block copy across tiles for enabling prediction from one constituent frame to another for frame-packed stereoscopic video, provided that intra block copy is adopted as a tool in VVC;

- Extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without VCL NAL unit modifications.

The proposed design goals are asserted to make VVC tiles suited for viewport-dependent 360° streaming.

The design goals are proposed to be used in evaluating merits of technical contributions and to be included as mandates of an appropriate JVET ad-hoc group.

# Introduction

Section 5.17 of MPEG N17074 [1] includes the following future video coding requirements for omnidirectional video and projections:

- The next-generation video codec shall efficiently code monoscopic and stereoscopic content captured using one or more lenses and the combined result   mapped using non-uniform projections to one or two rectangles, respectively.
Example: Good compression efficiency shall be achieved using wide field-of-view lenses, and equirectangular panoramas.

- The next-generation video codec should enable efficient extraction of a desired viewport from the compressed bit stream.
Example: An extracted viewport should have the substantially same coding efficiency as this perspective coded on its own.

- The next-generation video codec should enable inclusion of parameters in the bitstream related to any correction due to optics or pre-processing, which may be useful to the rendering device in order to accurately reproduce the scene.

This contribution proposes design goals for the VVC tile design that would reportedly make VVC suitable for tile-based viewport-dependent 360° video streaming. The design goals could be regarded as refinements of the above-quoted requirements.

The design goals are proposed to be used in evaluating merits of technical contributions and to be included as mandates of an appropriate JVET ad-hoc group.

The contribution is organized as follows:

- Section 2 of this contribution reviews approaches for tile-based viewport-dependent 360° HEVC video streaming.

- Section 3 describes why the HEVC tile design is asserted to be sub-optimal for viewport-dependent streaming and a design goal for making the VVC tile design better suited for viewport-dependent streaming.

- Sections 4 and 5 provide the references and patent right declaration, respectively.

# Background: tile-based viewport-dependent 360° video streaming

## Background: cubemap projection

Many examples that are provided below use cubemap projection, which is therefore briefly recapped in this section. In cubemap projection (CMP), 2D pictures comprise cube faces on which the spherical video is projected (Figure 1a). Figure 1b presents the default cube face arrangement.

Figure 1. a) Cubemap projection;			b) default cube face arrangement.

## Categorization of tile-based viewport-dependent streaming

Viewport-dependent video streaming is asserted to provide the following advantages over viewport-independent video streaming of 360° video:

- Better rate-distortion performance, where rate is understood as the streaming bitrate

- Higher effective resolution on the viewport with the same decoding capacity.

In tile-based viewport-dependent streaming approaches, projected pictures are partitioned into tiles that are coded as motion-constrained tile sets (MCTSs). Tile-based viewport-adaptive streaming schemes can be categorized as follows:

- Region-wise mixed quality (RWMQ) 360° video: Several versions of the content is encoded using MCTSs on the same tile grid, each version with different bitrate and picture quality. Players select on MCTS basis which version is received so that the quality of the MCTSs covering the viewport is higher than that of the other received MCTSs.

- Viewport + 360° video: MCTSs for a complete low-resolution omnidirectional picture and high-resolution tiles covering the viewport are received.

- Region-wise mixed resolution (RWMR) 360° video: Tiles are encoded at multiple resolutions. Players select a combination of high resolution tiles covering the viewport and low-resolution tiles for the remaining areas.

## Example of region-wise mixed quality (RWMQ) 360° video

The example is copied from clause D.4.2 of OMAF [2] and illustrated in Figure 2. A 4×2 tile grid has been used in forming of the motion-constrained tile sets for equirectangular projected pictures. Two HEVC bitstreams originating from the same source content are encoded at different picture qualities and bitrates. Each motion-constrained tile set sequence is included in one sub-picture track and an extractor track is also created. The OMAF player chooses the quality at which each sub-picture track is received based on the viewing orientation. In this example the OMAF player receives sub-picture tracks 1, 2, 5, and 6 at a particular quality and sub-picture tracks 3, 4, 7, and 8 at another quality. The extractor track is used to reconstruct a bitstream that could be decoded with a single HEVC decoder.

Figure 2. Example of MCTS-based RWMQ viewport-dependent 360° streaming.

## Example of 360° + viewport video

This example is illustrated in Figure 3 with cubemap projection. Another example with more details is provided in clause D.8 of OMAF [2].

Encoding: Content is encoded at two spatial resolutions: 448×448 per cube face and 1792×1792 per cube face. One MCTS per cube face is used in the low-resolution version, while 4×4 MCTSs per cube face are used in the high-resolution version, resulting into 96 MCTSs per picture.

Selection of streamed MCTSs: 36 MCTSs covering the viewport are selected from the high-resolution bitstream to be streamed. The selected MCTSs could cover e.g. a region of adjacent 6×6 MCTSs, which could be considered to correspond to a field of view of 135°×135°. All the six MCTSs of the low-resolution bitstream are also streamed and serve as a fallback e.g. for sudden viewport changes.

Merging MCTSs to a bitstream to be decoded: The received MCTSs of a single time instance are merged into a coded picture of 7×6 MCTSs, which corresponds to 3136×2688 (= 8 429 568) luma samples and thus complies with HEVC Level 5.1.

a) coded bitstreams		b) MCTSs selected for streaming		c) Picture merged 
 												from MCTSs

Figure 3. Example of 360° + viewport video.

## Example of region-wise mixed resolution (RWMR) 360° video

This scheme offers an increased effective spatial resolution on the viewport. Schemes where tiles covering the viewport originate from a 6K (6144×3072) ERP picture or an equivalent CMP resolution, illustrated in Figure 4, with "4K" decoding capacity (HEVC Level 5.1) were included in clauses D.6.3 and D.6.4 of OMAF [2] and also adopted in the VR Industry Forum Guidelines [3]. Such resolutions are asserted to be suitable for head-mounted displays using quad-HD (2560×1440) display panel.

Encoding: The content is encoded at two spatial resolutions with cube face size 1536×1536 and 768×768, respectively. In both bitstreams a 6×4 tile grid is used and an MCTS is coded for each tile position.

Selection of streamed MCTSs: 12 MCTSs from the high-resolution bitstream are selected and the complementary 12 MCTSs are extracted from the low-resolution bitstream. Thus, a hemi-sphere (180°×180°) of the streamed content originates from the high-resolution bitstream.

Merging MCTSs to a bitstream to be decoded: The received MCTSs of a single time instance are merged into a coded picture of 1920×4608, which conforms to HEVC Level 5.1. Another option for the merged picture is to have 4 tile columns of width 768, two tile columns of width 384, and three tile rows of height 768 luma samples, resulting into a picture of 3840×2304 luma samples.

a) coded bitstreams		b) MCTSs selected for streaming		c) Picture merged 	from MCTSs

Figure 4. Example MCTS-based RWMR viewport-dependent 360° streaming.

# Suggested tile design goals in VVC

## Reduced rate-distortion cost for encoding motion-constrained tile sets

### Motivation

The more fine-grained the MCTS grid is, the closer the viewport can be matched with high-resolution or high-quality streamed MCTSs, while the remaining areas can be covered with lower resolution or quality. However, fine-granularity MCTS grids also bring a rate-distortion cost in inter prediction.

The MCTS rate-distortion overhead of a few MCTS grids of ERP and CMP was studied. In all cases, the MCTS grid was identical to the tile grid, i.e., each tile was also an MCTS.

The results provide an insight of the bitrate increase caused by finer tiling. This bitrate increase would need to be compensated by streaming a smaller spherical area of high-resolution or high-quality tiles.

The ERP simulations were carried out with HM using the random access (RA) configuration for 97 frames for a range of 4K and 6K test sequences. Tile grids 6×1, 4×2, and 1281 were tested. The results are summarized in Table 1.

Table 1. Y-BDBR (%) of different MCTS grids for equirectangular projection compared to not using tiles.

|  | w | h | 6x1 | 4x2 | 12x8 |
| --- | --- | --- | --- | --- | --- |
| BearAttack | 3840 | 1920 | 1.15 | 1.04 | 7.06 |
| Daisy | 3840 | 1920 | 1.00 | 2.13 | 19.37 |
| LRRH | 3840 | 1920 | 0.40 | 0.46 | 3.57 |
| AerialCity | 3840 | 1920 | 4.70 | 5.63 | 22.64 |
| DrivingInCity | 3840 | 1920 | 3.84 | 7.54 | 20.08 |
| DrivingInCountry | 3840 | 1920 | 5.58 | 6.32 | 23.49 |
| PoleVault | 3840 | 1920 | 0.99 | 0.83 | 5.14 |
| Sheriff | 4096 | 2048 | 0.94 | 1.01 | 9.49 |
| Driving360 | 4096 | 2048 | 6.97 | 11.63 | 40.65 |
| Dancing360 | 4096 | 2048 | 0.99 | 1.30 | 12.83 |
| Balboa | 6144 | 3072 | 3.38 | 4.07 | 16.75 |
| BranCastle | 6144 | 3072 | 1.17 | 0.98 | 5.83 |
| Broadway | 6144 | 3072 | 2.57 | 2.59 | 14.12 |
| Landing | 6144 | 3072 | 1.79 | 1.63 | 10.02 |
| Average |  |  | 2.53 | 3.37 | 15.07 |

Cubemap simulations were carried out with HM using the random access (RA) configuration for 17 frames for each of the 8K test sequences in the JVET common test conditions and evaluation procedures for 360° video. The ERP input was first converted to CMP of different resolutions using 360Lib-5.0. The results are summarized in Table 2.

Table 2. Y-BDBR of different MCTS grids for cubemap projection compared to not using tiles.

| Cube face size |  | 1024x1024 |  |  | 1536x1536 |  |  | 2048x2048 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # MCTSs | 6 | 24 | 96 | 6 | 24 | 96 | 6 | 24 | 96 |
| # MCTSs / cube face | 1x1 | 2x2 | 4x4 | 1x1 | 2x2 | 4x4 | 1x1 | 2x2 | 4x4 |
| Trolley | 0.01 % | 1.01 % | 4.63 % | -0.11 % | 0.84 % | 3.90 % | -0.25 % | 0.36 % | 2.46 % |
| GasLamp | 0.55 % | 3.02 % | 12.51 % | 0.13 % | 2.59 % | 10.70 % | -0.17 % | 1.21 % | 6.57 % |
| Skateboarding_in_lot | 0.25 % | 9.07 % | 21.28 % | -0.78 % | 7.88 % | 18.60 % | -0.93 % | 4.48 % | 11.50 % |
| Chairlift | 1.56 % | 9.73 % | 29.43 % | 0.78 % | 8.80 % | 26.30 % | 0.23 % | 5.81 % | 18.72 % |
| KiteFlite | 0.09 % | 1.74 % | 5.73 % | -0.15 % | 1.54 % | 5.06 % | -0.40 % | 0.76 % | 3.30 % |
| Harbor | -0.26 % | 2.63 % | 11.52 % | -0.63 % | 2.25 % | 9.90 % | -0.54 % | 1.19 % | 6.11 % |
| Average | 0.37 % | 4.53 % | 14.18 % | -0.13 % | 3.98 % | 12.41 % | -0.35 % | 2.30 % | 8.11 % |

It can be observed that for some sequences the rate-distortion cost caused by MCTSs is significant.

### Suggested design goal

The tile design of VVC should enable encoding of motion-constrained tile sets (MCTSs) that are more efficient than HEVC MCTSs in terms of rate-distortion penalty.

## MCTS boundary visibility

### Motivation

In order to make MCTSs independent of each other, deblocking across MCTS boundaries is turned off for viewport-dependent video. Since no deblocking is applied and since prediction dependencies are disabled in tile boundaries, they can become visible. Figure 5 presents examples of visible tile boundaries.

Figure 5. Example of tile boundaries, Tampere Polevault, QP37.

The visibility of tile boundaries can be concealed by encoding slightly overlapped content in tiles. The overlapped regions can be blended as a decoder-side post-processing step. However, such an approach either requires greater decoding capacity or causes smaller effective picture sizes for region-wise mixed-quality or mixed-resolution viewport-dependent streaming. Moreover, region-wise mixed-quality streaming would no longer use the projected picture format as such but region-wise packing would be required.

### Suggested design goal

The tile design of VVC should enable avoiding visible MCTS boundaries with as small processing cost as possible.

## Intra block copy prediction across tiles for achieving inter-view prediction in stereoscopic video

### Motivation

In simulations of JVET-G0064 [4], HEVC screen content coding was tested for stereoscopic top-bottom frame-packed ERP content. HEVC SCC 16.15_SCM8.4 was used with without other screen content coding tools than intra block copy (IBC). The Y-BDBR in random access common test conditions was found to be -19.3% on average. This result suggests that IBC would significantly improve the rate-distortion performance of frame-packed stereoscopic video.

As discussed in previous sections, the use of motion-constrained tile sets is desired in viewport-dependent streaming.

As a consequence of the paragraphs above, it is proposed to enable intra block copy across tiles.

### Suggested design goal

Provided that intra block copy is adopted as a tool in VVC, the tile design of VVC should enable intra block copy across tiles for enabling prediction from one constituent frame to another for frame-packed stereoscopic video.

## MCTS reordering

### Motivation

As demonstrated in the examples of 360° + viewport video (Section 2.4) and region-wise mixed resolution 360° video (Section 2.5), the tile positions in the encoder outputs differ from those of the merged bitstream that is input to the decoder. Thus, parameter sets and slice headers need to be rewritten for the merged bitstream. Two approaches exist for such rewriting:

- Client-side slice header rewriting. Since this is not part a standardized decoding operation, it might not be supported by decoder APIs and many decoder implementations.

- Extractor track with rewritten slice header. ISO/IEC 14496-15 (the file format for NAL unit structured video) includes the design for extractors, which can be used for rewriting parameter sets and slice headers in the extractor track, while the tile data is included by reference. This approach might require one extractor track per each possible extracted combination, such as one per each range of 360° video viewing orientations that results in a different set of picked tiles.

### Suggested design goal

The tile design of VVC should enable extracting VCL NAL units of a subset of MCTSs from one VVC bitstream and reposition them to another VVC bitstream without VCL NAL unit modifications.

# References

- MPEG N17074, "Requirements for a Future Video Coding Standard v5," July 2017.

- MPEG N17563, "Second revised text of ISO/IEC FDIS 23090-2," April 2018.

- Virtual Reality Industry Forum Guidelines, draft version 1.1, May 2018. Online: https://www.vr-if.org/guidelines/, accessed 2018-06-24.

- F. Henry, J. Jung, A. Ouach, B. Ray, and P. Schwellenbach, "Stereoscopic 360 video compression with the next generation video codec," JVET-G0064, July 2017.

# Patent rights declaration(s)

Nokia Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).