| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0301-v1 |
| --- | --- |

| Title: | CE4-related: extension of merge and AMVP candidates for inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, and Shan Liu 661 Bryant St, Palo Alto, CA 94301 USA | Tel: Email: | +1-650-798-3300 guichunli@tencent.com xiaozhongxu@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to add two spatial neighboring positions, which are from the middle position at the left and top edge of the current block, into motion vector predictor candidate list. The new candidates are put at the beginning of the merge candidate list and the AMVP candidate list, if available. The number of allowed merge candidates or AMVP candidates is kept unchanged. The proposed method was tested on BMS-1.0. It is reported that with VTM configuration, the result has 0.14%, 0.12%, and 0.16% BD-rate improvement for Random Access, Low Delay B, and Low Delay P, respectively.

# Introduction

In current Versatile Video Coding Test Model (VTM), the positions of spatial motion vector predictors for inter merge candidates and AMVP candidates are as depicted in Figure 1.

Figure 1 - Merge Candidates Currently used in VTM

A0 is the left-bottom candidate, A1 is the left candidate, B0 is the above-right candidate, B1 is the above candidate, and B2 is the above-left candidate.

In inter merge mode, the order of spatial merge candidates is {A1, B1, B0, A0, B2}.

The following pruning process is performed in current VTM software is shown in Table 1.

Table 1-Pruning Process of Merge Candidates in VTM

| Candidate | Pruning |
| --- | --- |
| A1 | If A1 available |
| B1 | If B1 available, prune against A1 |
| B0 | If B0 available, prune against B1, A1 |
| A0 | If A0 available, prune against A1, B1, B0 |
| B2 | If B2 available, prune against A1, B1, A0, B0 |

In AMVP mode, the order of derivation of spatial motion vector predictor candidates on left side is

{A0, A1, Scaled A0, Scaled A1};

and the order of derivation on above side is

{B0, B1, B2, Scaled B0, Scaled B1, Scaled, B2}.

# Description of Proposed Method

Two new spatial candidate positions are included in the proposed method. As depicted in Figure 2, the new candidates are A2, left-middle candidate, and B3, above-middle candidate. These two candidates are added into both merge candidate list and AMVP candidate list. For both list, the number of allowed candidates, as well as the number of full RD evaluations, is kept unchanged.

Figure 2 - Merge Candidates used in Proposed Method

## Candidate extension in Inter Merge Mode

In inter merge mode, the order of spatial merge candidate derivation in the proposed method is {A2, B3, A1, B1, B0, A0, B2}. Pruning is applied to remove redundant merge candidates. And the derivation process stops when maximum number of merge candidates is met.

The derivation and pruning process is shown in order in the following Table 2. Additional pruning is applied only for B3, A1, and B1 to reduce the impact on complexity.

Table 2-Pruning Process of Merge Candidates in Proposed Method

| Candidate | Pruning |
| --- | --- |
| A2 | If A2 available |
| B3 | If B3 available, prune against A2 |
| A1 | If A1 available, prune against A2, B3 |
| B1 | If B1 available, prune against A1, A2, B3 |
| B0 | If B0 available, prune against B1, A1 |
| A0 | If A0 available, prune against A1, B1, B0 |
| B2 | If B2 available, prune against A1, B1, A0, B0 |

## Candidate extension in AMVP mode

In AMVP mode, the order of derivation of spatial motion vector predictor candidates on left side is

{A2, A0, A1, Scaled A2, Scaled A0, Scaled A1};

and the order of derivation on above side is

{B3, B0, B1, B2, Scaled B3, Scaled B0, Scaled B1, Scaled, B2}.

# Simulation Results

The proposed method is implemented in BMS-1.0 software [1]. And simulations were conducted on both VTM configuration and BMS configuration, using common test conditions in [2].

In the following tables (table 3, 4, and 5), the BD-rate, encoding time and decoding time changes for the proposed method are reported against the BMS-1.0 software with VTM configuration.

Table 3-Simulation result with VTM configuration, Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.20% | -0.01% | 101% | 105% |
| Class A2 | -0.23% | -0.18% | -0.16% | 100% | 103% |
| Class B | -0.11% | -0.11% | -0.08% | 101% | 101% |
| Class C | -0.16% | -0.20% | -0.15% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.16% | -0.10% | 101% | 103% |
| Class D | -0.12% | 0.02% | -0.07% | 100% | 104% |
| Class F (optional) | -0.08% | -0.04% | -0.05% | 101% | 104% |

Table 4-Simulation result with VTM configuration, Low Delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.08% | 0.26% | 103% | 107% |
| Class C | -0.12% | -0.10% | -0.10% | 101% | 101% |
| Class E | -0.17% | 0.10% | -0.36% | 101% | 103% |
| Overall | -0.12% | -0.04% | -0.02% | 102% | 104% |
| Class D | -0.20% | -0.17% | -0.07% | 102% | 106% |
| Class F (optional) | -0.21% | -0.37% | 0.37% | 101% | 102% |

Table 5-Simulation result with VTM configuration, Low Delay P

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | 0.00% | 0.01% | 103% | 107% |
| Class C | -0.13% | -0.05% | -0.11% | 101% | 110% |
| Class E | -0.27% | -0.18% | -0.13% | 96% | 103% |
| Overall | -0.16% | -0.06% | -0.07% | 101% | 107% |
| Class D | -0.17% | -0.51% | -0.21% | 98% | 96% |
| Class F (optional) | -0.08% | -0.32% | 0.63% | 101% | 103% |

It was observed that with the proposed method, in VTM configuration, there are 0.14%, 0.12%, and 0.16% BD-rate improvements for Random Access, Low Delay B, and Low Delay P, respectively.

In the following tables (table 6) the BD-rate, encoding time and decoding time results for the proposed method are reported against the BMS-1.0 software with BMS configuration.

Table 6-Simulation result with BMS configuration, Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.10% | -0.04% | 102% | 101% |
| Class A2 | -0.10% | -0.08% | -0.11% | 101% | 101% |
| Class B | -0.09% | -0.04% | -0.10% | 99% | 100% |
| Class C | -0.08% | -0.11% | -0.12% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.08% | -0.09% | 100% | 100% |
| Class D | -0.01% | 0.05% | -0.02% | 97% | 96% |
| Class F (optional) | -0.06% | -0.05% | -0.08% | 99% | 97% |

With BMS configuration, there is 0.08% BD-rate gain for Random Access.

# Conclusion

It was observed that the proposed method has BD-rate performance improvement with minor changes on the software. It is proposed to adopt the proposed method in the Versatile Video Coding standard or to include it into Core Experiments for further evaluation.

# References

[1] BMS-1.0 software: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).