| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0304-v1 |
| --- | --- |

| Title: | CE4-related: ranking based spatial merge candidate list for inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, and Shan Liu 661 Bryant St, Palo Alto, CA 94301 USA | Tel: Email: | +1-650-798-3300 guichunli@tencent.com xiaozhongxu@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to replace the original spatial merge candidate list construction method with a ranking based method and the positions of merge candidates are extended to include more potential candidates. The proposed spatial candidates are derived and put at the beginning of the merge candidate list. The number of maximum merge candidates is kept unchanged. Other non-spatial merge candidates are added afterwards until the merge list is full. The proposed method was tested on BMS-1.0 reference software. It is reported that with VTM configuration, the result has 0.20%, 0.38%, and 0.53% BD-rate improvement for Random Access, Low Delay B, and Low Delay P, respectively.

# Introduction

In current Versatile Video Coding Test Model (VTM), the positions of spatial motion vector predictors for inter merge candidates and AMVP candidates are as depicted in Figure 1.

A0 is the left-bottom candidate, A1 is the left candidate, B0 is the above-right candidate, B1 is the above candidate, and B2 is the above-left candidate.

Figure 1 - Merge Candidates Currently used in VTM

# Description of Proposed Method

In the proposed method, before constructing the merge candidate list, spatial neighboring blocks are scanned in a defined range, and the statistics of all available spatial merge candidates in the range are collected.

A histogram of the motion information is constructed, based on the count of neighboring blocks with same motion information.

In proposed method, each scanned spatial candidate block is in minimum block size (4x4 luma samples in VTM), such block will be referred to as min-block in this document. As depicted in Figure 3, the scanning range of spatial candidates includes min-blocks which are immediately adjacent to the current block (including top-left position), M min-blocks from the bottom-left neighboring block downwards, and N min-blocks from the above-right neighboring block to the right.

The value of M is related to the current block height H (in luma pixels). Where M = H/4.

The value of N is determined by the current block width W (in luma pixels). Where N = W/4.

Figure 2 – Scanning of Spatial candidates adjacent to current block and extended to more positions on left and above

After the scanning process, a histogram of neighboring blocks’ motion information is generated. Each histogram bin corresponds to an individual motion information from all the available spatial neighbors. Then all the motion information with non-zero counts in the histogram are sorted in descending order.

The motion information entries on the sorted list with largest counts will be added at the beginning of the merge candidate list, until the maximum allowed number of candidates is reached.

In case the number of candidates on the sorted list is smaller than max merge candidates, other non-spatial merge candidates will be derived in the original order as in VTM until the candidate list is fully constructed.

Collect statistics of all neighbors in defined range

Build a sorted histogram of available motion information

Fill the candidate list from top of the sorted list of motion info.

List Full?

Fill non-spatial merge candidates in original order

Finish

A diagram of the proposed method of merge candidate list construction is shown in Figure 3.

Figure 3 – Diagram of the proposed merge candidate list construction method

# Simulation Results

The proposed method is implemented in BMS-1.0 software [1]. And simulations were conducted on VTM configuration, using common test conditions in [2].

In the following tables (table 1, 2, and 3), the BD-rate, encoding time and decoding time changes for the proposed method are reported against the BMS-1.0 software with VTM configuration.

Table 1-Simulation result with VTM configuration, Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.23% | -0.26% | 103% | 105% |
| Class A2 | -0.10% | -0.10% | -0.10% | 104% | 111% |
| Class B | -0.24% | -0.23% | -0.16% | 105% | 114% |
| Class C | -0.21% | -0.25% | -0.12% | 103% | 114% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.21% | -0.16% | 104% | 112% |
| Class D | -0.05% | -0.09% | -0.11% | 103% | 113% |
| Class F (optional) | -0.02% | -0.02% | -0.03% | 103% | 114% |

Table 2-Simulation result with VTM configuration, Low Delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -0.15% | 0.31% | 99% | 108% |
| Class C | -0.14% | -0.27% | 0.07% | 99% | 104% |
| Class E | -0.88% | -0.85% | -1.30% | 104% | 109% |
| Overall | -0.38% | -0.36% | -0.18% | 100% | 107% |
| Class D | -0.07% | -0.24% | -0.28% | 104% | 118% |
| Class F (optional) | -0.23% | 0.01% | 0.36% | 102% | 107% |

Table 3-Simulation result with VTM configuration, Low Delay P

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.30% | -0.07% | 102% | 108% |
| Class C | -0.35% | -0.36% | -0.37% | 101% | 111% |
| Class E | -0.99% | -0.41% | -0.57% | 98% | 111% |
| Overall | -0.53% | -0.35% | -0.30% | 100% | 110% |
| Class D | -0.25% | -0.29% | -0.53% | 104% | 112% |
| Class F (optional) | -0.08% | -0.25% | 0.48% | 103% | 115% |

It was observed that with the proposed method, in VTM configuration, there are 0.20%, 0.38%, and 0.53% BD-rate improvement for Random Access, Low Delay B, and Low Delay P, respectively.

# Conclusion

It was observed that the proposed method has moderate BD-rate performance improvement. It is proposed to include it into a core experiment for further evaluation.

# References

[1] BMS-1.0 software: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).