| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0305-v1 |
| --- | --- |

| Title: | CE6 - Set of Transforms (Tests 3.2.1 and 3.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mischa Siekmann, Christian Bartnik, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-440 mischa.siekmann@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract:

A unified signaling and selection scheme for primary and secondary transforms [2] is tested together with two different sets of defined secondary transforms. Instead of signaling the secondary transform separately from the primary transform and allowing all combinations, a list of 5 combinations is provided for each TU as transform candidates and an index into this list is signaled. Test 3.2.1 achieves a luma BD bitrate saving of -4.65% at an encoder runtime of 239% over BMS1.1 with VTM-AI configuration and a loss of 0.45% at an encoder runtime of 39% with BMS-AI configuration. Test 3.2.2 achieves a luma BD bitrate saving of -5.81% at an encoder runtime of 255% over BMS1.1 with VTM-AI configuration and a luma BD bitrate saving of -0.44% at an encoder runtime of 55% with BMS-AI configuration. The performance gain of Test 3.2.2 over Test 3.2.1 comes with the cost of a significantly increased memory demand for storing the additional transform cores.

# Test description:

The transform selection and signaling scheme is the same in both Tests 3.2.1 and 3.2.2 and can be summarized as following:

The transform is selected among a set of 5 predefined transform candidates for each TU. Each of the transform candidates specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform (which may also be equal to the identity transform). The ordered list of transform candidates depends on the block size and the prediction mode. The transform chosen is signaled as follows: When cbf is equal to 1, a flag is transmitted which specifies whether the first transform in the candidate list is used. When this flag is equal to 1, the following applies: If the number of non-zero transform coefficient levels is greater than a threshold, another syntax element is transmitted which indicates the transform candidate used; otherwise, the second transform candidate of the list is used.

In Test 3.2.1 the vertical and horizontal primary transforms can be either of DCT-2, DCT-5, DCT-8, DST-1, DST-7 and can be combined with the 4x4 NSSTs as defined in BMS-1.1.

In addition to the transforms in Test 3.2.1, the Test 3.2.2 also provides a set of matrix-based secondary transforms with input shapes in {4x8, 8x4, 8x8}. Candidate lists of transform blocks with non-quadratic shapes use these transforms as their secondary transforms.

# Test results

According to the CE6 description in JVET-J1026 [1] and the common test conditions in JVET-J1010 [3], simulation were performed for intra-only (AI), random-access (RA) and low-delay B (LB) configuration with BMS-1.1 software in VTM and BMS configuration. The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | Off | AVX2 | GCC 7.2.1 | Linux |

## Test 6.3.2.1 - Set of transforms – comprising AMT and NSST transforms as defined in BMS.

Table 2 Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -4.25% | -3.70% | -3.99% | 225% | 129% |
| Class A2 | -3.86% | -3.06% | -3.00% | 240% | 123% |
| Class B | -4.36% | -3.36% | -3.00% | 245% | 123% |
| Class C | -4.85% | -2.83% | -3.10% | 251% | 114% |
| Class E | -6.05% | -4.39% | -4.29% | 227% | 117% |
| Overall | -4.65% | -3.42% | -3.40% | 239% | 121% |
| Class D | -4.80% | -2.31% | -2.20% | 261% | 114% |
| Class F (optional) |  |  |  |  |  |

Table 3 Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.93% | -2.35% | -2.91% | 132% | 109% |
| Class A2 | -1.46% | -1.19% | -1.03% | 122% | 109% |
| Class B | -2.26% | -2.36% | -2.27% | 125% | 105% |
| Class C | -2.44% | -1.60% | -1.79% | 124% | 99% |
| Class E |  |  |  |  |  |
| Overall | -2.08% | -1.92% | -2.02% | 125% | 105% |
| Class D | -2.05% | -1.19% | -1.13% | 124% | 97% |
| Class F (optional) |  |  |  |  |  |

Table 4 Result for VTM LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.73% | -1.01% | -0.92% | 116% | 102% |
| Class C | -0.70% | -0.59% | -0.55% | 113% | 95% |
| Class E | -1.13% | -1.59% | -0.86% | 103% | 98% |
| Overall | -0.82% | -1.01% | -0.78% | 111% | 99% |
| Class D | -0.49% | -0.41% | -0.61% | 110% | 102% |
| Class F (optional) |  |  |  |  |  |

Table 5 Result for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.59% | 1.69% | 2.02% | 40% | 102% |
| Class A2 | 0.65% | 1.58% | 1.68% | 39% | 100% |
| Class B | 0.46% | 1.06% | 1.48% | 39% | 101% |
| Class C | 0.36% | 0.69% | 1.02% | 37% | 102% |
| Class E | 0.22% | 0.43% | 0.69% | 43% | 101% |
| Overall | 0.45% | 1.06% | 1.37% | 39% | 101% |
| Class D | 0.45% | 0.57% | 0.70% | 35% | 101% |
| Class F (optional) |  |  |  |  |  |

Table 6 Result for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.31% | 1.76% | 1.58% | 64% | 99% |
| Class A2 | 0.17% | 1.41% | 1.29% | 71% | 102% |
| Class B | 0.14% | 1.48% | 1.75% | 69% | 99% |
| Class C | 0.14% | 1.24% | 0.96% | 68% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | 1.46% | 1.41% | 68% | 100% |
| Class D | 0.24% | 0.64% | 0.97% | 68% | 102% |
| Class F (optional) |  |  |  |  |  |

Table 7 Result for BMS LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.58% | 0.80% | 80% | 95% |
| Class C | -0.05% | 0.56% | 0.44% | 81% | 99% |
| Class E | 0.14% | 0.72% | 0.79% | 95% | 101% |
| Overall | 0.01% | 0.61% | 0.68% | 84% | 97% |
| Class D | 0.00% | 0.51% | 0.63% | 86% | 102% |
| Class F (optional) |  |  |  |  |  |

## Test 6.3.2.2 - Set of transforms – comprising AMT and NSST transforms as defined in BMS plus additionally proposed transforms.

Table 7 Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -5.60% | -4.39% | -4.49% | 243% | 130% |
| Class A2 | -4.90% | -3.82% | -3.68% | 256% | 125% |
| Class B | -5.43% | -4.46% | -4.30% | 263% | 124% |
| Class C | -5.85% | -3.65% | -4.00% | 267% | 115% |
| Class E | -7.51% | -5.73% | -5.53% | 241% | 119% |
| Overall | -5.81% | -4.37% | -4.37% | 255% | 122% |
| Class D | -5.20% | -2.84% | -2.81% | 276% | 114% |
| Class F (optional) |  |  |  |  |  |

Table 8 Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -2.75% | -2.57% | -2.87% | 134% | 110% |
| Class A2 | -2.12% | -1.52% | -1.30% | 123% | 106% |
| Class B | -3.03% | -3.14% | -3.17% | 128% | 104% |
| Class C | -3.06% | -2.03% | -2.23% | 127% | 102% |
| Class E |  |  |  |  |  |
| Overall | -2.80% | -2.41% | -2.48% | 128% | 105% |
| Class D | -2.32% | -1.30% | -1.38% | 127% | 98% |
| Class F (optional) |  |  |  |  |  |

Table 9 Result for VTM LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.06% | -1.23% | -0.99% | 116% | 102% |
| Class C | -0.95% | -0.70% | -0.72% | 114% | 103% |
| Class E | -1.61% | -1.97% | -1.40% | 104% | 98% |
| Overall | -1.16% | -1.24% | -1.00% | 112% | 101% |
| Class D | -0.66% | -0.53% | -1.00% | 110% | 101% |
| Class F (optional) |  |  |  |  |  |

Table 10 Result for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.60% | 0.97% | 1.38% | 43% | 101% |
| Class A2 | -0.16% | 0.86% | 1.08% | 41% | 101% |
| Class B | -0.33% | 0.12% | 0.39% | 41% | 101% |
| Class C | -0.44% | -0.17% | 0.16% | 38% | 102% |
| Class E | -0.75% | -0.35% | -0.02% | 45% | 99% |
| Overall | -0.44% | 0.24% | 0.55% | 41% | 101% |
| Class D | 0.14% | -0.08% | 0.11% | 36% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 11 Result for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.34% | 1.40% | 1.41% | 66% | 101% |
| Class A2 | -0.28% | 1.00% | 0.96% | 72% | 100% |
| Class B | -0.37% | 1.11% | 1.12% | 70% | 99% |
| Class C | -0.27% | 0.72% | 0.43% | 68% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | 1.04% | 0.96% | 69% | 100% |
| Class D | -0.02% | 0.49% | 0.49% | 69% | 98% |
| Class F (optional) |  |  |  |  |  |

Table 12 Result for BMS LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | 0.27% | 0.25% | 83% | 101% |
| Class C | -0.24% | 0.19% | 0.26% | 83% | 100% |
| Class E | -0.12% | 0.39% | 0.63% | 96% | 101% |
| Overall | -0.20% | 0.28% | 0.35% | 86% | 100% |
| Class D | -0.07% | 0.03% | 0.57% | 86% | 98% |
| Class F (optional) |  |  |  |  |  |

# Complexity metrics

In test 6.3.2.1 the transform candidate lists are populated with transforms defined in BMS-1.1. Hence, the complexity metrics in terms of memory requirements for storing and applying the transform cores and the number of needed arithmetic operations are the same as in BMS-1.1 and are summarized below. The table of candidate lists requires 1byte per candidate (i.e. [shape x intra mode x #candidates] 17x35x5 bytes in total).

## Primary transforms (AMT as in BMS-1.1)

Transform types: DCT-2, DCT-5, DCT-8, DST-1, DST-7

Transform block sizes: WxH with W,H in { 4, 8, 16, 32, 64 }

## HyGT-4x4 secondary transform as in BMS-1.1

| Type | HyGT – 4x4 |
| --- | --- |
| Number of passes: | 8 (2 rounds, 4 layers each) |
| Multiplications per pass: | 32 |
| Additions per pass: | 32 |
| Bit-shifts per pass: | 16 |
| Parameterized permutation layer: | 1 |
| Multiplications: | 256 |
| Additions: | 256 |
| Bit-shifts: | 128 |
| Parameter: | 64 x Givens-angles (8bit), 1 x permutation (16x4 bit) |
| Precision of arithmetic operations | +15 bits (rel. to precision of primary coefficients) |
| Precision intermediate data | +5 bits (rel. to precision of primary coefficients) |
| Storing single transform core: | 576 bits |
| Number of transforms: | 103 |
| Storing transform cores: | 59,328 bits |

## Additional Secondary transforms in 6.3.2.2

In addition to Test 3.2.1 the transform candidates in Test 3.2.2 non-quadratic block shapes have their own set of secondary transforms. These are implemented as matrix multiplication and their complexity metrics are summarized in the following table. The table of transform candidate lists for non-quadratic block shapes requires 14bit per candidate in Test 3.2.2.

| Type | Matrix multiplication (32) | Matrix multiplication (64) |
| --- | --- | --- |
| Multiplications: | 1024 | 4096 |
| Additions: | 1056 | 4160 |
| Bit-shifts: | 32 | 64 |
| Parameter: | 1024 coefficients (10bit) | 4096 coefficients (10bit) |
| Precision of arithmetic operations | +10bits (rel. to precision of primary coefficients) | +10 bits (rel. to precision of primary coefficients) |
| Precision intermediate data | +0 bits (rel. to precision of primary coefficients) | +0 bits (rel. to precision of primary coefficients) |
| Storing single transform core: | 10,240 bits | 40,960 bits |
| Number of transforms: | 142 | 174 |
| Storing transform cores: | 1,454,080 bits | 7,127,040 bits |

# References

- A. Said, X. Zhao “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, JVET-J1026, San Diego, USA, April 2018.

- M. Siekmann, B. Stallenberger, C. Bartnik, J. Pfaff, D. Marpe, H. Schwarz, T. Wiegand „Set of Transforms“, JVET-J0040, San Diego, USA, April 20018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).