| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0306-v1 |
| --- | --- |

| Title: | CE6 - related: “Set of Transforms” selection and signaling scheme tested with different types of secondary transforms sets. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mischa Siekmann, Christian Bartnik, Stefan Matlage, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-440 firstname.lastname @hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document provides an overview of the obtained performance, when the transform selection and signaling scheme (Set of Transforms [3]) is combined with different types of secondary transforms sets. In addition to the tests 3.2.1 and 3.2.2 in CE6 [1,2], SoT was also tested together with the secondary transforms of CE6-2.2 and CE6-2.3.

# Set of Transforms

The transform selection and signaling scheme is the same in all described tests and can be summarized as following:

The transform is selected among a set of 5 predefined transform candidates for each TU. Each of the transform candidates specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform (which may also be equal to the identity transform). The ordered list of transform candidates depends on the block size and the prediction mode. The transform chosen is signaled as follows: When cbf is equal to 1, a flag is transmitted which specifies whether the first transform in the candidate list is used. When this flag is equal to 1, the following applies: If the number of non-zero transform coefficient levels is greater than a threshold, another syntax element is transmitted which indicates the transform candidate used; otherwise, the second transform candidate of the list is used.

# Tests:

For all provided tests an individual mapping from block size and prediction mode to transform candidates is used. Each candidate has a vertical and horizontal primary transform and is either combined with one of the secondary transforms defined for the specific test or is a primary only transform. The list of defined primary transforms is the same for all tests, namely: DCT-2, DCT-5, DCT-8, DST-1 and DST-7.

## CE6-3.2.1: HyGT-4x4 as in BMS-1.1

This is the test 3.2.1 of CE6 [1,2]. The secondary transforms are chosen from the 35x3 HyGT-based 4x4 transforms as in BMS-1.1.

## CE6-3.2.2: HyGT-4x4 + matrix-based transforms for non-quadratic transform block shape

This is the test 3.2.1 of CE6 [1,2]. For quadratic block sizes the secondary transforms are chosen from the 35x3 HyGT-based 4x4 transforms as in BMS-1.1. The non-quadratic blocks have candidates with secondary transform chosen from a set of matrix-based transforms. This set includes 142 Mat-32 non-separable transforms for input shapes 8x4 and 4x8 and 74 Mat-64 transforms for 8x8 input shapes.

## SoT + CE6-2.2:  HyGT-4x4 + HSMT 8x8

In this test, the transform selection and signaling scheme of SoT is combined with the secondary transforms tested in CE6-2.2. As in CE6-2.2, transform blocks with a height or width equal to 4 can have the HyGT-4x4 transforms as secondary transforms, while transform candidates for larger block sizes use Hierarchically Structured Matrix-based Transforms (HSMT) [5] as their secondary transform. In total 35x3 HyGT-based 4x4 transforms (BMS-1.1) and, 13x3 HSMTs are defined.

## SoT + CE6-2.3: HyGT-4x4 + HyGT-8x8

In this test, the transform selection and signaling scheme of SoT is combined with the secondary transforms tested in CE6-2.3. This set of secondary transforms comprises 12x4 HyGT 4x4 transforms and 12x4 HyGT 8x8 transforms, which have been trained by the proponent [JVET-J0024, JVET-J0025]. In this combination test with SoT, the first transform candidate for each block is a primary only transform and obtained by a table lookup, while the remaining 4 candidates use DCT-II as vertical and horizontal primary transform and are combined with the same intra mode to NSST index mapping as in CE6-2.3.

# Test results

According to the CE6 description in JVET-J1026 [1] and the common test conditions in JVET-J1010 [3], simulation were performed for intra-only (AI), random-access (RA) and low-delay B (LB) configuration with BMS-1.1 software in VTM and BMS configuration. The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | Off | AVX2 | GCC 7.2.1 | Linux |

## Summary

The overall BD bitrate savings of the different tests are summarized for VTM and BMS configuration in this section. For the VTM configuration, also the results for VTM with AMT+NSST as defined in BMS-1.1 are included. (i.e. VTM-cfg + EMT=3; EMTFast=3 and NSST=1)

Table 2 Result for VTM – AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| AMT+NSST-4x4 | -5.28% | -4.89% | -5.03% | 594% | 119% |
| CE6 - 3.2.1 | -4.65% | -3.42% | -3.40% | 239% | 121% |
| CE6 - 3.2.2 | -5.81% | -4.37% | -4.37% | 255% | 122% |
| SOT w. CE6-2.2 | -6.32% | -4.78% | -4.94% | 246% | 122% |
| SOT w. CE6-2.3 | -5.82% | -4.23% | -4.47% | 252% | 116% |

Table 3 Result for VTM – RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| AMT+NSST-4x4 | -3.01% | -3.30% | -3.31% | 252% | 109% |
| CE6 - 3.2.1 | -2.08% | -1.92% | -2.02% | 125% | 105% |
| CE6 - 3.2.2 | -2.80% | -2.41% | -2.48% | 128% | 105% |
| SOT w. CE6-2.2 | -3.06% | -2.97% | -3.17% | 127% | 105% |
| SOT w. CE6-2.3 | -2.86% | -2.71% | -3.09% | 126% | 103% |

Table 4 Result for VTM – LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| AMT+NSST-4x4 | -1.87% | -0.26% | -0.26% | 202% | 109% |
| CE6 - 3.2.1 | -0.82% | -1.01% | -0.78% | 111% | 99% |
| CE6 - 3.2.2 | -1.16% | -1.24% | -1.00% | 112% | 101% |
| SOT w. CE6-2.2 | -1.25% | -1.14% | -1.30% | 113% | 102% |
| SOT w. CE6-2.3 | -1.14% | -1.15% | -1.09% | 112% | 102% |

Table 5 Result for BMS – AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| CE6 - 3.2.1 | 0.45% | 1.06% | 1.37% | 39% | 101% |
| CE6 - 3.2.2 | -0.44% | 0.24% | 0.55% | 41% | 101% |
| SOT w. CE6-2.2 | -0.79% | 0.18% | 0.47% | 40% | 100% |
| SOT w. CE6-2.3 | -0.09% | 0.86% | 1.32% | 41% | 98% |

Table 6 Result for BMS – RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| CE6 - 3.2.1 | 0.18% | 1.46% | 1.41% | 68% | 100% |
| CE6 - 3.2.2 | -0.32% | 1.04% | 0.96% | 69% | 100% |
| SOT w. CE6-2.2 | -0.49% | 0.83% | 0.75% | 69% | 99% |
| SOT w. CE6-2.3 | -0.15% | 1.17% | 1.19% | 69% | 101% |

Table 7 Result for BMS – LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| CE6 - 3.2.1 | 0.01% | 0.61% | 0.68% | 84% | 97% |
| CE6 - 3.2.2 | -0.20% | 0.28% | 0.35% | 86% | 100% |
| SOT w. CE6-2.2 | -0.27% | 0.13% | 0.43% | 85% | 98% |
| SOT w. CE6-2.3 |  |  |  |  |  |

## SoT + CE2.2

Table 8 Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -6.24% | -5.97% | -6.77% | 228% | 130% |
| Class A2 | -5.28% | -3.95% | -3.93% | 245% | 123% |
| Class B | -5.77% | -4.65% | -4.58% | 252% | 124% |
| Class C | -6.69% | -3.98% | -4.34% | 262% | 117% |
| Class E | -7.87% | -5.70% | -5.53% | 236% | 120% |
| Overall | -6.32% | -4.78% | -4.94% | 246% | 122% |
| Class D | -6.08% | -3.39% | -3.41% | 275% | 114% |
| Class F (optional) |  |  |  |  |  |

Table 9 Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.08% | -4.21% | -4.99% | 135% | 108% |
| Class A2 | -2.17% | -1.54% | -1.43% | 122% | 105% |
| Class B | -3.22% | -3.56% | -3.44% | 126% | 104% |
| Class C | -3.51% | -2.36% | -2.77% | 127% | 104% |
| Class E |  |  |  |  |  |
| Overall | -3.06% | -2.97% | -3.17% | 127% | 105% |
| Class D | -2.68% | -1.66% | -2.30% | 127% | 99% |
| Class F (optional) |  |  |  |  |  |

Table 10 Result for VTM LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.11% | -1.24% | -1.33% | 115% | 100% |
| Class C | -1.08% | -1.02% | -1.10% | 116% | 107% |
| Class E | -1.70% | -1.13% | -1.52% | 104% | 100% |
| Overall | -1.25% | -1.14% | -1.30% | 113% | 102% |
| Class D | -0.73% | -0.30% | -1.17% | 110% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 11 Result for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.24% | 0.15% | 0.53% | 40% | 101% |
| Class A2 | -0.46% | 0.96% | 1.09% | 40% | 100% |
| Class B | -0.53% | 0.20% | 0.51% | 40% | 99% |
| Class C | -0.93% | -0.23% | 0.10% | 38% | 100% |
| Class E | -0.91% | -0.07% | 0.24% | 44% | 98% |
| Overall | -0.79% | 0.18% | 0.47% | 40% | 100% |
| Class D | -0.44% | -0.44% | -0.30% | 36% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 12 Result for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.59% | 0.55% | 0.48% | 65% | 100% |
| Class A2 | -0.29% | 1.35% | 1.13% | 72% | 99% |
| Class B | -0.46% | 0.96% | 0.96% | 69% | 98% |
| Class C | -0.58% | 0.49% | 0.42% | 68% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.49% | 0.83% | 0.75% | 69% | 99% |
| Class D | -0.22% | -0.02% | 0.30% | 69% | 99% |
| Class F (optional) |  |  |  |  |  |

Table 13 Result for BMS LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.25% | 0.06% | 0.10% | 82% | 98% |
| Class C | -0.35% | 0.02% | -0.01% | 82% | 99% |
| Class E | -0.19% | 0.38% | 1.56% | 94% | 98% |
| Overall | -0.27% | 0.13% | 0.43% | 85% | 98% |
| Class D | -0.10% | -0.24% | 0.64% | 86% | 100% |
| Class F (optional) |  |  |  |  |  |

## SoT+CE2.3

Table 7 Result for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -6.21% | -5.18% | -6.26% | 233% | 115% |
| Class A2 | -4.80% | -3.62% | -3.79% | 251% | 119% |
| Class B | -5.22% | -4.20% | -3.79% | 260% | 118% |
| Class C | -6.17% | -3.55% | -4.18% | 267% | 114% |
| Class E | -6.98% | -4.81% | -4.84% | 240% | 113% |
| Overall | -5.82% | -4.23% | -4.47% | 252% | 116% |
| Class D | -5.47% | -3.17% | -3.08% | 281% | 114% |
| Class F (optional) |  |  |  |  |  |

Table 8 Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -3.08% | -3.69% | -5.22% | 131% | 102% |
| Class A2 | -1.92% | -1.49% | -1.51% | 121% | 105% |
| Class B | -2.98% | -3.29% | -3.06% | 126% | 103% |
| Class C | -3.23% | -2.18% | -2.72% | 126% | 102% |
| Class E |  |  |  |  |  |
| Overall | -2.86% | -2.71% | -3.09% | 126% | 103% |
| Class D | -2.44% | -1.64% | -2.04% | 127% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 9 Result for VTM LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.00% | -1.18% | -0.99% | 115% | 100% |
| Class C | -0.99% | -1.07% | -0.97% | 115% | 107% |
| Class E | -1.57% | -1.21% | -1.42% | 104% | 100% |
| Overall | -1.14% | -1.15% | -1.09% | 112% | 102% |
| Class D | -0.68% | -0.95% | -1.38% | 112% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 10 Result for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.83% | 1.43% | 2.05% | 41% | 94% |
| Class A2 | 0.21% | 1.58% | 1.59% | 41% | 98% |
| Class B | 0.25% | 0.76% | 1.42% | 41% | 98% |
| Class C | -0.15% | 0.30% | 0.79% | 39% | 99% |
| Class E | -0.12% | 0.45% | 0.87% | 45% | 97% |
| Overall | -0.09% | 0.86% | 1.32% | 41% | 98% |
| Class D | 0.37% | 0.11% | 0.48% | 36% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 11 Result for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.31% | 1.46% | 1.25% | 64% | 100% |
| Class A2 | 0.03% | 1.61% | 1.25% | 72% | 103% |
| Class B | -0.14% | 1.14% | 1.47% | 69% | 101% |
| Class C | -0.18% | 0.67% | 0.77% | 68% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | 1.17% | 1.19% | 69% | 101% |
| Class D | 0.10% | 0.34% | 0.41% | 69% | 100% |
| Class F (optional) |  |  |  |  |  |

Table 12 Result for BMS LB configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.29% | 0.38% | 0.04% | 83% | 101% |
| Class E | 0.02% | 0.47% | 1.00% | 94% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.25% | -0.40% | 86% | 100% |
| Class F (optional) |  |  |  |  |  |

# References

- A. Said, X. Zhao “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, JVET-J1026, San Diego, USA, April 2018.

- M. Siekmann, C. Bartnik, H. Schwarz, D. Marpe, T. Wiegand “CE6 - Set of transforms (Tests 3.2.1 and 3.2.2) “, JVET-K0305, Ljubljana, Slovenia, July 2018.

- M. Siekmann, B. Stallenberger, C. Bartnik, J. Pfaff, D. Marpe, H. Schwarz, T. Wiegand “Set of Transforms“, JVET-J0040, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

- A. Said, H. Egilmez, V. Seregin, M. Karczewicz, “Non-Separable Secondary Transform Implementations with Reduced Memory via Hierarchically Structured Matrix-based Transforms“, JVET-J0062, San Diego, USA, April 2018.

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).