| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0307-v1 |
| --- | --- |

| Title: | CE2-2.1.1: Long deblocking filters and fixes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, Z. Zhang, R. Sjöberg | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes long filters and decisions for luma. It also includes fixes to deblock implicitely split blocks for luma and enables parallel friendly deblocking for CU based deblocking for luma. When BMS is used it also applies deblocking of sub-blocks from motion vector prediction.

The proposed solution is claimed to improve subjective quality at low bitrates. It also provides a luma BD rate impact of -0.2/-0.2%/-0.1% for AI/RA/LD compared to VTM-1.0 and -0.1%/-0.1%/-0.1% for AI/RA/LD compared to BMS-1.0.  Decoding time 103%/103%/102% for AI/RA/LD compared to VTM-1.0 and 100%/102%/101% for AI/RA/LD compared to BMS-1.0.

The proposed solution without the long filters and decisions for luma cannot deal with block artifacts for large blocks but is also claimed to improve subjective quality over the anchors at low bitrates. It provides a luma BD rate impact of -0.2/-0.1%/-0.1% for AI/RA/LD compared to VTM-1.0 and -0.1%/-0.1%/-0.2% for AI/RA/LD compared to BMS-1.0. Decoding time 101%/101%/100% for AI/RA/LD compared to VTM-1.0 and 100%/101%/101% for AI/RA/LD compared to BMS-1.0.

# Introduction

The deblocking in the current test model is based on HEVC deblocking. The deblocking in this contribution is based on the deblocking in CfP response JVET-J0012 [1]. The contribution adds long filters and long filter decisions on top of the HEVC filters and decisions in section 2. The contribution also includes several fixes to the deblocking in the test model. Section 3 describes a fix to make the deblocking parallel friendly by not modifying samples that can be read when filtering neighbouring CU boundaries. In section 4 a fix to also deblock implicitely split TU boundaries is described. Section 5 describes a fix to also deblock prediction sub-block boundaries which only appears in BMS. In section 6 objective results in comparison to VTM-1.0 and BMS-1.0 are shown. Finally some conclusions in section 7.

# Long filters and decisions

The approach is to apply a long filter from a set of long filters to symmetrically filter 3, 5 or 7 samples on respective side of a block boundary for larger blocks and smooth regions. The filters are designed to not modify a plane/ramp and to be computationally efficient to implement.

The filtering of N samples on each side of a line of samples across the block boundary is performed as follows. First pre-filter to obtain three virtual samples, one centered at a midpoint refMiddle between current block Q and neighbouring block P, one refP centered between two samples in block P and one refQ centered between two samples in block Q. Then two tap linear interpolation between refMiddle and refP to determined filtered samples in block P, and between refMiddle and refQ to determine filtered samples in block Q. This procedure facilitates that a plan/ramp not is modified but that discontinuities between block P and Q can be reduced.

One of the longer filters is used when the strong weak filter decision (identical with HEVC/VTM/BMS) indicates strong filter for 8 lines of a 16 sample segment of a boundary to be filtered and that samples near the boundary are smooth. This gives less frequent use of long filters and also less frequent switching between long filters and weaker filters which are determined for segments of four samples. Filter decisions are shown in the Table 1. The 7 sample filter is used if the current and the neighbouring CUs have a width and height equal or greater than 32 and when 8 samples in current and neighbouring block are smooth. If that is not the case the 5 sample filter is used if the current and neighbouring CUs have a width and height equal or greater than 16 and when 6 samples in current and neighbouring block are smooth. If that is not the case the 3 sample filter is used if the current and neighbouring CUs have a width and height equal or greater than 8 and when 4 samples in current and neighbouring block are smooth. If that is not the case filters in VTM/BMS are used.

Table 1: Filter decisions

| Size | Conditions for each of the 8 lines of a 16 sample segment (line 0, 3, 4, 7, 8, 11, 12, 15) |
| --- | --- |
| 7 | HEVCstrong_weak&& p0-2*p2+p4+q0-2*q2+q4<3*β32 && p0-p2-p3+p5+q0-q2-q3+q5<3*β32&& p0-2*p3+p6+q0-2*q3+q6<3*β32 && p0-p3-p4+p7+q0-q3-q4+q7<3*β32 |
| 5 | HEVCstrong_weak&& p0-p4+q0-q4<β8 && p0-p5+q0-q5<β8 |
| 3 | HEVCstrong_weak&& p0-p1-p2+p3+q0-q1-q2+q3<3*β32 |
| 0-3 | HEVC filter and decisions apply but reducing maximum filter length to half the minimum distance between current and neighboring CU boundaries |

Block boundary samples pi and qi for i=0 to S-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles+64-fi*Ps)+32)≫6, clipped to pi±tc

- qi´=(fi*Middles+64-fi*Qs+32)≫6), clipped to qi±tc

where f, Middle, P and Q are given below:

Table 2: Filter kernels

| S | Filter kernels |
| --- | --- |
| 7 | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 Middle7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| 5 | fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+8)≫4 P5=(p4+p5+1)≫1, Q5=(q4+q5+1)≫1 |
| 3 | fi=53-i*21, can also be described as f=53,32,11 Middle3=(3*po+qo+p1+q1+2*(p2+q2)+8)≫4 P3=(p2+p3+1)≫1, Q3=(q2+q3+1)≫1 |

## Complexity

The filter complexity is illustrated in Table 3. It can be noted that the filtering of a line of pixel can to a large extent be done by 2-tap filtering. Same length of long filter is used for 16 line segments. Filter decisions can be made with relatively few checks considering the additional number of samples to filter due to the re-use of the strong/weak filter decisions for the first 4 samples on respective side of the boundary.

Table 3: Complexity numbers for filtering one line of samples

| Number of samples to filter | Switching frequency | Line buffer | Number of taps for filtering one line | Filter coefficient accuracy | Number of taps for decision |
| --- | --- | --- | --- | --- | --- |
| 3+3 | 16 lines | 4 | 1 6-tap + 8 2-tap | 2-bit to 6-bit | 2 2-tap |
| 5+5 | 16 lines | 6 | 1 10-tap + 12 2-tap | 2-bit to 6-bit | 4 2-tap |
| 7+7 | 16 lines | 8 | 1 14-tap + 16 2-tap | 2-bit to 6-bit | 4 3-tap + 4 4-tap |

# Non-recursive filtering

The deblocking in VTM/BMS is CU based and the smallest CU size is 4x4. If more than one sample on respective side of the current boundary of the smallest CU or a CU that has a width of 4 when filtering vertical boundaries or a CU that has a height of 4 when filtering horisontal boundaries will cause that samples used for filtering a neighbouring boundary 4 samples away will read modified samples. This means that the filtering of CU boundaries not can be performed in parallel.

The fix is to determine number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height >> 1) – 1 : (CUP_width>>1) - 1

lengthQ = edge==EDGE_HOR ? (CUQ_height >> 1) – 1 : (CUQ_width>>1) - 1

This means that only one sample will be filtered when the distance between boundaries are 4.

Then the weak filter can be used with two sample filtering on at least one side if lengthP or lengthQ is larger than 1. The strong filter can be used if both lengthP and lengthQ are larger than 2.

# Filtering of implicit split TU boundaries

Since the largest transform size is smaller than the largest CU size it can appear TU boundaries inside the CU. However VTM/BMS only filter CU boundaries which cause large blocks that not are deblocked at all.

The fix is to filter implicit split TU boundaries.

# Filtering of prediction sub-block boundaries

For BMS there is one tool (STMVP/ATMVP) that can create prediction sub-blocks inside CU. However BMS only filters CU boundaries which can cause significant boundary artifacts inside CU when sub-block prediction is used.

The fix is to first determine the max number of samples that can be modified to enable parallel deblocking of internal boundaries of the CU. No filtering outside the CU. Then filter the internal boundaries before filtering the CU boundary to still enable long filters on the CU boundary when there is a prediction sub-block boundary close to the CU boundary.

# Results

We provide results for CE2-2.1.1 which includes the fixes and also results without long filters for common test conditions [2]. Decoding performed on same machine for anchor and test but encoding on different machines. The software was released June 1 and can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE2/CE2-2.1.1

## With long filters

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.00% | 0.01% | 102%* | 102% | -0.07% | 0.00% | 0.00% | 106%* | 100% |
| Class A2 | -0.21% | 0.01% | 0.01% | 103%* | 103% | -0.12% | 0.01% | 0.00% | 110%* | 100% |
| Class B | -0.18% | 0.00% | 0.00% | 107%* | 104% | -0.12% | 0.00% | 0.00% | 104%* | 100% |
| Class C | -0.19% | 0.00% | 0.00% | 102%* | 103% | -0.13% | -0.01% | 0.00% | 107%* | 101% |
| Class E | -0.23% | -0.01% | -0.01% | 107%* | 103% | -0.20% | 0.00% | -0.01% | 102%* | 100% |
| Overall | -0.20% | 0.00% | 0.00% | 104%* | 103% | -0.13% | 0.00% | 0.00% | 106%* | 100% |
| Class D | -0.16% | 0.00% | 0.01% | 106%* | 102% | -0.12% | 0.00% | 0.01% | 97%* | 102% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.09% | -0.01% | 100%* | 104% | -0.12% | -0.03% | -0.01% | 107%* | 100% |
| Class A2 | -0.17% | -0.09% | -0.01% | 103%* | 101% | -0.13% | -0.06% | -0.06% | 104%* | 102% |
| Class B | -0.09% | -0.04% | 0.06% | 104%* | 103% | -0.07% | 0.03% | -0.01% | 101%* | 101% |
| Class C | -0.10% | -0.01% | -0.09% | 103%* | 103% | -0.09% | -0.03% | -0.13% | 100%* | 104% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.17% | -0.05% | -0.01% | 103%* | 103% | -0.10% | -0.02% | -0.05% | 103%* | 102% |
| Class D | -0.06% | 0.07% | 0.09% | 107%* | 101% | 0.02% | 0.08% | -0.08% | 103%* | 102% |

*

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.19% | 0.02% | 98%* | 101% | -0.04% | -0.23% | 0.17% | 103%* | 101% |
| Class C | -0.04% | -0.19% | 0.04% | 100%* | 102% | -0.14% | 0.17% | -0.13% | 105%* | 100% |
| Class E | -0.17% | 0.11% | -0.66% | 99%* | 102% | -0.20% | -0.31% | -0.16% | 102%* | 102% |
| Overall | -0.05% | 0.04% | -0.15% | 99%* | 102% | -0.11% | -0.12% | -0.01% | 104%* | 101% |
| Class D | -0.11% | -0.27% | 0.07% | 103%* | 101% | -0.11% | 0.00% | -0.03% | 97%* | 100% |

*Encoding for anchor and test performed on different machines.

## Without long filters

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.01% | -0.01% | 103%* | 101% | -0.07% | 0.00% | 0.00% | 108%* | 100% |
| Class A2 | -0.22% | 0.01% | 0.00% | 98%* | 102% | -0.12% | 0.00% | 0.00% | 111%* | 98% |
| Class B | -0.19% | 0.00% | 0.00% | 105%* | 102% | -0.12% | 0.01% | 0.01% | 103%* | 101% |
| Class C | -0.19% | 0.00% | 0.00% | 104%* | 100% | -0.13% | -0.01% | 0.00% | 108%* | 101% |
| Class E | -0.19% | -0.01% | -0.01% | 104%* | 99% | -0.15% | 0.00% | -0.01% | 104%* | 101% |
| Overall | -0.19% | 0.00% | 0.00% | 103%* | 101% | -0.12% | 0.00% | 0.00% | 106%* | 100% |
| Class D | -0.16% | 0.00% | 0.01% | 100%* | 101% | -0.12% | 0.00% | 0.01% | 103%* | 102% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.04% | 0.02% | 102%* | 100% | -0.17% | -0.19% | 0.01% | 103%* | 101% |
| Class A2 | -0.17% | -0.07% | 0.02% | 107%* | 103% | -0.19% | 0.07% | 0.06% | 103%* | 101% |
| Class B | -0.09% | 0.03% | 0.11% | 102%* | 101% | -0.14% | 0.04% | 0.02% | 100%* | 101% |
| Class C | -0.09% | 0.01% | 0.05% | 104%* | 101% | -0.09% | 0.01% | -0.04% | 97%* | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.12% | -0.01% | 0.06% | 104%* | 101% | -0.14% | -0.01% | 0.01% | 101%* | 101% |
| Class D | -0.07% | 0.09% | 0.01% | 103%* | 101% | -0.04% | 0.03% | -0.06% | 88%* | 101% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.09% | 0.21% | 0.15% | 99%* | 100% | -0.13% | -0.36% | 0.00% | 101%* | 100% |
| Class C | -0.06% | -0.07% | 0.07% | 100%* | 99% | -0.14% | 0.11% | -0.05% | 101%* | 101% |
| Class E | -0.21% | 0.23% | -0.25% | 108%* | 101% | -0.37% | 0.00% | -0.86% | 101%* | 102% |
| Overall | -0.11% | 0.12% | 0.02% | 102%* | 100% | -0.19% | -0.11% | -0.23% | 101%* | 101% |
| Class D | -0.10% | -0.06% | -0.28% | 100%* | 101% | -0.03% | 0.12% | 0.12% | 93%* | 100% |

*Encoding for anchor and test performed on different machines.

# Conclusions

We propose to include the fixes of the problems we found with the test model into the next version of the test model:

- Deblocking of implicitely split TU.

- Non-recursive deblocking.

- Deblocking of prediction sub-block boundaries (applies for BMS).

We also propose to include the long filters and decisions since they can help to reduce blocking artifacts for large blocks.

# References

[1] R. Sjöberg, K. Andersson, R. Yu, Z. Zhang, P. Wennersten, J. Lainema, A. Hallapuro, A. Aminlou, M. Hannuksela, R. Ghaznavi-Youvalari, J. Ridge, “Description of SDR and HDR video coding technology proposal by Ericsson and Nokia”, JVET-J0012, San Diego, April 2018

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego,  April 2018

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).