| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0308-v1 |
| --- | --- |

| Title: | CE12: HDR In-loop Reshaping (CE12-5, 12-6, 12-7 and 12-8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Walt Husak, Sean McCarthy, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, tlu}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This proposal reports HDR in-loop reshaping CE12 subtests based on JVET-J0015: CE12-5, in-loop luma pixel-level reshaping; and the additional coding features CE12-6, luma-based Chroma QP Offset; CE12-7, loop-filter application; and CE12-8, Region-of-Interest. For all tests, parameters of the reshaping are derived to be aligned to bitrate redistribution function (deltaQP settings) employed by the HDR anchor.

Summary performance gains for in-loop reshaping of HDR PQ Content under HDR fixed QP CTC with suggested setting compared to VTM HDR Anchor are:

- AI: DE100:-1.5%; PSNRL100:-2.8%;  wPsnrY: -2.0%, wPsnrU/V{-1.9%, -4.8%}

Enc time: 105%, Dec time: 105%

- RA: DE100:-1.3%; PSNRL100:-2.6%;  wPsnrY: -2.0%, wPsnrU/V{0.0%, -0.1%}

Enc time: 104%, Dec time: 108%

# Introduction

In-loop reshaping is proposed in JVET-J0015 [2]. In CE12 [1], 4 subtests are designed to evaluate all the coding features in in-loop reshaping architecture: CE12-5, in-loop luma pixel-level reshaping; CE12-6, luma-based Chroma QP Offset; CE12-7, loop-filter application; CE12-8, Region-of-Interest.

JVET-J0015 proposes in-loop reshaping. It does not require any pre-/post processing. In addition, it has more flexibility and allows adaptive slice/region/CTU reshaping easily. CE12-5 is designed to test in-loop reshaping architecture. In JVET-J0015, two modes are supported: default mode and adaptive mode. The default reshaper uses a static Luma mapping curve derived from the weights in the wPSNR metric which is aligned with luma-based adaptive QP formulae used in HDR Anchor. The adaptive reshaper allows adaptive adjustment to the Luma mapping curve for better HDR subjective quality. The default mode can be used to evaluate the in-loop reshaping architecture using current wPSNR metrics, while the adaptive mode enables future commercial optimization using better HVS models.

The sub-CEs 6 through 8 are designed to evaluate additional coding features step by step. To evaluate the in-loop reshaping architecture using current wPSNR metrics and for fair comparison with Anchor, the default reshaping curve is used in this CE. For chroma components, chroma QP adaptation used in Anchor is applied. Because luma reshaping curve modifies the relationship between luma and chroma, the coding tool evaluated in CE12-6 luma-based chroma QP Offset is designed to compensate the interaction. The in-loop reshaping architecture allows the in-loop filter to be applied in either the original domain or the  reshaped domain. CE12-7 is designed to assess the performance of in-loop filter applied in different domains. The in-loop reshaping architecture also enables adaptive slice/region/CTU reshaping easily. CE12-8 provides a setting to show how CTU adaptation can work with the in-loop reshaping architecture.

Considering the trade-offs between performance and complexity, the recommended settings based on all CE12-5 to CE12-7 tests are as follows: enable luma-level pixel mapping; enable luma-based chroma QP Offset; and apply loop-filter in reshaped domain for both intra and inter slices. The corresponding BDRate of HDR metrics for HDR PQ Content under HDR fixed QP CTC [3] compared to VTM HDR Anchor are:

- AI: DE-100:-1.5%; PSNRL-100:-2.8%;  wPsnrY: -2.0%, wPsnrU/V{-1.9%, -4.8%}

Enc time: 105%, Dec time: 105%

- RA: DE-100:-1.3%; PSNRL-100:-2.6%;  wPsnrY: -2.0%, wPsnrU/V{0.0%, -0.1%}

Enc time: 104%, Dec time: 108%

It is recommended that in-loop reshaping is a good candidate for VVC coding tool.

# CE subtests algorithm description

## CE12-5: in-loop luma reshaping

### in-loop reshaping architecture

The in-loop Luma reshaper is implemented and configured as a pair of one dimensional, 10-bit, 1024-entry mapping tables (1D-LUT). The first LUT is referred to as FwdLUT, which can be used to map an input Luma code value Yi to an altered value Yr: Yr=FwdLUT[Yi]. The second LUT is referred to as InvLUT, which can be used to map altered code value Yr for “the reconstruction of Yi” as Yi: Yi=InvLUT[Yr]. Only one of the two LUTs need to be signaled as the other one can be computed from the signaled LUT.

The in-loop reshaper maps the Luma code values based on the slice type received by the decoder.

For an intra slice, reshaping is performed in an explicit way. The HDR intra reconstruction process is performed in the same manner as in SDR decoding until the reconstructed samples are about to be stored into the DPB buffer. The reconstructed Yr samples are modified by the inverse reshaping process before the final intra reconstructed samples Yi are stored into the DPB buffer by Yi=InvLUT[Yr]. The simplified HDR intra slice decoding workflow is shown in Figure 1.

Figure 1. HDR Intra slice reconstruction with in-loop Luma Reshaper

For an inter slice, reshaping is implicitly performed. The decoding reconstruction process (e.g., Yr= Yres+Ypred) is modified so that reconstructed sample values are produced with the influence of the reshaper. The reconstructed samples are generated by Yr=InvLUT[Yres+FwdLUT[Ypred]]. All the other inter decoding tools are used in the same way as the SDR decoding process, such as entropy decoding, motion compensation, reference picture handling, etc. The simplified HDR inter slice decoding workflow is shown in Figure 2.

Figure 2. HDR Inter slice reconstruction with in-loop Luma Reshaper

### Derivation of luma reshaping curve

In HDR Anchor, the luma-based QP adaptation derives a local dQP value per CTU based on the average of luma sample values (-dQP is used in encoding).

dQP is defined using the formula:

dQP(Y) = max(-3, min(6, 0.015*Y - 1.5 – 6 ) ), where Y is luma value from [0 maxY], maxY=1023 for 10 bit.

For wPSNR calculation, the luma based weight for squared distortion is calculated as

W_SSE(Y) = pow(2.0, dQP(Y)/3.0);

The default luma reshaping curve is derived as:

- compute the slope of the reshaping curve, which is also the luma sample importance (weight) slope[Y] = sqrt(W_SSE(Y)) =  = 2^(dQP(Y)/6);

If signal is in standard range, slope(Y) = 0 for [0 64) and (940 1023];

- integral reshaping curve F(Y)

F(0) = 0;

for Y =0: maxY-1

{

F[Y+1] = F[Y] + slope[Y];   // F[Y] is the integral of slope[Y]

}

- normalize F() to [0 maxY]

for Y=0: maxY

{

FwdLUT[Y] = clip3(0, 1023, round(F[Y]*maxY/F[maxY]));  // rounding and normalization to get FwdLUT

}

InvLUT is simply derived by mapping FwdLUT.

### Encoder mode decision

At encoder, SSE distortion is used for Intra slices for Y component. Weighted SSE distortion is used for Inter slices for Y component. For U and V components, the same weighted SSE distortion used for HDR anchors is used for all Intra and Inter slices.

## CE12-6: luma-based Chroma QP Offset

Luma reshaping modifies luma values.  To compensate for the Luma signal interaction with the Chroma signal, an additional Chroma QP Offset is used to adjust Chroma quantization parameters for Chroma component decoding. The Chroma QP Offset cQPO is derived as follows:

cQPO=-6*log2(FwdLUT'YCU) = - dQP(YCU)

where FwdLUT' is the slope (first order derivative) of the FwdLUT. For inter slice,  YCU is the average predicted Luma value of the CU. For intra slice,   YCU is inverse reshaped value of the average predicted Luma value of the CU.  The final chromaQP is computed described with simplified form as:

chromaQP = QP_luma + chromaQPOffset + cQPO;

During discussion, it has been pointed out that luma-based chroma QP offset can cause pipeline dependency in hardware implementation at decoder: Chroma inverse quantization and inverse transform cannot start until luma predicted value is available. To solve the issue, alternatively, it can be implemented by scaling the chroma residue signal in each TU.

cScale = FwdLUT'YCU

Encoder side: cResScale = cRes * cScale;  cResScale is the actual residue signal to be transformed and quantized.

Decoder side: cRes = cResScale/cScale.

In this way, decoder can start inverse quantization and transform immediately after parsing to produce cResScale.

CE12-6 contains 2 sub-CE. CE12-6.1 uses chroma QP Offset approach directly. CE12-6.2 uses chroma residue scaling approach. The chroma scaling approach is implemented with fixed point integer operations.

## CE12-7: Loop Filter application

The loop filter can be applied in either the original domain or the reshaped domain. CE12-7 is designed to evaluate performance for loop filter application.

For intra slice, the reshaped picture is coded. If loop filter is in original domain, we first inverse reshape picture after decoding, then apply loop filter before storing the picture in DPB. If loop filter is in reshape domain, we perform decoding, apply loop filter, then inverse reshape picture before storing the picture in DPB. For both cases, we need to perform inverse reshaping once, therefore both approaches have similar complexity.

For inter slice, the original picture is coded. If loop filter is in original domain, we just follow normal decoding. There is no additional step. If loop filter is in reshaped domain, we first perform decoding,  then we need to reshape reconstructed picture, then apply loop filter, then inverse reshape picture before storing it in DPB. Performing loop filter in reshaped domain requires a pair of forward and inverse LUT mapping. The simulation shows that we do not need to perform loop filter in reshaped domain for all inter pictures. The processing is only needed for pictures with TemporalId<= TemporalId_Thr. Under VTM CTC RA case, TemporalID_Thr=2 gives best performance and complexity trade-off.

## CE12-7: Region of Interest

In-loop reshaping architecture allows CTU adaptation. In JVET-J0015, ROI is originally designed for subjective benefit using adaptive reshaping curve. Because this CE is designed to use default reshaping curve to evaluate in-loop reshaping architecture with HDR metrics, we developed a heuristic algorithm to illustrate CTU adaptation feature. For fair comparison with Anchor, both Anchor and CE12-7 uses the same algorithm. CTU adaptation is implemented for inter slices only.

The algorithm is described as follows: For each CTU, we compute average luma value CTU_avg_lum_value for this CTU, then we compare it with the threshold THR. If CTU_avg_lum_value < THR, we turn on lumaDQP (in the case of Anchor) or reshaping (in the case of Test) for this CTU. The threshold selected in CE12-7 is 767.

# Syntax

The reshaping related syntax is added in SPS() (Table 1), slice header() (Table 2), and CTU() (Table 3) to allow sequence/slice/CTU level adaptation. The reshaper model can be signalled in SPS and slice header. In this CE, because default reshaping curve is used, sps_reshaper_model() is used. The default FwdLUT is signalled by linear dQP model, which is about 40bits. More detail about reshaping model syntax and semantics is presented in Annex A.

Table 1. Reshaping syntax in SPS

| SPS() | Descriptor |
| --- | --- |
| …… |  |
| sps_reshaper_enable_flag /*1: reshaping on, else off */ | u(1) |
| if (sps_reshaper_enable_flag) { |  |
| sps_reshaper_signal_type /* 0:SDR, 1:PQ*/ | u(2) |
| sps_reshaper_ILF_opt /* loop filter in which domain: 2 bits inter/intra */ | u(2) |
| if ( sps_reshaper_ILF_opt == 3) |  |
| sps_reshaper_ILF_Tid | u(3) |
| sps_reshaper_chromaAdj /* 1: chromaDQP; 2: chroma scaling/ | u(2) |
| sps_reshaper_model_present_flag /*1: present*/ | u(1) |
| if (sps_reshaper_model_present_flag) |  |
| sps_reshaper_model () |  |
| } |  |

sps_reshaper_enable_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

sps_reshaper_signal_type indicates the original codeword representation: sps_reshaper_signal_type equal to 0 specifies SDR (gamma); sps_reshaper_signal_type equal to 1 specifies PQ; and sps_reshaper_signal_type equal to 2 specifies HLG.

sps_reshaper_ILF_opt indicates whether the in-loop filter should be applied in the original domain or the reshaped domain for intra and inter slices. The LSB of the binary representation of sps_reshaper_ILF_opt indicates the domain in which to apply in-loop filter in intra slice and the MSB of the binary representation of sps_reshaper_ILF_opt indicates the domain in which to apply in-loop filter in inter slice. sps_reshaper_ILF_opt equal to 0 indicates that in-loop filter is applied in original domain for both intra and inter slices. sps_reshaper_ILF_opt equal to 1 indicates that in-loop filter is applied in original domain for inter slices and reshaped domain for intra slices. sps_reshaper_ILF_opt equal to 2 indicates that in-loop filter is applied in reshaped domain for inter slices and original domain for intra slices. sps_reshaper_ILF_opt equal to 3 indicates that in-loop filter is applied in reshaped domain for both inter and intra slices.

sps_reshaper_ILF_Tid specifies the highest TemporalID where in-loop filter is applied for a reshaped slice in the reshaped domain.

sps_reshaper_chromaAdj equal to 1 indicates that chroma adjustment is done using chroma QP offset. sps_reshaper_chromaAdj equal to 2 indicates that chroma adjustment is done using chroma residue scaling. sps_reshaper_chromaAdj equal to 0 indicates that chroma adjustment is not enabled. The sps_reshaper_chromaAdj can only take values of 0, 1 and 2.

sps_reshaper_model_present_flag equal to 1 indicates sps_reshaper_model() is present in SPS. sps_reshaper_model_present_flag equal to 0 indicates sps_reshaper_model() is not present in SPS.

Table 2. Reshaping syntax in slice header

| slice_header() | Descriptor |
| --- | --- |
| …… |  |
| slice_reshaper_model_present_flag | u(1) |
| if ( slice_reshaper_model_present_flag ) |  |
| slice_reshaper_model ( ) |  |
| slice_reshaper_enable_flag | u(1) |
| if ( slice_reshaper_enable_flag ) { |  |
| reshaper_CTU_control_flag | u(1) |
| } |  |
| …… |  |

slice_reshaper_model_present_flag equal to 1 indicates slice_reshaper_model() is present in slice header. slice_reshaper_model_present_flag equal to 0 indicates slice_reshaper_model() is not present in slice header.

slice_reshaper_enable_flag equal to 1 specifies that reshaper is enabled for the current slice. slice_reshaper_enable_flag equal to 0 specifies that reshaper is not enabled for the current slice.

reshaper_CTU_control_flag equal to 1 indicates that reshaper adaption is allowed for each CTU in current slice. reshaper_CTU_control_flag equal to 0 indicates that reshaper is not allowed to be adapted for each CTU in current slice. When reshaper_CUT_control_flag is not present, the value shall be inferred to be 0.

Table 3. Reshaping syntax in CTU

| coding_tree_unit() | Descriptor |
| --- | --- |
| …… |  |
| if ( reshape_CTU_control_flag ) { |  |
| reshaper_CTU_flag | ae(v) |
| } |  |
| …… |  |

reshaper_CTU_flag equal to 1 specifies that reshaper is used for the current CTU. reshaper_CUT_flag equal to 0 specifies that reshaper is not used for the current CTU. When reshaper_CTU_flag is not present, the value shall be inferred to equal to slice_reshaper_enabled_flag.

# Simulation Results

The CE tests are implemented in BMS1.1 software with VTM tool settings (JEM_TOOLS = 0) and with JEM-comparable HDR implementation (WCG_EXT=1). The decoding process is implemented using fixed point. The test is done by disabling BMS tools and used VTM CTC with AI and RA. The test is performed for HDR PQ clips because HDR HLG anchor does not use luma QP adaptation technology. AVX2 is used in run time for SIMD optimization. The encoding time is measured using cluster. The decoding time is measured using a PC station specified by Table 4. The following tables shows BDRate of HDR metrics for fixed QP test condition.

Table 4. Technical specification of the PC workstation for decoding

| CPU | Intel Core i7-3930K / Clock speed 3.20GHz |
| --- | --- |
| Memory | 64 GB |
| SDD characteristics | Intel SSD 520 Series -Capacity: 240GB -Interface: SATA 3.0 6Gb/s |
| Compiler | Visual Studio 2015 |
| Operating System (OS) | Windows 7 64 bit |

The setting for all sub-CEs is described in Table 5.

Table 5. Simulation Setting for CE12-5 to CE12-8

|  | sps_reshaper_chromaAdj | sps_reshaper_ILF_opt |
| --- | --- | --- |
| CE12-5 | 0 | 3(RA), 1 (AI) |
| CE12-6.1 | 1 | 3(RA), 1 (AI) |
| CE12-6.2 | 2 | 3(RA), 1 (AI) |
| CE12-7.1 | 1 | 0 |
| CE12-7.2 | 1 | 1 |
| CE12-7.3 | 1 | 3(RA) |
| CE12-8 | 1 | 3(RA) |

## CE12-5: In-loop luma reshaping

Table 6 shows the BDRate results for CE12-5 for HDR metrics. The average BDRate for PSNRL100 is -2.8% for AI and -2.6% for RA; for wPsnrY is -2.1% for AI and -2.0% for RA; for DE100 is 13.7% for AI and 11% for RA. There is more than 2% gain in PSNRL100 and wPSNRY, but there is loss for DE100.

Table 6. BDRate for CE12-5: AI and RA

|  | AI: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-5 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 16.9% | -3.9% | -3.3% | 25.9% | 30.2% |
| SunRise | 25.6% | -3.1% | -2.5% | 23.3% | 32.9% |
| ShowGirl2 | -4.4% | -1.3% | -0.6% | -6.4% | -17.3% |
| BalloonFestival | 6.7% | -4.1% | -3.1% | 4.1% | 5.3% |
| Hurdles | 38.9% | -4.6% | -4.1% | 53.0% | 69.9% |
| Starting | 20.6% | -4.9% | -4.2% | 17.6% | 24.7% |
| Cosmos1 | -8.7% | 2.4% | 3.3% | -31.3% | -54.9% |
| Average HDR-B | 13.7% | -2.8% | -2.1% | 12.3% | 13.0% |
| Enc Time[%] | 101% |  |  |  |  |
| Dec Time[%] | 104% |  |  |  |  |

|  | RA: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-5 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 17.0% | -2.8% | -2.4% | 25.0% | 32.7% |
| SunRise | 11.9% | -1.7% | -1.2% | 22.6% | 21.5% |
| ShowGirl2 | -2.1% | -0.9% | -0.4% | -0.8% | -10.6% |
| BalloonFestival | 7.4% | -3.8% | -2.9% | 4.9% | 5.0% |
| Hurdles | 26.9% | -4.8% | -4.1% | 41.6% | 50.0% |
| Starting | 23.0% | -4.7% | -3.8% | 18.5% | 21.6% |
| Cosmos1 | -7.2% | 0.5% | 0.5% | -27.7% | -47.9% |
| Average HDR-B | 11.0% | -2.6% | -2.0% | 12.0% | 10.3% |
| Enc Time[%] | 104% |  |  |  |  |
| Dec Time[%] | 108% |  |  |  |  |

## CE12-6: luma-based Chroma QP Offset

Table 7 shows the BDRate results for CE12-6.1 (chroma QP Offset) for HDR metrics. The average BDRate of CE12-6.1 with chroma QP offset for PSNRL100 is -2.8% for AI and -2.6% for RA; for wPsnrY is -2.0% for AI and -2.0% for RA; for DE100 is -1.5% for AI and -1.3% for RA. Compared to CE12-5, DE100 shows some gain instead of loss. And the gain for PSNRL100 and wPsnrY metrics is maintained.

Table 8 shows the BDRate results for CE12-6-2 (chroma residue scaling) for HDR metrics. The average BDRate of CE12-6-2 with chroma residue scaling for PSNRL100 is -2.8% for AI and -2.6% for RA; for wPsnrY is -2.1% for AI and -2.0% for RA; for DE100 is 0.5% for AI and 2.0% for RA. Compared to CE12-5, DE100 loss has kept under 2.0%. The gain for PSNRL100 and wPsnrY metrics is maintained. The test shows luma-based chroma adjustment techniques can effectively improve chroma performance.

Table 7. BDRate for CE12-6.1: AI and RA (12-6.1: chroma QP Offset)

|  | AI: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-6.1 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 0.7% | -3.5% | -2.9% | -1.0% | -4.0% |
| SunRise | -0.2% | -2.2% | -1.5% | -3.0% | -7.1% |
| ShowGirl2 | -3.6% | -2.1% | -1.6% | -2.9% | -8.3% |
| BalloonFestival | -0.4% | -3.6% | -2.4% | -2.9% | -3.8% |
| Hurdles | -1.7% | -2.4% | -1.8% | -0.9% | -2.1% |
| Starting | -2.3% | -3.0% | -2.2% | -2.6% | -2.5% |
| Cosmos1 | -3.1% | -2.5% | -1.8% | 0.1% | -5.6% |
| Average HDR-B | -1.5% | -2.8% | -2.0% | -1.9% | -4.8% |
| Enc Time[%] | 105% |  |  |  |  |
| Dec Time[%] | 105% |  |  |  |  |

|  | RA: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-6.1 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 1.2% | -2.5% | -2.1% | 0.4% | 1.9% |
| SunRise | -2.2% | -1.2% | -0.5% | -1.0% | -4.3% |
| ShowGirl2 | -3.4% | -1.6% | -1.2% | -2.7% | -4.0% |
| BalloonFestival | -0.4% | -3.5% | -2.5% | -2.0% | -2.6% |
| Hurdles | -1.3% | -3.8% | -3.1% | -0.3% | 5.0% |
| Starting | -1.8% | -3.3% | -2.3% | -1.4% | 2.1% |
| Cosmos1 | -1.4% | -2.1% | -2.2% | 7.0% | 1.4% |
| Average HDR-B | -1.3% | -2.6% | -2.0% | 0.0% | -0.1% |
| Enc Time[%] | 104% |  |  |  |  |
| Dec Time[%] | 108% |  |  |  |  |

Table 8. BDRate for CE12-6.2: AI and RA (12-6.2: chroma residue scaling)

|  | AI: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-6.2 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | -0.8% | -3.5% | -3.0% | 4.1% | 4.6% |
| SunRise | 16.8% | -1.6% | -0.9% | 2.5% | 12.0% |
| ShowGirl2 | -4.7% | -1.7% | -1.3% | -2.9% | -9.4% |
| BalloonFestival | 4.9% | -4.1% | -3.1% | 3.1% | 7.0% |
| Hurdles | -4.7% | -1.9% | -1.3% | -1.3% | -4.6% |
| Starting | -7.3% | -3.5% | -2.7% | 1.6% | 6.4% |
| Cosmos1 | -0.5% | -3.0% | -2.2% | 6.5% | 2.2% |
| Average HDR-B | 0.5% | -2.8% | -2.1% | 2.0% | 2.6% |
| Enc Time[%] | 106% |  |  |  |  |
| Dec Time[%] | 106% |  |  |  |  |

|  | RA: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-6.2 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 0.9% | -2.6% | -2.2% | 6.8% | 11.9% |
| SunRise | 14.8% | -0.7% | 0.0% | 11.5% | 15.1% |
| ShowGirl2 | -3.7% | -1.5% | -1.1% | -2.4% | -3.6% |
| BalloonFestival | 9.0% | -3.8% | -3.0% | 5.6% | 9.7% |
| Hurdles | -3.3% | -3.6% | -2.9% | 0.6% | 5.5% |
| Starting | -5.1% | -3.7% | -2.7% | 3.6% | 10.6% |
| Cosmos1 | 1.4% | -2.4% | -2.4% | 15.9% | 13.2% |
| Average HDR-B | 2.0% | -2.6% | -2.0% | 5.9% | 8.9% |
| Enc Time[%] | 104% |  |  |  |  |
| Dec Time[%] | 108% |  |  |  |  |

## CE12-7: loop-filter application

Table 9 shows the BDRate results for CE12-7.1 (Intra: original domain; Inter: original domain) for HDR metrics. The average BDRate for PSNRL100 is -2.1% for AI and -1.7% for RA; for wPsnrY is -1.5% for AI and -1.3% for RA; for DE100 is -1.4% for AI and -1.1% for RA.

Table 10 shows the BDRate results for CE12-7.2 (Intra: reshaped domain; Inter: original domain) for HDR metrics. The average BDRate for PSNRL100 is -2.8% for AI and -2.4% for RA; for wPsnrY is -2.0% for AI and -1.8% for RA; for DE100 is -1.5% for AI and -1.4% for RA.

Table 11 shows the BDRate results for CE12-7.3 (Intra: reshaped domain; Inter: reshaped domain) for HDR metrics. For AI, CE12-7.3 is not a valid test setting since there is no inter coding in AI test condition. The average BDRate for PSNRL100 is -2.4% for RA; for wPsnrY is -2.6% for RA; for DE100 is -1.3% for RA.

The test shows that applying loop-filter in reshaped domain improve HDR metrics performance. The major gain comes from applying loop-filter in reshaped domain for intra slices (CE12-7.2). And this setting has similar decoding complexity as applying loop-filter in original domain (CE12-7.1). This is for both CE12-7.1 and CE12-7.2, for intra slices, we only need to perform InvLUT once. CE12-7.2 further improved HDR metrics by applying loop filter for inter slices for lower temporal level with only 2% decoding running time increase. This is our recommended setting for in-loop reshaping architecture.

Table 9. BDRate for CE12-7.1: AI and RA (Intra: original domain; Inter: original domain)

|  | AI: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-7.1 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 1.1% | -2.1% | -1.8% | -1.0% | -4.0% |
| SunRise | -0.3% | -1.5% | -1.0% | -3.0% | -7.0% |
| ShowGirl2 | -3.4% | -1.8% | -1.3% | -2.9% | -7.8% |
| BalloonFestival | -0.3% | -3.4% | -2.1% | -2.9% | -3.8% |
| Hurdles | -1.4% | -0.8% | -0.3% | -0.9% | -2.2% |
| Starting | -2.0% | -2.2% | -1.5% | -2.6% | -2.5% |
| Cosmos1 | -3.7% | -2.8% | -2.5% | 0.2% | -5.5% |
| Average HDR-B | -1.4% | -2.1% | -1.5% | -1.9% | -4.7% |
| Enc Time[%] | 106% |  |  |  |  |
| Dec Time[%] | 105% |  |  |  |  |

|  | RA: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-7.1 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 1.9% | -0.7% | -0.8% | 0.8% | 2.2% |
| SunRise | -2.3% | -0.6% | -0.4% | -0.9% | -4.3% |
| ShowGirl2 | -3.3% | -1.4% | -0.9% | -2.2% | -3.4% |
| BalloonFestival | -0.4% | -3.1% | -2.1% | -2.2% | -2.8% |
| Hurdles | -0.7% | -2.0% | -1.8% | -0.6% | 4.5% |
| Starting | -1.6% | -2.2% | -1.5% | -1.4% | 2.0% |
| Cosmos1 | -1.3% | -2.0% | -2.0% | 7.3% | 1.6% |
| Average HDR-B | -1.1% | -1.7% | -1.3% | 0.1% | 0.0% |
| Enc Time[%] | 103% |  |  |  |  |
| Dec Time[%] | 106% |  |  |  |  |

Table 10. BDRate for CE12-7.2: AI and RA (Intra: reshaped domain; Inter: original domain)

|  | AI: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-7.2 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 0.7% | -3.5% | -2.9% | -1.0% | -4.0% |
| SunRise | -0.2% | -2.2% | -1.5% | -3.0% | -7.1% |
| ShowGirl2 | -3.6% | -2.1% | -1.6% | -2.9% | -8.3% |
| BalloonFestival | -0.4% | -3.6% | -2.4% | -2.9% | -3.8% |
| Hurdles | -1.7% | -2.4% | -1.8% | -0.9% | -2.1% |
| Starting | -2.3% | -3.0% | -2.2% | -2.6% | -2.5% |
| Cosmos1 | -3.1% | -2.5% | -1.8% | 0.1% | -5.6% |
| Average HDR-B | -1.5% | -2.8% | -2.0% | -1.9% | -4.8% |
| Enc Time[%] | 105% |  |  |  |  |
| Dec Time[%] | 105% |  |  |  |  |

|  | RA: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-7.2 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 1.3% | -2.0% | -1.7% | 0.5% | 1.9% |
| SunRise | -2.4% | -1.3% | -0.7% | -1.1% | -4.4% |
| ShowGirl2 | -3.7% | -1.5% | -1.1% | -2.8% | -3.7% |
| BalloonFestival | -0.4% | -3.4% | -2.3% | -2.0% | -2.7% |
| Hurdles | -1.2% | -3.2% | -2.6% | -0.5% | 4.8% |
| Starting | -1.8% | -3.0% | -2.0% | -1.4% | 1.9% |
| Cosmos1 | -1.5% | -2.1% | -2.0% | 7.0% | 1.2% |
| Average HDR-B | -1.4% | -2.4% | -1.8% | -0.1% | -0.1% |
| Enc Time[%] | 104% |  |  |  |  |
| Dec Time[%] | 106% |  |  |  |  |

Table 11. BDRate for CE12-7.3: RA (Intra: reshaped domain; Inter: reshaped domain)

|  | RA: BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-7.3 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 1.2% | -2.5% | -2.1% | 0.4% | 1.9% |
| SunRise | -2.2% | -1.2% | -0.5% | -1.0% | -4.3% |
| ShowGirl2 | -3.4% | -1.6% | -1.2% | -2.7% | -4.0% |
| BalloonFestival | -0.4% | -3.5% | -2.5% | -2.0% | -2.6% |
| Hurdles | -1.3% | -3.8% | -3.1% | -0.3% | 5.0% |
| Starting | -1.8% | -3.3% | -2.3% | -1.4% | 2.1% |
| Cosmos1 | -1.4% | -2.1% | -2.2% | 7.0% | 1.4% |
| Average HDR-B | -1.3% | -2.6% | -2.0% | 0.0% | -0.1% |
| Enc Time[%] | 104% |  |  |  |  |
| Dec Time[%] | 108% |  |  |  |  |

## CE12-8: ROI (Inter CTU Adaptation)

Table 12 shows the BDRate results for CE12-8 (Inter CTU Adaptation) for HDR metrics. For fair comparison, HDR anchor is modified such that both Anchor and CE test use the same algorithm for CTU level on/off decision. The average BDRate under RA: PSNRL100: -2.2% ; wPsnrY: -1.7%; DE100:  -1.2%. The simple, heuristic algorithm is based on a luminance threshold and is aimed at exhibiting  the functionality of CTU adaptation for in-loop reshaping, rather than demonstrating of absolute performance gain. Other improved and complex CTU adaptation algorithms can be targeted to optimize the BDrate gain or to boost subjective quality.

Table 12. BDRate for CE12-8: AI and RA

|  | BD-rate |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CE12-8 | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | 1.2% | -2.3% | -1.9% | 0.6% | 2.2% |
| SunRise | -1.9% | -1.0% | -0.3% | -0.9% | -3.9% |
| ShowGirl2 | -3.7% | -0.8% | -0.5% | -2.6% | -4.8% |
| BalloonFestival | -0.3% | -3.5% | -2.5% | -2.0% | -2.5% |
| Hurdles | -1.0% | -3.7% | -3.0% | -0.3% | 5.2% |
| Starting | -1.3% | -2.2% | -1.7% | -1.1% | 2.4% |
| Cosmos1 | -1.5% | -2.1% | -2.2% | 6.7% | 0.9% |
| Average HDR-B | -1.2% | -2.2% | -1.7% | 0.1% | -0.1% |
| Enc Time[%] | 103% |  |  |  |  |
| Dec Time[%] | 103% |  |  |  |  |

## Complexity analysis

The major computational complexity of in-loop reshaping is the mapping for Luma samples using the pair of FwdLUT and InvLUT. The pair of reshaping LUTs are implemented using 2-byte (10-bit), 1024 entry integer arrays. All the related operations, such as mapping and LUT construction, are performed with fixed point arithmetic at the decoder side. The in-loop reshaper performs Luma sample mapping in-place, meaning no extra storage buffer is needed for Luma samples. The running time for simulation shown in this contribution is with SIMD AVX2 enabled. It is known that there is no effective LUT command in SIMD implementation. Even with this constraint, for recommended setting, the decoding running time increase over VTM1.0 is limited under 10%. In addition, the complexity of in-loop reshaping and its components is mainly determined by picture resolution and will not change if decoder complexity increases due to more complex coding tools are enabled (e.g. BMS coding tools or other JEM coding tools). For class B (HD sequences) RA, the decoding complexity of VTM1.0 is 76% over HM16.18 and the decoding complexity of BMS1.0 is 278% over VTM1.0. Therefore, 10% decoding complexity over VTM1.0 is relative to 7% over HM16.18 and 3% over BMS1.0.

## Summary

Table 13 shows a summary table for HDR metrics for all CE 12-5 to 12-7. CE12-6.1 (same as CE12-7.3) show best trade-off among luma and chroma components. The following settings are enabled for the best trade-off: luma-level pixel mapping; luma-based chroma QP Offset; and applying loop-filter in reshaped domain for both intra and inter slices. The setting shows that more -2% gain for wPsnrY/PSNRL100, and no loss for chroma metrics. The encoder running time is about 105% and decoding running time is about 108% of VTM Anchor. CE12-6.2 has 2% loss in RA for DE100 and maintains similar gain in PSNRL100 and wPsnrY. The advantage is that it removes hardware dependency issue. It is another recommended setting to be considered.

The gain from reshaping is mainly due to: 1) Reshaping is a pixel level operation while the Anchor uses block level operation; 2) quantization in reshaping has much finer granularity than transform quantization.

Table 13. BDRate Summary for CE12-5 to CE12-7: AI and RA

| AI | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE12-5 | 13.70% | -2.80% | -2.10% | 12.30% | 13.00% | 101% | 104% |
| CE12-6.1 | -1.50% | -2.80% | -2.00% | -1.90% | -4.80% | 105% | 105% |
| CE12-6.2 | 0.50% | -2.80% | -2.10% | 2.00% | 2.60% | 106% | 106% |
| CE12-7.1 | -1.40% | -2.10% | -1.50% | -1.90% | -4.70% | 106% | 106% |
| CE12-7.2 | -1.50% | -2.80% | -2.00% | -1.90% | -4.80% | 105% | 105% |

| RA | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | Enc time | Dec time |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE12-5 | 11.00% | -2.60% | -2.00% | 12.00% | 10.30% | 104% | 108% |
| CE12-6.1 | -1.30% | -2.60% | -2.00% | 0.00% | -0.10% | 104% | 108% |
| CE12-6.2 | 2.00% | -2.60% | -2.00% | 5.90% | 8.90% | 104% | 108% |
| CE12-7.1 | -1.10% | -1.70% | -1.30% | 0.10% | 0.00% | 103% | 106% |
| CE12-7.2 | -1.40% | -2.40% | -1.80% | -0.10% | -0.10% | 104% | 106% |
| CE12-7.3 | -1.30% | -2.60% | -2.00% | 0.00% | -0.10% | 104% | 108% |

# Conclusion

The in-loop reshaping tested in this CE is designed to have a fair comparison with HDR Anchor under VTM CTC using HDR Metrics. The recommended setting (luma-level pixel mapping, luma-based chroma QP Offset, and applying loop-filter in reshaped domain for both intra and inter) shows that more than 2% gain for wPsnrY/PSNRL100, and no loss for chroma metrics. The encoder running time is about 105% and decoding running time is about 108%. It is recommended that in-loop reshaping is a good candidate for VVC coding tool. In [4], it shows that in-loop reshaping is not only effective for HDR sequences. It can also improve coding performance for SDR UHD/HD sequences.

# Reference

- E. Francois, D. Rusanovskyy, P. Yin, “Description of Core Experiment 12 (CE12): mapping for HDR content”, JVET-J0032, San Diego, USA, April, 2018.

- X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. Ye, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen, “Description of SDR, HDR, and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, San Diego, USA, April, 2018.

- A. Segall, E. Francois, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-J1011, San Diego, USA, April, 2018.

- F. Pu, T. Lu, P. Yin, W. Husak, S. McCarthy, T. Chen, “SDR In-loop Reshaping”, JVET-K0309, Ljubljana, SI,  July , 2018.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Acknowledgement

We would like to thank Qualcomm, Sharp and Technicolor for crosschecking our CE results.

# Annex A : Syntax on reshaping model

| sps_reshaper_model() { | descriptor |
| --- | --- |
| full_range_input_flag | u(1) |
| dQP_model_scale_int_prec | ue(v) |
| if (dQP_model_scale_int_prec > 0) { |  |
| dQP_model_scale_int | u(v) |
| } |  |
| dQP_model_scale_frac_prec_minus16 | ue(v) |
| dQP_model_scale_frac | u(v) |
| if (dQPModelScaleAbs) { |  |
| dQP_model_scale_sign | u(1) |
| } |  |
| dQP_model_offset_int_prec_minus3 | ue(v) |
| dQP_model_offset_int | u(v) |
| dQP_model_offset_frac_prec_minus1 | ue(v) |
| dQP_model_offset_frac | u(v) |
| if (dQPModelOffsetAbs) { |  |
| dQP_model_offset_sign | u(1) |
| } |  |
| dQP_model_abs_prec_minus3 | ue(v) |
| dQP_model_max_abs | u(v) |
| if (dQP_model_max_abs) { |  |
| dQP_model_max_sign | u(1) |
| } |  |
| dQP_model_min_abs | u(v) |
| if (dQP_model_min_abs) { |  |
| dQP_model_min_sign | u(1) |
| } |  |
| } |  |

full_range_input_flag specifies the input video signal range. A full_range_input_flag of 0 corresponds to a standard dynamic range input video signal. A full_range_input_flag of 1 corresponds to full range input video signal.  When not present, full_range_input_flag is inferred to be 0.

dQP_model_scale_int_prec specifies the number of bits used for the representation of dQP_model_scale_int. A dQP_model_scale_int_prec equals to 0 indicates dQP_model_scale_int is not signaled and is inferred to be 0.

dQP_model_scale_int specifies the integer value of dQP model scale.

dQP_model_scale_frac_prec_minus16 plus 16 specifies the number of bits used for the representation of dQP_model_scale_frac.

dQP_model_scale_frac specifies the fractional value of the dQP model scale.

The variable dQPModelScaleAbs is derived as:

dQPModelScaleAbs = dQP_model_scale_int << (dQP_model_scale_frac_prec_minus16 + 16) + dQP_model_scale_frac

dQP_model_scale_sign specifies the sign of dQP model scale. When dQPModelScaleAbs equals 0, dQP_model_scale_sign is not signaled and it is inferred to be 0.

dQP_model_offset_int_prec_minus3 plus 3 specifies the number of bits used for the reprentation of dQP_model_offset_int.

dQP_model_offset_int specifies the integer value of dQP model offset.

dQP_model_ offset _frac_prec_minus1 plus 1 specifies the number of bits used for the representation of dQP_model_ offset_frac.

dQP_model_ offset_frac specifies the fractional value of the dQP model offset.

The variable dQPModelOffsetAbs is derived as:

dQPModelOffsetAbs = dQP_model_offset_int << (dQP_model_offset_frac_prec_minus1 + 1) + dQP_model_offset_frac

dQP_model_offset_sign specifies the sign of dQP model offset. When dQPModelOffsetAbs equals 0, dQP_model_offset_sign is not signaled and is inferred to be 0.

dQP_model_abs_prec_minus3 plus 3 specifies the number of bits used for the representation of dQP_model_max_abs and dQP_model_min_abs.

dQP_model_max_abs specifies the integer value of dQP model max.

dQP_model_max_sign specifies the sign of dQP model max. When dQP_model_max_abs equals 0, dQP_model_max_sign is not signaled and is inferred to be 0.

dQP_model_min_abs specifies the integer value of dQP model min.

dQP_model_min_sign specifies the sign of dQP model min. When dQP_model_min_abs equals 0, dQP_model_min_sign is not signaled and is inferred to be 0.