| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0309-v1 |
| --- | --- |

| Title: | In-loop Reshaping for SDR Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Sean McCarthy, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, fpu}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

In-loop Reshaping is proposed for HDR video in JVET-J0015 and is further tested in JVET-K00xx for CE12: Mapping for HDR content. This contribution proposes to use the same in-loop reshaping coding tool for SDR video.

Compared to VTM SDR Anchor, the contribution shows the following BDRate for UHD and HD clips under CTC:

- AI {Y, U, V}:

Class A1:{-1.50%, 0.56%, 0.43%}; Class A2:{-1.37%, 3.14%, 3.43%}; Class B:{-0.76%, 0.83%, 0.04%}

Enc time: 105%, Dec time: 104%

- RA {Y, U, V}:

Class A1:{-1.58%, 2.49%, 2.49%}; Class A2{-1.28%, 2.47%, 2.57%}; Class B:{-1.09%, 0.89%, -0.27%}

Enc time: 110%, Dec time: 103%

# Introduction

Reshaping is to re-allocate codeword more efficiently based on content characteristics. In-loop reshaping has been proposed for HDR video in JVET-J0015 [2] and its coding performance has been further tested in JVET-J1032 [1] and JVET-K0308 CE12 [3]. However, the performance of using in-loop reshaper as a general coding tool for SDR coding has not been studied by earlier work.

In this contribution, we propose to use in-loop reshaping for SDR video coding. The results on UHD/HD clips for AI and RA has demonstrated the compression capability of the in-loop reshaping coding tool for SDR video.

# Algorithm Description

## In-loop reshaping architecture

The details of in-loop reshaping architecture can refer to JVET-K00xx [3]. To be brief, the in-loop Luma reshaper is implemented and configured as a pair of one dimensional, 10-bit, 1024-entry mapping tables (1D-LUT). The first LUT is referred to as FwdLUT, which can be used to map an input Luma code value Yi to an altered value Yr: Yr=FwdLUT[Yi]. The second LUT is referred to as InvLUT, which can be used to map altered code value Yr for “the reconstruction of Yi” as Yi: Yi=InvLUT[Yr]. Only FwdLUT need to be signaled and InvLUT is computed from the FwdLUT. How the in-loop reshaper maps the Luma code values is based on the slice type received by the decoder. Figure 1 and Figure 2 shows how in-loop reshaping works at decoder. In this proposal, in-loop reshaping is used only for luma component.

Figure 1. HDR Intra slice reconstruction with in-loop Luma Reshaper

Figure 2. HDR Inter slice reconstruction with in-loop Luma Reshaper

To compensate luma and chroma interaction due to luma reshaping, an additional luma-based chroma QP Offset is used to adjust chroma QP. The Chroma QP Offset cQPO is derived as follows:

cQPO=-6*log2(FwdLUT'YCU) = - dQP(YCU)                                            (eq:1)

where FwdLUT' is the slope (first order derivative) of the FwdLUT. For inter slice,  YCU is the average predicted Luma value of the CU. For intra slice, YCU is inverse reshaped value of the average predicted Luma value of the CU.  The final chromaQP is computed described at the simplified form as

chromaQP = QP_luma + chromaQPOffset + cQPO;

For intra slice, if reshaping is applied, loop-filter is applied in reshaped domain. For inter slice, if reshaping is applied, loop-filter is applied in original domain.

## Construct luma reshaping curve

The reshaping curve is constructed by assigning a number of codewords to each bin(/piece) directly. We divide all codewords into 32 bins. which means each bin contains 32 codewords for 10 bit full range video. The goal of the reshaping is to find an optimized codeword allocation for each bin, such that best RDO can be achieved. In the current implementation, each bin is assigned from one of 4 codeword number {M0, M1, M2, M3}, M0=0, M1=32, M2<M2<=64 and 0<M3<32. For limited video range, no codewords (i.e., M0) will be assigned to bin {0, 1, 30, 31}. M2 is defined to be around 38. The number is calculated by linearly mapping narrow range signal to full range, such that each bin will contain about CEIL(1023/(940-64)*32)=38 codewords. The FwdLUT is an integral of codewords assignment to each bin.

The encoder applies the following algorithm for every intra slice.

- Analyze the image to gather variance (bin_var) for each bin. The bin variance is computed by averaging the pixel variance of all pixels belonging to that bin.  The pixel variance is computed by 5x5 windows.

- Compare bin_var with two thresholds: Thr1 and Thr2

if bin_var = 0, the bin is allocated M0 (zero) codeword.

else if bin_var < Thr1, the bin is allocated M2 codewords;

else if bin_var > Thr2, the bin is allocated M3 codewords;

else

the bin is allocated M1 codewords.

- if sum of the number of codewords in each bin exceed 1024, we normalize it to [0 1023].

- For each bin, divide bin assignment by 32 to find codeword mapping for each codeword cw[i].

- Construct FwdLUT by integral all codeword assignment.

FwdLUT(0) = 0;

for Y =0: maxY-1   (maxY=1023 for 10 bit)

FwdLUT[Y+1] = Clip3(0, maxY, (round) (FwdLUT[Y] + cw[Y]));

- construct InvLUT by mapping FwdLUT.

In the syntax, the number of codewords assignment for each bin is signaled. At decoder, it will repeat step 3) to 6) above to construct FwdLUT and InvLUT.

For chroma coding, luma-based chroma QP Offset is derived using codewords assignment:

FwdLUT’() = slope = (cw_bin(k)/32), assuming piecewise linear model. Because cw_bin is limited to [1 64], we can precompute cQPO LUT using eq1.

## Encoder optimization

Our current algorithm only applies for limited range video sequences.

### Mode decision

If luma reshaper is applied to the current CTU for an inter slice, for luma, we will use weighted SSE. For all luma values in each bin, the same weight is used. The weight is derived from codeword assignment in that bin as follows:

w_rsp(k) = (cw_bin(k)/32)^2, w_rsp(k) is weight for k-th bin.

For intra slice, SSE is used for luma in mode decision.

### Slice Adaptation

In-loop reshaping allows slice level and CTU level adaptation, therefore allows flexible rate-distortion optimization between slices and CTUs.

# Syntax

The reshaping related syntax is added in SPS() (Table 1), slice header() (Table 2), and CTU() (Table 3) to allow sequence/slice/CTU level adaptation, the same as in [3]. The reshaper model can be signalled in SPS and slice header. In this contribution, because adaptive reshaping curve is used, slice_reshaper_model() is used, which is about 80 bits. More detail about slice reshaping model syntax and semantics can refer to Annex A.

Table 1. Reshaping syntax in SPS

| SPS() | Descriptor |
| --- | --- |
| …… |  |
| sps_reshaper_enable_flag /*1: reshaping on, else off */ | u(1) |
| if (sps_reshaper_enable_flag) { |  |
| sps_reshaper_signal_type /* 0:SDR, 1:PQ*/ | u(2) |
| sps_reshaper_ILF_opt /* loop filter in which domain: 2 bits inter/intra */ | u(2) |
| if ( sps_reshaper_ILF_opt == 3) |  |
| sps_reshaper_ILF_Tid | u(3) |
| sps_reshaper_chromaAdj /* 1: chromaDQP; 2: chroma scaling/ | u(2) |
| sps_reshaper_model_present_flag /*1: present*/ | u(1) |
| if (sps_reshaper_model_present_flag) |  |
| sps_reshaper_model () |  |
| } |  |

sps_reshaper_enable_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

sps_reshaper_signal_type indicates the original codewords distribution or representation. sps_reshaper_signal_type equal to 0 specifies SDR (gamma); sps_reshaper_signal_type equal to 1 specifies PQ; and sps_reshaper_signal_type equal to 2 specifies HLG.

sps_reshaper_ILF_opt indicates whether the in-loop filter should be applied in the original domain or the reshaped domain for intra and inter slices.  The LSB of the binary representation of sps_reshaper_ILF_opt indicates the domain in which to apply in-loop filter in intra slice and the MSB of the binary representation of sps_reshaper_ILF_opt indicates the domain in which to apply in-loop filter in inter slice. sps_reshaper_ILF_opt equal to 0 indicates that in-loop filter is applied in original domain for both intra and inter slices. sps_reshaper_ILF_opt equal to 1 indicates that in-loop filter is applied in original domain for inter slices and reshaped domain for intra slices. sps_reshaper_ILF_opt equal to 2 indicates that in-loop filter is applied in reshaped domain for inter slices and original domain for intra slices. sps_reshaper_ILF_opt equal to 3 indicates that in-loop filter is applied in reshaped domain for both inter and intra slices.

sps_reshaper_ILF_Tid specifies the highest TemporalID where in-loop filter is applied for a reshaped slice in the reshaped domain.

sps_reshaper_chromaAdj equal to 1 indicates that chroma QP adjustment is done using chroma QP offset. sps_reshaper_chromaAdj equal to 2 indicates that chroma QP adjustment is done using chroma residue scaling. sps_reshaper_chromaAdj equal to 0 indicates that chroma adjustment is not enabled. The sps_reshaper_chromaAdj can only take values of 0, 1 and 2.

sps_reshaper_model_present_flag equal to 1 indicates sps_reshaper_model() is present in sps. sps_reshaper_model_present_flag equal to 0 indicates sps_reshaper_model() is not present in SPS.

Table 2. Reshaping syntax in slice header

| slice_header() | Descriptor |
| --- | --- |
| …… |  |
| slice_reshaper_model_present_flag | u(1) |
| if ( slice_reshaper_model_present_flag ) |  |
| slice_reshaper_model ( ) |  |
| slice_reshaper_enable_flag | u(1) |
| if ( slice_reshaper_enable_flag ) { |  |
| reshaper_CTU_control_flag | u(1) |
| } |  |
| …… |  |

slice_reshaper_model_present_flag equal to 1 indicates slice_reshaper_model() is present in slice header. slice_reshaper_model_present_flag equal to 0 indicates slice_reshaper_model() is not present in slice header.

slice_reshaper_enable_flag equal to 1 specifies that reshaper is enabled for the current slice. slice_reshaper_enable_flag equal to 0 specifies that reshaper is not enabled for the current slice.

reshaper_CTU_control_flag equal to 1 indicate that reshaper is allowed to be adapted for each CTU in current slice. reshaper_CTU_control_flag equal to 0 indicate that reshaper is not allowed to be adapted for each CTU in current slice. When reshaper_CUT_control_flag is not present, the value shall be inferred to be 0.

Table 3. Reshaping syntax in CTU

| coding_tree_unit() | Descriptor |
| --- | --- |
| …… |  |
| if ( reshape_CTU_control_flag ) { |  |
| reshaper_CTU_flag | ae(v) |
| } |  |
| …… |  |

reshaper_CTU_flag equal to 1 specifies that reshaper is used for the current CTU. reshaper_CUT_flag equal to 0 specifies that reshaper is not used for the current CTU. When reshaper_CTU_flag is not present, the value shall be inferred to equal to slice_reshaper_enabled_flag.

# Simulation Results

The proposal is implemented in VTM1.1 software. The decoding process is implemented using fixed point.  The test is done under VTM CTC for AI and RA. AVX2 is used in run time for SIMD optimization. The encoding time is measured using cluster. The decoding time is measured using a PC station specified by Table 4.

Table 4. Technical specification of the PC workstation for decoding

| CPU | Intel Core i7-3930K / Clock speed 3.20GHz |
| --- | --- |
| Memory | 64 GB |
| SDD characteristics | Intel SSD 520 Series -Capacity: 240GB -Interface: SATA 3.0 6Gb/s |
| Compiler | Visual Studio 2015 |
| Operating System (OS) | Windows 7 64 bit |

Table 5 show the BDRate results and Encoding/Decoding complexity for UHD (Class A1, A2) and HD (Class B) clips under RA and AI for QP {22, 27, 32, 37}.  Table 6 show the BDRate results for UHD (Class A1, A2) and HD (Class B) clips under RA and AI for QP {27, 32, 37, 42}. The decoding time ranges from 102% to 104% compared to VTM Anchor. The encoding time is 105% for AI and 110% for RA. The encoding time increase is mainly due the calculation of pixel variance for every intra slice. The decoding complexity is mainly due to FwdLUT and InvLUT construction and LUT mapping.

Table 5. BDRate under VTM (QP: 22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.50% | 0.56% | 0.43% | 108% | 106% |
| Class A2 | -1.37% | 3.14% | 3.43% | 105% | 104% |
| Class B | -0.76% | 0.83% | 0.04% | 103% | 103% |
| Overall | -1.13% | 1.38% | 1.07% | 105% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.58% | 2.49% | 2.49% | 111% | 105% |
| Class A2 | -1.28% | 2.47% | 2.57% | 110% | 102% |
| Class B | -1.09% | 0.89% | -0.27% | 109% | 103% |
| Overall | -1.28% | 1.76% | 1.26% | 110% | 103% |

Table 6. BDRate under VTM (QP: 27, 32, 37, 42)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.83% | -1.39% | -1.43% | 107% | 105% |
| Class A2 | -1.19% | 3.10% | 3.23% | 105% | 104% |
| Class B | -1.08% | 0.22% | -0.84% | 103% | 103% |
| Overall | -1.31% | 0.57% | 0.11% | 105% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.23% | 1.43% | 1.69% | 110% | 103% |
| Class A2 | -1.11% | 2.19% | 2.27% | 110% | 102% |
| Class B | -1.34% | -0.39% | -1.64% | 109% | 102% |
| Overall | -1.25% | 0.81% | 0.33% | 110% | 102% |

# Conclusion

In this contribution, the in-loop reshaping shows potential coding performance for SDR video coding. We propose to continue investigate the coding benefit using in-loop reshaping.

# Reference

- E. Francois, D. Rusanovskyy, P. Yin, “Description of Core Experiment 12 (CE12): mapping for HDR content”, JVET-J0032, San Diego, USA, April, 2018.

- X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. Ye, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen, “Description of SDR, HDR, and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, San Diego, USA, April, 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: HDR In-loop Reshaping”, JVET-K0308, Ljubljana, SI,  July , 2018.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A : Syntax on slice reshaping model

| slice_reshaping_model() { |  |
| --- | --- |
| reshape_model_profile_type |  |
| reshape_model_scale_idx |  |
| reshape_model_min_bin_idx |  |
| reshape_model_delta_max_bin_idx |  |
| reshape_model_num_cw_minus1 |  |
| for ( i = 0; i < reshape_model_num_cw_minus1 + 1; i++) { |  |
| reshape_model_delta_abs_CW [ i ] |  |
| if ( reshape_model_delta_abs_CW > 0 ) |  |
| reshape_model_delta_sign_CW [ i ] |  |
| } |  |
| for ( i = reshape_model_min_bin_idx; i <= reshape_model_max_bin_idx; i++ ) { |  |
| reshape_model_bin_profile_delta [ i ] |  |
| } |  |

reshape_model_profile_type specifies the profile type to be used in the reshaper construction process.

reshape_model_scale_idx specifies the index value of a scale factor (denoted as ScaleFactor) to be used in the reshaper construction process.

reshape_model_min_bin_idx specifies the minimum bin index to be used in the reshaper construction process. The value of reshape_model_min_bin_idx shall be in the range of 0 to 31, inclusive.

reshape_model_delta_max_bin_idx  is set equal to the maximum allowed bin index (31) minus the maximum bin index to be used in the reshaper construction process.

reshape_model_num_cw_minus1 plus 1 specifies the number of codewords to be signalled.

reshape_model_delta_abs_CW [ i ] specifies the i-th absolute delta codeword value.

reshape_model_delta_sign_CW [ i ] specifies the sign for the i-th delta codeword.

reshape_model_delta_CW[i] = (1-2* reshape_model_delta_sign_CW [i]) * reshape_model_delta_abs_CW [ i ];

reshape_model_CW [ i ] = 32 + reshape_model_delta_CW [ i ].

reshape_model_bin_profile_delta [ i ]  specifies the delta value to be used to adjust the profile of the i-th bin in the reshaper construction process. The value of reshape_model_bin_profile_delta [ i ] shall be in the range of 0 to 1 when reshape_model_num_cw_minus1 is equal to 0. The value of reshape_model_bin_profile_delta [ i ] shall be in the range of 0 to 2 when reshape_model_num_cw_minus1 is equal to 1.

CW=32 when reshape_model_bin_profile_delta [ i ]  is set equal to 0,  CW= reshape_model_CW [ 0 ] when  reshape_model_bin_profile_delta [ i ]  is set equal to 1; CW= reshape_model_CW [ 1 ] when reshape_model_bin_profile_delta [ i ]  is set equal to 2.