| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0315-v1 |
| --- | --- |

| Title: | CE2: Deblocking Improvements for Large CUs (Test 2.1.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu Kiran Misra Andrew Segall Phil Cowan | Tel: Email: | +1-360-834-8054 zhuw@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com cowanp@sharplabs.com |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The contribution proposes a deblocking process designed to reduce the blocking artifacts resulting from the use of large transforms and large block sizes. Compared to the deblocking approach in the VVC draft 1[1], the process incorporates stronger filters for both luma and chroma. Additionally, the process includes a control process that considers the block sizes on both sides of the boundary being deblocked.  In the proposal, the proposed stronger filters are used for luma samples that correspond to larger block sizes, while the existing VVC deblocking filters are used for luma samples corresponding to smaller block sizes.  For chroma, the stronger filter is applied when chroma samples on either sides of the deblocking boundary belong to a large block. It is asserted that the proposed deblocking approach improves subjective quality at low bit-rates when compared with the deblocking process existing in the VTM software.

Introduction

The use of large transforms and block sizes results in significant blocking artifacts, especially in low bitrate scenarios. Figure 1 below shows the blocking artifacts that result in POC 28 for the FoodMarket sequence (used in CTC[2]) when coded with QP 42 and using BMS with random access VTM configuration[3].

Figure 1: Blocking artifacts at QP 42 resulting from use of large transforms in VVC

In studying this problem, we have observed that to overcome this issue, it is beneficial to use a stronger filter with more support than what is available in the VTM.

Proposed Scheme

Several modifications are proposed in this contribution. To facilitate this explanation, we use the following nomenclatures:  First, samples on each side of a block boundary (perpendicular to the boundary edge) are represented as:

… p8 p7 p6 p5 p4 p3 p2 p1 p0 | q0 q1 q2 q3 q4 q5 q6 q7 q8 …

where | represents the block boundary edge.

Thus, in Figure 2a, samples py,x and qy,x correspond to line R[x] when deblocking the vertical edge, while for Figure 2b, samples py,x and qy,x correspond to line R[y] when deblocking the horizontal edge.

| (a) | (b) |
| --- | --- |

Figure 2: Deblocking boundary for (a) vertical edge (b) horizontal edge

Stronger deblocking filter for luma

For luma deblocking, we use the stronger filter proposed in JVET-J0028[4] when a sample belongs to a large block.  The filter is listed below, and a sample belonging to a large block is defined as when the width >= 32 for a vertical edge, and when height>=32 for a horizontal edge.

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (7*q7+2*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (6*q7+q6+2*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (5*q7+q6+q5+2*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (4*q7+q6+q5+q4+2*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (3*q7+q6+q5+q4+q3+2*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (2*q7+q6+q5+q4+q3+q2+2*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

As can be seen, the number of deblocked samples is extended to seven above.  For the samples at small block sides, the strong filter in VVC is used for deblocking, which is given as:.

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

q0′ = ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3

q1′ = ( p0 + q0 + q1 + q2 + 2 )  >>  2

q2′= ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3

Figure 3 shows an illustration where the samples at side P are located in a large block and the samples at side Q are located in a small block. As described, the number of deblocked samples would then be three for side Q and seven for side P.  To be clear, the filter in [4] would then be used for deblocking the samples at the side P and the strong filter in the VTM would be used for deblocking the samples at the side Q.

Figure 3: An illustration of the proposed deblocking scheme

Deblocking control for luma

The deblocking decision process is described in this sub-section.  To begin, we define the following gradient calculation functions:

xCalDQp(R[x]): 		abs(p2 −2*p1+p0)

xCalDQq(R[x]): 		abs(q2 −2*q1+q0)

xCalDQpLargeBlock(R[x]): 	Max(Max(Max(Max(Max(abs(p2−2*p1+p0), 
					abs(p3−2*p2+p1)),
					abs(p4−2*p3+p2)), 
					abs(p5−2*p4+p3)),
					abs(p6 −2*p5+p4)), 
					abs(p7 −2*p6+p5))

xCalDQqLargeBlock(R[x]): 	Max(Max(Max(Max(Max(abs(q2−2*q1+q0), 
					abs(q3−2*q2+q1)),
					abs(q4−2*q3+q2)),
					abs(q5−2*q4+q3)),
					abs(q6 −2*q5+q4)),
					abs(q7 −2*q6+q5))

Wider-stronger luma filter is filters are used only if all of the Condition1, Condition2 and Condition 3 are TRUE.

The condition 1 is the “large block condition” in Fig.4. This condition detects whether the samples at P-side and Q-side belong to large blocks, which are represented by the variable bSidePisLargeBlk and bSideQisLargeBlk respectively. The bSidePisLargeBlk and bSideQisLargeBlk are defined as follows.

bSidePisLargeBlk = ((edge type is vertical and p0 belongs to CU with width >= 32) | | (edge type is horizontal and p0 belongs to CU with height >= 32))? TRUE: FALSE

bSideQisLargeBlk = ((edge type is vertical and q0 belongs to CU with width >= 32) | | (edge type is horizontal and q0 belongs to CU with height >= 32))? TRUE: FALSE

Based on bSidePisLargeBlk and bSideQisLargeBlk, the condition 1 is defined as follows.

Condition1 = (bSidePisLargeBlk || bSidePisLargeBlk) ? TRUE: FALSE

Next, if Condition 1 is true, the condition 2(The “large block gradient condition” in Fig.4) will be further checked. First, the following variables are derived:

d0P = bSidePisLargeBlk? XCalDQpLargeBlock(R[0]) : XCalDQp (R[0])

d1P = bSidePisLargeBlk? XCalDQpLargeBlock(R[3]) : XCalDQp (R[3])

d0Q = bSideQisLargeBlk? XCalDQqLargeBlock(R[0]) : XCalDQq (R[0])

d1Q = bSideQLargeBlk? XCalDQqLargeBlock(R[3]) : XCalDQq (R[3])

d0L = d0P + d0Q

d3L = d1P + d1Q

dL = d0L + d3L

Then the condition 2 is defined as follows.

Condition2 = (dL < β) ? TRUE: FALSE

Finally, if both the Condition 1 and Condition 2 are valid, the proposed deblocking method will check the condition 3(The Large Block Strong Filter Condition in Fig.4), which is defined as follows.

Condition3 =

( xUseStrongFilteringLargeBlock(R[0], d0L, bSidePisLargeBlk, bSideQisLargeBlk) &&

xUseStrongFilteringLargeBlock(R[3], d3L, bSidePisLargeBlk, bSideQisLargeBlk)) ? TRUE:FALSE;

In the condition 3, xUseStrongFilteringLargeBlock (R[x], d, bSidePisLargeBlk, bSideQisLargeBlk) computes a Boolean variable, as follows:

(bSidePisLargeBlk ? max(abs(p7- p4), max(abs(p4- p0), abs(p7- p0)) : abs(p4 - p0))  +

(bSideQisLargeBlk ? max(abs(q7- q4), max(abs(q4- q0), abs(q7- q0)) : abs(p4 - p0)) < (β>> 3))

&& (d < (β >> 2))

&& (abs(q0- p0) < ((tC * 5 + 1) >> 1)))? TRUE:FALSE

Strong deblocking filter for chroma

The following strong deblocking filter for chroma is defined as:

p0′= (p2 +2*p1+2*p0+2*q0+q1 +4) >> 3

p1′= (p2+p1+ p0+q0+2) >> 2

p2′= (p4+2*p3+3*p2+ p1+ p0+4) >> 3

q0′= (q2 +2*q1+2*q0+2*p0+p1 +4) >> 3

q1′= (q2+q1+ q0+p0+2) >> 2

q2′= (q4+2*q3+3*q2+ q1+ q0+4) >> 3

Deblocking control for chroma

The chroma strong filters are used on both sides of the block boundary.  Specifically, the strong filter is used when any of the following conditions is true:

- The edge type is vertical an p0 belongs to CU with width >= 32

- The edge type is horizontal and p0 belongs to CU with height >= 32

- The edge type is vertical and q0 belongs to CU with width >= 32

- The edge type is horizontal and q0 belongs to CU with height >= 32

The above test is the Large Block Condition test for chroma.

Flowchart of luma and chroma deblocking control process

A flowchart for the luma and chroma deblocking control process is shown below:

Luma deblocking control:

Figure 4: Flowchart of luma deblocking control

Where, text in red corresponds to the proposed modifications, while the text in black corresponds to existing VVC deblocking. Note, the large block condition here is same as with large block condition test for chroma (section 2.4).

Chroma deblocking control:

Figure 5: Flowchart of luma deblocking control

Deblocking along implicit TU boundaries

TU boundaries will not align with CU boundaries when the CU size is larger than TU size. On this case, the proposed deblocking method will be performed along both CU and TU boundaries.

Experimental Results

Results using the proposed method were generated and viewed.  A still image from the result is shown in Figure 6, which demonstrates the reduction in blocking artifacts as a result of the proposed deblocking modification.

- (b)

Figure 6: Comparison of (a) VTM versus (b) VTM+proposed deblocking modification when QP=42

The objective performance is also reported by following the test condition in [2] on top of BMS1.0[3]. Simulation results with VTM and BMS configurations are given as follows.

Table 1 Coding Performance over BMS1.0 with VTM configuration

Table 2 Coding Performance over BMS1.0 with BMS configuration

Conclusion

In this proposal, a new adaptive long-tap deblocking filtering method is proposed. It is asserted that the proposed method achieves significant subjective quality improvements without any objective quality loss. We propose adopting the method into the versatile video coding working draft.

Reference

- B. Bross “Versatile Video Coding (Draft 1)”, JVET-J1001, April 2018, San Diego, United States.

- J. Boyce, K. Suehring, X. Li, V. Seregin “JVET common test conditions and software reference configurations”, JVET-J1010, April 2018, San Diego, United States.

- BMS 1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/

- T. Suzuki, M. Ikeda, K. Sharman, “Description of SDR and HDR video coding technology proposal by Sony”, doc. JVET-J0028, April 2018.

Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).