| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SLOVENIA, 10–20 Jul. 2018 | Document: JVET-K0316 |
| --- | --- |

| Title: | CE1: Implicit QT, BT and MTT Partitions on Picture Boundary (Test 2.0.15) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu Kiran Misra Andrew Segall | Tel: Email: | +1-360-834-8054 zhuw@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The contribution proposes a method for implicitly partitioning CTUs or CUs spanning across a picture boundary. Specifically, for picture dimensions that are not an integer multiple of the CTU size, CTUs at the right and/or bottom of the picture will contain pixel regions that do not correspond to the intended, reconstructed image. In the proposed method, these CTUs undergo implicit partitioning that makes use of the available square and rectangular partitioning types until all the samples at the leaf of the partitioning tree created using implicit splits do not span across the picture boundary. The implicit splits can include both quad tree and binary tree splits. Moreover, the quad-tree splitting can be further controlled by receiving a minimum number of required quad-tree splits of the CTU within a parameter set. Experimental results show that the proposed method achieves a coding efficiency improvement of 0.20% and 0.25% relative to the VTM and BMS, respectively, using CTC random access test conditions. This includes a gain of 0.42% and 0.54%, respectively, for the Class B test sequences.  In all cases, the impact on encoder run-time is measured to be in the range of 102%-104%.

Introduction

In the current VVC draft[1], a QT partition is always selected for a CTU or CU spanning across the picture boundary. Binary tree and triple tree only can be employed on CUs where all pixels in the CU are inside the bottom or/and right picture boundaries. Here, we assert that the flexibility of picture partitioning can be further enhanced by enabling binary tree and triple tree on picture boundaries partition.

Proposed Scheme

Picture boundary partition

In this proposal, a series of implicit quad splits, binary splits and triple tree splits are performed recursively for CUs spanning across the picture boundary according to the following process: First, the quad-tree splitting are performed recursively until the QT depth is larger than or equal to a threshold which is received from a parameter set or, alternatively, until the current region is inside the picture boundary. Second, the binary tree and triple tree will be further employed and the partition mode that results in the split edge being closest to the picture boundary is selected. Moreover, between the two split direction candidates, the direction that does not partition the block of samples inside the picture boundary is chosen – or if both directions partition the block of samples inside the picture boundaries, the horizontal partition is simply chosen. In addition, the maximal BT depth of the whole CTU will be increased by 1 if one implicit BT partition is selected for a boundary CU. This process is performed recursively. Once complete, explicit signaling in the bitstream then determines if the resulting partitions will undergo further splitting after the implicit partitioning has been carried out. Some examples of the proposed method are shown in Fig.1 and Fig.2. In Fig.1 and Fig.2, the black dash line denotes the picture boundary, the two red lines are the partition boundaries of a triple tree and the yellow line is a binary tree partition boundary. The proposed method first measures the distances between the split boundary and the picture boundary for different partition modes and the partition mode with minimal distance will then be selected to partition the current block. In Fig.1, the triple tree will be selected to further partition the current block since its distance to the bottom picture boundary is shorter than binary tree split boundaries. Similarly, the binary tree will be selected to further partition the current block for the case in Fig.2.

Fig.1 An illustration of the proposed picture boundary partition on bottom boundaries

Fig.2 An illustration of the proposed picture boundary partition on right boundaries

Signaling of QT depth threshold

In the proposed picture boundary partitioning approach, a QT depth threshold is employed to indicate how many implicit QT splits to perform for partitions that span the picture boundary. This depth information is proposed to be signaled using the syntax below:

|  | Descriptor |
| --- | --- |
| Sequence_parameter_set{ |  |
| ……. |  |
| infer_min_QT_for_boundary_enable_flag | u(1) |
| If (infer_min_QT_for_boundary_enable_flag){ |  |
| same_min_QT_split_value_for_all_boundary_CTUs_flag | u(1) |
| if(same_min_QT_split_value_for_all_boundary_CTUs_flag) |  |
| min_QT_split_for_CTU_at_boundary | ue(v) |
| else { |  |
| min_QT_split_for_CTU_at_right_picture_edge | ue(v) |
| min_QT_split_for_CTU_at_bottom_picture_edge | ue(v) |
| min_QT_split_for_CTU_at_rightbottom_picture_edge | ue(v) |
| } |  |
| sps_slice_level_override_present | u(1) |
| ….. |  |
| } |  |

| Slice_header{ |  |
| --- | --- |
| ……. |  |
| If (sps_slice_level_override_present){ |  |
| slice_level_override_present | u(1) |
| If(slice_level_override_present){ |  |
| slice_same_min_QT_split_value_for_all_boundary_CTUs_flag | u(1) |
| if(slice_same_min_QT_split_value_for_all_boundary_CTUs_flag) |  |
| slice_min_QT_split_for_CTU_at_boundary | ue(v) |
| } |  |
| else { |  |
| slice_min_QT_split_for_CTU_at_right_picture_edge | ue(v) |
| slice_min_QT_split_for_CTU_at_bottom_picture_edge | ue(v) |
| slice_min_QT_split_for_CTU_at_rightbottom_picture_edge | ue(v) |
| } |  |
| ….. |  |
| } |  |

infer_min_QT_for_boundary_enable_flag a flag to enable the signaling of QT depth thresholds.

same_min_QT_split_value_for_all_boundary_CTUs_flag a flag to indicate whether the QT depth thresholds at different location(bottom and/or right) have different values.

min_QT_split_for_CTU_at_boundary the value of the QT threshold used on all boundary CTUs.

min_QT_split_for_CTU_at_right_picture_edge the value of the QT threshold used on the right boundary CTUs.

min_QT_split_for_CTU_at_bottom_picture_edge the value of the QT threshold used on the bottom boundary CTUs.

min_QT_split_for_CTU_at_rightbottom_picture_edge the value of the QT threshold used on the right-bottom boundary CTUs.

sps_slice_level_override_present a flag to indicate whether to update QT threshold in each slice header.

slice_level_override_present a flag whether to update QT threshold in the current slice header

slice_same_min_QT_split_value_for_all_boundary_CTUs_flag a flag to indicate whether the QT depth thresholds at different location(bottom and/or right) have different values.

slice_min_QT_split_for_CTU_at_boundary the slice-updated value of the QT threshold used on all boundary CTUs.

slice_min_QT_split_for_CTU_at_right_picture_edge the slice-updated value of the QT threshold used on the right boundary CTUs.

slice_min_QT_split_for_CTU_at_bottom_picture_edge the slice-updated value of the QT threshold used on the bottom boundary CTUs.

slice_min_QT_split_for_CTU_at_rightbottom_picture_edge the slice-updated value of the QT threshold used on the right-bottom boundary CTUs.

Experimental Results

Results using the proposed method are reported by following the test condition in [2] on top of BMS1.0[3]. Simulation results with VTM and BMS configurations are given as follows.

Table 1 Coding Performance over BMS1.0 with VTM configuration

Table 2 Coding Performance over BMS1.0 with BMS configuration

Conclusion

In this proposal, a picture boundary partitioning method is proposed where quad-tree, binary partitions and triple tree partition can be adaptively used on the boundary partitioning. The performance achieves 0.20% BD- bitrate saving and 0.25% BD- bitrate saving under the VTM and BMS random-access configurations, respectively. This includes a gain of 0.42% and 0.54% for the Class B test sequences.  In all cases, the impact on encoder run-time is measured to be in the range of 102%-104%. We propose adopting the method into the versatile video coding working draft.

Reference

- B. Bross “Versatile Video Coding (Draft 1)”, JVET-J1001, April 2018, San Diego, United States.

- J. Boyce, K. Suehring, X. Li, V. Seregin “JVET common test conditions and software reference configurations”, JVET-J1010, April 2018, San Diego, United States.

- BMS 1.0, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/

Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)