| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0320-v3 |
| --- | --- |

| Title: | CE1-related: Zero-Unit for Picture Boundary Handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane, San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, Zero-Unit (ZU) is proposed to handle the coding structure at picture boundaries. A ZU is defined as a special Coding Unit (CU) with width or height not in a form of 2N. A ZU is coded as a normal CU, except that it has no residues. cbf_flag is not signaled and inferred to be zero in a ZU. A ZU can also be binary-split or quarter-split into sub-CUs or sub-ZUs as a normal CU. Simulation results reportedly show 0.33% BD-rate saving on the Y component for Random Access (RA) configuration in average compared with VTM-1.1 with almost the same encoding and decoding time.

# Introduction

In VTM, a quadtree (QT)/binary tree (BT)/ternary tree (TT) coding structure is applied. The width or height of a Coding Unit (CU) is always in a form of 2N. After a quarter-, ternary-, or binary- split, a CU can be partitioned into four, three, or two sub-CUs, the widths or heights of which are still in a form of 2N. For Coding Tree Units (CTUs) at picture boundaries, the width or height of the CTU inside the picture may not be in the form of 2N. To handle this case, VTM applies a forced QT strategy. The CTU is forced to recursively take quarter-split, until all the widths and heights of sub-CUs inside the picture are in the form of 2N.

The forced QT strategy sacrifices the partitioning flexibility of regions close to picture boundaries. A region close to the picture boundary may be partitioned into unnecessary small blocks with a lot of overhead bits for signaling modes and other coded information.

# Zero-Unit

To address the problem of picture boundary handling in VTM, Zero-Unit is proposed in this contribution.

A ZU is defined as a special CU with width or height not in a form of 2N. A ZU is coded as a normal CU, except that it has no residues. Therefore, cbf_flag is not signaled and inferred to be zero in a ZU. A ZU can also be binary-split or quarter-split into sub-blocks, which can be a ZU or a normal CU.

Suppose the size of a ZU is W×H, then {w0, w1, h0, h1} are calculated as:

w0=2N, w1=W- w0; where 2N <W<=2N+1;

h0=2M, h1=H- h0; where 2M <H<=2M+1.

If a ZU is split by QT, the four sub-blocks are with sizes w0× h0, w1× h0, w0× h1 and w1× h1, respectively. If a ZU is split by vertical BT, the two sub-blocks are with sizes w0× H and w1×H, respectively. In addition, a ZU can be split by horizontal BT, and the two sub-blocks are with sizes W× h0 and W× h1, respectively. It should be noted that a sub-block may be a ZU or normal CU depending on whether both its width and height are in the form of 2N.

Fig. 1 shows some examples of how a ZU with size 64×48 can be split. By QT, it is split into two normal CUs with the size 32×32 and two normal CUs with the size 32×16, as shown in Fig. 1 (a). By horizontal BT, it is split into one normal CU with the size 64×32 and the other with the size 64×16, as shown in Fig. 1 (b). By vertical BT, it is split into two ZUs with the size 32×48 as shown in Fig. 1 (c).

(a)

(b)

(c)

Fig. 1 Examples of ZU split.

A ZU can also be split by TT. {w0, w1, w2, h0, h1, h2} are calculated as:

w0=2N, w1=2N+1, w2=W- w0- w1; where 3*2N <W<=3*2N+1;

h0=2N, h1=2N+1, h2=H- h0- h1; where 3*2N <H<=3*2N+1;

If a ZU is split by vertical TT, the three sub-blocks are with sizes w0× H, w1× H and w1×H, respectively. If a ZU is split by horizontal TT, and the three sub-blocks are with sizes W× h0, W× h1 and W× h2, respectively.

Fig. 2(a) shows an example of how a ZU with size 64×48 is split by horizontal TT. It is split into two normal CUs with the sizes 64×8 and 64×16, and one ZU with the size 64×24. By vertical TT, it is split into three ZUs with the sizes 16×48, 32×48 and 16×48, as shown in Fig. 2 (b).

(a)

(b)

Fig. 2 Examples of TT split.

# Experimental Results

The proposed method are implemented on VTM-1.1 and BMS-1.1. The simulations were performed following the JVET CTC [1]. Results of Random Access (RA) configuration based on VTM-1.1 and BMS-1.1 if TT is not allowed for ZU are summarized in Table 1 and Table 2, respectively. Results of RA configuration based on VTM-1.1 and BMS-1.1 if TT is allowed for ZU are summarized in Table 3 and Table 4, respectively. More detail results can be found in the attached spreadsheets.

Table 1: ZU w/o TT compared with VTM-1.1 (RA)

|  | Over VTM-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.50% | -0.46% | 100% | 101% |
| Class A2 | -0.13% | -0.19% | -0.12% | 101% | 102% |
| Class B | -0.65% | -0.78% | -0.78% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.02% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.40% | -0.37% | 101% | 102% |

Table 2: ZU w/o TT compared with BMS-1.1 (RA) (Running time is not reliable)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.67% | -0.65% | 110% | 98% |
| Class A2 | -0.19% | -0.15% | -0.15% | 114% | 106% |
| Class B | -0.78% | -1.08% | -0.96% | 109% | 115% |
| Class C | 0.08% | 0.04% | -0.16% | 97% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.51% | -0.52% | 107% | 105% |

Table 3: ZU with TT compared with VTM-1.1 (RA) (Running time is not reliable)

|  | Over VTM-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.50% | -0.49% | 87% | 89% |
| Class A2 | -0.14% | -0.22% | -0.14% | 87% | 85% |
| Class B | -0.67% | -0.81% | -0.82% | 97% | 94% |
| Class C | 0.00% | -0.02% | 0.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.42% | -0.38% | 94% | 93% |

Table 4: ZU with TT compared with BMS-1.1 (RA) (Running time is not reliable)

|  | Over BMS-1.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | -0.70% | -0.74% | 75% | 96% |
| Class A2 | -0.20% | -0.19% | -0.20% | 72% | 97% |
| Class B | -0.78% | -1.07% | -0.96% | 73% | 74% |
| Class C | 0.06% | -0.04% | -0.17% | 96% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | -0.55% | -0.55% | 79% | 88% |

# References

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).