| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0321-v2 |
| --- | --- |

| Title: | CE 7.1.4: JEM 7.0 coefficient coding with complexity reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Cheung Auyeung Jianle Chen | Tel: Email: | 1-408-330-4690 cheung.auyeung@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents the CE 7.1.4 results of a method to reduce the computation complexity of JEM-7.0 coefficient coding by reducing the number of decisions to select the context index group for the coding of significant_coeff_flag, coeff_abs_greater1_flag, and coeff_abs_greaer2_flag. In comparing BMS-1.0 with and without the complexity reduction modification, the BD-Rate of the test configuration without SDH changed by 0.05/0.05/-0.04% in AI/RA/LB BMS. For luminance significant_coeff_flag, coeff_abs_greater1_flag, and coeff_abs_greaer2_flag, the number of decisions for context group selection in the implementation of BMS-1.0 is 2 per flag, independent on the coefficient group (CG). In comparison, the number of decisions in the implementation of CE 7.1.4 depends on the CG. For the first CG, it has 2 per significant_coeff_flag, 1 per coeff_abs_greater1_flag, and 1 per coeff_abs_greaer2_flag. For the other CG, it is 0 for significant_coeff_flag, coeff_abs_greater1_flag, and coeff_abs_greaer2_flag.  In addition, the CE 7.1.4 implementation has an overhead of 1 decision per CG to decide if the CG is the first CG. For chrominance, there are no algorithmic difference between BMS-1.0 and CE 7.1.4.

# Introduction

This report presents the CE 7.1.4 results of a method to reduce the computation complexity of JEM-7.0 coefficient coding by reducing the number of decisions. In JEM-7.0, the selection of context model of a transform coefficient is based on the location of the coefficient within a transform unit. As shown in Figure 1, for an 8x8 TU, the decision lines for selecting context index group for significant_coeff_flag, coeff_abs_greater1_flag and coeff_abs_greaer2_flag cross multiple coefficient groups. Consequently the implementation of BMS1 has two decisions per coded flags for luminance to determine the context index group.

In JVET-J10027 [1], CE 7.1.4 is intended to study the effect of reducing the number of decision needed to determine the context index group. As shown in  Figure 2, the proposed context model selection method is the same as in the Figure 1, except that the transform coefficient positional based context switching is constrained to the first 4x4 coefficient group to reduce the number of decisions.

In this report, the number of decisions are further reduced by using the context group assignments as shown in Figure 3 for 8x8 blocks.

Figure 1: JEM-7.0 context index assignments.

Figure 2: Context index assignments for CE 7.1.4 as documented in JVET-J10027.

Figure 3: Context index assignments for this report.

# Simulation results

The following BD-rate results are obtained according to the common test conditions defined in JCTVC-J1010 [2]. The execution times are measured in a heterogeneous computing environment and are approximate.

Four sets of BD-rate results are presented in Table 1 to Table 4:

- Table 1 reports the results of VTM test configuration without sign data hiding (SDH). It was generated with VTM configuration and options --AltResiComp=1 -SBH 0.

- Table 2 reports the results of BMS test configuration without SDH. It was generated with BMS configuration and options --AltResiComp=1 -SBH 0.

- Table 3 reports the results of VTM test configuration with sign SDH. It was generated with VTM configuration and options --AltResiComp=1 -SBH 1.

- Table 4 reports the results of BMS test configuration with SDH. It was generated with BMS configuration and options --AltResiComp=1 -SBH 1.

In particular, Table 2 compares BMS-1.0 with and without the complexity reduction modification. It showed that the BD-Rate changed by 0.05/0.05/-0.04% in AI/RA/LB BMS configuration without SDH.

Table 1: Results of VTM test configuration without SDH.

Table 2: Results of BMS test configuration without SDH.

Table 3: Results of VTM test configuration with SDH.

Table 4: Results of BMS configuration with SDH

# Complexity Comparison

Table 5 and Table 6 compare the number of decisions for context index group selection in the implementation of BMS-1.0 and CE 7.1.4 in the JVET SVN server for luminance TU and chrominance TU respectively. This is an implementation specific comparison. Other implementation is likely to result in different number of decisions.

Let L00, L01, L02, L10, L11, L12 be the number of luminance coded flags for significant_coeff_flag, coef_abs_greater1_flag and coef_abs_greater2_flags as define in Table 5. And let M be the number of coded CG. Then the number of positional decisions for luminance TU in BMS-1.0 is (L00+L01+L02+L10+L11+L12) and for CE7-1.4.0 is 2L00+L01+L02+M.

Let K0, K1 be the number of chrominance coded flags for significant_coeff_flag for first CG and the other CG as define in Table 6. And let N be the number of coded CG. Then the number of positional decisions for chrominance TU in BMS-1.0 is K0+K1 and for CE7-1.4 is K0+N. This difference is a result of implementation and not a result of algorithm, since the context index group assignments for chrominance are the same for BSM-1.0 in Figure 1 and CE 7.1.4 in Figure 3.

Table 5: Number of positional decisions for luminance TU.

|  | Flags in first CG | Flags in (M-1) other CG | BMS1 | CE7-1.4 |
| --- | --- | --- | --- | --- |
| significant_coeff_flag | L00 | L10 | 2(L00+ L10) | 2L00 |
| coef_abs_greater1_flag | L01 | L11 | 2(L01+ L11) | L01 |
| coef_abs_greater2_flag | L02 | L12 | 2(L02+ L12) | L02 |
| First/not first CG decison |  |  | 0 | M |

Table 6: Number of postitional decisions for chormiance TU.

|  | Flags in first CG | Flags in (N-1) other CG | BMS1 | CE7-1.4 |
| --- | --- | --- | --- | --- |
| significant_coeff_flag | K0 | K1 | K0+K1 | K0 |
| coef_abs_greater1_flag | 0 | 0 | 0 | 0 |
| coef_abs_greater2_flag | 0 | 0 | 0 | 0 |
| First/not first CG decision |  |  | 0 | N |

# References

| [1] | J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditins and software reference configurations," JVET-J1010, 10th Meeting: San Diego, CA, USA, 10–20 Apr. 2018. |
| --- | --- |
| [2] | H. Schwarz and M. Coban, "Description of Core Experiment 7 (CE7): Quantization and coefficient coding," JVET-J1027, 10th Meeting: San Diego, CA, USA, 10–20 Apr. 2018. |

# Patent rights declaration(s)

Huawei Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).